Flight ID: 070914I

Sensor or system						Number or Name

INE								ine1

Accelerometer							acci1

Temperature Probe						tt1 (Rosemount)

Dew Point Probe						tdm2x (Edgetech Vigilant)

Altitude (for vertical wind)					ra159 (GPS aided inertial alt)

Static Pressure							psf (Rosemount Fuselage)

Dynamic Pressure						pqf1 (Rosemount Fuselage)

Time Source							Micro 99

Dropsondes Deployed						Good: 13  Bad: 0

Local Met Data:		Takeoff (1950Z)			Landing (0401Z)

Aircraft Static Pressure		1005.2 	 mb			1006.8	 mb

Tower Pressure			1005.0	 mb			1007.0	 mb

Top of Form 1

Notes

No data gaps

Ashtech data spikes at: 002626Z-002644Z, 014722Z-014731Z, 022008Z-022826Z, 035956Z-040347Z

hdgi1 had a data spike at 224806Z-224809Z. Substituted data from hdgi2 and saved as hdgi1X

tt3 sensor not installed

tdm2 sensor balancing at 022210Z-030227Z. Substituted tdm1 data with 0 offset during this time and saved as tdm2X

The HG9550 radar altimeter data has a consistent 1% positive bias above 1,500 feet that was discovered after flight. The correction formula is:

(((reported altitude -  1,500) * 0.988) +1,500) = corrected altitude

This formula should only be needed for this hurricane season’s flight data as the problem should be corrected automatically during next season’s flights.

There were periods where the dewpoint temperature exceeded the ambient temperature resulting in RH values > 100%. This was likely due to heavy rain, a wet-bulb effect on the total temperature sensor, and/or an artificial warming of the dewpoint sensor as it tried to burn off excess moisture. Corrections were not made to the data.