1

N42RF ERROR SUMMARY

GENESIS OF TS DOLLY

Flight ID: 080720H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           RA159X

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There were no data gaps.

There were no liquid water sensors available for the flight.

The APN-159 (RA159X) radar altimeter output was replaced with RINU-1550 blended GPS altitude data during the low level (1500 feet) portion of the flight from 115900Z – 150216Z. The replacement values were used to extrapolate sea level pressure from altitude for the 1353Z VORTEX message that was transmitted to NHC.

During a descent the measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited erroneous data from 120219Z - 120450Z. The erroneous data was removed by substituting TDM3(Maycomm/TDL) output using the following equation,

TDM2 = TDM3 – 1.50

All other instruments worked optimally during the flight.

Seven (7) GPS dropsondes were deployed all being good. Their corresponding tempdrop messages were successfully sent to NHC.

Takeoff		Landing

0819Z		1723Z

Aircraft Static Pressure                   1016.3mb		1018.3mb

Corrected Tower Pressure               1016.3mb	           1018.6mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073