1

N42RF ERROR SUMMARY

TS DOLLY

Flight ID: 080721H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           RA159X(GPS)

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

Beginning at 172310Z....approaching MacDill AFB for landing.....and ending at block in, there were a number of sporadic one (1) second data gaps attributed to the aircraft data system.

There were no liquid water sensors available for the flight.

The APN-159 (RA159X) radar altimeter output became erroneous at 101041Z and eventually failed completely at approximately 1500Z. Its data was replaced with RINU-1550 blended GPS altitude data beginning at 101041Z and continued until the end of the mission. This data was used to extrapolate sea level pressure from altitude for the 1310Z VORTEX message that was transmitted to NHC.

Measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited erroneous data from 101921Z - 102240Z. The erroneous data was removed by substituting TDM3(Maycomm/TDL) output using the following equation,

TDM2 = TDM3 – 1.53

There were two other occurrences of erroneous output from TDM2. From 120215Z – 120500Z TDM2's erroneous data was replaced with data from TDM3 using statistical techniques.

From 172416Z – 172700Z TDM2's erroneous data was removed by substituting TDM3 values with the equation,

TDM2 = TDM3 – 3.1

All other instruments worked optimally during the flight.

Seven (7) GPS dropsondes were deployed with 6 being good. Their corresponding tempdrop messages were successfully sent to NHC. Twelve (12) AXBTs were deployed internally from the aircraft with 8 good and 4 failures.

Takeoff		Landing

0857Z		1729Z

Aircraft Static Pressure                   1015.7mb	1016.6mb

Corrected Tower Pressure               1015.9mb	           1016.9mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073