1

N42RF P3 ERROR SUMMARY

FERRY TO BARBADOS

Flight ID: 080812H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM1X (Buck)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           RA159X

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There were no data gaps. However a lag (~ 3-4 seconds) was observed between the two inertial outputs with INE2 lagging INE1. INE1 was chosen since it best corresponded with the analog output.

There were no liquid water sensors available for the flight.

The APN-159 (RA159X) radar altimeter output displayed erroneous data during the following time frames: 141500Z – 141700Z and 141800Z – 142000Z. The erroneous data was removed using statistical techniques.

Measured dewpoint temperature #1 (EdgeTech)...TDM1...exhibited erroneous data during the following time frames: 132856Z – 133233Z, 133856Z – 134251Z, 134822Z – 135417Z, 143320Z – 143429Z, 145420Z – 145747Z, 152130Z – 152158Z and

183658Z - 185143Z. The erroneous data was removed by substituting TDM3(Maycomm/TDL) output,

TDM1 = TDM3

All other instruments worked optimally during the flight.

Takeoff		Landing

1314Z		1849Z

Aircraft Static Pressure                   1012.3mb	1013.8mb

Corrected Tower Pressure               1012.7mb	           NA

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073