NOAA P-3 N43RF

OCEAN HEAT CONTENT PRE-IKE

Flight ID: 080908I

Sensor or system						Number or Name

INE								1

Accelerometer				1

Temperature Probe						1

Dew Point Probe						2 (EdgeTech)

Altitude (for vertical wind)					Blended GPS/RINU

Static Pressure			Rosemount Fuselage PSF

Dynamic Pressure						Rosemount Fuselage PQF1

Time Source							Micro99

Constants File		n43_hur08.adc

Notes:

There were no time/data gaps during this flight.

Due to hardware changes in some of the navigational equipment, a number of parameters have been adjusted in the netCDF file. RA159 and ALTI1 both display the blended altitude solution of the GPS/RINU system, RA232 displays the corrected HG9550 radar altitude, ALTI2 displays the uncorrected HG9550 radar altitude, and ALTCLN displays the NOVATEL radar altitude data.

Temperature data from sensor number 3 is erroneous.

There was an ~40 mb difference in differential attack pressure between the fuselage and radome.

No Collins or Ashtech GPS data exist for this flight.

The King Liquid Water Sensor was inoperative throughout the flight.

Data from dew point sensor number 2 was edited in order to bring relative humidity values to ~100% during times when oversaturation occurred. These corrections occurred from 155242Z-155255Z, 192021Z-192034Z, 192113Z-192136Z, and 204852Z-204904Z.

SPECIAL NOTE!!!	Vertical ground, vertical air and vertical speeds, computed

using Dave Jorgensen’s vertical wind algorithm should be used for vertical

wind analysis.

Takeoff           		Landing

Aircraft Static Pressure 	1017.6 mb     		1011.5 mb

Corrected Tower Pressure	1015.9 mb		1013.2 mb

Flight Director:			Martin Mayeaux   (813) 828-3310 ext. 3086