1

N42RF ERROR SUMMARY

H. IKE OVER WESTERN CUBA

Flight ID: 080909H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           GPS(Blended)

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There were no data gaps. Beginning with the H080829 H. Gustav flight and continuing through the H080912 H. Ike flight, there was observed a 3 to 4 second lag between all INE1 and INE2 outputs. INE1was selected since it best corresponded with the analog output.

There were no liquid water sensors available for the flight.

The APN-159 (RA159X) was inop for the entire flight. The blended GPS altitude from INE1 (ALTI1) was used.

Measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited a couple of  time periods of erroneous data during the flight.  For 014446Z – 014522Z the erroneous data was removed by substituting TDM3(Maycomm/TDL) output,

TDM2 = TDM3

For 203914Z – 203926Z  the erroneous TDM2 data was removed and replaced using statistical techniques.

For 000472Z – 000536Z  the erroneous TDM2 data was removed by substituting TDM1(Buck) output,

TDM2 = TDM1

All other instruments worked optimally during the flight.

Twenty (20) GPS dropsondes were deployed with all being good. Twenty (20) tempdrop messages were sent to NHC. Also three (3) prototype dropsondes were deployed.

Seven (7) AXBTs were deployed with six (6) being good.

Takeoff		Landing

2010Z		0428Z

Aircraft Static Pressure                   1009.9mb	1012.0mb

Corrected Tower Pressure               1010.2mb	           1012.5mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073