NOAA P-3 N43RF

Tail Doppler Radar Ike

Flight ID: 080911I

Sensor or system						Number or Name

INE								1

Accelerometer				1

Temperature Probe						1

Dew Point Probe						2 (EdgeTech)

Altitude (for vertical wind)					Blended GPS (alti1)

Static Pressure							Rosemount Fuselage PSF

Dynamic Pressure						Rosemount Fuselage PQF1

Time Source							Micro99

Constants File		n43_hur08.adc

There were no time/data gaps during this flight.

Due to hardware changes in some of the navigational equipment, a number of parameters have

been adjusted in the netCDF file. RA159 and ALTI1 both display the blended altitude solution of the

GPS/RINU system, RA232 displays the corrected HG9550 radar altitude, ALTI2 displays the

uncorrected HG9550 radar altitude, and ALTCLN displays the NOVATEL radar altitude data.

Temperature data from sensor number 3 is erroneous.

There was an ~40 mb difference in differential attack pressure between the fuselage and radome.

There was an ~12 mb difference in differential sideslip pressure between the fuselage and radome.

No Collins or Ashtech GPS data exist for this flight.

The King Liquid Water Sensor was inoperative throughout the flight.

Due to oversaturation, relative humidity values occasionally spike above 100%.  No corrections were

made to correct this.

Dewpoint sensor number one displayed erroneous spikes during the following times: 103356Z-103754Z,

110254Z-110555Z, and 163037Z-163110Z.

SPECIAL NOTE!!!	Vertical ground, vertical air and vertical speeds, computed

using Dave Jorgensen’s vertical wind algorithm should be used for vertical

wind analysis.

Takeoff		Landing

0810Z                             1655Z

Aircraft Static Pressure 	1012.6 mb     		1013.9 mb

Corrected Tower Pressure	1012.7 mb		1014.8 mb

Flight Director:			Martin Mayeaux   (813) 828-3310 ext. 3086