1

N42RF ERROR SUMMARY

DISTURBANCE NEAR HISPANOLA

Flight ID: 080924H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           GPS(Blended)

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There were five (5) data gaps: 222351Z – 222359Z, 22501Z – 222531Z,

223041Z – 223111Z, 223351Z – 223356Z and 223421Z - 223440Z.

No lag was observed between INE1 and INE2 outputs.

There were no liquid water sensors available for the flight.

The APN-159 (RA159X) was inop for the entire flight. The blended GPS altitude from INE1 (ALTI1) was used.

Measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited several  time periods of erroneous data during the flight.  For 203616Z – 203623Z, 203844Z – 203847Z,

031622Z - 031752Z and 035747Z - 035815Z the erroneous data was removed by substituting TDM3(Maycomm/TDL) output,

TDM2 = TDM3

It should be noted that TDM3 failed between 223440Z – 234340Z.

For 000240Z – 000549Z  the erroneous TDM2 data was removed by substituting TDM1(Buck) output,

TDM2 = TDM1

All other instruments worked optimally during the flight.

Six (6) GPS dropsondes were deployed with all being good. Six (6)  tempdrop messages were sent to NHC.

Takeoff		Landing

1951Z		0412Z

Aircraft Static Pressure                   1012.0mb	1014.0mb

Corrected Tower Pressure               1011.9mb	           1014.8mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073