1

N42RF ERROR SUMMARY

TS KYLE AND DISTURBANCE NEAR SOUTH CAROLINA COAST

Flight ID: 080925H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1X

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           GPS(Blended)

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There were two (2) data gaps: 234501Z – 234506Z and 030701Z - 030710Z.

No lag was observed between INE1 and INE2 outputs.

There were no liquid water sensors available for the flight.

The APN-159 (RA159X) was inop for the entire flight. The blended GPS altitude from INE1 (ALTI1) was used.

Measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited several  time periods of erroneous data during the flight.  For 211608Z – 211617Z, 215532Z – 215541Z,

215749Z – 215803Z, 021337Z – 021348Z, 021447Z - 021522Z and 025400Z - 025446Z the erroneous data was removed by substituting TDM3(Maycomm/TDL) output,

TDM2 = TDM3

For 011630Z – 011830Z erroneous TDM2 data was removed using statistical techniques.

For 235910Z – 000523Z  the erroneous TDM2 data was removed by substituting TDM1(Buck) output,

TDM2 = TDM1

Total temperature #1 (TT1) failed prior to take-off but came back at eleven (11) minutes later...195201Z – 200331Z. During this time output from total temperature #2 (TT2)

was used

TT1 = TT2

All other instruments worked optimally during the flight.

Nine (9) GPS dropsondes were deployed with 8 being good. Eight (8)  tempdrop messages were sent to NHC.

Takeoff		Landing

1955Z		0456Z

Aircraft Static Pressure                   1011.0mb	1011.8mb

Corrected Tower Pressure               1011.2mb	           1012.3mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073