1

N43RF ERROR SUMMARY

HURRICANE 2009

NESDIS CAL FLIGHT

Flight ID: 090701I

Sensor or system						Number or Name

INE	(for wind derivation)				INE1

Accelerometer						ACC1

Temperature Probe				          TT1

Dew Point Probe						TDM2 (EDGETECH)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						          ALTI1

Constants File                                                           n43_hur09v2.adc

Project Directory						/acdata/2009/hur09/P3

Notes:

Takeoff was delayed due to severe weather over/near MacDill Air Force base.

There were no data gaps.

Dewpoint sensor #1...TDM1 (Buck)...and TDM3 (TDL) behaved erratically throughout the flight so TDM2 (EDGETECH) was selected.

Total temperature probe one...TT1...had several occurrences of erroneous data during the following time frames: 170100Z – 170400Z, 170400Z – 170700Z,

181100Z – 181300Z and 184700Z – 184900Z. The erroneous data was corrected untlizing statistical techiques using TT2 values as a reference.

All other instruments worked optimally during the flight.

One (1) dropsonde was deployed. It was good and its corresponding tempdrop message was transmitted.

SPECIAL NOTE!!!	The variable names dpj_wgs, dpj_was and dpj_wz in the netCDF file represent vertical ground, vertical air and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

Takeoff		Landing

1534Z                   1901Z

Aircraft Static Pressure                1011.8mb	  1012.4mb

Corrected Tower Pressure           1011.5mb	  1012.2mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073