NOAA G-IV N49RF ERROR SUMMARY

1

HURRICANE 2011

26 Aug 2011 Irene (NOAA9 2809A IRENE)

NHC (PHS) KMCFKMCF

Flight ID: 20110826N2

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.2

Dew Point Probe						TD.1

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PS.2

Slip Angle							SA.2

Temperature Probe				                     TTM.1

True Airspeed						TASF.1

Constants File                                                           49cal111

Flight Directory						acdata/2011/MET/20110826N2

Local Met Data			Takeoff (1725z)		Landing (0120z)

Aircraft Static Pressure		  1008.0 mb		               1006.4 mb

Tower Pressure (corrected)	  1009.7 mb			     1009.3 mb

__________________________________________________________________

Notes:

There was a glitch in the AAMPS data system that caused a data gap for all analog and inertial data beginning 5 seconds after 00z that lasted 48 seconds.

There was an abnormally large spread (2.9 millibars for PSF.2) between Raw Static Pressure (PSF) and MacDill Station Pressure during landing rollout. PSF.2 was the closer of the two systems and the default.

TTM.3 displayed an oscillation of around 0.5C (with a period of approx 3 minutes) during cruise above 41K. TTM.1 is the default.

In creation of higher MET parameters, PDAlphaF.2 was selected (under Attack Pressure Probe Selection choices) to utilize AA.2.

GDIFF check: Mean of 1914 meters of drift for INE #1 (with respect to the Novatel) 3647 meters for INE#2. In creation of higher MET parameters, INE #1 selected.

Vertical Winds were very close to zero during the cruise portion of flight (with a mean UWZ.1 of 0.00 from 1840z to 0000z).

All other flight level instruments worked optimally during the flight.

- 35 drop points assigned by NHC for ingestion into 27/0000z models

- 36 AVAPS II dropsondes deployed, all 35 assigned points covered with an ob

- Drop #30 at 2346z had no launch detect but the backup was good

- All 35 transmitted sondes coded surface winds.

Flight Directors: Richard Henning (813) 828-3310 ext. 3086 and LTJG Nick Morgan.