Flight ID: 20111018H1

Sensor or system						Variable Name

Inertial	(for wind derivation)					ine1

Accelerometer							acci1

Temperature Probe						tt1

Dew Point Probe						tdm2

Altitude (for vertical wind)					alti1X

Static Pressure							psf

Dynamic Pressure						pqf1

Constants file							/acdata/adc/42_11v5.adc

Flight Directory						/acdata/2011/MET/20111018H1

Local Met Data:			Takeoff (1558Z)		Landing (2325Z)

Aircraft Static Pressure		 1007.1 mb			1001.9  mb

Tower Pressure			 1007.0  mb			1002.2  mb

Notes

There were three data gaps, during these data gaps all the digital data went to zero but the analog data had a gap.  The times were:  180534Z – 180551Z, 194323Z – 194432Z, and 224938Z - 225034Z.

Alti1 stair stepped erroneously from 173831Z – 173834Z, and altnvl, novatel altitude was used to substitute in for this stairstep using the following equation:  alti1X = altnvl + 2.39 offset.  Alti1X is default.

There were periods where the dewpoint temperature exceeded the ambient temperature resulting in RH values > 100%. This was likely due to heavy rain, a wet-bulb effect on the total temperature sensor, and/or an artificial warming of the dewpoint sensor as it tried to burn off excess moisture. Corrections were not made to the data.

All other instruments appeared to work optimally.

A total of 89 instruments were dropped from the aircraft:

20 Dropsondes, 19 AXBT’s, 25 AXCP’s, 25 AXCTD’s