Flight ID: 20120824N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3x

Attack Angle		PDAlpha.2

Dew Point Probe						TDM.2

Dynamic Pressure						PQM.2

Geopotential Altitude						ALTGA.d (AltGPS.3x)

Inertial Selected						INE.1

Static Pressure							PSM.2

Slip Angle		PDBeta.2

Temperature Probe						TTM.1x

Constants File		49cal102

Flight Directory		acdata/MET/2012/20120824N1

Local Met Data:			Takeoff (1744Z)		Landing (0156Z)

Aircraft Static Pressure		1015.1mb			1013.6mb

Tower Pressure (corrected)		1016.1mb			1015.4mb

Notes:

At 00z there was a system wide data gap that appeared from 00:00:08z to 00:00:56z.

TDM.1 and TDM.2 are both not rated for use under -50 deg C.  Cannot be considered reliable for dewpoints colder than -50C.  When descending to land, TDM.1 as it approached a dewpoint of 0C looked to have malfunctioned.  TDM.2 chosen as default for derived parameters.

There was a downward spike at 19:52:45z with amplitude of approximately 700m in the AltGPS.3(Novatel unit) data.  The .AXC file was created to filter out the spike of erroneous data in AltGPS.3 and AltGPS.3x is used as the default altitude.

TTM.1 had a spike of about 3 deg C at 21:21:17 which was not seen in either TTM.3 or TTM.4.  The spike was filtered out in the .AXC file and the new parameter is named TTM.1x.  In addition to the spike previously mentioned, there was a period of brief turbulence at 21:06z that caused some disagreement among TTM.1 2 and 3.  TTM.1 reflects the middle between the largest spike TTM.4 and TTM.3 which barely notices a temperature change at all.  TTM.1x should be chosen as the reference total temperature.

.

The position data for the GPS.2 (Honeywell unit) has many scattered gaps throughout the flight in the position data.  GPS.3 is advised as the best performing GPS at this time.

PSM.1 and PSM.2 differed by 0.92mb upon taxiing at departure and 1.77mb while taxiing upon return indicating neither looked to have wandered throughout the flight.  From 21:25:14 to 21:31:16 PSM.1 read erroneously so PSM.2 is chosen as default for computations.

From 22:24z to 23:15z an example can be seen of a disagreement in our GPS altitude sources.  GPS.3 (Novatel) seems to read about a 20m difference compared to GPS.1 and GPS.1 altitudes.  Still with this disagreement it is advised to use GPS.3.