Flight ID: 20120825N2

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3x

Attack Angle		PDAlpha.2

Dew Point Probe						TDM.1

Dynamic Pressure						PQM.2

Geopotential Altitude						ALTGA.d (AltGPS.3x)

Inertial Selected						INE.1

Static Pressure							PSM.2

Slip Angle		PDBeta.2

Temperature Probe						TTM.1

Constants File		49cal102

Flight Directory		acdata/MET/2012/20120825N2

Local Met Data:			Takeoff (1727Z)		Landing (0147Z)

Aircraft Static Pressure		1014.1mb			1013.5mb

Tower Pressure (corrected)		1015.4mb			1015.4mb

Notes:

At 00z there was a system wide data gap that appeared from 00:00:08z to 00:00:54z.

TDM.1 and TDM.2 are both not rated for use under -50 deg C.  Cannot be considered reliable for dewpoints colder than -50C.

At 23:58z the tech on board was troubleshooting PSM.1 and PQM.1 to sort out recent issues with performance.  This caused both parameters to error.

There was a downward spike at 21:27:08z with amplitude of approximately 700m in the AltGPS.3(Novatel unit) data.  The .AXC file was created to filter out the spike of erroneous data in AltGPS.3 and AltGPS.3x is used as the default altitude.

From 22:51z to 23:50z an example can be seen of a disagreement in our GPS altitude sources.  GPS.3 (Novatel) seems to read about a 20m difference compared to GPS.1 and GPS.1 altitudes.  Still with this disagreement it is advised to use GPS.3.