Flight ID: 20120826H1

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.2

Dew Point Probe						TDM.2

Static Pressure							PSM.2

Dynamic Pressure						PQM.2

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2012/20120826H1

Constants File	              20120826H1/AAMPSConfig/core/n42.xml

Local Met Data:			Takeoff (2013Z)		Landing (0246Z)

Aircraft Static Pressure (PSM.2)	1011.5 mb			1014.1 mb

Tower Pressure (corrected)		1010.7 mb			1014.5 mb

Notes:

All Navigational Data, and therefore all derived parameters, were lost from 21:15:40Z until 21:24:31Z, when the data system was restarted.  From 21:24:31Z to 21:37:50Z there was a data gap in all parameters due to the system restart.  Because of this, there are two QC’d 1 hz flight level data files, 20120826H1_AC.nc which runs until 21:24:31Z and 20120826H2_BC.nc which runs from 21:37:50Z until block in.

There was a data gap in all parameters from 00:00:09Z – 00:00:51Z.

The Edgetech dewpoint, TDM.2, performed best and was used as default.  Dew point values intermittently exceeded ambient temperature values during portions of flight where the aircraft was in precipitation, causing RH values greater than 100%.

The Novatel Alt, Lat and Lon (GPS.3) had 5 data spikes at 21:40:10Z, 21:43:42Z, 21:43:49Z, 21:44:58Z and 22:47:36Z.  The blended inertial-GPS solution Alt, Lat and Lon (I-GPS.1) is the default position source.

SPECIAL NOTE!!!    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds, vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 28 GPS dropsondes and 18 AXBT’s released from the aircraft.