Flight ID: 20120828H1

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.1

Dew Point Probe						TDM.2X

Static Pressure							PSF.2

Dynamic Pressure						PQM.2

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2012/20120828H1

Constants File	              	AAMPSConfig/core/n43.xml

Local Met Data:			Takeoff (0746Z) KJAX	Landing (1604Z) KJAX

Aircraft Static Pressure		1012.4  mb			1013.4 mb

Tower Pressure (corrected)		1011.8 mb			1013.6 mb

Notes:

Takeoff/Landing data:  Data during landing and takeoff are suspect.  It is recommended that ground data not be used for scientific analysis.

SPECIAL NOTE:  The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

During approach into KJAX, TDM.2 recorded erroneous data.  They were directly substituted with values recorded by TDM.3 during the following times 160131 – 160940.

All other AOC instruments utilized in deriving higher order parameters performed optimally.

There were 34 GPS dropsondes, 20 AXBT’s, 8 CPs and 11 AXCTD’s  deployed from the aircraft.

1 GPS dropsonde, 2 AXBTs, 1 AXCTD, and 3 AXCPs failed.

Beginning at time 15:15:00Z and continuing until time 16:07:44Z, 1-second data gaps were observed in several derived parameter values (those with the .d extension). These occurrences were very infrequent and greatly varied among the list of derived parameter values. In other words wind direction (WD.d) may show a 1-second gap once during the flight but no other derived parameter values would exhibit a data gap for that same time. Likewise after a period of time elapsed a different derived parameter's value, say true airspeed (TAS.d), would be missing but other derived parameter outputs would be present.
  
Since it would be tedious and time consuming to list all the individual derived parameters values missing for a specific time, it was decided to provide the
the customer the beginning and end times for the time period when these data gaps occurred.