NOAA WP-3D N42RF ERROR SUMMARY

1

HURRICANE ISAAC 2012

30 Aug 2012 WA09A ISAAC

HRD POST ISAAC OHC (PHX) KMCF KMCF

Flight ID: 20120830H1

Sensor or system						Number or Name

Altitude							AltGPS.1 (RINU)

Accelerometer						AccZfilterI-GPS.1

Dew Point Probe						TDM.2X

Dynamic Pressure						PQM.2

Inertial Selected						INE1

Static Pressure						PSM.2

Temperature Probe				                     TTM.2

Constants File                                                           AAMPSConfig/core/n43.xml

Flight Directory						acdata/2012/MET/20120830H1

Local Met Data			Takeoff (1225z)		Landing (2104z)

Aircraft Static Pressure		  1016.1 mb			     1015.1 mb

Tower Pressure (corrected)	  1016.3 mb			     1016.8 mb

__________________________________________________________________

Notes:

Most of mission was flown at 6,000 feet PA then 8,000 feet after approx 1930z. There were no data gaps during the flight.

The Edgetech (TDM.2) spiked from 14:06:10z to 14:08:50. The values for the Buck (TDM.1) were substituted for the reference TDM.2 during this interval.

Vertical Winds during the mission cruise portion of the flight from 13:20z to 20:25z showed a mean UWZ of -0.12 m/s on the D.nc file off the plane as well as the QC’d DXC.nc file. SPECIAL NOTE: The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm. It is recommended that these values be used for vertical wind analysis.

All other flight level instruments worked optimally during the flight.

- 20 dropsondes deployed, 2 bad, 18 good and transmitted.

- 22 externally loaded AXBTs plus one internally launched BT were deployed (3 were bad)

- 24 CTs were deployed (1 was bad)

- 20 CPs were deployed (3 were bad)

Flight Directors: Richard Henning (813) 828-3310 ext. 3086 and A Barry Damiano