Flight ID: 20120907N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3x

Attack Angle		PDAlpha.2

Dew Point Probe						TDM.2

Dynamic Pressure						PQM.2

Geopotential Altitude						ALTGA.d (AltGPS.3x)

Inertial Selected						INE.1

Static Pressure							PSM.2

Slip Angle		PDBeta.2

Temperature Probe						TTM.1

Constants File		49cal102

Flight Directory		acdata/MET/2012/20120907N1

Local Met Data:			Takeoff (2115Z)		Landing (0359Z)

Aircraft Static Pressure		1011.1mb			1010.7mb

Tower Pressure (corrected)		1012.7mb			1013.7mb

Notes:

At 00z there was a system wide data gap that appeared from 00:00:05z to 00:00:53z.

TDM.1 and TDM.2 are both not rated for use under -50 deg C.  Cannot be considered reliable for dewpoints colder than -50C.  TDM.2 is used as reference but on occasion is prone to unrealistic relative humidity values (supersaturation).

PQM.1 Malfunctioned throughout the flight and PQM.2 should be used in derived parameters.

There was a downward spike at 03:27:24z with amplitude of approximately 700m in the AltGPS.3(Novatel unit) data.  The .AXC file was created to filter out the spike of erroneous data in AltGPS.3 and AltGPS.3x is used as the default altitude.

There is a gap in AltRa.1 (and AltRa1.c) at 22:48.24 where the radar altimeter loses the ground.  Comes back immediately however.

Static pressure (PS.c) read high upon return to KMCF when compared to tower pressure.  There was a 3mb difference between the two.  Tower Pressure = 1013.7mb, Aircraft Pressure = 1010.7mb.