Flight ID: 20121016H1

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.1

Dew Point Probe						TDM.2

Static Pressure							PSM.2

Dynamic Pressure						PQM.2

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2012/20121016H1

Constants File	              20121016H1/AAMPSConfig/core/n42.xml

Local Met Data:			Takeoff (1827Z)		Landing (0431Z)

Aircraft Static Pressure (PSM.2)	1013.3 mb			1009.6 mb

Tower Pressure (corrected)		1012.1 mb			1011.4 mb

Notes:

There was a data gap from 00:00:07Z – 00:00:51Z.

The Edgetech dewpoint, TDM.2, performed best and was used as default.   However, both TDM.1 and 2 spiked on descent into landing, from 04:24:57Z to 04:26:30Z.  Dew point values intermittently exceeded ambient temperature values during portions of flight where the aircraft was in precipitation, causing RH values greater than 100%.

The Novatel Alt, Lat and Lon (GPS.3) had one data spike at 00:32:50Z.  The blended inertial-GPS solution Alt, Lat and Lon (I-GPS.1) is the default position source.

SPECIAL NOTE!!!    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds, vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 13 GPS dropsondes (13 sent) and 6 AXBT’s released from the aircraft.  There were 4 hurricane penetrations.