| MISSION PLAN |  |  |  |
| --- | --- | --- | --- |
| FLIGHT ID | 20240926H1 | STORM | AL09 |
| MISSION ID | 1709A | TAIL NUMBER | NOAA-42 |
| TASKING | NHC/EMC TDR | PLANNED PATTERN | Rotated Fig-4 |
| MISSION SUMMARY |  |  |  |
| TAKEOFF [UTC] | 1952 | LANDING [UTC] |  |
| TAKEOFF LOCATION | KEFD | LANDING LOCATION | KEFD |
| FLIGHT TIME | Fractional hr, Takeoff to Landing Time | BLOCK TIME | Get from onboard LPS or Flight Director |
| TOTAL REAL-TIME RADAR ANALYSES (Transmitted) | 4 (4) | TOTAL DROPSONDES Deployed (Tx to GTS) | 24 (20) |
| OCEAN EXPENDABLES deployed (good) | 8 UM AXBTs (7) | sUAS (Type) | 2 Blackswift s0 |
| APHEX EXPERIMENTS / MODULES | Exact name of the Experiment in the HFP Plan; identify relevant experiments / module even if not a research tasking |  |  |
| HRD CREW MANIFEST |  |  |  |
| LPS ONBOARD | Hazelton | LPS GROUND | Holbach/Aberson |
| TDR ONBOARD | Hazelton | TDR GROUND | Gamache/Hollingshead |
| ASPEN ONBOARD | n/a | ASPEN GROUND | Sippel/Dahl |
| NESDIS SCIENTISTS | Chang, Jelenak |  |  |
| GUESTS (Affiliation) |  |  |  |
| AOC CREW MANIFEST |  |  |  |
| PILOTS | Abitol, Wood, Keith |  |  |
| NAVIGATOR |  |  |  |
| FLIGHT ENGINEERS |  |  |  |
| FLIGHT DIRECTOR |  |  |  |
| DATA TECHNICIAN |  |  |  |
| AVAPS |  |  |  |

| PRE-FLIGHT |  |
| --- | --- |
| Flight Plan | Planned flight pattern is a modified butterfly with a coastal run. 105 n mi legs. |
| Expendable Distribution | Endpoint, midpoint, and center drops. Coastal drops every ~10 n mi. AXBTs at locations noted in the flight plan. |
| Preflight Weather Briefing | As of 5pm ET Helene is a category 3 hurricane with max winds of 110 kt and minimum pressure of 951 mb. It is moving north-northeastward at 025/20 kt. NHC 5pm discussion: Helene has rapidly intensified today while nearing landfall in the Florida Big Bend. Doppler radar and aircraft data indicate that the eyewall is now completely closed and the eye has become more circular and is clearing out. The aircraft data also indicated that the inner core has contracted significantly today while the tropical-storm-force winds have expanded. Based on all of the data, the initial intensity is set at 110 kt. The latest minimum pressure based on the aircraft data is around 951 mb. Tropical-storm-force winds are occurring across portions of west-central and southwestern Florida, and conditions are expected to deteriorate rapidly in the hurricane warning area during the next several hours. It should be emphasized that Helene is at the upper bound of hurricanes in terms of storm size and impacts are and will occur well away from the center. The large hurricane is now accelerating north-northeastward, with the latest initial motion estimated to be 025/20 kt. This general motion is expected to continue, taking the core of the major hurricane to the Florida Big Bend later this evening. After landfall, a turn to the north over Georgia is expected late tonight and early Friday, followed by a slowdown or a complete stall over the Tennessee Valley late Friday and Saturday when Helene merges with a mid- to upper-level low. The hurricane has intensified by 30 kt since sunrise and given the conducive environmental conditions and contracting inner core, it seems likely that Helene will be at or very near category 4 strength when it makes landfall in the Florida Big Bend this evening. The fast forward speed when Helene moves inland will result in a far inland penetration of strong winds over parts of the southeastern United States, including strong gusts over the higher terrain of the southern Appalachians. Accordingly, a higher-than-normal gust factor is indicated in the official forecast while Helene is inland. |
| Instrument Notes | [What instruments are working, not working, not functioning nominally, not installed?] |

| IN-FLIGHT |  |
| --- | --- |
| Time [UTC] | Event |
| 1952 | Take-off from EFD |
| 2038 | Modified flight track: ahazelton_n42_LPS Looks very soupy for the coastal run, and the TC is further S than anticipated ahazelton_n42_LPS So we are doing an audible to what's basically a rotated figure 4 Droning_On A second audible is we are doing only 1 sUAS (@ the IP 105nmi west of the center) ahazelton_n42_LPS We will do a W-E run first, downwind, NE-SW, downwind, S-N, downwind, NW to the center, and from there either out SE or W depending on if we can land in Lakeland or have to go back to Houston Droning_On It will be an inflow mission ending with (hopefully, time permitting) a partial orbit in the eyewall |
| 2123 | Drop 1 IP west with S0 |
| 2130 | S0 at 1000m. Following a trajectory that should get it to the storm center/eyewall. Wind direction 245 deg. |
| 2135 | Drop 2 midpoint, combo drop BT 1, started reporting when T=20C |
| 2142 | Onboard LPS notes an elliptical eye with possible mesovortices |
| 2149 | Drop 3 center, combo drop BT 2 29.5C URNT12 KWBC VORTEX DATA MESSAGE AL092024 A. 26/21:50:06Z B. 27.90 deg N 084.56 deg W C. NA D. 948 mb E. 190 deg 08 kt F. OPEN W G. C30 H. NA I. NA J. 317 deg 97 kt K. 220 deg 12 nm 21:47:05Z L. NA M. NA N. 185 deg 136 kt O. 090 deg 31 nm 21:57:21Z P. 20 C / 2448 m Q. 23 C / 2441 m R. 14 C / NA S. 12345 / NA T. 0.01 / 3 nm U. NOAA2 1709A HELENE OB 99 MAX FL WIND 136 KT 090 / 31 NM 21:57:21Z MAX FL TEMP 24 C 089 / 13 NM FROM FL CNTR TDR center info from 240926H1_2150_xy.nc: Alt (km) Lat (deg) Lon (deg W) 0.5 99.99 99.99 2.0 27.89 84.49 3.0 27.91 84.47 6.0 27.94 84.41 2-6-km Vortex Tilt: 10.0 km at 53 deg |
| 2156 | Drop 4 RMW E |
| 2200 | Drop 5 MP combo drop BT 3 29.4C |
| 2210 | Drop 6 EP |
| 2218 | Lost comms with S0 |
| 2220 | NHC upgraded Helene to a category 4 hurricane with winds 115 kt and min pressure 947 based on data from our first pass. |
| 2225 | Drop 7 IP NE |
| 2226 | Regained comms with S0 |
| 2234 | Drop 8 MP |
| 2238 | Drops 9, 10, 11, 3-sonde sequence in northeast eyewall |
| 2247 | Drop 12, center combo BT 29.1 URNT12 KWBC VORTEX DATA MESSAGE AL092024 A. 26/22:46:11Z B. 28.26 deg N 084.42 deg W C. NA D. 944 mb E. NA F. CLOSED G. E14/35/25 H. NA I. NA J. 150 deg 127 kt K. 066 deg 20 nm 22:40:35Z L. NA M. NA N. 298 deg 85 kt O. 033 deg 1 nm 23:30:39Z P. 17 C / 2441 m Q. 25 C / 2306 m R. 18 C / NA S. 12345 / NA T. 0.01 / 2 nm U. NOAA2 1709A HELENE OB 99 MAX FL WIND 136 KT 090 / 31 NM 21:57:21Z TDR center info from 240926H1_2246_xy.nc: Alt (km) Lat (deg) Lon (deg W) 0.5 99.99 99.99 2.0 28.25 84.35 3.0 28.27 84.33 6.0 28.31 84.31 2-6-km Vortex Tilt: 7.2 km at 34 deg |
| 2250 | Turned in eye to reposition. Looked like center was tucked to west side where eyewall curved in. Continued circling due to comms issues |
| 2321 | Onboard LPS noted that eyewall seems to be closing off on the MMR and nose |
| 2322 | Plan is to go out SW, then downwind to S, S to N, downwind to NW, inbound from NW and then either out SE or back out NW depending on time and where they will be landing |
| 2323 | Release of second S0 |
| 2326 | Onboard LPS noted lots of small scale and medium scale features in the eyewall |
| 2338 | Droning_On update for s0....100kts at 900m...just entering east eyewall...plan to drop to 700m soon...should get north semicircle of eyewall...maybe more cov Droning_On 140kts.. sim_home You measured 140 kt, or hoping you do? ahazelton_n42_LPS It did ahazelton_n42_LPS I think there was a 148 Droning_On whats crazy is winds are HIGHLY variable...not usually what we see...140, 110, 130, 100...all within a min of flight time...meso vortex action ?? Droning_On we are at 700m..droppong to 500m |
| 2345 | Sonde 13 MP combo BT bad |
| 2328 | Inadvertent eye/eyewall mixing module |
| 2340 | S0 reported 148 kt at 900 m |
| 2353 | Sonde 14 EP |
| 2347 | Trimming SW leg a little to save some time and there is not much precip out that way |
| 0011 | Sonde 15 IP S |
| 0019 | Sonde 16 MP, bad sonde |
| 0020 | Sonde 17 MP combo BT bad |
| 0030 | No center sonde TDR center info from 240926H1_2430_xy.nc: Alt (km) Lat (deg) Lon (deg W) 0.5 99.99 99.99 2.0 99.99 99.99 3.0 99.99 99.99 6.0 28.95 84.09 2-6-km Vortex Tilt: 999.9 km at 999 deg |
| 0031 | Decided not to go out north, going out northeast. Scalloping in eyewall |
| 0032 | URNT12 KNHC 270056 VORTEX DATA MESSAGE AL092024 A. 27/00:32:30Z B. 28.98 deg N 084.20 deg W C. 700 mb 2622 m D. 942 mb E. 175 deg 11 kt F. OPEN S G. C30 H. 101 kt I. 059 deg 17 nm 00:27:00Z J. 136 deg 123 kt K. 056 deg 23 nm 00:25:00Z L. 79 kt M. 202 deg 15 nm 00:37:30Z N. 301 deg 100 kt O. 199 deg 14 nm 00:37:00Z P. 10 C / 3045 m Q. 20 C / 3047 m R. 14 C / NA S. 12345 / 7 T. 0.02 / 1.5 nm U. AF306 1809A HELENE OB 09 MAX FL WIND 134 KT 085 / 28 NM 23:46:30Z |
| 0034 | Sonde 18 RMW NE |
| 0042-3 | Sondes 19-21 RMW NE |
| 0044 | S0 got 138 kt at 150 m |
|  | Quick turnaround, back in northeast |
| 0047 | Lost comms with S0 |
| 0050 | Sonde 22 Center URNT12 KWBC VORTEX DATA MESSAGE AL092024 A. 27/00:49:57Z B. 29.01 deg N 084.17 deg W C. NA D. 943 mb E. 285 deg 09 kt F. OPEN S G. E12/40/25 H. NA I. NA J. 165 deg 136 kt K. 068 deg 23 nm 00:43:27Z L. NA M. NA N. 302 deg 87 kt O. 239 deg 8 nm 00:52:05Z P. 17 C / 2535 m Q. 22 C / 2437 m R. 16 C / NA S. 1234 / NA T. 0.01 / NA U. NOAA2 1709A HELENE OB 99 MAX FL WIND 139 KT 172 / 33 NM 00:35:17Z MAX FL TEMP 24 C 076 / 16 NM FROM FL CNTR TDR center info from 240926H1_2449_xy.nc: Alt (km) Lat (deg) Lon (deg W) 0.5 99.99 99.99 2.0 29.00 84.08 3.0 29.06 84.10 6.0 29.11 84.08 2-6-km Vortex Tilt: 12.0 km at 0 deg |
| 0100 | Sonde 23 MP W |
| 0116 | Sonde 24 EP |

| POST-FLIGHT |  |
| --- | --- |
| Mission Summary | [Short description of interesting observations from the flight; what objectives were successful? What was unsuccessful? Was the planned pattern flown? What deviations occurred? [Don’t forget to fill in Tables on page 1] [Sonde and ocean expendable accounting: how many total of each? How many are charged to each account?] |
| Actual Standard Pattern Flown | [Butterfly, Rotated Figure-4, Lawnmower, etc] |
| APHEX Experiments / Modules Flown | [Linked to HFP Plan; fill in regardless of whether the mission was operationally or research tasked] |
| Plain Language Summary | [Boil down the above into a couple of bullet points in “plain language”. This will help us when we report to management & OAR Public Affairs and prepare storm mission summaries] |
| Instrument Notes | [Notes about instrument status from during and after the mission] |
| Final Mission Track |  |