NOAA Data Report, OAR AOML-XX

HYDROGRAPHIC MEASUREMENTS COLLECTED ABOARD THE R.V. OCEANUS, APRIL-MAY 2001: WESTERN BOUNDARY TIME SERIES CRUISE 1 (ABACO 2001)

C. Fonseca

M. Baringer

Atlantic Oceanographic and Meteorological Laboratory

Miami, Florida

April 2009

noaa

NATIONAL OCEANIC AND

ATMOSPHERIC ADMINISTRATION

Office of Oceanic and

Atmospheric Research

NOAA  Data Report OAR AOML-XX

HYDROGRAPHIC MEASUREMENTS COLLECTED ABOARD THE R.V. OCEANUS, APRIL-MAY 2001: WESTERN BOUNDARY TIME SERIES CRUISE 1(ABACO 2001)

Carlos Fonseca

University of Miami/ Cooperative Institute for Marine and Atmospheric Studies,

Miami, Florida

Molly Baringer

NOAA/ Atlantic Oceanographic and Meteorological Laboratory

Miami, Florida

April 2009

UNITED STATES	NATIONAL OCEANIC AND	Office of Oceanic and

DEPARTMENT OF COMMERCE	ATMOSPHERIC ADMINISTRATION	Atmospheric Research

Carlos M. Gutierrez	VADM Conrad C. Lautenbacher, Jr.	Dr. Richard W. Spinrad

Secretary	Undersecretary for Oceans and	Assistant Administrator

Atmosphere/Administrator

# Disclaimer

NOAA does not approve, recommend, or endorse any proprietary product or material mentioned in this document. No reference shall be made to NOAA or to this document in any advertising or sales promotion, which would indicate or imply that NOAA approves, recommends, or endorses any proprietary product or proprietary material herein or which has as its purpose any intent to cause directly or indirectly the advertised product to be used or purchased because of this document.

The findings and conclusion on this report are those of the authors and do not necessarily represent the views of the funding agency.

# Table of Contents

Disclaimer	3

Table of Contents	4

List of Tables	5

List of Figures	6

Abstract	8

1.	Introduction	8

2.	Cruise Narrative	12

3.	Inverted Echo-Sounder Operations	13

4.	Standards and Pre-Cruise Calibrations	13

4.1	Conductivity	14

4.2	Temperature	15

4.3	Pressure	16

4.4	Dissolved Oxygen	17

5.	Data Acquisition	18

5.1	Data Acquisition/System Problems	19

5.2	Salinity Analyses	19

5.3	Oxygen Analyses	20

6.	Processing of CTD data	21

7.	Post-Cruise Calibrations	23

7.1	CTD Data Processing	23

7.2	Pressure	24

7.3	Temperature	25

7.4	Conductivity	26

7.5	Dissolved Oxygen	32

8.	Final  CTD Data Presentation	38

9.	Acknowledgements	45

10.	References	45

Appendix A - WOCE Summary File	46

Appendix B – WOCE Bottle Summary File	51

Appendix C - Hydrographic - CTD Data	71

# List of Tables

Table 1.1 – Cruise Participants of the Western Boundary Time Series Cruise (2001)	11

Table 1.2 – ABACO-01 CTD Cast Summary	11

Table 3.1 - Inverted echo-sounders deployment locations.	13

Table 4.1 - Equipment used during ABACO-01	14

Table 4.2 – Calibration coefficients for the conductivity sensors.	15

Table 4.3 - Calibration coefficients for the temperature sensors.	16

Table 4.4 - Calibration coefficients for the pressure sensor.	17

Table 4.5 - Calibration coefficients for the dissolved oxygen sensors.	18

Table 7.1 - Calibration coefficients for the temperature sensors	23

Table 7.2 - Calibration coefficients for the conductivity sensors	23

Table 7.3 – Time drift correction for the temperature sensors	25

Table 7.4 - Time drift correction for the conductivity sensors	26

Table 7.5 – Final statistics of the conductivity calibration	30

Table 7.6 – Final Statistics of the oxygen calibration (values in μmol/kg)	36

# List of Figures

Figure 1-1 - ABACO-01 Cruise Track and CTD station locations.	9

Figure 1-2 - Abaco section CTD station locations (7–33) and IES deployment locations.	10

Figure 1-3 - Northwest Providence Channel section CTD station locations (2-6).	10

Figure 5-1 - Bottle locations for 26.5ºN Deep Western Boundary Current section east of Abaco Island (Abaco section).	20

Figure 5-2 - Bottle locations for Northwest Providence Channel section.	21

Figure 7-1 – Pressure recorded by the CTD on deck before cast	24

Figure 7-2 – Temperature differences between sensors by station number for p > 500 dbar.	25

Figure 7-3 - Temperature differences between sensors at each bottle stop plotted vs. pressure.	26

Figure 7-4 - Bottle - Uncalibrated CTD salinity differences plotted against station number. The crosses are data points above 500 dbar and the stars are the data points below 500 dbar. The average values for each group by station number are also plotted (squares and triangles respectively).	27

Figure 7-5 - Bottle - Uncalibrated CTD salinity differences plotted against pressure. The crosses are data points above 500 dbar and the stars are the data points below 500 dbar.	28

Figure 7-6 - Bottle - Calibrated CTD salinity differences plotted against station number	28

Figure 7-7 - Bottle - Calibrated CTD salinity differences plotted against pressure.	29

Figure 7-8 - Bottle - Calibrated CTD salinity differences plotted against station number (data below 500 dbar).	29

Figure 7-9 - Bottle - Calibrated CTD salinity differences plotted against pressure (data below 500 dbar).	30

Figure 7-10 –Temperature – Salinity diagram for stations located on the Northwest Providence Channel section area. The solid black lines are the data collected during this cruise; the gray dots are data from the historical database.	31

Figure 7-11 - Potential Temperature – Salinity diagram for stations located on the Abaco section area. The solid black lines are the data collected during this cruise; the gray dots are data from the historical database.	31

Figure 7-12 - Potential Temperature – Salinity diagram for stations located on the Abaco section area with emphasis on the properties of the deep water. The solid black lines are the data collected during this cruise; the gray dots are data from the historical database.	32

Figure 7-13 - Bottle - Uncalibrated CTD oxygen differences plotted against pressure. The crosses are data points above 500 dbar and the stars are the data points below 500 dbar. The average values for each group by station number are also plotted (squares and triangles respectively).	33

Figure 7-14 - Bottle - Uncalibrated CTD oxygen differences plotted against pressure. The crosses are data points above 500 dbar and the stars are the data points below 500 dbar.	34

Figure 7-15 - Bottle - Calibrated CTD oxygen differences plotted against station number	34

Figure 7-16 - Bottle - Calibrated CTD oxygen differences plotted against pressure	35

Figure 7-17 - Bottle - Calibrated CTD oxygen differences plotted against station number for data points below 500 dbar.	35

Figure 7-18 - Bottle - Calibrated CTD oxygen differences plotted against station number (data points below 500 dbar).	36

Figure 7-19 - Potential Temperature – Dissolved Oxygen diagram for stations located on the Abaco section area. The solid black lines are the data collected during this cruise; the gray dots are data from the historical database.	37

Figure 7-20 - Potential Temperature – Dissolved Oxygen diagram for stations located on the Abaco section area with emphasis on the properties of the deep water. The solid black lines are the data collected during this cruise; the gray dots are data from the historical database.	37

Figure 8-1 – Potential Temperature (oC) section for the Abaco Section. Contour intervals are 0.1oC from 1-2 oC, 0.2oC from 2-3oC, 0.5oC from 3-5oC and 1oC for 5-35oC.  Dashed vertical lines are the CTD station locations.	39

Figure 8-2 – Salinity (PSS 78) section for the Abaco section. Contour intervals are 0.01 from 34-35, 0.05 from 35-35.1and 0.1 from 35.1-38.  Dashed vertical lines are the CTD station locations.	40

Figure 8-3 – Dissolved Oxygen (μmol/kg) section for the Abaco Section. Contour intervals are 10 μmol/kg from 100-300 μmol/kg in the upper panel. In the lower panel, contours are 20 μmol/kg from 100-220 μmol/kg and 5 μmol/kg form 220-255 μmol/kg and  2 μmol/kg from 256-270 μmol/kg.  Dashed vertical lines are the CTD station locations.	41

Figure 8-4 – Neutral density (kg/m3) section for the Abaco Section. Contour intervals are 0.2 kg/m3 for 24-26 kg/m3, 0.1 kg/m3 for 26-28 kg/m3 in the upper panel. In the lower panel, contour intervals are 0.05 kg/m3 for 27-27.8 kg/m3 and 0.01 kg/m3 to 27.8 to 29 kg/m3.   Dashed vertical lines are the CTD station locations.	42

Figure 8-5 – Potential Temperature (oC) section for the Northwest Providence Channel Section. Contour intervals are 0.1oC from 1-2 oC, 0.2oC from 2-3oC, 0.5oC from 3-5oC and 1oC for 5-35oC. Dashed vertical lines are the CTD station locations.	43

Figure 8-6 – Salinity (PSS 78) section for the Northwest Providence Channel section. Contour intervals are 0.01 from 34-35, 0.05 from 35-35.1and 0.1 from 35.1-38.  Dashed vertical lines are the CTD station locations.	43

Figure 8-7 – Dissolved Oxygen (μmol/kg) section for the Northwest Providence Channel Section. Contour intervals are 10 μmol/kg. Dashed vertical lines are the CTD station locations.	44

Figure 8-8 – Neutral density (kg/m3) section for the Northwest Providence Channel Section. Contour intervals are 0.2 kg/m3 for 24-26 kg/m3, 0.1 kg/m3 for 26-28 kg/m3. Dashed vertical lines are the CTD station locations.	44

# Abstract

Summaries of CTD/O2 measurements and hydrographic data acquired on an oceanographic research cruise during the spring of 2001 aboard the R/V Oceanus are presented.  Data acquisition and processing systems are described and calibration procedures are documented.  Station location, CTD/O2 data listings, and profiles are included for each station.

# Introduction

The Abaco time series began in August 1984 when NOAA extended its Straits of Florida program to include measurements of western boundary current transports and watermass properties east of Abaco, the Bahamas. Since 1986, more than 21 hydrographic sections have been completed east of Abaco, most including direct velocity observations by Pegasus and/or Lowered Acoustic Doppler Current Profiler (LADCP). Transient tracer (CFC) measurements have been made on 7 of these sections, at roughly 2-year intervals. Current meter arrays were also maintained from April 1986 to April 1997.

The repeated hydrographic and tracer sampling at Abaco has established a high-resolution record of watermass properties in the Deep Water Boundary Current (DWBC) at 26oN, which for temperature and salinity can be reasonably constructed back to about 1955 [Molinari, et al., 1998; Vaughan and Molinari, 1997]. Events such as the intense convection period in the Labrador Sea and renewal of classical Labrador Sea Water in the 1980's are clearly reflected in the cooling and freshening of the DWBC waters off Abaco, and the arrival of a strong CFC pulse, approximately 10 years later.  This array is unique in that it is not just a single time series site but also a transport section, of which very few are available in the ocean that approach a decade in length.

To achieve the goals of NOAA's strategic plan in terms of understanding the Atlantic Ocean's role in decadal and longer time scale climate variability, these continued time series observations at Abaco are seen as serving three main purposes:

1. Monitoring of the DWBC for watermass and transport signatures related to changes in the strengths and regions of high latitude water mass formation in the North Atlantic.  Monitoring watermass properties in the DWBC at key locations is one part of an effort to track decadal changes in large-scale watermass properties.

2. Serving as a western boundary endpoint of a subtropical Meridional Overturning Circulation (MOC) heat flux monitoring system designed to measure the interior dynamic height difference across the Atlantic basin and the associated baroclinic heat transport.

3. Monitoring the intensity of the Antilles current as an index (together with the Florida Current) of inter-annual variability in the strength of the subtropical gyre.  Variations in the strength of the subtropical gyre in relation to the North Atlantic Oscillation (NAO) has been proposed as an important mechanism in the atmosphere-ocean feedback within

coupled models (e.g. [Latif and Barnett, 1996]

During the 2001 survey, a total of 33 hydrographic stations were occupied in the Florida Straits and East of Abaco Island, Bahamas (Figure 1-1) aboard the R/V Oceanus. Table 1.2 provides a summary of cast information.  At each station, profiles of temperature, salinity (conductivity) and dissolved oxygen concentration were collected to within approximately 20 m of the bottom. Water samples for calibration of the salinity and dissolved oxygen profiles were collected at each station using 10L Niskin-type bottles.

Figure 1-1 - ABACO-01 Cruise Track and CTD station locations.

Figure 1-2 - Abaco section CTD station locations (7–33) and IES deployment locations.

Figure 1-3 - Northwest Providence Channel section CTD station locations (2-6).

Table 1.1 – Cruise Participants of the Western Boundary Time Series Cruise (2001)

| Name | Responsibility | Affiliation | Nationality |
| --- | --- | --- | --- |
| Molly Baringer | Principal Investigator, Chief Scientist | AOML | USA |
| David Bitterman | ET and LADCP watch | AOML | USA |
| Robert Roddy | Oxygen analysis | AOML | USA |
| Doug Anderson | ET and LADCP watch | AOML | USA |
| Sonia Bauer | CTD watch stander | AOML | USA |

Table 1.2 – ABACO-01 CTD Cast Summary

| Station | Date | Time | Latitude | Longitude | Depth |
| --- | --- | --- | --- | --- | --- |
|  |  | (GMT) | N | W | (m) |
| 001 | 04/26/01 | 2031 | 27.024 | 79.919 | 157 |
| 002 | 04/27/01 | 1200 | 26.442 | 78.668 | 741 |
| 003 | 04/27/01 | 1321 | 26.325 | 78.713 | 656 |
| 004 | 04/27/01 | 1430 | 26.240 | 78.772 | 507 |
| 005 | 04/27/01 | 1544 | 26.161 | 78.806 | 443 |
| 006 | 04/27/01 | 1659 | 26.065 | 78.858 | 283 |
| 007 | 04/28/01 | 0649 | 26.501 | 76.914 | 978 |
| 008 | 04/28/01 | 1048 | 26.512 | 76.843 | 1005 |
| 009 | 04/28/01 | 1319 | 26.507 | 76.754 | 3913 |
| 010 | 04/28/01 | 1709 | 26.513 | 76.614 | 4729 |
| 011 | 04/28/01 | 2355 | 26.527 | 76.492 | 4842 |
| 012 | 04/29/01 | 0420 | 26.529 | 76.316 | 4828 |
| 013 | 04/29/01 | 0853 | 26.528 | 76.127 | 4807 |
| 014 | 04/29/01 | 1343 | 26.531 | 75.928 | 4748 |
| 015 | 04/29/01 | 2111 | 26.551 | 75.728 | 4690 |
| 016 | 04/29/01 | 0237 | 26.531 | 75.515 | 4683 |
| 017 | 04/30/01 | 0724 | 26.515 | 75.304 | 4636 |
| 018 | 04/30/01 | 1228 | 26.529 | 75.091 | 4603 |
| 019 | 04/30/01 | 1728 | 26.502 | 74.797 | 4508 |
| 020 | 04/30/01 | 2257 | 26.512 | 74.518 | 4496 |
| 021 | 05/01/01 | 0433 | 26.519 | 74.243 | 4536 |
| 022 | 05/01/01 | 0958 | 26.520 | 73.963 | 4642 |
| 023 | 05/01/01 | 1543 | 26.519 | 73.588 | 4920 |
| 024 | 05/01/01 | 2056 | 26.517 | 73.234 | 5058 |
| 025 | 05/02/01 | 1723 | 26.580 | 72.858 | 5107 |
| 026 | 05/02/01 | 0002 | 26.515 | 72.462 | 5138 |
| 027 | 05/03/01 | 0613 | 26.531 | 72.098 | 5278 |
| 028 | 05/03/01 | 1259 | 26.529 | 71.729 | 5373 |
| 029 | 05/03/01 | 1957 | 26.534 | 71.337 | 5465 |
| 030 | 05/04/01 | 0210 | 26.537 | 71.009 | 5470 |
| 031 | 05/04/01 | 1118 | 26.540 | 70.681 | 5470 |
| 032 | 05/04/01 | 1803 | 26.521 | 70.200 | 5471 |
| 033 | 05/04/01 | 0027 | 26.520 | 69.806 | 5483 |

# Cruise Narrative

The cruise left Miami Thursday April 26th and proceeded northward to a test cast in the Florida Straits before reaching the first CTD cast on the west end of the 27oN section.  The weather leaving Miami was beautiful and calm, but steadily deteriorated as we moved northward.  The first CTD cast was conducted in approximately 26 knot sustained winds from the north and 8-10 ft seas in the Florida Current moving in excess of 2 knots.  The Captain decided it was too rough to reliably hold the CTD wire off the hull of the ship, so the 27oN section was abandoned and we proceeded to the Northwest Providence Channel section. The weather forced the cancellation of the small piggyback project to measure stable isotopes in the Florida Straits (Dr. Jordan UC Santa Barbara).  Thanks to coordination with the ship, the project was moved to another cruise.

Winds changed over the night to 15 knots out of the east and the CTD stations proceeded as planned until the weather started to decline on Monday April 30th and the transit time between stations slowed to about 6 knots. Over the evening and morning of May 1-2 the ship remained hove-to with 25-35 knot winds and 10-14 ft seas.  The morning of May 2nd winds dropped to 20-25 knots and seas to about 8 ft and operations resumed.  The last CTD station was completed along the 26.5oN section on the evening of Friday May 4, 2008 and the ship docked back in Miami at the anticipated time Monday May 7.  Returning to Miami downwind, the ship experienced 20-30 degree tilts athwart ship.

The ship collected standard VMDAS shipboard ADCP and the standard pingdata data files were stored with no on board processing or quality control.  The LADCP system used included a 300 kHz RDI workhorse.   The first unit used had flooded two months prior on a Windward Island Monitoring Program cruise (D. Wilson, pers. Comm.), but was quickly repaired and returned by RDI in time for this cruise.  No problems were encountered until station 9, the first station over 1000 m.  The LADCP flooded again and we swapped to a secondary unit on loan to AOML from RDI.  For the secondary unit several communications problems prevented the data from being downloaded via any cables.  After station 14 the data was successfully downloaded after removing the memory card from the unit and reading using a PCIMA memory slot on a portable laptop (further details in the LADCP Data Report).

The RV Oceanus has no dynamic positioning capabilities and limited ability to work in rough weather and hold station.  Through most of the cruise the stations were held no better than about 1 nm from beginning to end of the CTD cast.  Most of the displacements occurred in the north/south direction. It is our conclusion that use of a Class II ship for the Western Boundary Hydrographic Surveys should account for additional ship days (2 or more) in case of weather related problems.  We would also recommend using Class II ships with dynamic positioning.

A total of 33 stations were completed including one station in the Florida Straits, five stations across the Northwest Providence Channel and 27 stations east of Abaco Island out to 69o 50’ W. Also four inverted Echo-Sounders (IES) were deployed east of Abaco Island. Deep western Boundary Current velocities were in the range of 20-40 cm/sec southward.

# Inverted Echo-Sounder Operations

An inverted echo sounder is consisting mainly of a transducer, which can produce sound waves and hear sound waves, and a precise clock. The inverted echo sounders used here at AOML send out a series of 24 10kHz or 12kHz sound pulses each hour. These pulses reflect when they hit the ocean surface, and 1-8 seconds later the IES records the precise amount of time between when each pulse is sent out and when the pulse is heard returning to the IES. The median value of the 24 pulses is then taken as the travel time for that hour (multiple pulses are needed to average out the changes in travel time due to waves at the ocean surface and other sources of noise). Because the speed of sound in seawater is dependent on temperature, as the water temperatures (and salinities) above the IES change over time the travel time measurement of the IES changes. The travel time measurement of the IES is combined with other ocean measurements of temperature and salinity in order to estimate full-water-column profiles of temperature, salinity, and density. The result is a time series of profiles of these quantities at each IES site.

During the April 2001 cruise four inverted echo-sounders (University of Rhode Island URI model 6.1c) were deployed. Deployments were planned for two years.

Bathymetric surveys were used to triangulate over the projected area of IES deployment to find a location with a gentle slope (we aimed for slopes less than 5o).  The acoustic transmissions on the ship (possibly due to ambient ocean noise due to the harsh weather resulted in some problems recording IES ping transmissions on the LSR/PDR.

It is useful to monitor the IES deployment by tracking the IES descent and verifying the first travel time observation after deployment.  Generally deployments went very well, ranging in time between 2.5 and 4 hours.  Eventually we discovered that the PDR/LSR tracking was improved if the ship maintained a slow uniform speed, suggesting the poor transmissions were, in fact, due to environmental conditions.

Table 3.3 - Inverted echo-sounders deployment locations.

| Instrument SN# | Latitude | Longitude |
| --- | --- | --- |
| 24 | 26o 30.700 N | 76o 50.412 W |
| 21 | 26o 29.986 N | 76o 28.788 W |
| 23 | 26o 30.070 N | 75o 42.224 W |
| 22 | 26o 30.171 N | 70o 41.101 W |

# Standards and Pre-Cruise Calibrations

The CTD/O2 system is a real-time data acquisition system with the data from a Sea-Bird Electronics, Inc. (SBE) 9plus underwater unit transmitted via a conducting cable to a SBE 11plus deck unit.  The serial data from the underwater unit is sent to the deck unit in RS-232 NRZ format.  The deck unit decodes the serial data and sends it to a personal computer for display and storage in a disk file using Sea-Bird Seasave software (version 4.22).

The SBE 911plus system transmits data from primary and auxiliary sensors in the form of binary numbers equivalent to the frequency or voltage outputs from those sensors.  These are referred to as the raw data.  The SBE software performs the calculations required to convert raw data to engineering units.

The SBE 911plus system is electrically and mechanically compatible with the standard, unmodified carousel water sampler, also made by Sea-Bird Electronics, Inc.  A modem and carousel interface allows the 911plus system to control the operations of the carousel directly without interrupting the flow of data from the CTD.

The SBE 9plus underwater unit is configured with dual standard modular temperature (SBE 3) and conductivity (SBE 4) sensors, which are mounted near the lower end cap.  The conductivity cell entrance is co-planar with the tip of the temperature sensor probe.  The pressure sensor is mounted inside the underwater unit main housing.  A centrifugal pump module flushes water through sensor tubing at a constant rate independent of the CTD’s motion to improve dynamic performance.  Dual dissolved oxygen sensors (YSI-type) are added to the pumped sensor configuration following the temperature-conductivity (TC) pair.

Table 4.4 - Equipment used during ABACO-01

| Instrument | SN | Stations | Note |
| --- | --- | --- | --- |
| Sea-Bird SBE32 24-place Carousel Water Sampler | 328531-0031 | 1-33 |  |
| Sea-Bird SBE9plus CTD | 09P10779-363 | 0-1 |  |
| Sea-Bird SBE9plus CTD | 09P8531-367 | 2-33 |  |
| Paroscientific Digiquartz Pressure Sensor | 58808 | 0-1 |  |
| Paroscientific Digiquartz Pressure Sensor | 50619 | 2-33 |  |
| Sea-Bird SBE3plus Temperature Sensor | 1075 | 1-33 | primary |
| Sea-Bird SBE3plus Temperature Sensor | 1609 | 1-33 | secondary |
| Sea-Bird SBE4C Conductivity Sensor | 1346 | 1-33 | primary |
| Sea-Bird SBE4C Conductivity Sensor | 1347 | 1-33 | secondary |
| Sea-Bird SBE13Y YSI Dissolved Oxygen Sensor | 381 | 1-33 | primary |
| Sea-Bird SBE13Y YSI Dissolved Oxygen Sensor | 384 | 1-33 | secondary |
| Sea-Bird SBE5T Pump | 1211 | 1-33 | primary |
| Sea-Bird SBE5T Pump | 1072 | 1-33 | secondary |
| Simrad 807 Altimeter | AOML | 1-33 | range 280m |
| AOML Pinger | 7000-1 | 1-33 |  |
| RDI LADCP - 300 kHz Workhorse | Doug Wilson | 1-33 | Downward |
| RDI LADCP - 300 kHz Workhorse | UM | 1-33 | Downward |

## Conductivity

The flow-through conductivity-sensing element is a glass tube (cell) with three platinum electrodes (Seabird model SBE 4).  The resistance measured between the center electrode and the end electrode pair is determined by the cell geometry and the specific conductance of the fluid within the cell, and controls the output frequency of a Wein Bridge circuit.  The sensor has a frequency output of approximately 3 to 12 kHz corresponding to conductivity from 0 to 7 Siemens/meter (0 to 70 mmho/cm).  The SBE 4 has a typical accuracy/stability of 0.0003 S/m and resolution of 0.00004 S/m at 24 scans per second.

Table 4.5 – Calibration coefficients for the conductivity sensors.

| s/n 1346 | s/n 1347 |
| --- | --- |
| November 14, 2000 | November 14, 2000 |
| g = -4.07585521e+00 | g = -3.71068959e+00 |
| h = 5.36912871e-01 | h = 4.88160793e-01 |
| I = -4.9006457e-05 | i =- 1.53920017e-04 |
| j = 3.37480389e-05 | j = 3.64058142e-05 |
| ctcor = 3.2500e-06 | ctcor = 3.2500e-06 |
| cpcor = -6.957e-08 | cpcor = -7.7600e-08 |

Two conductivity sensors were used during ABACO-01, serial numbers (s/n) 1346 and 1347.  Pre-cruise sensor calibrations were performed at Sea-Bird Electronics, Inc. in Bellevue, Washington on November 14, 2000.  The coefficients show in Table 4.5 were entered into Seasave using the configuration file.

Conductivity calibration certificates show an equation containing the appropriate pressure-dependent correction term to account for the effect of hydrostatic loading (pressure) on the conductivity cell:

C(Siemensmeter)=g+h⋅f2+i⋅f3+j⋅f410⋅1+ctcor⋅t+cpcor⋅p

where g, h, i, j, ctcor, and cpcor are the calibrations coefficients shown above, f is the instrument frequency (kHz), t is the water temperature (degrees Celsius), and p is the water pressure (dbar).  SEASAVE automatically implements this equation.

## Temperature

The temperature-sensing element is a glass-coated thermistor bead, pressure protected by a stainless steel tube.  The sensor output frequency ranges from 5-13 kHz corresponding to temperature from –5 to 35 C.  The output frequency is inversely proportional to the square root of the thermistor resistance, which controls the output of a patented Wein Bridge circuit.  The thermistor resistance is exponentially related to temperature.  The SBE 3 thermometer has a typical accuracy/stability of 0.004C per year and resolution of 0.0003 C at 24 samples per second.  The SBE 3 thermometer has a fast response time of 0.070 seconds.

The two temperature sensors used during ABACO-01 were s/n 1075 and 1609.  Pre-cruise sensor calibrations were performed at Sea-Bird Electronics, Inc. in Bellevue, Washington on November 14, 2000.  The following (Table 4.6) coefficients were entered into SEASAVE using the configuration file:

Table 4.6 - Calibration coefficients for the temperature sensors.

| s/n 1075 | s/n 1609 |
| --- | --- |
| November 14, 2000 | November 14, 2000 |
| g = 4.81093723e-03 | g = 4.86579329e-03 |
| h = 6.68988344e-04 | h = 6.79748362e-04 |
| I = 2.51627419e-05 | i = 2.61064749e-05 |
| j = 1.99051925e-06 | j = 2.00700808e-06 |
| f0 = 1000.0 | f0 = 1000.0 |

Temperature (ITS-90) is computed according to:

T(C)=1g+h⋅lnf0f+i⋅ln2f0f+j⋅ln3f0f−273.15

where g, h, i, j and f0 are the calibration coefficients above and f is the instrument frequency (kHz).  SEASAVE automatically implements this equation and converts between ITS-90 and IPTS-68 temperature scales as desired.

## Pressure

The Paroscientific series 4000 Digiquartz high pressure transducer uses a quartz crystal resonator whose frequency of oscillation varies with pressure induced stress measuring changes in pressure as small as 0.01 parts per million with an absolute range of 0 to 10,000 psia (0 to 6885 dbar).  Repeatability, hysteresis and pressure conformance are 0.002% FS.  The nominal pressure frequency (0 to full scale) is 34 to 38 kHz.  The nominal temperature frequency is 172 kHz + 50 ppm/C.

The pressure sensor utilized during ABACO-01 was s/n 50619.  Pre-cruise sensor calibrations were performed at Sea-Bird Electronics, Inc. in Bellevue, Washington on April 5, 1994.  The coefficients (Table 4.7) were entered into SEASAVE using the configuration file.

Pressure coefficients are first formulated into:

c=c1+c2⋅U+c3⋅U2

d=d1+d2⋅U

t0=t1+t2⋅U+t3⋅U2+t4⋅U3+t5⋅U4

Where U is temperature in degrees Celsius.  Pressure is computed according to:

P(psia)=c⋅1−t02t⋅1−d⋅1−t02t

Where t is pressure period (s).  SEASAVE automatically implements this equation.

Table 4.7 - Calibration coefficients for the pressure sensor.

s/n 50619

April 5, 1994

c1 = -4.010525e+04

c2 = 6.815307e-01

c3 = 1.288650e-02

d1 = 3.974700e-02

d2 = 0.0

t1 = 3.029263e+01

t2 = -5.273358e-05

t3 = 4.378350e-06

t4 = 2.065070e-09

t5 = 0.0

## Dissolved Oxygen

The SBE 13Y dissolved oxygen sensor Yellow Springs Instrument (YSI) uses a polarographic type element.  Oxygen sensors determine the dissolved oxygen concentration by counting the number of oxygen molecules per second (flux) that diffuse through a membrane.  By knowing the flux of oxygen and the geometry of the diffusion path, the concentration of oxygen can be computed.  The permeability of the membrane to oxygen is a function of temperature and ambient pressure.  The interface electronics output voltages proportional to membrane current (oxygen current) and membrane temperature (oxygen temperature).  Oxygen temperature is used for internal temperature compensation.  Initial computation of dissolved oxygen in engineering units is done in the software.  The range for dissolved oxygen is 0 to 670 mol/kg; nominal accuracy is 4.45 mol/kg; resolution is 0.4 mol/kg.  Response times are roughly 2 seconds at 25C and 5 seconds at 0C.

Oxygen sensors 130381 and 130364 were used during ABACO-01. The oxygen calibrations (Table 4.8) were entered into SEASAVE using the configuration file.

The use of these constants in linear equations of the form I = mV + b and T = kV + c yield sensor membrane current and temperature (with maximum error of about 0.5C) as a function of sensor output voltage.

Dissolved oxygen concentration is calculated according to:

O(mll)=Soc⋅Oc+tau⋅dOcdt+Boc⋅etcor⋅T+wt.T0−T+pcor⋅P⋅OXSAT(T,S)

Where Soc, Boc, wt, tcor, pcor, and tau are the calibration coefficients above and Oc is the instrument current (µA) and dOc/dt (µA/s) is the slope of the current. T, S and P are the temperature, salinity and pressure measured by the CTD and OXSAT is the oxygen saturation value calculated according to:

| A1 = -173.4292 | A2 = 249.6339 | A3 = 143.3483 | A4 = -21.8492 |
| --- | --- | --- | --- |
| B1 = -0.033096 | B2= 0.014259 | B3 = -0.00170 |  |

OXSAT(θ,S)=expA1+A2⋅100θ+A3⋅lnθ100+A4⋅θ100+S⋅B1+B2⋅θ100+B3⋅θ1002

Where  is the absolute temperature (K). SEASAVE automatically implements this equation.

Table 4.8 - Calibration coefficients for the dissolved oxygen sensors.

| s/n 130381 | s/n 130364 |
| --- | --- |
| March 16, 2001 | March 16, 2001 |
| M = 2.4496e-07 | M = 2.4614e-07 |
| B = -2.7680e-10 | B = -5.0212e-10 |
| Soc = 2.8062 | Soc = 2.7018 |
| Boc =- 0.0123 | Boc =- 0.0113 |
| tcor =- 0.033 | tcor =- 0.033 |
| pcor = 1.50e-04 | pcor = 1.50e-04 |
| tau= 2.0 | tau= 2.0 |
| wt= 0.67 | wt= 0.67 |
| k=9.0214 | k=9.0037 |
| c=-6.7355 | c=-6.8110 |

# Data Acquisition

CTD/O2 measurements were made using a SBE 9plus CTD with dual sensor configuration.  Each set of sensors included a temperature, conductivity, and dissolved oxygen sensor.  The sets were placed as mirror images to each other mounted low in the CTD main housing with the intakes approximately 6-8 inches apart.  The TC pairs were monitored for calibration drift and shifts by examining the differences between the two pairs on each CTD and comparing CTD salinity values with bottle salinity measurements.

AOML’s SBE 9plus CTD/O2 s/n 09P10779-0363 (sampling rate 24Hz) was mounted in a 24-position frame and employed as the primary package.  Auxiliary sensors included a Lowered Acoustic Doppler Current Profiler (LADCP) and a Simrad altimeter.  Water samples were collected using a SBE bottle carousel and 10-liter Niskin bottles.  Data from the secondary sensor pair was determined to fit the sample data best and hence was used for all final data in this report.

The deck unit and CTD were powered on approximately 15 minutes before the start of each CTD cast with continuous data storage (to allow for on deck reading prior to the start of the cast and any deck unit transients to subside).  The package entered the water from the starboard side of the ship and was held within 5 meters of the surface for 1 minute in order to activate the pump.  The package was lowered at a rate of 30m/min to 100 m and 60 m/min generally to within 20 meters of the bottom, slowing on the approach.  The altimeter monitored the position of the package relative to the bottom.  The package was then brought back up to the surface stopping at intervals for the firing of the carousel and retrieval of water samples at pre-specified depths.  The CTD was typically held at the depth of each water sample for no more than 20 seconds (several rolls of the ship).  Upon completion of the cast, sensors were flushed repeatedly and stored with a dilute Triton-X solution in the tubing.  Niskin bottles were then sampled first for oxygen and then salinity.

A SBE 11plus deck unit received the data signal from the CTD.  Digitized data were forwarded to a personal computer equipped with Seasave acquisition and processing software SBEDataProc version 5.27c.  Preliminary temperature, salinity, and oxygen profiles were displayed in real time.  Raw data files were archived to SyQuest 230mb removable drives as well to compact discs.

## Data Acquisition/System Problems

The performance of the sensors was outstanding.  Problems with the configuration of the oxygen sensors occurred during the test cast.  Several issues arose with the lowered acoustic doppler current profiler (LADCP), which will be outlined in a separate report.

The 10L Niskin bottles all worked well, holding their water.  We had substantial problems initially getting the PDR and altimeter to work and hence to track the CTD package, the first station in the west end of the Florida Straits was stopped well above the anticipated bottom (from the initial depth reading at the start of the cast).

Bottle confirmation problems were extensive during the test cast.  The CTD cables were replaced and checked connectors to no avail.  After the test cast and station one we changed from CTD2 to CTD1 and had no further problems with confirmations.  Out of the Straits of Florida, the altimeter worked exceedingly well, typically targeting in on the bottom more than 350 m from the bottom.  The PDR/LSR recording improved, but could not continuously track the descent of the CTD package or the bottom depth while underway.

## Salinity Analyses

Bottle salinity analyses were performed in the ship’s temperature-controlled salinity laboratory using a Guildline Model 8400B inductive autosalinometer, and a dedicated PC.  Software allowed the user to standardize the autosalinometer.  IAPSO Standard Seawater was used as the standard.  The autosal was standardized before each case of samples was analyzed, or every 24 samples.

Duplicate samples were taken on several casts.  Bottle salinities were compared with preliminary CTD salinity values to monitor CTD conductivity cell performance and drift.  The expected precision of the autosalinometer is 0.001 PSS, with an accuracy of 0.0003 PSS.

Figure 5-4 - Bottle locations for 26.5ºN Deep Western Boundary Current section east of Abaco Island (Abaco section).

## Oxygen Analyses

Bottle oxygen analyses were also performed in the ship’s temperature-controlled salinity laboratory using a photometric automatic Winkler method titration with a Carpenter modification, and a dedicated PC.  The water samples are drawn (without air bubbles) from Niskin bottles immediately upon arrival on deck.  Manganese sulfate (or chloride) is added to the sample, followed by the addition of an alkaline sodium hydroxide-sodium iodide solution.  These solutions “pickle” the sample causing it to precipitate and react with the dissolved oxygen in the water sample.  The sample is then dissolved and photometrically titrated to an end point with a standardized sodium thiosulphate solution.  The content of oxygen value is calculated utilizing the volume of the water sample bottle and the amount of added thiosulphate.  Automated titrating systems can attain a precision of about 4.46 mol/kg [Friederich, et al., 1991].

Figure 5-5 - Bottle locations for Northwest Providence Channel section.

# Processing of CTD data

SBEDataProc consists of modular menu driven routines for acquisition, display, processing, and archiving of oceanographic data acquired with SBE equipment.  The software is designed to work with a PC with Windows operational system.  Raw data are acquired from the instruments and are stored unmodified.  The conversion module DATCNV uses the instrument configuration and pre-cruise factory calibration coefficients to create a converted engineering unit data file that is utilized by all SBEDataProc post processing modules.  Unless otherwise noted, all calibration parameters given are factory default values recommended by Sea Bird Electronics, Inc.  The following is the SBEDataProc processing module sequence and specifications used in the reduction of CTD/O2 data from this cruise:

 DATCNV converted the raw data to pressure, temperature, conductivity, oxygen current, and oxygen temperature, and computed salinity, the time rate of change of oxygen current, and preliminary oxygen.  DATCNV also extracted bottle information where scans were marked with a bottle confirmation bit during acquisition.

 ALIGNCTD aligns conductivity, temperature and oxygen in time relative to pressure to ensure that all calculations are made using measurements from the same parcel of water, which minimizes salinity spiking and density errors.  The SBE 11plus deck unit has factory settings to advance the primary conductivity cell; therefore, ALIGNCTD was not performed on this cell.  The secondary conductivity cell, however, is not advanced in the deck unit and so was advanced 0.073 seconds in the ALIGNCTD module.  Since the SBE3 temperature sensor response is fast, (0.06 seconds), it is not necessary to advance temperature relative to pressure. Oxygen sensors were not advanced

 CELLTM uses a recursive filter to remove conductivity cell thermal mass effects from measured conductivity.  Both conductivity cells are epoxy coated and therefore the thermal anomaly amplitude (alpha) and the time constant (1/beta) were 0.03 and 9.0 respectively for each sensor.

 ROSSUM created a summary of the bottle data.  Bottle position, date, and time were output automatically.  Pressure, temperature, conductivity, salinity, oxygen current, oxygen temperature, time rate of change of oxygen current, and preliminary oxygen values were averaged over a 2 second interval from 5 to 3 seconds prior to the confirm bit.  ROSSUM computed potential temperature and sigma-theta.

 WILDEDIT marked extreme outliers in the data files.  The first pass of WILDEDIT obtained an accurate estimate of the true standard deviation of the data.  The data were read in blocks of 200 scans.  Data greater than two standard deviations were flagged.  The second pass computed a standard deviation over the same 200 scans excluding the flagged values.  Values greater than 16 standard deviations were marked as bad values.

 FILTER performed a low pass filter on pressure data with a time constant of 0.15 seconds.  In order to produce no time shift, the filter first runs forward through the data file and then runs backwards through the data file.

 LOOPEDIT marks data scans where the CTD package was moving less than a minimum velocity of 0.25 m/s or traveling backwards due to ship roll.

 DERIVE was used to re-compute the time rate of change of oxygen current (dox/dt) and oxygen (mol/kg) with a time window size of 2 seconds.

 BINAVG averages the data into 1 decibar (dbar) pressure bins starting at 1 dbar with no surface bin.  The center value of the first bin was set to equal the bin size.  The bin minimum and maximum values are the center values  half the bin size.  Scans with pressure values greater than the minimum and less than or equal to the maximum were averaged.  Scans were interpolated so that a data record exists for every decibar.  The number of points averaged in each bin was added to the variables listed in the data file.

 DERIVE recomputed salinity and calculates other oceanographic parameters (e.g. density, etc.).

 SPLIT data into up-cast (pressure decreasing) and down-cast (pressure increasing) files.

 TRANS converts the binary data file into ASCII format.

# Post-Cruise Calibrations

Post cruise sensor calibrations were done at Sea-Bird Electronics, Inc. during June 2001.  Primary TC pair T1609/C1347 and secondary oxygen sensor (s/n 130364) were selected for final data reduction for all stations. In addition to the Seasoft processing modules, a group of Matlab script files called AOML/CTDCAL Toolbox were used. These scripts were based in earlier work of different groups as well in modern statistical tools.

Table 7.9 - Calibration coefficients for the temperature sensors

| s/n 1075 | s/n 1609 |
| --- | --- |
| June 19, 2001 | June 19, 2001 |
| g = 4.81077385e-03 | g = 4.86584016e-03 |
| h = 6.68822271e-04 | h = 6.79809001e-04 |
| I = 2.51066059e-05 | i = 2.61315502e-05 |
| j = 1.98467480e-06 | j = 2.01051410e-06 |
| f0 = 1000.0 | f0 = 1000.0 |

Table 7.10 - Calibration coefficients for the conductivity sensors

| s/n 1346 | s/n 1347 |
| --- | --- |
| June 19, 2001 | June 19, 2001 |
| g = -4.0734695e+00 | g = -3.70629517e+00 |
| h = 5.35702670e-01 | h = 4.87122911e-01 |
| i = 1.47891074e-04 | i =-5.16052716e-06 |
| j = 2.67641211e-05 | j = 3.06056848e-05 |
| ctcor = 3.2500e-06 | ctcor = 3.2500e-06 |
| cpcor = -6.957e-08 | cpcor = -7.7600e-08 |

AOML/CTDCAL Toolbox covers all the steps of the CTD data processing from the preliminary comparisons between sensors or with bottle samples to data reductions and final sensors calibrations.

## CTD Data Processing

By using the post cruise sensors calibrations; time drifts were estimated for the temperature and conductivity sensors (for estimated time drifts see the appropriate sections below). The processing module sequence used at sea is done again to include the time drifts as well the pressure correction. After this step the following Matlab scripts based on PMEL programs are applied to the CTD data:

 FILL_SURFACE was used to copy the first good value of salinity; potential temperature, oxygen and oxygen current back to the surface. The program then calculated temperature and conductivity, and zeroed doc/dt of oxygen current for those records.

 DESPIKE1 removed spikes from primary oxygen current and oxygen temperature data, as well as removing spikes from the primary conductivity sensor.  Data were linearly interpolated over de-spiked records.  Conductivity was back calculated, and sigma-theta and potential temperature were recomputed for the interpolated records.

 DESPIKE2 removed spikes from secondary sensors in the same method as DESPIKE1.

 Package slowdown and reversals due to ship roll can move mixed water in tow in front of the CTD sensors.  This mixture can create artificial density inversions and other artifacts.  In addition to SEASOFT module LOOPEDIT, PMEL program DELOOP computed values of density locally referenced between every 1 dbar of pressure to compute N2 = (- g/p) (dp/dz) and linearly interpolated measured parameters over those records where N2  -1.0e –05s-2.

## Pressure

Pressure sensor calibration coefficients derived from the pre-cruise calibrations were applied to raw pressure data during each cast.  On deck pressures were constantly monitored throughout the cruise and show an average value of 1 dbar, which was applied as and offset in the configuration file, and a variability of 0.1 dbar demonstrating that no other adjustments will be required.

Figure 7-6 – Pressure recorded by the CTD on deck before cast

## Temperature

Temperature sensor calibration coefficients derived from the pre-cruise calibrations were applied to raw primary and secondary temperature data during each cast. Usually the temperature sensors (SBE 3) drift by changing the offset. In order to correct this drift, the calibration coefficients from the post cruise calibration is necessary. Using the post-cruise calibration coefficients and the pre-cruise calibration data, a residual over the calibration temperature range is calculated. The mean residual is divided by the number of days since the pre-cruise calibration. This number is the offset per day. The offset per day is multiplied by the number of days between pre-cruise calibration and the day the data was collected to get the offset that should be entered into configuration file. Based on the post and pre calibrations time drifts were estimated for the sensors s/n 1075 and 1606 and were applied to the original configuration files.

Table 7.11 – Time drift correction for the temperature sensors

| sensor s/n | temperature offset (oC) | time interval between calibrations (days) | offset/day (oC/day) | time interval between pre-cal and cruise (days) | offset (oC) |
| --- | --- | --- | --- | --- | --- |
| 1075 | 0.00072 | 216 | 3.33e-6 | 163-174 | 5.60e-4 |
| 1609 | 0.00020 | 216 | 9.26e-7 | 163-174 | 1.56e-4 |

Data accuracy, reproducibility and stability were examined by tabulating the difference between the two different temperature sensors over a range of pressures (bottle trip locations) for each cast. These comparisons are summarized in Figure 7-7.  The median difference between the sensors for all data (below 500 dbar) was -0.00007 oC (-0.00008 oC) for all stations with one standard deviation of 0.0005 oC (0.0003 oC).

Figure 7-7 – Temperature differences between sensors by station number for p > 500 dbar.

Figure 7-8 - Temperature differences between sensors at each bottle stop plotted vs. pressure.

## Conductivity

The conductivity sensors (SBE 4) usually lose sensitivity as they drift, that way the drift takes a form of a slope. In order to correct this drift, the calibration coefficients from the post cruise calibration is necessary. By using the post-cruise calibration data and pre cruise calibrations coefficients together with the post cruise calibration coefficients a “postslope” is calculated. This postslope is then adjusted accordingly with the time interval between the calibrations and between the pre cruise calibration and the cruise period. Based on the post and pre calibrations time drifts were estimated for the sensors s/n 1346 and 1347 and were applied to the original configuration files.

Table 7.12 - Time drift correction for the conductivity sensors

| sensor s/n | slope correction | time interval between calibrations (days) | time interval between pre-cal and cruise (days) |  |  | New slope |
| --- | --- | --- | --- | --- | --- | --- |
| 1346 | 0.999780 | 216 |  | 163-174 | 1.000171 |  |
| 1347 | 0.999434 | 216 |  | 163-174 | 1.000441 |  |

Comparisons between the primary and secondary sensors and between each of the sensors to conductivity calculated from bottle salinities were evaluated and the secondary pair was determined to contain the lowest residuals.  The secondary sensor pair was thus used for all final data values. Based on the differences between the samples and the CTD sensor outliers were removed and initially 96.5% of the samples were kept to perform the calculations.

Figure 7-9 - Bottle - Uncalibrated CTD salinity differences plotted against station number. The crosses are data points above 500 dbar and the stars are the data points below 500 dbar. The average values for each group by station number are also plotted (squares and triangles respectively).

Despite the large variability of the data, in particular below 500 dbar, the samples of the first 6 casts were used for the final data reduction. Note also that the first six CTD stations were in the Florida Straits and Northwest Providence Channel where bottom depths do not exceed 800 m. The AOML/CTDCAL Toolbox automatically applies a quality control to the data based on comparison with a normal distribution. After these procedures 605 data points (91.8%) were used in the final calculations.

In order to calibrate the CTD conductivity data against the sample conductivity we assume a constant additive correction (offset), multiplicative correction (slope), time drift correction (represented by station number) and where needed, a linear pressure-dependent term. In this way the function to be minimized is

Cbottle−m∗CCTD+b+p1∗station+pcor∗P

where Cbottle is bottle conductivity (S/m), CCTD is pre-cruise calibrated CTD conductivity (S/m), m is the conductivity slope, b is the offset (S/m), P is the pressure, pcor is the pressure correction coefficient, station is the station number and p1 is the polynomial coefficient.

Figure 7-10 - Bottle - Uncalibrated CTD salinity differences plotted against pressure. The crosses are data points above 500 dbar and the stars are the data points below 500 dbar.

Figure 7-11 - Bottle - Calibrated CTD salinity differences plotted against station number

The coefficients estimated by the equation above were then applied to the CTD conductivities and the final results show a residual of 6.5.10-7 psu (9.2.10-5 psu for the data below 500 dbar) and a standard deviation of 0.0017 psu (0.0011 psu for the data below 500dbar). Also 85.9% of the residuals for the data are within the confidence limits determined by the WOCE (± 0.002 psu) and this number increase to 93.4% if we consider only the data below 500 dbar.

Figure 7-12 - Bottle - Calibrated CTD salinity differences plotted against pressure.

Figure 7-13 - Bottle - Calibrated CTD salinity differences plotted against station number (data below 500 dbar).

Figure 7-14 - Bottle - Calibrated CTD salinity differences plotted against pressure (data below 500 dbar).

Table 7.13 – Final statistics of the conductivity calibration.  “Raw” values and “QC” values represent differences between uncalibrated CTD measurements and all unedited bottle values (Raw) and quality controlled (QC) bottle values.  “Calibrated” statistics are for final processed and quality controlled CTD and bottle values.  Note the QC bottle values are for large statistical outliers and represent only the first pass of QC for the bottle files.  A small number of additional bottles are flagged as questionable during the CTD calibration process (hence the % data used is slightly lower for fully calibrated data).

| Data | % data used | Residual (all) | Standard deviation (all) | Residual (>500dbar) | Standard deviation (>500dbar) | Residual (>3000dbar) | Standard deviation (>3000dbar) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Raw | 100 | -0.1320 | 0.1874 | -0.0221 | 0.0298 | -0.0266 | 0.0466 |
| QC | 92.4 | -0.0206 | 0.0028 | -0.0219 | 0.0018 | -0.0229 | 0.0008 |
| Calibrated | 91.8 | 6.5e-7 | 0.0017 | -9.2e-5 | 0.0011 | 1.7e-5 | 0.0008 |

A final verification about the quality of the data was made by comparing the results of this cruise with some historical data available at the location of the Northwest Providence Channel section (Figure 7-15) as well at the location of the Abaco section (Figure 7-16 and Figure 7-17). Water mass properties are very stable, specially for deeper layers of the ocean, that way by comparing these values we can have a very good estimative of the quality of these data.

Figure 7-15 –Temperature – Salinity diagram for stations located on the Northwest Providence Channel section area. The solid black lines are the data collected during this cruise; the gray dots are data from the historical database.

Figure 7-16 - Potential Temperature – Salinity diagram for stations located on the Abaco section area. The solid black lines are the data collected during this cruise; the gray dots are data from the historical database.

Figure 7-17 - Potential Temperature – Salinity diagram for stations located on the Abaco section area with emphasis on the properties of the deep water. The solid black lines are the data collected during this cruise; the gray dots are data from the historical database.

## Dissolved Oxygen

Oxygen samples collected during CTD/O2 profiles were used for calibration. Due to hysteresis problem with oxygen sensors, the CTD upcast data for each bottle stop was replaced with the corresponding processed downcast CTD/O2 data at common neutral density levels

The algorithm used for converting oxygen sensor current and probe temperature measurements as described by [Owens and Millard, 1985] requires a non-linear least squares regression technique in order to determine the best fit coefficients of the model for oxygen sensor behavior to the water sample observations.  A Matlab sub-routine called oxift.m from the AOML CTD/CAL TOOLBOX performs non-linear least squares regression using the Gauss-Newton algorithm with Levenberg-Marquardt modifications for global convergence. This algorithm is independent of the first coefficients guess and demonstrates excellent convergence.  This oxfit.m routine includes an optional time drift term (related with the station number), allowing all stations to be calibrated without breaking into discrete groupings.  The Owens and Millard (1985) algorithm was modified as follows:

O(mll)=Soc⋅Oc+tau⋅dOcdt+Boc+p1∗sta+p2∗sta2⋅etcor⋅T+wt.T0−T+pcor⋅P⋅OXSAT(T,S),

where Oc is the instrument current (µA) and dOc/dt (µA/s) is the slope of the current  and Soc, Boc, wt, tcor, pcor, tau, p1, p2, p3 and p4 are the estimated coefficients. The T, S and P are the temperature, salinity and pressure measured by the CTD, sta is the station number and OXSAT is the oxygen saturation (see section 4.4 Dissolved Oxygen).

Figure 7-18 - Bottle - Uncalibrated CTD oxygen differences plotted against pressure. The crosses are data points above 500 dbar and the stars are the data points below 500 dbar. The average values for each group by station number are also plotted (squares and triangles respectively).

A comparison between the primary and secondary sensors and between each of the sensors to bottle oxygen were evaluated and the primary sensor was chose to perform the calibration. Based on the differences between the samples and the CTD sensor outliers were removed and initially 97% of the samples were kept to perform the calculations.

Despite the large variability of the data, in particular below 500 dbar, the samples of the first 8 casts were used for the final data reduction. Note that the first six stations were in the Florida Straits and Northwest Providence Channel (where bottom depths do not exceed 800m).  Stations 7 and 8 were also in shallow water (less than about 1000 m depth).  Also, analogous to the conductivity, AOML/CTDCAL Toolbox automatically applies a quality control to the data based on comparison with a normal distribution. After these procedures 616 data points (94.3%) were used in the final calculations.

Figure 7-19 - Bottle - Uncalibrated CTD oxygen differences plotted against pressure. The crosses are data points above 500 dbar and the stars are the data points below 500 dbar.

Figure 7-20 - Bottle - Calibrated CTD oxygen differences plotted against station number

By minimizing the differences between the oxygen samples and the CTD oxygen estimated from the equation described on this section, new coefficients were calculated and then applied to the CTD original data. The residual is –0.02 μmol/kg (–0.14 μmol/kg for the data below 500 dbar) and the standard deviation 2.75 μmol/kg (–1.81 μmol/kg for the data below 500 dbar). ). Also 68% of the residuals for the data are within the confidence limits determined by the WOCE (± 1% of the dissolved oxygen measured) and this number increase to 82% if we consider only the data below 500 dbar.

Figure 7-21 - Bottle - Calibrated CTD oxygen differences plotted against pressure

Figure 7-22 - Bottle - Calibrated CTD oxygen differences plotted against station number for data points below 500 dbar.

Figure 7-23 - Bottle - Calibrated CTD oxygen differences plotted against station number (data points below 500 dbar).

Table 7.14 – Final Statistics of the oxygen calibration (values in μmol/kg).  “Raw” values and “QC” values represent differences between uncalibrated CTD measurements and all unedited bottle values (Raw) and quality controlled (QC) bottle values.  “Calibrated” statistics are for final processed and quality controlled CTD and bottle values.  Note the QC bottle values are for large statistical outliers and represent only the first pass of QC for the bottle files.  A small number of additional bottles are flagged as questionable during the CTD calibration process (hence the % data used is slightly lower for fully calibrated data).

| Data | % data used | Residual (all) | Standard deviation (all) | Residual (>500dbar) | Standard deviation (>500dbar) |
| --- | --- | --- | --- | --- | --- |
| Raw | 100 | 6.18 | 6.68 | 3.87 | 5.52 |
| QC | 94.3 | 6.07 | 5.52 | 3.97 | 4.51 |
| Calibrated | 93.2 | -0.02 | 2.75 | -0.14 | 1.81 |

A final verification about the quality of the data, like in the salinity data, was made by comparing the results of this cruise with some historical data available at the location of the Abaco section (Figure 7-24 and Figure 7-25), unfortunately no dissolved oxygen data were available in the database in the area of the Northwest Providence Channel section. Again by investigating water mass properties, particularly for deeper layers of the ocean, we can have an estimative of the quality of these data.

Figure 7-24 - Potential Temperature – Dissolved Oxygen diagram for stations located on the Abaco section area. The solid black lines are the data collected during this cruise; the gray dots are data from the historical database.

Figure 7-25 - Potential Temperature – Dissolved Oxygen diagram for stations located on the Abaco section area with emphasis on the properties of the deep water. The solid black lines are the data collected during this cruise; the gray dots are data from the historical database.

# Final  CTD Data Presentation

AOML/CTDCAL Toolbox processes finalized CTD/O2 data and computes ITS-90 temperature, ITS-90 potential temperature and dynamic height.  The final calibrated data files were used to produce the tables and station profile plots presented in Appendix B for each CTD station.  The table on the left is in “standard depths” and its corresponding profile plot is shown on the facing page. Niskin bottle depths are presented on the right side of the profile plot.  Bottle salinity and oxygen values are plotted as points in the three smaller plots.

Vertical sections of potential temperature, CTD salinity, neutral density, and CTD oxygen are contoured with pressure as the vertical axis and, for Abaco Sections longitude as horizontal axis (Figure 8-26 to Figure 8-29). Nominal vertical exaggerations are 100:1 below 1000 dbar (lower panels) and 40:1 above 1000 dbar (upper panels). For the Northwest Providence Channel Sections latitude is used as horizontal axis (Figure 8-30 to Figure 8-33)

Post-cruise calibrations were applied to CTD data associated with bottle data using Matlab sub-routines (apply_calibration.m). WOCE quality flags were appended to bottle data records. “Bad values” (WOCE quality control value = 4) were flagged if they bottle samples failed the initial quality control and were not used for the calibration (which meant they typically fell outside 5 standard deviations of the difference between samples and uncalibrated CTD values).  Questionable flags (WOCE quality control value = 3) were defined by using the value of 2.8 times the standard deviation of the difference between calibrated CTD values and bottle samples.

Figure 8-26 – Potential Temperature (oC) section for the Abaco Section. Contour intervals are 0.1oC from 1-2 oC, 0.2oC from 2-3oC, 0.5oC from 3-5oC and 1oC for 5-35oC.  Dashed vertical lines are the CTD station locations.

Figure 8-27 – Salinity (PSS 78) section for the Abaco section. Contour intervals are 0.01 from 34-35, 0.05 from 35-35.1and 0.1 from 35.1-38.  Dashed vertical lines are the CTD station locations.

Figure 8-28 – Dissolved Oxygen (μmol/kg) section for the Abaco Section. Contour intervals are 10 μmol/kg from 100-300 μmol/kg in the upper panel. In the lower panel, contours are 20 μmol/kg from 100-220 μmol/kg and 5 μmol/kg form 220-255 μmol/kg and  2 μmol/kg from 256-270 μmol/kg.  Dashed vertical lines are the CTD station locations.

Figure 8-29 – Neutral density (kg/m3) section for the Abaco Section. Contour intervals are 0.2 kg/m3 for 24-26 kg/m3, 0.1 kg/m3 for 26-28 kg/m3 in the upper panel. In the lower panel, contour intervals are 0.05 kg/m3 for 27-27.8 kg/m3 and 0.01 kg/m3 to 27.8 to 29 kg/m3.   Dashed vertical lines are the CTD station locations.

Figure 8-30 – Potential Temperature (oC) section for the Northwest Providence Channel Section. Contour intervals are 0.1oC from 1-2 oC, 0.2oC from 2-3oC, 0.5oC from 3-5oC and 1oC for 5-35oC. Dashed vertical lines are the CTD station locations.

Figure 8-31 – Salinity (PSS 78) section for the Northwest Providence Channel section. Contour intervals are 0.01 from 34-35, 0.05 from 35-35.1and 0.1 from 35.1-38.  Dashed vertical lines are the CTD station locations.

Figure 8-32 – Dissolved Oxygen (μmol/kg) section for the Northwest Providence Channel Section. Contour intervals are 10 μmol/kg. Dashed vertical lines are the CTD station locations.

Figure 8-33 – Neutral density (kg/m3) section for the Northwest Providence Channel Section. Contour intervals are 0.2 kg/m3 for 24-26 kg/m3, 0.1 kg/m3 for 26-28 kg/m3. Dashed vertical lines are the CTD station locations.

# Acknowledgements

The assistance of the captain and the crew of the R/V Oceanus are gratefully acknowledged. This research was carried out [in part] under the auspices of the Cooperative Institute for Marine and Atmospheric Studies (CIMAS), a Joint Institute of the University of Miami and the National Oceanic and Atmospheric Administration (NOAA).  The National Science Foundation and the National Oceanic and Atmospheric Administration provided funding for the Deep Western Current Climate Time Series. This cruise was sponsored by NOAA's Climate Program Office.

# References

Benson, B. B., and D. Krause, Jr. (1984), The Concentration and Isotopic Fractionation of Oxygen Dissolved in Freshwater and Seawater in Equilibrium with the Atmosphere, Limnology and Oceanography, 29, 620-632.

Carpenter, J. H. (1965a), The accuracy of the Winkler method for dissolved oxygen analysis, Limnology and Oceanography, 10, 135-140.

Carpenter, J. H. (1965b), The Chesapeake Bay Institute Technique for the Winkler dissolved oxygen method, Limnology and Oceanography, 10, 141-143.

Friederich, G., et al. (1991), An Easy-to-Construct Automated Winkler Titration System, Monterey Bay Aquarium Research Institute Technical Report, 91, 31.

Kawano, T., et al. (2006), The latest batch-to-batch difference table of standard seawater and its application to the WOCE onetime sections, J. Oceanogr., 62, 777-792.

Latif, M., and T. P. Barnett (1996), Decadal climate variability over the North Pacific and North America: dynamics and predictability, Journal of Climate, Boston, MA, 9, 2407-2423.

Molinari, R. L., et al. (1998), The arrival of recently formed Labrador Sea Water in the Deep Western Boundary Current at 26.5 degree N, Geophysical Research Letters, 25, 2249-2252.

Owens, W., and R. C. Millard, Jr. (1985), New algorithm for CTD oxygen calibration, Journal of Physical Oceanography, Boston, 15, 621-631.

Vaughan, S. L., and R. L. Molinari (1997), Temperature and salinity variability in deep western boundary current, Journal of Physical Oceanography, Boston, MA, 27, 749-761.

# Appendix A - WOCE Summary File

(This page is intentionally left blank)

| SHIP/CRS | WOCE |  |  | CAST | CAST | UTC | EVENT | LATITUDE |  |  | LONGITUDE |  |  |  | UNC | HT ABOVE | WIRE | MAX | NO. OF | PARAMETERS | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EXPOCODE | SECT | STNNBR | CASTNO | TYPE | DATE | TIME | CODE | DEG | MIN |  | DEG | MIN |  | NAV | DEPTH | BOTTOM | OUT | PRESS | BOTTTLES |  |  |
| OC365/9 | AB01 | 1 | 1 | ROS | 42601 | 2019 | BE | 27 | 0.716 | N | 79 | 55.351 | W | GPS |  |  |  |  |  |  | CTD2 NO CNFRM BTLS 2 5 8 |
| OC365/9 | AB01 | 1 | 1 | ROS | 42601 | 2031 | BO | 27 | 1.442 | N | 79 | 55.159 | W | GPS | 157 |  |  | 139.6 | 9 | 1,2 |  |
| OC365/9 | AB01 | 1 | 1 | ROS | 42601 | 2048 | EN | 27 | 2.730 | N | 79 | 54.680 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 2 | 2 | ROS | 42601 | 1125 | BE | 26 | 26.033 | N | 78 | 40.007 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 2 | 2 | ROS | 42601 | 1200 | BO | 26 | 26.507 | N | 78 | 40.104 | W | GPS | 741 |  |  | 746.4 | 12 | 1,2 | CTD2 BTL 3 NO FIRE |
| OC365/9 | AB01 | 2 | 2 | ROS | 42601 | 1208 | EN | 26 | 26.610 | N | 78 | 40.102 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 3 | 3 | ROS | 42701 | 1301 | BE | 26 | 19.887 | N | 78 | 42.841 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 3 | 3 | ROS | 42701 | 1321 | BO | 26 | 19.519 | N | 78 | 42.786 | W | GPS | 656 |  |  | 660.4 | 9 | 1,2 | CTD1 PSBL LEAK STOPPER ON SFC BOTTLE |
| OC365/9 | AB01 | 3 | 3 | ROS | 42701 | 1339 | EN | 26 | 19.033 | N | 78 | 42.974 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 4 | 4 | ROS | 42701 | 1417 | BE | 26 | 14.705 | N | 78 | 46.181 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 4 | 4 | ROS | 42701 | 1430 | BO | 26 | 14.392 | N | 78 | 46.349 | W | GPS | 507 |  |  | 510.3 | 10 |  |  |
| OC365/9 | AB01 | 4 | 4 | ROS | 42701 | 1451 | EN | 26 | 13.973 | N | 78 | 46.595 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 5 | 5 | ROS | 42701 | 1533 | BE | 26 | 9.837 | N | 78 | 48.242 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 5 | 5 | ROS | 42701 | 1544 | BO | 26 | 9.667 | N | 78 | 48.358 | W | GPS | 443 |  |  | 446.1 | 10 | 1,2 |  |
| OC365/9 | AB01 | 5 | 5 | ROS | 42701 | 1600 | EN | 26 | 9.837 | N | 78 | 48.292 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 6 | 6 | ROS | 42701 | 1652 | BE | 26 | 3.926 | N | 78 | 51.319 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 6 | 6 | ROS | 42701 | 1659 | BO | 26 | 3.897 | N | 78 | 51.467 | W | GPS | 283 |  |  | 284.6 | 8 | 1,2 |  |
| OC365/9 | AB01 | 6 | 6 | ROS | 42801 | 1708 | EN | 26 | 3.846 | N | 78 | 51.656 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 7 | 7 | ROS | 42801 | 616 | BE | 26 | 29.932 | N | 76 | 54.704 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 7 | 7 | ROS | 42801 | 649 | BO | 26 | 30.075 | N | 76 | 54.855 | W | GPS | 978 | 8 | 970 | 978.0 | 13 | 1,2 |  |
| OC365/9 | AB01 | 7 | 7 | ROS | 42801 | 715 | EN | 26 | 30.183 | N | 76 | 54.720 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 8 | 8 | ROS | 42801 | 1015 | BE | 26 | 30.646 | N | 76 | 50.453 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 8 | 8 | ROS | 42801 | 1048 | BO | 26 | 30.724 | N | 76 | 50.585 | W | GPS | 1005 | 8.6 | 1000 | 997.7 | 12 | 1,2 |  |
| OC365/9 | AB01 | 8 | 8 | ROS | 42801 | 1116 | EN | 26 | 30.853 | N | 76 | 50.668 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 9 | 9 | ROS | 42801 | 1206 | BE | 26 | 30.039 | N | 76 | 44.046 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 9 | 9 | ROS | 42801 | 1319 | BO | 26 | 30.394 | N | 76 | 45.219 | W | GPS | 3913 | 10.1 | 4102 | 3903.7 | 21 | 1,2 |  |
| OC365/9 | AB01 | 9 | 9 | ROS | 42801 | 1439 | EN | 26 | 31.186 | N | 76 | 46.681 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 10 | 10 | ROS | 42801 | 1546 | BE | 26 | 30.114 | N | 76 | 37.839 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 10 | 10 | ROS | 42801 | 1709 | BO | 26 | 30.800 | N | 76 | 36.867 | W | GPS | 4729 | 7.3 | 4767 | 4788.6 | 24 | 1,2 |  |
| OC365/9 | AB01 | 10 | 10 | ROS | 42801 | 1833 | EN | 26 | 31.862 | N | 76 | 37.968 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 11 | 11 | ROS | 42801 | 2214 | BE | 26 | 30.093 | N | 76 | 28.922 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 11 | 11 | ROS | 42801 | 2355 | BO | 26 | 31.624 | N | 76 | 29.541 | W | GPS | 4842 | 12.1 | 5074 | 4925.3 | 24 | 1,2 |  |
| OC365/9 | AB01 | 11 | 11 | ROS | 42801 | 129 | EN | 26 | 33.276 | N | 76 | 30.156 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 12 | 12 | ROS | 42901 | 242 | BE | 26 | 30.072 | N | 76 | 18.022 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 12 | 12 | ROS | 42901 | 420 | BO | 26 | 31.738 | N | 76 | 18.977 | W | GPS | 4828 | 12.6 | 5055 | 4913.0 | 24 | 1,2 |  |
| OC365/9 | AB01 | 12 | 12 | ROS | 42901 | 552 | EN | 26 | 33.434 | N | 76 | 19.982 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 13 | 13 | ROS | 42901 | 717 | BE | 26 | 30.081 | N | 76 | 7.040 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 13 | 13 | ROS | 42901 | 853 | BO | 26 | 31.704 | N | 76 | 7.637 | W | GPS | 4807 | 12.8 | 5320 | 4889.5 | 24 | 1,2 |  |
| OC365/9 | AB01 | 13 | 13 | ROS | 42901 | 1044 | EN | 26 | 33.432 | N | 76 | 8.783 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 14 | 14 | ROS | 42901 | 1205 | BE | 26 | 30.073 | N | 75 | 54.999 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 14 | 14 | ROS | 42901 | 1343 | BO | 26 | 31.841 | N | 75 | 55.657 | W | GPS | 4748 | 10.3 | 5473 | 4834.0 | 24 | 1,2 | 2 FIRED AT 65M |
| OC365/9 | AB01 | 14 | 14 | ROS | 42901 | 1524 | EN | 26 | 33.592 | N | 75 | 56.265 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 15 | 15 | ROS | 42901 | 1922 | BE | 26 | 30.303 | N | 75 | 42.373 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 15 | 15 | ROS | 42901 | 2111 | BO | 26 | 33.038 | N | 75 | 43.689 | W | GPS | 4690 | 10.1 | 5967 | 4768.9 | 24 | 1,2 |  |
| OC365/9 | AB01 | 15 | 15 | ROS | 42901 | 2306 | EN | 26 | 35.660 | N | 75 | 45.110 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 16 | 16 | ROS | 42901 | 111 | BE | 26 | 30.443 | N | 75 | 30.111 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 16 | 16 | ROS | 42901 | 237 | BO | 26 | 31.877 | N | 75 | 30.908 | W | GPS | 4683 | 9.8 | 5005 | 4762.5 | 24 | 1,2 |  |
| OC365/9 | AB01 | 16 | 16 | ROS | 42901 | 405 | EN | 26 | 33.356 | N | 75 | 31.224 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 17 | 17 | ROS | 43001 | 605 | BE | 26 | 30.051 | N | 75 | 18.138 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 17 | 17 | ROS | 43001 | 724 | BO | 26 | 30.929 | N | 75 | 18.232 | W | GPS | 4636 | 6.6 | 4697 | 4718.6 | 23 | 1,2 | BOTTLE 24 DID NOT FIRE |
| OC365/9 | AB01 | 17 | 17 | ROS | 43001 | 857 | EN | 26 | 32.133 | N | 75 | 18.448 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 18 | 18 | ROS | 43001 | 1057 | BE | 26 | 30.025 | N | 75 | 5.000 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 18 | 18 | ROS | 43001 | 1228 | BO | 26 | 31.720 | N | 75 | 5.450 | W | GPS | 4603 | 9.5 | 5011 | 4685.4 | 24 | 1,2 |  |
| OC365/9 | AB01 | 18 | 18 | ROS | 43001 | 1401 | EN | 26 | 33.080 | N | 75 | 5.218 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 19 | 19 | ROS | 43001 | 1608 | BE | 26 | 29.780 | N | 74 | 48.115 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 19 | 19 | ROS | 43001 | 1728 | BO | 26 | 30.144 | N | 74 | 47.793 | W | GPS | 4508 | 8.1 | 4605 | 4612.5 | 24 | 1,2 |  |
| OC365/9 | AB01 | 19 | 19 | ROS | 43001 | 1857 | EN | 26 | 30.291 | N | 74 | 48.206 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 20 | 20 | ROS | 43001 | 2127 | BE | 26 | 30.064 | N | 74 | 30.906 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 20 | 20 | ROS | 43001 | 2257 | BO | 26 | 30.715 | N | 74 | 31.106 | W | GPS | 4496 | 10 | 4580 | 4558.1 | 24 | 1,2 |  |
| OC365/9 | AB01 | 20 | 20 | ROS | 43001 | 32 | EN | 26 | 31.844 | N | 74 | 30.938 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 21 | 21 | ROS | 50101 | 306 | BE | 26 | 30.117 | N | 74 | 14.148 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 21 | 21 | ROS | 50101 | 433 | BO | 26 | 31.112 | N | 74 | 14.555 | W | GPS | 4536 | 6.1 | 4836 | 4608.8 | 24 | 1,2 |  |
| OC365/9 | AB01 | 21 | 21 | ROS | 50101 | 551 | EN | 26 | 31.826 | N | 74 | 13.851 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 22 | 22 | ROS | 50101 | 822 | BE | 26 | 30.031 | N | 73 | 57.988 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 22 | 22 | ROS | 50101 | 958 | BO | 26 | 31.081 | N | 73 | 57.766 | W | GPS | 4642 | 6.6 | 4827 | 4716.2 | 24 | 1,2 |  |
| OC365/9 | AB01 | 22 | 22 | ROS | 50101 | 1134 | EN | 26 | 32.157 | N | 73 | 57.404 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 23 | 23 | ROS | 50101 | 1411 | BE | 26 | 30.025 | N | 73 | 34.977 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 23 | 23 | ROS | 50101 | 1543 | BO | 26 | 31.138 | N | 73 | 35.256 | W | GPS | 4920 | 6.8 | 5035 | 4937.5 | 24 | 1,2 |  |
| OC365/9 | AB01 | 23 | 23 | ROS | 50101 | 1703 | EN | 26 | 31.951 | N | 73 | 35.750 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 24 | 24 | ROS | 50101 | 1920 | BE | 26 | 30.002 | N | 73 | 13.100 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 24 | 24 | ROS | 50101 | 2056 | BO | 26 | 31.030 | N | 73 | 14.011 | W | GPS | 5058 | 8.8 | 5202 | 5133.9 | 24 | 1,2 |  |
| OC365/9 | AB01 | 24 | 24 | ROS | 50101 | 2233 | EN | 26 | 31.860 | N | 73 | 15.267 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 25 | 25 | ROS | 50201 | 1540 | BE | 26 | 33.517 | N | 72 | 51.104 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 25 | 25 | ROS | 50201 | 1723 | BO | 26 | 34.783 | N | 72 | 51.486 | W | GPS | 5107 | 8.4 | 5298 | 5164.0 | 24 | 1,2 |  |
| OC365/9 | AB01 | 25 | 25 | ROS | 50201 | 1908 | EN | 26 | 35.925 | N | 72 | 51.873 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 26 | 26 | ROS | 50201 | 2222 | BE | 26 | 30.036 | N | 72 | 27.946 | W | GPS |  |  |  |  |  |  | BOTTLES 3 AND 23 MISFIRED |
| OC365/9 | AB01 | 26 | 26 | ROS | 50201 | 2 | BO | 26 | 30.904 | N | 72 | 27.742 | W | GPS | 5138 | 9.6 | 5262 | 5263.3 | 24 | 1,2 | BOTTLE 14 LANYARD KEPT FROM SEALING |
| OC365/9 | AB01 | 26 | 26 | ROS | 50201 | 147 | EN | 26 | 32.305 | N | 72 | 28.040 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 27 | 27 | ROS | 50301 | 422 | BE | 26 | 29.990 | N | 72 | 6.111 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 27 | 27 | ROS | 50301 | 613 | BO | 26 | 31.859 | N | 72 | 5.894 | W | GPS | 5278 | 6.8 | 5606 | 5366.0 | 24 | 1,2 |  |
| OC365/9 | AB01 | 27 | 27 | ROS | 50301 | 822 | EN | 26 | 34.004 | N | 72 | 5.727 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 28 | 28 | ROS | 50301 | 1116 | BE | 26 | 30.118 | N | 71 | 43.939 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 28 | 28 | ROS | 50301 | 1259 | BO | 26 | 31.742 | N | 71 | 43.718 | W | GPS | 5373 | 9.1 | 5622 | 5469.0 | 24 | 1,2 |  |
| OC365/9 | AB01 | 28 | 28 | ROS | 50301 | 1450 | EN | 26 | 33.419 | N | 71 | 43.224 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 29 | 29 | ROS | 50301 | 1800 | BE | 26 | 30.061 | N | 71 | 21.089 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 29 | 29 | ROS | 50301 | 1957 | BO | 26 | 32.024 | N | 71 | 20.237 | W | GPS | 5465 | 9.1 | 5853 | 5586.8 | 24 | 1,2 |  |
| OC365/9 | AB01 | 29 | 29 | ROS | 50301 | 2147 | EN | 26 | 33.899 | N | 71 | 19.098 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 30 | 30 | ROS | 50401 | 30 | BE | 26 | 30.107 | N | 71 | 0.076 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 30 | 30 | ROS | 50401 | 210 | BO | 26 | 32.248 | N | 71 | 0.536 | W | GPS | 5470 | 8.09 | 6020 | 5585.5 | 24 | 1,2 |  |
| OC365/9 | AB01 | 30 | 30 | ROS | 50401 | 414 | EN | 26 | 34.255 | N | 71 | 0.596 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 31 | 31 | ROS | 50401 | 926 | BE | 26 | 30.263 | N | 70 | 41.009 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 31 | 31 | ROS | 50401 | 1118 | BO | 26 | 32.379 | N | 70 | 40.887 | W | GPS | 5470 | 7.3 | 6088 | 5591.3 | 24 | 1,2 |  |
| OC365/9 | AB01 | 31 | 31 | ROS | 50401 | 1311 | EN | 26 | 34.191 | N | 70 | 40.057 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 32 | 32 | ROS | 50401 | 1604 | BE | 26 | 29.993 | N | 70 | 14.021 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 32 | 32 | ROS | 50401 | 1803 | BO | 26 | 31.264 | N | 70 | 11.994 | W | GPS | 5471 | 8.8 | 6194 | 5591.0 | 24 | 1,2 |  |
| OC365/9 | AB01 | 32 | 32 | ROS | 50401 | 2014 | EN | 26 | 31.231 | N | 70 | 9.419 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 33 | 33 | ROS | 50401 | 2236 | BE | 26 | 30.002 | N | 69 | 50.065 | W | GPS |  |  |  |  |  |  |  |
| OC365/9 | AB01 | 33 | 33 | ROS | 50401 | 27 | BO | 26 | 31.194 | N | 69 | 48.341 | W | GPS | 5483 | 9.1 | 5997 | 5594.2 | 24 | 1,2 |  |
| OC365/9 | AB01 | 33 | 33 | ROS | 50401 | 219 | EN | 26 | 32.309 | N | 69 | 46.618 | W | GPS |  |  |  |  |  |  |  |

# Appendix B – WOCE Bottle Summary File

(This page is intentionally left blank)

| EXPOCODE | SECT_ID | STNNBR | CASTNO | SAMPNO | BTLNBR | BTLNBR | DATE | TIME | LATITUDE | LONGITUDE | DEPTH | CTDPRS | CTDTMP | CTDSAL | CTDSAL | SALNTY | SALNTY | CTDOXY | CTDOXY | OXYGEN | OXYGEN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | FLAG |  |  |  |  |  | DBARs | ITS-90 | PSS-78 | FLAG | PSS-78 | FLAG | UMOL/KG | FLAG | UMOL/KG | FLAG |
| 31WBTSOC | AB01 | 1 | 1 | 1 | 24 | 2 | 20010426 | 2022 | 27.0158 | -79.9213 | 159 | 139.6 | 10.4301 | 35.3285 | 2 | 35.3199 | 2 | 126.3 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 1 | 1 | 2 | 1 | 2 | 20010426 | 2022 | 27.0158 | -79.9213 | 159 | 127.5 | 11.8637 | 35.5302 | 2 | 35.5321 | 2 | 132.9 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 1 | 1 | 3 | 3 | 2 | 20010426 | 2022 | 27.0158 | -79.9213 | 159 | 112 | 13.9258 | 35.8105 | 2 | 35.8263 | 3 | 135.3 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 1 | 1 | 4 | 4 | 2 | 20010426 | 2022 | 27.0158 | -79.9213 | 159 | 77.3 | 18.0814 | 36.2409 | 2 | 36.249 | 2 | 170.1 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 1 | 1 | 5 | 6 | 2 | 20010426 | 2022 | 27.0158 | -79.9213 | 159 | 49 | 23.8687 | 36.439 | 2 | 36.4411 | 2 | 224.2 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 1 | 1 | 6 | 13 | 2 | 20010426 | 2022 | 27.0158 | -79.9213 | 159 | 24.5 | 26.0002 | 36.2327 | 2 | 36.2403 | 2 | 214.4 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 2 | 1 | 1 | 24 | 2 | 20010427 | 1127 | 26.4348 | -78.6667 | 763 | 749 | 9.474 | 35.2761 | 2 | 35.2748 | 2 | 147.9 | 2 | 152 | 2 |
| 31WBTSOC | AB01 | 2 | 1 | 2 | 1 | 2 | 20010427 | 1127 | 26.4348 | -78.6667 | 763 | 649.7 | 11.4486 | 35.5002 | 2 | 35.4984 | 2 | 154 | 2 | 154.4 | 2 |
| 31WBTSOC | AB01 | 2 | 1 | 3 | 2 | 2 | 20010427 | 1127 | 26.4348 | -78.6667 | 763 | 547.8 | 13.8439 | 35.8431 | 2 | -999 | 9 | 168.6 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 2 | 1 | 4 | 3 | 2 | 20010427 | 1127 | 26.4348 | -78.6667 | 763 | 449.9 | 15.9879 | 36.1857 | 2 | 36.1833 | 2 | 182.4 | 2 | 175.6 | 2 |
| 31WBTSOC | AB01 | 2 | 1 | 5 | 4 | 2 | 20010427 | 1127 | 26.4348 | -78.6667 | 763 | 374.8 | 17.3085 | 36.4004 | 2 | 36.4019 | 2 | 192.8 | 2 | 188.1 | 2 |
| 31WBTSOC | AB01 | 2 | 1 | 6 | 5 | 2 | 20010427 | 1127 | 26.4348 | -78.6667 | 763 | 300.7 | 18.2799 | 36.5474 | 2 | 36.5477 | 2 | 196.8 | 2 | 193.6 | 2 |
| 31WBTSOC | AB01 | 2 | 1 | 7 | 6 | 2 | 20010427 | 1127 | 26.4348 | -78.6667 | 763 | 225.1 | 20.0271 | 36.7426 | 2 | 36.7448 | 2 | 193.7 | 2 | 190 | 2 |
| 31WBTSOC | AB01 | 2 | 1 | 8 | 12 | 2 | 20010427 | 1127 | 26.4348 | -78.6667 | 763 | 173.6 | 21.5201 | 36.8389 | 2 | -999 | 9 | 203.7 | 2 | 194.3 | 2 |
| 31WBTSOC | AB01 | 2 | 1 | 9 | 13 | 2 | 20010427 | 1127 | 26.4348 | -78.6667 | 763 | 124.6 | 22.959 | 36.8877 | 2 | 36.8404 | 4 | 203.4 | 2 | 196.1 | 2 |
| 31WBTSOC | AB01 | 2 | 1 | 10 | 14 | 2 | 20010427 | 1127 | 26.4348 | -78.6667 | 763 | 74 | 24.0485 | 36.5556 | 2 | 36.5756 | 4 | 205.6 | 2 | 203.1 | 2 |
| 31WBTSOC | AB01 | 2 | 1 | 11 | 15 | 2 | 20010427 | 1127 | 26.4348 | -78.6667 | 763 | 49 | 25.1356 | 36.3057 | 2 | 36.3201 | 3 | 212.7 | 2 | 205 | 2 |
| 31WBTSOC | AB01 | 2 | 1 | 12 | 16 | 2 | 20010427 | 1127 | 26.4348 | -78.6667 | 763 | 3.7 | 25.1114 | 36.3078 | 2 | 36.3086 | 2 | 209.5 | 2 | 205.6 | 2 |
| 31WBTSOC | AB01 | 3 | 1 | 1 | 24 | 2 | 20010427 | 1302 | 26.331 | -78.7138 | 691 | 676.5 | 9.0961 | 35.2636 | 2 | 35.2677 | 2 | 157.8 | 2 | 156.9 | 2 |
| 31WBTSOC | AB01 | 3 | 1 | 2 | 1 | 2 | 20010427 | 1302 | 26.331 | -78.7138 | 691 | 548.3 | 13.4185 | 35.7809 | 2 | 35.7761 | 2 | 166.9 | 2 | 156 | 2 |
| 31WBTSOC | AB01 | 3 | 1 | 3 | 2 | 2 | 20010427 | 1302 | 26.331 | -78.7138 | 691 | 451.1 | 15.7061 | 36.1389 | 2 | 36.1384 | 2 | 182.2 | 2 | 170.9 | 2 |
| 31WBTSOC | AB01 | 3 | 1 | 4 | 3 | 2 | 20010427 | 1302 | 26.331 | -78.7138 | 691 | 351.9 | 17.616 | 36.4524 | 2 | 36.4532 | 2 | 195 | 2 | 185.4 | 2 |
| 31WBTSOC | AB01 | 3 | 1 | 5 | 4 | 2 | 20010427 | 1302 | 26.331 | -78.7138 | 691 | 251 | 19.7497 | 36.7115 | 2 | 36.7108 | 2 | 194.6 | 2 | 186.1 | 2 |
| 31WBTSOC | AB01 | 3 | 1 | 6 | 5 | 2 | 20010427 | 1302 | 26.331 | -78.7138 | 691 | 174.3 | 21.791 | 36.855 | 2 | 36.8545 | 2 | 202.5 | 2 | 194.6 | 2 |
| 31WBTSOC | AB01 | 3 | 1 | 7 | 6 | 2 | 20010427 | 1302 | 26.331 | -78.7138 | 691 | 123.9 | 23.3987 | 36.9121 | 2 | 36.9138 | 2 | 203.9 | 2 | 197.5 | 2 |
| 31WBTSOC | AB01 | 3 | 1 | 8 | 12 | 2 | 20010427 | 1302 | 26.331 | -78.7138 | 691 | 48.9 | 25.2466 | 36.2959 | 2 | 36.2985 | 2 | 210.1 | 2 | 203.6 | 2 |
| 31WBTSOC | AB01 | 3 | 1 | 9 | 13 | 2 | 20010427 | 1302 | 26.331 | -78.7138 | 691 | 2.8 | 25.7322 | 36.2192 | 2 | 36.2201 | 2 | 207 | 2 | 203.5 | 3 |
| 31WBTSOC | AB01 | 4 | 1 | 1 | 24 | 2 | 20010427 | 1418 | 26.2448 | -78.7697 | 526 | 509.9 | 14.7271 | 35.9843 | 2 | 35.9869 | 2 | 165.6 | 2 | 164.3 | 2 |
| 31WBTSOC | AB01 | 4 | 1 | 2 | 1 | 2 | 20010427 | 1418 | 26.2448 | -78.7697 | 526 | 434.3 | 16.2 | 36.2214 | 2 | 36.2165 | 2 | 182.8 | 2 | 174.2 | 2 |
| 31WBTSOC | AB01 | 4 | 1 | 3 | 2 | 2 | 20010427 | 1418 | 26.2448 | -78.7697 | 526 | 359.2 | 17.4736 | 36.4317 | 2 | 36.43 | 2 | 193.4 | 2 | 183.9 | 2 |
| 31WBTSOC | AB01 | 4 | 1 | 4 | 3 | 2 | 20010427 | 1418 | 26.2448 | -78.7697 | 526 | 319.2 | 18.0227 | 36.5174 | 2 | 36.5144 | 2 | 195.8 | 2 | 186.9 | 2 |
| 31WBTSOC | AB01 | 4 | 1 | 5 | 4 | 2 | 20010427 | 1418 | 26.2448 | -78.7697 | 526 | 279.8 | 18.8647 | 36.6157 | 2 | 36.6159 | 2 | 197 | 2 | 188.6 | 2 |
| 31WBTSOC | AB01 | 4 | 1 | 6 | 5 | 2 | 20010427 | 1418 | 26.2448 | -78.7697 | 526 | 249.9 | 19.9635 | 36.7233 | 2 | 36.7247 | 2 | 195.4 | 2 | 186.2 | 2 |
| 31WBTSOC | AB01 | 4 | 1 | 7 | 6 | 2 | 20010427 | 1418 | 26.2448 | -78.7697 | 526 | 180 | 22.1968 | 36.8549 | 2 | 36.8521 | 2 | 210.5 | 2 | 200.3 | 2 |
| 31WBTSOC | AB01 | 4 | 1 | 8 | 12 | 2 | 20010427 | 1418 | 26.2448 | -78.7697 | 526 | 139.3 | 23.1781 | 36.9066 | 2 | 36.885 | 4 | 203.9 | 2 | 196.9 | 2 |
| 31WBTSOC | AB01 | 4 | 1 | 9 | 13 | 2 | 20010427 | 1418 | 26.2448 | -78.7697 | 526 | 48.9 | 25.4683 | 36.3309 | 2 | 36.3543 | 4 | 206.4 | 2 | 200.3 | 2 |
| 31WBTSOC | AB01 | 4 | 1 | 10 | 14 | 2 | 20010427 | 1418 | 26.2448 | -78.7697 | 526 | 3.1 | 25.8169 | 36.2134 | 2 | 36.2158 | 2 | 207.4 | 2 | 203.5 | 2 |
| 31WBTSOC | AB01 | 5 | 1 | 1 | 24 | 2 | 20010427 | 1533 | 26.1638 | -78.804 | 462 | 445.1 | 16.6594 | 36.2881 | 2 | 36.2842 | 2 | 169.4 | 2 | 168.8 | 2 |
| 31WBTSOC | AB01 | 5 | 1 | 2 | 1 | 2 | 20010427 | 1533 | 26.1638 | -78.804 | 462 | 415 | 17.2887 | 36.4002 | 2 | 36.3963 | 2 | 187.9 | 2 | 183.3 | 2 |
| 31WBTSOC | AB01 | 5 | 1 | 3 | 2 | 2 | 20010427 | 1533 | 26.1638 | -78.804 | 462 | 348.9 | 18.0517 | 36.5241 | 2 | 36.5201 | 2 | 195.3 | 2 | 187.5 | 2 |
| 31WBTSOC | AB01 | 5 | 1 | 4 | 3 | 2 | 20010427 | 1533 | 26.1638 | -78.804 | 462 | 299 | 18.5155 | 36.5826 | 2 | 36.5796 | 2 | 195.7 | 2 | 187.4 | 2 |
| 31WBTSOC | AB01 | 5 | 1 | 5 | 4 | 2 | 20010427 | 1533 | 26.1638 | -78.804 | 462 | 249.1 | 19.2552 | 36.6595 | 2 | 36.6587 | 2 | 195.1 | 2 | 187.2 | 2 |
| 31WBTSOC | AB01 | 5 | 1 | 6 | 5 | 2 | 20010427 | 1533 | 26.1638 | -78.804 | 462 | 198.7 | 20.8387 | 36.7873 | 2 | 36.7858 | 2 | 199.8 | 2 | 190.1 | 2 |
| 31WBTSOC | AB01 | 5 | 1 | 7 | 6 | 2 | 20010427 | 1533 | 26.1638 | -78.804 | 462 | 149.9 | 23.3493 | 36.9126 | 2 | 36.914 | 2 | 207.4 | 2 | 196.3 | 2 |
| 31WBTSOC | AB01 | 5 | 1 | 8 | 12 | 2 | 20010427 | 1533 | 26.1638 | -78.804 | 462 | 101.1 | 24.6527 | 36.4088 | 2 | 36.4369 | 4 | 203.1 | 2 | 196.1 | 2 |
| 31WBTSOC | AB01 | 5 | 1 | 9 | 13 | 2 | 20010427 | 1533 | 26.1638 | -78.804 | 462 | 49.6 | 25.7753 | 36.2218 | 2 | 36.2363 | 3 | 209.5 | 2 | 202.3 | 2 |
| 31WBTSOC | AB01 | 5 | 1 | 10 | 14 | 2 | 20010427 | 1533 | 26.1638 | -78.804 | 462 | 2.2 | 25.8041 | 36.2205 | 2 | 36.2207 | 2 | 207.1 | 2 | 202.7 | 2 |
| 31WBTSOC | AB01 | 6 | 1 | 1 | 24 | 2 | 20010427 | 1652 | 26.0653 | -78.8552 | 301 | 282.8 | 18.6941 | 36.59 | 2 | 36.5889 | 2 | 176.3 | 2 | 177 | 2 |
| 31WBTSOC | AB01 | 6 | 1 | 2 | 1 | 2 | 20010427 | 1652 | 26.0653 | -78.8552 | 301 | 247.8 | 19.275 | 36.6541 | 2 | 36.6462 | 2 | 185 | 2 | 179.8 | 2 |
| 31WBTSOC | AB01 | 6 | 1 | 3 | 2 | 2 | 20010427 | 1652 | 26.0653 | -78.8552 | 301 | 248.4 | 19.2725 | 36.6539 | 2 | 36.6488 | 2 | 185.4 | 2 | 179.6 | 2 |
| 31WBTSOC | AB01 | 6 | 1 | 4 | 3 | 2 | 20010427 | 1652 | 26.0653 | -78.8552 | 301 | 198.8 | 19.9882 | 36.7258 | 2 | 36.7241 | 2 | 194.2 | 2 | 186.8 | 2 |
| 31WBTSOC | AB01 | 6 | 1 | 5 | 4 | 2 | 20010427 | 1652 | 26.0653 | -78.8552 | 301 | 150.9 | 22.1631 | 36.8333 | 2 | 36.8289 | 2 | 196.1 | 2 | 186.1 | 2 |
| 31WBTSOC | AB01 | 6 | 1 | 6 | 5 | 2 | 20010427 | 1652 | 26.0653 | -78.8552 | 301 | 99.3 | 24.2938 | 36.4626 | 2 | 36.4975 | 4 | 215.9 | 2 | 205.7 | 2 |
| 31WBTSOC | AB01 | 6 | 1 | 7 | 6 | 2 | 20010427 | 1652 | 26.0653 | -78.8552 | 301 | 48.9 | 25.1788 | 36.3105 | 2 | 36.3143 | 2 | 213 | 2 | 205.4 | 2 |
| 31WBTSOC | AB01 | 6 | 1 | 8 | 12 | 2 | 20010427 | 1652 | 26.0653 | -78.8552 | 301 | 1.7 | 25.6711 | 36.188 | 2 | 36.1894 | 2 | 208.6 | 2 | 204.5 | 3 |
| 31WBTSOC | AB01 | 7 | 1 | 1 | 24 | 2 | 20010428 | 618 | 26.4988 | -76.9127 | 980 | 968.6 | 6.112 | 35.0837 | 2 | 35.0834 | 2 | 201.9 | 2 | 200.5 | 2 |
| 31WBTSOC | AB01 | 7 | 1 | 2 | 1 | 2 | 20010428 | 618 | 26.4988 | -76.9127 | 980 | 798.6 | 8.0739 | 35.1459 | 2 | 35.1395 | 2 | 160.2 | 2 | 158.2 | 2 |
| 31WBTSOC | AB01 | 7 | 1 | 3 | 2 | 2 | 20010428 | 618 | 26.4988 | -76.9127 | 980 | 647.7 | 11.4967 | 35.5075 | 2 | 35.5057 | 2 | 155.9 | 2 | 148.5 | 2 |
| 31WBTSOC | AB01 | 7 | 1 | 4 | 3 | 2 | 20010428 | 618 | 26.4988 | -76.9127 | 980 | 549.5 | 13.6815 | 35.8193 | 2 | 35.8138 | 2 | 167.1 | 2 | 159.3 | 2 |
| 31WBTSOC | AB01 | 7 | 1 | 5 | 4 | 2 | 20010428 | 618 | 26.4988 | -76.9127 | 980 | 448.8 | 16.9884 | 36.3438 | 2 | 36.3357 | 2 | 193.3 | 2 | 181.2 | 2 |
| 31WBTSOC | AB01 | 7 | 1 | 6 | 5 | 2 | 20010428 | 618 | 26.4988 | -76.9127 | 980 | 346.5 | 18.7792 | 36.6037 | 2 | 36.604 | 2 | 199.9 | 2 | 190.7 | 2 |
| 31WBTSOC | AB01 | 7 | 1 | 7 | 6 | 2 | 20010428 | 618 | 26.4988 | -76.9127 | 980 | 306.4 | 19.4811 | 36.6678 | 2 | 36.6694 | 2 | 197 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 7 | 1 | 8 | 7 | 2 | 20010428 | 618 | 26.4988 | -76.9127 | 980 | 248.5 | 20.5316 | 36.7704 | 2 | 36.7707 | 2 | 199.7 | 2 | 181 | 2 |
| 31WBTSOC | AB01 | 7 | 1 | 9 | 8 | 2 | 20010428 | 618 | 26.4988 | -76.9127 | 980 | 200.1 | 21.8498 | 36.8715 | 2 | 36.8735 | 2 | 198.7 | 2 | 191.7 | 2 |
| 31WBTSOC | AB01 | 7 | 1 | 10 | 9 | 2 | 20010428 | 618 | 26.4988 | -76.9127 | 980 | 148.9 | 22.7944 | 36.8468 | 2 | 36.8491 | 2 | 215.3 | 2 | 207 | 2 |
| 31WBTSOC | AB01 | 7 | 1 | 11 | 10 | 2 | 20010428 | 618 | 26.4988 | -76.9127 | 980 | 99.3 | 24.0719 | 36.6284 | 2 | 36.6361 | 2 | 215.8 | 2 | 207.7 | 2 |
| 31WBTSOC | AB01 | 7 | 1 | 12 | 11 | 2 | 20010428 | 618 | 26.4988 | -76.9127 | 980 | 47.9 | 24.2036 | 36.477 | 2 | 36.4781 | 2 | 211.6 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 7 | 1 | 13 | 12 | 2 | 20010428 | 618 | 26.4988 | -76.9127 | 980 | 1.9 | 24.1599 | 36.4651 | 2 | -999 | 9 | 208.6 | 2 | 206.2 | 3 |
| 31WBTSOC | AB01 | 8 | 1 | 1 | 24 | 2 | 20010428 | 1026 | 26.5112 | -76.8418 | 1007 | 996.1 | 5.794 | 35.0748 | 2 | 35.0755 | 2 | 213.9 | 2 | 209.4 | 2 |
| 31WBTSOC | AB01 | 8 | 1 | 2 | 1 | 2 | 20010428 | 1026 | 26.5112 | -76.8418 | 1007 | 900 | 7.1283 | 35.0978 | 2 | 35.0964 | 2 | 175.4 | 2 | 171.2 | 2 |
| 31WBTSOC | AB01 | 8 | 1 | 3 | 2 | 2 | 20010428 | 1026 | 26.5112 | -76.8418 | 1007 | 750.7 | 10.1411 | 35.3325 | 2 | 35.3312 | 2 | 151.9 | 2 | 145.4 | 2 |
| 31WBTSOC | AB01 | 8 | 1 | 4 | 3 | 2 | 20010428 | 1026 | 26.5112 | -76.8418 | 1007 | 625.9 | 12.872 | 35.697 | 2 | 35.6951 | 2 | 160.3 | 2 | 151.8 | 2 |
| 31WBTSOC | AB01 | 8 | 1 | 5 | 4 | 2 | 20010428 | 1026 | 26.5112 | -76.8418 | 1007 | 500.5 | 16.147 | 36.2054 | 2 | 36.2021 | 2 | 189.9 | 2 | 178.1 | 2 |
| 31WBTSOC | AB01 | 8 | 1 | 6 | 5 | 2 | 20010428 | 1026 | 26.5112 | -76.8418 | 1007 | 399.4 | 17.9522 | 36.5075 | 2 | 36.508 | 2 | 197 | 2 | 186.7 | 2 |
| 31WBTSOC | AB01 | 8 | 1 | 7 | 6 | 2 | 20010428 | 1026 | 26.5112 | -76.8418 | 1007 | 309.5 | 19.085 | 36.6422 | 2 | 36.6426 | 2 | 196 | 2 | 188 | 2 |
| 31WBTSOC | AB01 | 8 | 1 | 8 | 7 | 2 | 20010428 | 1026 | 26.5112 | -76.8418 | 1007 | 224.3 | 20.6644 | 36.7708 | 2 | 36.7737 | 2 | 212.5 | 2 | 204 | 2 |
| 31WBTSOC | AB01 | 8 | 1 | 9 | 8 | 2 | 20010428 | 1026 | 26.5112 | -76.8418 | 1007 | 160.4 | 21.8493 | 36.823 | 2 | 36.8164 | 2 | 216.2 | 2 | 209 | 2 |
| 31WBTSOC | AB01 | 8 | 1 | 10 | 9 | 2 | 20010428 | 1026 | 26.5112 | -76.8418 | 1007 | 119.4 | 23.2314 | 36.7842 | 2 | 36.7884 | 2 | 215.9 | 2 | 208.3 | 2 |
| 31WBTSOC | AB01 | 8 | 1 | 11 | 10 | 2 | 20010428 | 1026 | 26.5112 | -76.8418 | 1007 | 60.8 | 24.2821 | 36.5751 | 2 | 36.5905 | 3 | 215.5 | 2 | 208.4 | 2 |
| 31WBTSOC | AB01 | 8 | 1 | 12 | 11 | 2 | 20010428 | 1026 | 26.5112 | -76.8418 | 1007 | 2.6 | 24.6943 | 36.5601 | 2 | 36.56 | 2 | 208.7 | 2 | 206.5 | 3 |
| 31WBTSOC | AB01 | 9 | 1 | 1 | 24 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 3902.1 | 2.3649 | 34.9006 | 2 | 34.9007 | 2 | 267 | 2 | 266.6 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 2 | 1 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 3599.4 | 2.5038 | 34.9113 | 2 | -999 | 9 | 266 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 9 | 1 | 3 | 2 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 3297.1 | 2.677 | 34.9226 | 2 | 34.9226 | 2 | 263.2 | 2 | 267 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 4 | 3 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 3000.9 | 2.8797 | 34.9342 | 2 | 34.9342 | 2 | 259.8 | 2 | 265 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 5 | 4 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 2698.2 | 3.115 | 34.9489 | 2 | 34.9487 | 2 | 256.4 | 2 | 263.2 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 6 | 5 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 2398.5 | 3.3671 | 34.9621 | 2 | 34.9619 | 2 | 254.5 | 2 | 262 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 7 | 6 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 2100.8 | 3.6198 | 34.9708 | 2 | 34.9702 | 2 | 253.7 | 2 | 261.9 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 8 | 7 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 1802.2 | 3.7444 | 34.9675 | 2 | 34.9682 | 2 | 253.5 | 2 | 262.4 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 9 | 8 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 1486.6 | 4.0591 | 34.9861 | 2 | 34.9849 | 2 | 248.1 | 2 | 257.8 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 10 | 9 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 1200.7 | 4.9812 | 35.0543 | 2 | 35.0546 | 2 | 227.1 | 2 | 235.2 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 11 | 10 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 1005.4 | 6.2774 | 35.079 | 2 | 35.078 | 2 | 182.6 | 2 | 188.5 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 12 | 11 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 854.2 | 7.9064 | 35.1142 | 2 | 35.1137 | 2 | 148.1 | 2 | 151.7 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 13 | 12 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 701.8 | 11.1485 | 35.464 | 2 | 35.4704 | 2 | 152.1 | 2 | 151.5 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 14 | 13 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 546.3 | 15.1375 | 36.0468 | 2 | 36.0474 | 2 | 171.6 | 2 | 164.7 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 15 | 14 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 450.7 | 17.3042 | 36.3991 | 2 | 36.3987 | 2 | 192.7 | 2 | 184.6 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 16 | 15 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 349.7 | 18.458 | 36.5724 | 2 | 36.5689 | 2 | 196.1 | 2 | 189.3 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 17 | 16 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 249.8 | 19.3016 | 36.6442 | 2 | 36.6482 | 2 | 207.1 | 2 | 202.2 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 18 | 17 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 150.3 | 21.3603 | 36.7825 | 2 | 36.7844 | 2 | 219.3 | 2 | 213 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 19 | 18 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 99.6 | 22.9455 | 36.875 | 2 | 36.87 | 2 | 216.6 | 2 | 210.8 | 3 |
| 31WBTSOC | AB01 | 9 | 1 | 20 | 19 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 49.3 | 24.5513 | 36.6409 | 2 | 36.642 | 2 | 212.8 | 2 | 207.7 | 2 |
| 31WBTSOC | AB01 | 9 | 1 | 21 | 20 | 2 | 20010428 | 1208 | 26.5 | -76.7345 | 3861 | 3 | 24.6451 | 36.6022 | 2 | 36.602 | 2 | 206.9 | 2 | 207 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 1 | 24 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 4790.4 | 2.2035 | 34.8771 | 2 | 34.8761 | 2 | 255.5 | 2 | 256.8 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 2 | 1 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 4500.2 | 2.2771 | 34.8896 | 2 | 34.8923 | 2 | 260.2 | 2 | 263 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 3 | 2 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 4246.9 | 2.2874 | 34.8929 | 2 | 34.8881 | 3 | 259.8 | 2 | 264.7 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 4 | 3 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 3869.6 | 2.3698 | 34.9025 | 2 | 34.9019 | 2 | 260.8 | 2 | 266.9 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 5 | 4 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 3496.6 | 2.51 | 34.9132 | 2 | 34.9129 | 2 | 260 | 2 | 267.5 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 6 | 5 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 3120.4 | 2.741 | 34.9278 | 2 | 34.9269 | 2 | 257.6 | 2 | 265.9 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 7 | 6 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 2747 | 3.0453 | 34.9444 | 2 | 34.9434 | 2 | 254.1 | 2 | 263.4 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 8 | 7 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 2446.8 | 3.2561 | 34.9532 | 2 | 34.9521 | 2 | 253.2 | 2 | 263.1 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 9 | 8 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 2147 | 3.4484 | 34.9552 | 2 | 34.9537 | 2 | 254.6 | 2 | 264.3 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 10 | 9 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 1849.8 | 3.6974 | 34.961 | 2 | 34.9596 | 2 | 253.8 | 2 | 264 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 11 | 10 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 1550.9 | 4.0299 | 34.9784 | 2 | 34.9773 | 2 | 248.6 | 2 | 258.9 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 12 | 11 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 1247.4 | 4.9799 | 35.0506 | 2 | 35.0501 | 2 | 227 | 2 | 226.7 | 4 |
| 31WBTSOC | AB01 | 10 | 1 | 13 | 12 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 1046.4 | 6.2539 | 35.0883 | 2 | 35.0873 | 2 | 188.8 | 2 | 194.3 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 14 | 13 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 896.7 | 7.7348 | 35.1052 | 2 | 35.1052 | 2 | 151.1 | 2 | 154.8 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 15 | 14 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 749.8 | 10.3582 | 35.3678 | 2 | 35.3678 | 2 | 148.8 | 2 | 148.2 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 16 | 15 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 600.7 | 14.0575 | 35.8719 | 2 | 35.877 | 2 | 168.7 | 2 | 163.9 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 17 | 16 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 485 | 16.4019 | 36.2505 | 2 | 36.2601 | 2 | 183.8 | 2 | 179.5 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 18 | 17 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 355.7 | 18.1624 | 36.5382 | 2 | 36.5427 | 2 | 193.7 | 2 | 189.7 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 19 | 18 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 276.9 | 18.7808 | 36.6039 | 2 | 36.6042 | 2 | 205 | 2 | 201.1 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 20 | 19 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 200.7 | 19.7284 | 36.6794 | 2 | 36.6789 | 2 | 203.7 | 2 | 200.1 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 21 | 20 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 151.4 | 20.6365 | 36.7587 | 2 | 36.758 | 2 | 209.9 | 2 | 205.9 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 22 | 21 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 100.6 | 22.3793 | 36.9216 | 2 | 36.921 | 2 | 198.6 | 2 | 194.8 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 23 | 22 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 49.2 | 23.6756 | 36.866 | 2 | 36.8654 | 2 | 209.1 | 2 | 205.6 | 2 |
| 31WBTSOC | AB01 | 10 | 1 | 24 | 23 | 2 | 20010428 | 1548 | 26.502 | -76.6135 | 4726 | 3.6 | 24.239 | 36.6921 | 2 | 36.6954 | 2 | 211.6 | 2 | 208.7 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 1 | 24 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 4923.4 | 2.3021 | 34.8863 | 2 | 34.885 | 2 | 260.5 | 2 | 261.6 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 2 | 1 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 4549.1 | 2.291 | 34.8895 | 2 | -999 | 9 | 260.1 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 11 | 1 | 3 | 2 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 4149.3 | 2.3038 | 34.8945 | 2 | -999 | 9 | 259.7 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 11 | 1 | 4 | 3 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 3800.5 | 2.3674 | 34.901 | 2 | 34.9013 | 2 | 259 | 2 | 266.9 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 5 | 4 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 3449.1 | 2.4596 | 34.9088 | 2 | 34.3192 | 4 | 258.3 | 2 | 268.2 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 6 | 5 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 3149.5 | 2.6766 | 34.923 | 2 | 34.9226 | 2 | 257.2 | 2 | 267.1 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 7 | 6 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 2800.9 | 2.9214 | 34.9386 | 2 | 34.9383 | 2 | 253.5 | 2 | 264.3 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 8 | 7 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 2500.6 | 3.168 | 34.9478 | 2 | 34.9472 | 2 | 252.9 | 2 | 263.8 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 9 | 8 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 2200.1 | 3.3618 | 34.9557 | 2 | 34.956 | 2 | 251.9 | 2 | 263.3 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 10 | 9 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 1901.5 | 3.6634 | 34.965 | 2 | 34.9645 | 2 | 251.7 | 2 | 263.8 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 11 | 10 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 1602.9 | 3.9995 | 34.9802 | 2 | 34.9804 | 2 | 248.2 | 2 | 259.4 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 12 | 11 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 1374.8 | 4.4979 | 35.0145 | 2 | 35.0154 | 2 | 237.3 | 2 | 247.9 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 13 | 12 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 1174.3 | 5.2842 | 35.0609 | 2 | 35.0599 | 2 | 218.7 | 2 | 227.1 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 14 | 13 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 978.9 | 6.8101 | 35.0701 | 2 | 35.1546 | 4 | 166.1 | 2 | 158.5 | 4 |
| 31WBTSOC | AB01 | 11 | 1 | 15 | 14 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 803.7 | 9.6314 | 35.2756 | 2 | 35.278 | 2 | 144.8 | 2 | 146 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 16 | 15 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 652.2 | 13.0774 | 35.7274 | 2 | 35.7399 | 3 | 161.8 | 2 | 160 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 17 | 16 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 503.2 | 16.2138 | 36.2103 | 2 | 36.2128 | 2 | 183.8 | 2 | 177.1 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 18 | 17 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 402.1 | 17.7832 | 36.4788 | 2 | 36.4819 | 2 | 195.8 | 2 | 189.7 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 19 | 18 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 301.8 | 18.634 | 36.5854 | 2 | 36.5861 | 2 | 205.5 | 2 | 200.7 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 20 | 19 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 224.4 | 19.1891 | 36.6329 | 2 | 36.6343 | 2 | 203.7 | 2 | 199.9 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 21 | 20 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 160.3 | 20.7266 | 36.7745 | 2 | 36.7735 | 2 | 219 | 2 | 213.7 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 22 | 21 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 110.4 | 21.92 | 36.8723 | 2 | 36.8785 | 2 | 201.4 | 2 | 196.7 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 23 | 22 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 54.4 | 23.4655 | 36.8177 | 2 | 36.8219 | 2 | 217.4 | 2 | 212.8 | 2 |
| 31WBTSOC | AB01 | 11 | 1 | 24 | 23 | 2 | 20010428 | 2224 | 26.5002 | -76.48 | 4855 | 3.5 | 24.4553 | 36.6369 | 2 | 36.6379 | 2 | 210.5 | 2 | 208.2 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 1 | 24 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 4907.8 | 2.3164 | 34.8878 | 2 | 34.8883 | 2 | 262.2 | 2 | 262.1 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 2 | 1 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 4794.6 | 2.333 | 34.8904 | 2 | 34.8909 | 2 | 262.4 | 2 | 263.7 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 3 | 2 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 4668.5 | 2.3407 | 34.8925 | 2 | -999 | 9 | 263 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 12 | 1 | 4 | 3 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 4498 | 2.3476 | 34.8944 | 2 | 34.8944 | 2 | 261.8 | 2 | 264.8 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 5 | 4 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 4046.9 | 2.3354 | 34.8974 | 2 | 34.8974 | 2 | 259.6 | 2 | 265.7 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 6 | 5 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 3600.1 | 2.4339 | 34.9062 | 2 | 34.9066 | 2 | 259.4 | 2 | 267.9 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 7 | 6 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 3199.3 | 2.6382 | 34.9198 | 2 | 34.9198 | 2 | 257.8 | 2 | 267.3 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 8 | 7 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 2797.3 | 2.8595 | 34.933 | 2 | 34.9332 | 2 | 255 | 2 | 265.5 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 9 | 8 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 2449.7 | 3.1308 | 34.9495 | 2 | 34.9497 | 2 | 252.2 | 2 | 262.9 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 10 | 9 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 2099.1 | 3.404 | 34.9543 | 2 | 34.955 | 2 | 253 | 2 | 263.9 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 11 | 10 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 1799.8 | 3.6443 | 34.9601 | 2 | 34.9603 | 2 | 252.4 | 2 | 264.2 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 12 | 11 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 1500 | 4.1771 | 34.9919 | 2 | 34.9924 | 2 | 244.9 | 2 | 255.6 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 13 | 12 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 1198.3 | 5.1086 | 35.0497 | 2 | 35.0497 | 2 | 222.3 | 2 | 230.7 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 14 | 13 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 950.1 | 7.3677 | 35.1007 | 2 | 35.1005 | 2 | 161.7 | 2 | 165.6 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 15 | 14 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 825.3 | 9.4671 | 35.2564 | 2 | 35.2571 | 2 | 144.1 | 2 | 145.4 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 16 | 15 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 678.4 | 12.4547 | 35.6367 | 2 | 35.6562 | 3 | 159.8 | 2 | 148.7 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 17 | 16 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 551.9 | 15.3304 | 36.0708 | 2 | 36.0777 | 2 | 177.2 | 2 | 172.9 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 18 | 17 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 460.7 | 17.0087 | 36.345 | 2 | 36.3428 | 2 | 187.1 | 2 | 182.3 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 19 | 18 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 371 | 17.9952 | 36.5073 | 2 | 36.5096 | 2 | 194.1 | 2 | 191 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 20 | 19 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 230 | 19.0776 | 36.6232 | 2 | 36.6234 | 2 | 201.5 | 2 | 199 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 21 | 20 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 176.4 | 20.0402 | 36.7129 | 2 | 36.7152 | 2 | 201.7 | 2 | 199.4 | 3 |
| 31WBTSOC | AB01 | 12 | 1 | 22 | 21 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 120.1 | 21.8598 | 36.8691 | 2 | 36.8668 | 2 | 199.2 | 2 | 195.8 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 23 | 22 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 60.3 | 23.1095 | 36.8272 | 2 | 36.8271 | 2 | 215.4 | 2 | 212.8 | 2 |
| 31WBTSOC | AB01 | 12 | 1 | 24 | 23 | 2 | 20010429 | 252 | 26.504 | -76.3028 | 4840 | 2.8 | 24.2519 | 36.714 | 2 | 36.7156 | 2 | 210.5 | 2 | 209.1 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 1 | 24 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 4888.3 | 2.3073 | 34.8872 | 2 | 34.8872 | 2 | 262 | 2 | 262 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 2 | 1 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 4848.5 | 2.3022 | 34.8871 | 2 | 34.8872 | 2 | 261.3 | 2 | 261.9 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 3 | 2 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 4600.8 | 2.3207 | 34.8914 | 2 | -999 | 9 | 261.8 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 13 | 1 | 4 | 3 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 4298.5 | 2.314 | 34.8937 | 2 | 34.8936 | 2 | 259.8 | 2 | 265.1 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 5 | 4 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 3898.7 | 2.3687 | 34.9003 | 2 | -999 | 9 | 260.5 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 13 | 1 | 6 | 5 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 3634 | 2.461 | 34.9076 | 2 | 34.9072 | 2 | 259.8 | 2 | 267.7 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 7 | 6 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 3249.2 | 2.6196 | 34.9182 | 2 | 34.9178 | 2 | 258.1 | 2 | 267.6 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 8 | 7 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 2898 | 2.7916 | 34.9293 | 2 | 34.9295 | 2 | 255.7 | 2 | 266.3 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 9 | 8 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 2549.8 | 3.13 | 34.9465 | 2 | 34.9468 | 2 | 253.4 | 2 | 263.8 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 10 | 9 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 2199.5 | 3.3726 | 34.953 | 2 | 34.9535 | 2 | 253.4 | 2 | 264.1 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 11 | 10 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 1902.5 | 3.571 | 34.9527 | 2 | 34.9525 | 2 | 254 | 2 | 265.2 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 12 | 11 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 1599.6 | 3.7318 | 34.9593 | 2 | 34.9613 | 2 | 251.5 | 2 | 263.6 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 13 | 12 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 1377.7 | 4.2196 | 34.9971 | 2 | 34.9987 | 2 | 243.2 | 2 | 253.8 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 14 | 13 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 1150.8 | 5.3472 | 35.0673 | 2 | 35.0682 | 2 | 216.7 | 2 | 225 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 15 | 14 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 928 | 7.7772 | 35.113 | 2 | 35.1117 | 2 | 151 | 2 | 155.1 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 16 | 15 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 724.6 | 11.4175 | 35.4896 | 2 | 35.4905 | 2 | 149.1 | 2 | 152 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 17 | 16 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 560.1 | 15.318 | 36.0628 | 2 | 36.0628 | 2 | 177.6 | 2 | 171.6 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 18 | 17 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 450.4 | 17.0494 | 36.3607 | 2 | 36.3615 | 2 | 184.4 | 2 | 180.3 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 19 | 18 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 349 | 18.0915 | 36.5303 | 2 | 36.5298 | 2 | 193.1 | 2 | 188.7 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 20 | 19 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 219.3 | 19.7243 | 36.7212 | 2 | 36.7212 | 2 | 186.3 | 2 | 182.8 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 21 | 20 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 160 | 21.0358 | 36.7754 | 2 | 36.7799 | 2 | 205.2 | 2 | 200.4 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 22 | 21 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 109.3 | 22.5834 | 36.911 | 2 | 36.9043 | 2 | 195.5 | 2 | 193.6 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 23 | 22 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 48.2 | 24.8366 | 36.6016 | 2 | 36.6498 | 4 | 210.4 | 2 | 206 | 2 |
| 31WBTSOC | AB01 | 13 | 1 | 24 | 23 | 2 | 20010429 | 720 | 26.5022 | -76.1175 | 4821 | 2.4 | 24.8784 | 36.5419 | 2 | 36.5417 | 2 | 206.5 | 2 | 206.3 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 1 | 24 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 4831.8 | 2.2453 | 34.8813 | 2 | 34.8824 | 2 | 257.4 | 2 | 258.6 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 2 | 1 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 4618.2 | 2.2653 | 34.8864 | 2 | 34.8872 | 2 | 258.8 | 2 | 261.5 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 3 | 2 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 4317.7 | 2.2906 | 34.8925 | 2 | 34.8923 | 2 | 260.1 | 2 | 264.7 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 4 | 3 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 3922.1 | 2.3703 | 34.9015 | 2 | 34.9021 | 2 | 260.8 | 2 | 267.3 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 5 | 4 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 3624 | 2.4808 | 34.9091 | 2 | 34.9092 | 2 | 260.1 | 2 | 267.9 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 6 | 5 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 3224 | 2.6675 | 34.9215 | 2 | 34.9218 | 2 | 258.3 | 2 | 267.2 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 7 | 6 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 2923.3 | 2.8816 | 34.9351 | 2 | 34.9346 | 2 | 255.1 | 2 | 265.5 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 8 | 7 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 2599.4 | 3.1514 | 34.95 | 2 | 34.9511 | 2 | 252.8 | 2 | 263 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 9 | 8 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 2225.3 | 3.4185 | 34.9534 | 2 | 34.9505 | 2 | 253.7 | 2 | 264.3 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 10 | 9 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 1924.3 | 3.5815 | 34.9555 | 2 | 34.9566 | 2 | 252.8 | 2 | 264.6 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 11 | 10 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 1626.9 | 3.9471 | 34.9745 | 2 | 34.9735 | 2 | 249.3 | 2 | 260.6 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 12 | 11 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 1400.9 | 4.4502 | 35.0156 | 2 | 35.0164 | 2 | 238.6 | 2 | 248.8 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 13 | 12 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 1203.9 | 5.342 | 35.072 | 2 | 35.0743 | 2 | 215.7 | 2 | 223.5 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 14 | 13 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 1051.2 | 6.6153 | 35.088 | 2 | 35.0875 | 2 | 177.3 | 2 | 182.8 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 15 | 14 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 953.3 | 7.5817 | 35.0954 | 2 | 35.0971 | 2 | 152 | 2 | 160.3 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 16 | 15 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 751.3 | 11.4349 | 35.4941 | 2 | 35.5076 | 3 | 148.8 | 2 | 156.5 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 17 | 16 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 576.5 | 15.2863 | 36.0651 | 2 | 36.0688 | 2 | 174.9 | 2 | 173 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 18 | 17 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 460.2 | 17.1252 | 36.3734 | 2 | 36.3755 | 2 | 184.1 | 2 | 185.1 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 19 | 18 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 362.4 | 18.337 | 36.5616 | 2 | 36.5653 | 2 | 189.3 | 2 | 190 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 20 | 19 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 228.6 | 19.7483 | 36.7144 | 2 | 36.7152 | 2 | 188.8 | 2 | 189.9 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 21 | 20 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 228.8 | 19.7451 | 36.7143 | 2 | 36.7148 | 2 | 188.9 | 2 | 187.7 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 22 | 21 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 173.1 | 20.6808 | 36.8103 | 2 | 36.8088 | 2 | 186.2 | 2 | 185.9 | 2 |
| 31WBTSOC | AB01 | 14 | 1 | 23 | 22 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 124 | 21.6709 | 36.8753 | 2 | -999 | 9 | 192.2 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 14 | 1 | 24 | 23 | 2 | 20010429 | 1209 | 26.5027 | -75.9167 | 4767 | 3.7 | 24.9026 | 36.4966 | 2 | 36.4965 | 2 | 208.5 | 2 | 215.4 | 4 |
| 31WBTSOC | AB01 | 15 | 1 | 1 | 24 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 4769.9 | 2.1939 | 34.8756 | 2 | 34.8765 | 2 | 253.7 | 2 | 260.2 | 4 |
| 31WBTSOC | AB01 | 15 | 1 | 2 | 1 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 4394.9 | 2.2707 | 34.8898 | 2 | 34.8897 | 2 | 259.2 | 2 | 265.7 | 4 |
| 31WBTSOC | AB01 | 15 | 1 | 3 | 2 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 3996.8 | 2.3 | 34.896 | 2 | 34.8964 | 2 | 259.6 | 2 | 274 | 4 |
| 31WBTSOC | AB01 | 15 | 1 | 4 | 3 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 3696 | 2.3604 | 34.9026 | 2 | 34.9027 | 2 | 259.3 | 2 | 280 | 4 |
| 31WBTSOC | AB01 | 15 | 1 | 5 | 4 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 3398.7 | 2.4762 | 34.9108 | 2 | 34.9106 | 2 | 258.4 | 2 | 274.3 | 4 |
| 31WBTSOC | AB01 | 15 | 1 | 6 | 5 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 3100.6 | 2.6727 | 34.9224 | 2 | 34.9218 | 2 | 257.4 | 2 | 280.1 | 4 |
| 31WBTSOC | AB01 | 15 | 1 | 7 | 6 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 2802 | 2.9104 | 34.937 | 2 | 34.9367 | 2 | 254.3 | 2 | 265.6 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 8 | 7 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 2501.4 | 3.2259 | 34.9518 | 2 | 34.9513 | 2 | 252.7 | 2 | 263.9 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 9 | 8 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 2203.1 | 3.4884 | 34.9536 | 2 | 34.9531 | 2 | 254 | 2 | 265 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 10 | 9 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 2003.1 | 3.6841 | 34.9664 | 2 | 34.9664 | 2 | 252.3 | 2 | 262.8 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 11 | 10 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 1804.1 | 3.876 | 34.9726 | 2 | 34.9729 | 2 | 250.9 | 2 | 261.2 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 12 | 11 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 1603.6 | 4.2217 | 35.0002 | 2 | 34.9977 | 2 | 244.6 | 2 | 254.5 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 13 | 12 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 1409.8 | 4.6782 | 35.033 | 2 | 35.0359 | 3 | 234.4 | 2 | 243.5 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 14 | 13 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 1207.7 | 5.6841 | 35.0849 | 2 | 35.0877 | 2 | 205.7 | 2 | 199.6 | 4 |
| 31WBTSOC | AB01 | 15 | 1 | 15 | 14 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 1006.2 | 7.4148 | 35.0912 | 2 | 35.092 | 2 | 155.8 | 2 | 160.2 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 16 | 15 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 805.9 | 10.9455 | 35.4381 | 2 | 35.4439 | 2 | 149.8 | 2 | 150.3 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 17 | 16 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 605.1 | 15.0335 | 36.0303 | 2 | 36.0419 | 3 | 167.3 | 2 | 163.7 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 18 | 17 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 483 | 17.0949 | 36.3663 | 2 | 36.3708 | 2 | 184.6 | 2 | 181 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 19 | 18 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 354.9 | 18.3824 | 36.5569 | 2 | 36.557 | 2 | 195.3 | 2 | 190 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 20 | 19 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 222.6 | 19.781 | 36.6881 | 2 | 36.69 | 2 | 205.7 | 2 | 201.7 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 21 | 20 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 154.3 | 21.1015 | 36.7752 | 2 | 36.7757 | 2 | 215 | 2 | 211.5 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 22 | 21 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 89.7 | 22.5436 | 36.8304 | 2 | 36.8309 | 2 | 219.2 | 2 | 219.1 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 23 | 22 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 45.5 | 23.3788 | 36.8042 | 2 | 36.813 | 2 | 215.7 | 2 | 213.7 | 2 |
| 31WBTSOC | AB01 | 15 | 1 | 24 | 23 | 2 | 20010429 | 1930 | 26.5088 | -75.7067 | 4706 | 3.2 | 24.4865 | 36.6194 | 2 | 36.62 | 2 | 209.6 | 2 | 207.8 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 1 | 24 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 4760.3 | 2.1727 | 34.8733 | 2 | 34.8738 | 2 | 253.7 | 2 | 255.2 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 2 | 1 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 4496.8 | 2.2546 | 34.8871 | 2 | 34.8866 | 2 | 259 | 2 | 261.7 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 3 | 2 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 4199.5 | 2.2734 | 34.8918 | 2 | 34.8913 | 2 | 259.5 | 2 | 264.2 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 4 | 3 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 3899.1 | 2.2948 | 34.8962 | 2 | 34.8956 | 2 | 259.4 | 2 | 266 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 5 | 4 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 3599.9 | 2.3605 | 34.9024 | 2 | 34.9023 | 2 | 259.4 | 2 | 267.4 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 6 | 5 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 3301 | 2.5313 | 34.9151 | 2 | 34.9165 | 2 | 258.7 | 2 | 267.5 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 7 | 6 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 2999.9 | 2.7342 | 34.927 | 2 | 34.9271 | 2 | 256.7 | 2 | 265.9 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 8 | 7 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 2698.5 | 3.0457 | 34.9459 | 2 | 34.9466 | 2 | 253.2 | 2 | 263 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 9 | 8 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 2450.9 | 3.2974 | 34.9605 | 2 | 34.9605 | 2 | 251.3 | 2 | 261.2 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 10 | 9 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 2202 | 3.5293 | 34.968 | 2 | 34.9678 | 2 | 251.4 | 2 | 261.3 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 11 | 10 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 1951.4 | 3.7656 | 34.9711 | 2 | 34.9718 | 2 | 251.5 | 2 | 261.6 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 12 | 11 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 1701.1 | 4.1141 | 34.9946 | 2 | 34.995 | 2 | 246.3 | 2 | 256.3 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 13 | 12 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 1452.8 | 4.6765 | 35.0327 | 2 | 35.029 | 2 | 234.6 | 2 | 243.4 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 14 | 13 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 1176.1 | 6.1322 | 35.0857 | 2 | 35.0849 | 2 | 191.8 | 2 | 197.7 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 15 | 14 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 1051.6 | 7.2975 | 35.096 | 2 | 35.0971 | 2 | 161.3 | 2 | 165.4 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 16 | 15 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 851.3 | 10.6271 | 35.3917 | 2 | 35.3907 | 2 | 147.8 | 2 | 147.6 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 17 | 16 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 649.3 | 14.6002 | 35.957 | 2 | 35.9579 | 2 | 166 | 2 | 161.5 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 18 | 17 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 458.1 | 17.808 | 36.4806 | 2 | 36.4817 | 2 | 196 | 2 | 190.1 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 19 | 18 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 344.1 | 18.6503 | 36.587 | 2 | 36.5859 | 2 | 204.7 | 2 | 201.1 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 20 | 19 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 259.5 | 19.213 | 36.6288 | 2 | 36.6283 | 2 | 202.6 | 2 | 199.8 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 21 | 20 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 180.2 | 20.6145 | 36.7685 | 2 | 36.7686 | 2 | 214.4 | 2 | 210.4 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 22 | 21 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 109.8 | 21.5934 | 36.8071 | 2 | 36.8065 | 2 | 213.2 | 2 | 210.7 | 2 |
| 31WBTSOC | AB01 | 16 | 1 | 23 | 22 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 55 | 23.2231 | 36.7926 | 2 | -999 | 9 | 217.6 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 16 | 1 | 24 | 23 | 2 | 20010430 | 113 | 26.5077 | -75.5022 | 4697 | 2.5 | 23.9683 | 36.7248 | 2 | -999 | 9 | 211 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 17 | 1 | 1 | 24 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 4716.4 | 2.2206 | 34.8799 | 2 | 34.8805 | 2 | 257.6 | 2 | 258.3 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 2 | 1 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 4297.6 | 2.2445 | 34.8891 | 2 | 34.8887 | 2 | 259.2 | 2 | 262.9 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 3 | 2 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 3921.4 | 2.2906 | 34.8964 | 2 | 34.896 | 2 | 260.5 | 2 | 266 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 4 | 3 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 3571.1 | 2.3732 | 34.904 | 2 | 34.9039 | 2 | 260.3 | 2 | 267.9 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 5 | 4 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 3224.8 | 2.5348 | 34.9149 | 2 | 34.934 | 4 | 259 | 2 | 267.9 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 6 | 5 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 2901.6 | 2.8301 | 34.9341 | 2 | 34.9521 | 4 | 255.4 | 2 | 264.8 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 7 | 6 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 2621.7 | 3.1369 | 34.9515 | 2 | 34.9623 | 4 | 252.9 | 2 | 262.1 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 8 | 7 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 2424.7 | 3.3514 | 34.9626 | 2 | 34.9694 | 4 | 251.8 | 2 | 261 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 9 | 8 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 2223.2 | 3.5408 | 34.9704 | 2 | 34.9702 | 2 | 251.7 | 2 | 261.1 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 10 | 9 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 2025.5 | 3.7139 | 34.9702 | 2 | 34.971 | 2 | 252.5 | 2 | 262 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 11 | 10 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 1824.5 | 3.9018 | 34.9754 | 2 | 34.9759 | 2 | 250.9 | 2 | 260.6 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 12 | 11 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 1623.1 | 4.2438 | 34.9997 | 2 | 35.0003 | 2 | 244.7 | 2 | 254.2 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 13 | 12 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 1424.9 | 4.8391 | 35.044 | 2 | 35.0859 | 2 | 231 | 2 | 205.1 | 3 |
| 31WBTSOC | AB01 | 17 | 1 | 14 | 13 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 1223.8 | 5.9736 | 35.0853 | 2 | 35.0859 | 2 | 199.3 | 2 | 205.1 | 3 |
| 31WBTSOC | AB01 | 17 | 1 | 15 | 14 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 1027.7 | 7.8241 | 35.1071 | 2 | 35.1062 | 2 | 150 | 2 | 153.6 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 16 | 15 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 825.3 | 11.4616 | 35.4994 | 2 | 35.4976 | 2 | 153.7 | 2 | 152.5 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 17 | 16 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 622.8 | 15.6365 | 36.1167 | 2 | 36.1156 | 2 | 179 | 2 | 173.3 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 18 | 17 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 448.6 | 18.0008 | 36.5101 | 2 | 36.5027 | 2 | 196.1 | 2 | 191.5 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 19 | 18 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 325.8 | 18.8742 | 36.6115 | 2 | 36.6107 | 2 | 207.5 | 2 | 203.9 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 20 | 19 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 224.6 | 19.9773 | 36.697 | 2 | 36.692 | 2 | 209.7 | 2 | 193.2 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 21 | 20 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 150.8 | 20.9131 | 36.7777 | 2 | 36.7775 | 2 | 215.3 | 2 | 212.9 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 22 | 21 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 99.7 | 21.9376 | 36.7989 | 2 | 36.7991 | 2 | 216.7 | 2 | 213.1 | 2 |
| 31WBTSOC | AB01 | 17 | 1 | 23 | 22 | 2 | 20010430 | 607 | 26.503 | -75.3027 | 4654 | 2.8 | 24.2747 | 36.6877 | 2 | 36.6878 | 2 | 210.5 | 2 | 208.5 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 1 | 24 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 4684.7 | 2.2032 | 34.8785 | 2 | 34.8773 | 2 | 257 | 2 | 257.6 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 2 | 1 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 4400.1 | 2.2456 | 34.8875 | 2 | 34.8877 | 2 | 259.4 | 2 | 261.9 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 3 | 2 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 4249.5 | 2.2527 | 34.8898 | 2 | 34.8903 | 2 | 259.8 | 2 | 264.1 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 4 | 3 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 3872.6 | 2.2748 | 34.8954 | 2 | 34.8948 | 2 | 259.6 | 2 | 265.8 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 5 | 4 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 3499.4 | 2.3714 | 34.9043 | 2 | 34.9047 | 2 | 260 | 2 | 268.2 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 6 | 5 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 3127.3 | 2.6082 | 34.9193 | 2 | 34.9194 | 2 | 258.8 | 2 | 267.6 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 7 | 6 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 2749.3 | 2.9607 | 34.9418 | 2 | 34.942 | 2 | 254 | 2 | 264.1 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 8 | 7 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 2448.4 | 3.297 | 34.9599 | 2 | 34.9599 | 2 | 252.2 | 2 | 262.2 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 9 | 8 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 2153.2 | 3.5974 | 34.9696 | 2 | 34.9712 | 2 | 251.8 | 2 | 261.6 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 10 | 9 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 1850.1 | 3.878 | 34.9755 | 2 | 34.9759 | 2 | 250.8 | 2 | 261.1 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 11 | 10 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 1549.3 | 4.459 | 35.0176 | 2 | 35.0184 | 2 | 239.8 | 2 | 249.1 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 12 | 11 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 1251.9 | 5.8678 | 35.0859 | 2 | 35.0865 | 2 | 202.2 | 2 | 208.5 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 13 | 12 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 1049.8 | 7.7304 | 35.0966 | 2 | 35.0967 | 2 | 149.8 | 2 | 153.7 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 14 | 13 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 900.5 | 10.4381 | 35.3671 | 2 | 35.3706 | 2 | 147.8 | 2 | 147.5 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 15 | 14 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 749.8 | 13.6597 | 35.8072 | 2 | 35.8095 | 2 | 166 | 2 | 162.7 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 16 | 15 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 601.5 | 16.2197 | 36.2115 | 2 | 36.2124 | 2 | 183 | 2 | 179.5 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 17 | 16 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 475.4 | 17.8883 | 36.493 | 2 | 36.4932 | 2 | 193.6 | 2 | 190.4 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 18 | 17 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 350.9 | 18.7004 | 36.5943 | 2 | 36.5974 | 2 | 202.2 | 2 | 200.2 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 19 | 18 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 276.2 | 19.0266 | 36.6206 | 2 | 36.6198 | 2 | 205.5 | 2 | 203.2 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 20 | 19 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 183.7 | 20.5487 | 36.765 | 2 | 36.7646 | 2 | 215.3 | 2 | 211.6 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 21 | 20 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 148.4 | 20.9099 | 36.7802 | 2 | 36.7799 | 2 | 213 | 2 | 211.8 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 22 | 21 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 99.5 | 22.0584 | 36.8166 | 2 | 36.8184 | 2 | 217.4 | 2 | 215.6 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 23 | 22 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 48.2 | 23.1307 | 36.8653 | 2 | 36.8644 | 2 | 215.4 | 2 | 214 | 2 |
| 31WBTSOC | AB01 | 18 | 1 | 24 | 23 | 2 | 20010430 | 1059 | 26.5008 | -75.0832 | 4624 | 3.5 | 23.3266 | 36.7886 | 2 | 36.7892 | 2 | 212 | 2 | 212.4 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 1 | 24 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 4609.1 | 2.1952 | 34.8781 | 2 | 34.8795 | 2 | 256.9 | 2 | 257.6 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 2 | 1 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 4344.7 | 2.2178 | 34.8852 | 2 | 34.8848 | 2 | 258.3 | 2 | 260.9 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 3 | 2 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 4019.5 | 2.2444 | 34.8912 | 2 | 34.8913 | 2 | 259.3 | 2 | 264.1 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 4 | 3 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 3695.1 | 2.3028 | 34.8992 | 2 | 34.8986 | 2 | 259.6 | 2 | 266.6 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 5 | 4 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 3369.9 | 2.4445 | 34.9105 | 2 | 34.9102 | 2 | 259.8 | 2 | 268 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 6 | 5 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 3050.1 | 2.6697 | 34.9233 | 2 | 34.9234 | 2 | 258.1 | 2 | 266.7 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 7 | 6 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 2701.1 | 2.9727 | 34.9403 | 2 | 34.9395 | 2 | 254.7 | 2 | 264.5 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 8 | 7 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 2342.5 | 3.3382 | 34.9578 | 2 | 34.9564 | 2 | 252.6 | 2 | 262.7 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 9 | 8 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 2123.1 | 3.5686 | 34.9635 | 2 | 34.9564 | 3 | 253.2 | 2 | 263.5 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 10 | 9 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 1921.6 | 3.7475 | 34.965 | 2 | 34.9657 | 2 | 253 | 2 | 263 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 11 | 10 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 1722 | 4.0368 | 34.9823 | 2 | 34.9814 | 2 | 248.9 | 2 | 258.6 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 12 | 11 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 1522.1 | 4.4419 | 35.014 | 2 | 35.0119 | 2 | 240.3 | 2 | 249.6 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 13 | 12 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 1321.9 | 5.2835 | 35.0668 | 2 | 35.0641 | 2 | 219.9 | 2 | 227.6 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 14 | 13 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 1126.3 | 6.7781 | 35.0809 | 2 | 35.0814 | 2 | 171.8 | 2 | 176.3 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 15 | 14 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 921.5 | 9.8075 | 35.2935 | 2 | 35.2931 | 2 | 145 | 2 | 145.4 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 16 | 15 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 724.2 | 13.9137 | 35.8448 | 2 | 35.8446 | 2 | 169.2 | 2 | 163.9 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 17 | 16 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 548.7 | 17.1051 | 36.3597 | 2 | 36.3611 | 2 | 190.3 | 2 | 183.4 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 18 | 17 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 381.4 | 18.4405 | 36.5673 | 2 | 36.5679 | 2 | 195 | 2 | 191.2 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 19 | 18 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 263.1 | 19.2596 | 36.645 | 2 | 36.6456 | 2 | 196.9 | 2 | 194.8 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 20 | 19 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 191.3 | 20.3919 | 36.7496 | 2 | 36.7503 | 2 | 198.9 | 2 | 198.3 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 21 | 20 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 144.5 | 21.5846 | 36.821 | 2 | 36.8241 | 2 | 206.7 | 2 | 204.5 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 22 | 21 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 91.8 | 22.5076 | 36.8146 | 2 | 36.8154 | 2 | 214.8 | 2 | 213.1 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 23 | 22 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 45.6 | 23.4297 | 36.7806 | 2 | 36.7819 | 2 | 213 | 2 | 212.1 | 2 |
| 31WBTSOC | AB01 | 19 | 1 | 24 | 23 | 2 | 20010430 | 1612 | 26.497 | -74.8017 | 4550 | 3 | 23.5002 | 36.7768 | 2 | 36.7767 | 2 | 210.8 | 2 | 211.7 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 1 | 24 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 4557.4 | 2.1923 | 34.8791 | 2 | 34.8795 | 2 | 257.1 | 2 | 258.3 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 2 | 1 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 4235.5 | 2.2335 | 34.8883 | 2 | 34.8881 | 2 | 259.6 | 2 | 262.7 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 3 | 2 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 3873.6 | 2.2667 | 34.8947 | 2 | 34.895 | 2 | 260.1 | 2 | 265.7 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 4 | 3 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 3600 | 2.311 | 34.8989 | 2 | 34.8994 | 2 | 260.1 | 2 | 271 | 3 |
| 31WBTSOC | AB01 | 20 | 1 | 5 | 4 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 3347.8 | 2.4353 | 34.9099 | 2 | 34.91 | 2 | 259.8 | 2 | 267.9 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 6 | 5 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 3097.8 | 2.6233 | 34.9215 | 2 | 34.9212 | 2 | 258.4 | 2 | 267.1 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 7 | 6 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 2847.7 | 2.8275 | 34.9339 | 2 | 34.9342 | 2 | 255.5 | 2 | 264.7 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 8 | 7 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 2598.7 | 3.0882 | 34.9494 | 2 | 34.9489 | 2 | 253.1 | 2 | 262.5 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 9 | 8 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 2373.2 | 3.3264 | 34.9632 | 2 | 34.9637 | 2 | 251.5 | 2 | 260.8 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 10 | 9 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 2123.4 | 3.5835 | 34.9724 | 2 | 34.959 | 4 | 251.2 | 2 | 261.4 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 11 | 10 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 1973.9 | 3.7314 | 34.9719 | 2 | 34.9723 | 2 | 252 | 2 | 262 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 12 | 11 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 1620.8 | 4.2098 | 34.9978 | 2 | 35.0005 | 2 | 245.7 | 2 | 255.1 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 13 | 12 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 1400.2 | 4.8424 | 35.0398 | 2 | 35.0406 | 2 | 230.9 | 2 | 240 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 14 | 13 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 1173.8 | 6.3001 | 35.0885 | 2 | 35.0893 | 2 | 189.5 | 2 | 194.9 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 15 | 14 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 976.4 | 8.7057 | 35.1769 | 2 | 35.1785 | 2 | 144.3 | 2 | 146.6 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 16 | 15 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 778.3 | 12.4198 | 35.6282 | 2 | 35.6377 | 2 | 157.3 | 2 | 156.1 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 17 | 16 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 628.8 | 15.4662 | 36.0896 | 2 | 36.0911 | 2 | 175.8 | 2 | 171.9 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 18 | 17 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 477 | 17.7425 | 36.4673 | 2 | 36.4688 | 2 | 193.3 | 2 | 189.1 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 19 | 18 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 386.1 | 18.4582 | 36.5676 | 2 | 36.5667 | 2 | 195.7 | 2 | 193.7 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 20 | 19 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 294.6 | 18.946 | 36.6189 | 2 | 36.6188 | 2 | 203.3 | 2 | 202.1 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 21 | 20 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 212.6 | 20.1217 | 36.7203 | 2 | 36.7208 | 2 | 199.6 | 2 | 197.7 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 22 | 21 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 162.4 | 21.2286 | 36.7992 | 2 | 36.7993 | 2 | 204.9 | 2 | 203.7 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 23 | 22 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 109.9 | 22.2151 | 36.8381 | 2 | 36.8382 | 2 | 210.1 | 2 | 209.3 | 2 |
| 31WBTSOC | AB01 | 20 | 1 | 24 | 23 | 2 | 20010430 | 2132 | 26.5017 | -74.5148 | 4500 | 60.1 | 23.4068 | 36.792 | 2 | 36.7952 | 2 | 212.9 | 2 | 211.4 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 1 | 24 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 4608 | 2.206 | 34.8798 | 2 | 34.8815 | 2 | 258.4 | 2 | 258.5 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 2 | 1 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 4397.1 | 2.2382 | 34.8865 | 2 | 34.8874 | 2 | 260.5 | 2 | 261.6 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 3 | 2 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 4097.3 | 2.263 | 34.8921 | 2 | 34.8925 | 2 | 261.1 | 2 | 264.6 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 4 | 3 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 3795.3 | 2.2892 | 34.8962 | 2 | 34.8968 | 2 | 260.7 | 2 | 265.9 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 5 | 4 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 3499.3 | 2.3617 | 34.9028 | 2 | 34.9043 | 2 | 260.7 | 2 | 267.9 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 6 | 5 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 3197.7 | 2.5548 | 34.9169 | 2 | 34.918 | 2 | 259.4 | 2 | 267.4 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 7 | 6 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 2900.5 | 2.7994 | 34.9308 | 2 | 34.9316 | 2 | 257.4 | 2 | 265.9 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 8 | 7 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 2598.2 | 3.1089 | 34.9509 | 2 | 34.9519 | 2 | 252.4 | 2 | 261.7 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 9 | 8 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 2388 | 3.3275 | 34.9636 | 2 | 34.9641 | 2 | 251.7 | 2 | 260.7 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 10 | 9 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 2187.1 | 3.4969 | 34.9671 | 2 | 34.968 | 2 | 252.2 | 2 | 261.7 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 11 | 10 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 1988.9 | 3.6867 | 34.9713 | 2 | 34.9718 | 2 | 251.9 | 2 | 261.7 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 12 | 11 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 1791.2 | 3.8817 | 34.9727 | 2 | 34.9747 | 2 | 251.6 | 2 | 261 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 13 | 12 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 1488.3 | 4.5138 | 35.0233 | 2 | 35.0239 | 2 | 239.2 | 2 | 247.9 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 14 | 13 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 1289.1 | 5.2757 | 35.068 | 2 | 35.069 | 2 | 220.6 | 2 | 227.6 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 15 | 14 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 989.6 | 7.9335 | 35.1126 | 2 | 35.1131 | 2 | 149.2 | 2 | 151.8 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 16 | 15 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 788.3 | 11.7831 | 35.543 | 2 | 35.6831 | 4 | 155.7 | 2 | 158.5 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 17 | 16 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 662.3 | 14.4863 | 35.9313 | 2 | 35.945 | 2 | 173.1 | 2 | 168.3 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 18 | 17 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 529.5 | 16.8245 | 36.3151 | 2 | 36.3161 | 2 | 185 | 2 | 180.7 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 19 | 18 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 400.8 | 18.1796 | 36.5331 | 2 | 36.534 | 2 | 193.8 | 2 | 190.5 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 20 | 19 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 272.7 | 19.0137 | 36.6224 | 2 | 36.6234 | 2 | 201.4 | 2 | 200.1 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 21 | 20 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 170.5 | 20.7481 | 36.7803 | 2 | 36.7809 | 2 | 206.4 | 2 | 205.2 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 22 | 21 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 171.3 | 20.7085 | 36.774 | 2 | 36.7783 | 2 | 207.3 | 2 | 204.8 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 23 | 22 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 113 | 22.0809 | 36.8414 | 2 | 36.8416 | 2 | 209.3 | 2 | 207.9 | 2 |
| 31WBTSOC | AB01 | 21 | 1 | 24 | 23 | 2 | 20010501 | 311 | 26.503 | -74.2487 | 4549 | 54.6 | 23.5125 | 36.7471 | 2 | 36.7477 | 2 | 212.4 | 2 | 211.2 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 1 | 24 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 4716.7 | 2.2285 | 34.8805 | 2 | 34.8817 | 2 | 259.4 | 2 | 258.9 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 2 | 1 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 4448 | 2.2433 | 34.8864 | 2 | 34.8864 | 2 | 260.3 | 2 | 261.8 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 3 | 2 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 4198.4 | 2.2602 | 34.8904 | 2 | 34.8901 | 2 | 260.7 | 2 | 264 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 4 | 3 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 3898.5 | 2.2913 | 34.8966 | 2 | 34.8964 | 2 | 261.2 | 2 | 266.1 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 5 | 4 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 3547.6 | 2.3894 | 34.9053 | 2 | 34.9055 | 2 | 261 | 2 | 267.6 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 6 | 5 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 3124.7 | 2.6372 | 34.9206 | 2 | 34.9208 | 2 | 259.1 | 2 | 267.3 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 7 | 6 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 2747.3 | 2.9464 | 34.94 | 2 | 34.9403 | 2 | 254.7 | 2 | 264.1 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 8 | 7 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 2447.8 | 3.2559 | 34.9566 | 2 | 34.9574 | 2 | 252.7 | 2 | 261.7 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 9 | 8 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 2149.4 | 3.5535 | 34.9689 | 2 | 34.969 | 2 | 251.7 | 2 | 261.7 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 10 | 9 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 1841.8 | 3.7869 | 34.9717 | 2 | 34.9721 | 2 | 251.6 | 2 | 261.6 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 11 | 10 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 1549.5 | 4.3449 | 35.011 | 2 | 35.0117 | 2 | 242 | 2 | 251.4 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 12 | 11 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 1252.1 | 5.3378 | 35.0723 | 2 | 35.0731 | 2 | 217.1 | 2 | 224.2 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 13 | 12 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 1049.6 | 7.1041 | 35.0934 | 2 | 35.0934 | 2 | 167.7 | 2 | 170.6 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 14 | 13 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 899.9 | 9.183 | 35.2201 | 2 | 35.2204 | 2 | 142.8 | 2 | 144.8 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 15 | 14 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 750.1 | 12.217 | 35.5968 | 2 | 35.5935 | 2 | 154.9 | 2 | 153.2 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 16 | 15 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 599.2 | 15.3561 | 36.0699 | 2 | 36.0696 | 2 | 177.2 | 2 | 173.1 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 17 | 16 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 473.4 | 17.3847 | 36.4088 | 2 | 36.5669 | 2 | 188.9 | 2 | 185.7 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 18 | 17 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 351.1 | 18.442 | 36.5677 | 2 | 36.6373 | 2 | 193.1 | 2 | 191.3 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 19 | 18 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 248.1 | 19.1641 | 36.6377 | 2 | 36.7325 | 2 | 197.7 | 2 | 196 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 20 | 19 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 202.2 | 20.1873 | 36.7343 | 2 | 36.8116 | 2 | 196.1 | 2 | 183.2 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 21 | 20 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 150.9 | 21.2736 | 36.8153 | 2 | 36.8124 | 2 | 200 | 2 | 198.8 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 22 | 21 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 100.7 | 22.3391 | 36.8371 | 2 | 36.836 | 2 | 213.3 | 2 | 211.5 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 23 | 22 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 49.9 | 23.2501 | 36.727 | 2 | 36.7263 | 2 | 212.6 | 2 | 212.1 | 2 |
| 31WBTSOC | AB01 | 22 | 1 | 24 | 23 | 2 | 20010501 | 827 | 26.5015 | -73.9658 | 4655 | 3.7 | 23.2329 | 36.7282 | 2 | 36.1499 | 4 | 210.2 | 2 | 211.8 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 1 | 24 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 4936 | 2.2089 | 34.8748 | 2 | 34.8746 | 2 | 256.8 | 2 | 256.2 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 2 | 1 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 4800 | 2.2431 | 34.8812 | 2 | 34.8815 | 2 | 258.7 | 2 | 259.1 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 3 | 2 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 4550.4 | 2.2684 | 34.8879 | 2 | 34.8881 | 2 | 260.2 | 2 | 262.5 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 4 | 3 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 4145.9 | 2.2969 | 34.8946 | 2 | 34.894 | 2 | 261.1 | 2 | 265.5 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 5 | 4 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 3799.6 | 2.3821 | 34.903 | 2 | 34.9033 | 2 | 261.6 | 2 | 267.8 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 6 | 5 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 3497.8 | 2.4999 | 34.9122 | 2 | 34.9119 | 2 | 260.2 | 2 | 267.3 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 7 | 6 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 3149.8 | 2.7237 | 34.9264 | 2 | 34.9271 | 2 | 257.6 | 2 | 265.6 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 8 | 7 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 2799.7 | 2.9795 | 34.9412 | 2 | 34.9411 | 2 | 254.8 | 2 | 263.6 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 9 | 8 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 2551.2 | 3.187 | 34.9524 | 2 | 34.9529 | 2 | 253.5 | 2 | 253.4 | 3 |
| 31WBTSOC | AB01 | 23 | 1 | 10 | 9 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 2251 | 3.4637 | 34.9645 | 2 | 34.9647 | 2 | 252.2 | 2 | 262 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 11 | 10 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 1899.4 | 3.7678 | 34.9722 | 2 | 34.9718 | 2 | 251.5 | 2 | 261.7 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 12 | 11 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 1584.3 | 4.1912 | 34.9938 | 2 | 34.9946 | 2 | 245.3 | 2 | 255.2 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 13 | 12 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 1273 | 5.3478 | 35.0808 | 2 | 35.0816 | 2 | 215.9 | 2 | 223 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 14 | 13 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 999.2 | 7.6108 | 35.1155 | 2 | -999 | 9 | 157.9 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 23 | 1 | 15 | 14 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 850.6 | 9.8838 | 35.299 | 2 | 35.2985 | 2 | 143.7 | 2 | 144.7 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 16 | 15 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 699.1 | 13.1705 | 35.7341 | 2 | 35.7326 | 2 | 160 | 2 | 156.1 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 17 | 16 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 550.7 | 16.1351 | 36.2031 | 2 | 36.2057 | 2 | 180.8 | 2 | 175.5 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 18 | 17 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 400.3 | 18.1221 | 36.5277 | 2 | 36.5302 | 2 | 195.2 | 2 | 190.5 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 19 | 18 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 299.6 | 18.9115 | 36.6203 | 2 | 36.622 | 2 | 207.2 | 2 | 203.6 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 20 | 19 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 226.4 | 19.3852 | 36.6564 | 2 | 36.6583 | 2 | 196.4 | 2 | 195.7 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 21 | 20 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 160.5 | 20.8193 | 36.7875 | 2 | 36.788 | 2 | 198.4 | 2 | 196.9 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 22 | 21 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 110.2 | 21.6972 | 36.8123 | 2 | 36.814 | 2 | 218.1 | 2 | 216 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 23 | 22 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 50.5 | 22.5556 | 36.8479 | 2 | 36.8553 | 2 | 215.6 | 2 | 214.7 | 2 |
| 31WBTSOC | AB01 | 23 | 1 | 24 | 23 | 2 | 20010501 | 1417 | 26.501 | -73.5823 | 4868 | 2 | 22.5624 | 36.8467 | 2 | 36.8585 | 3 | 213.5 | 2 | 214.7 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 1 | 24 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 5133 | 2.1232 | 34.8615 | 2 | 34.862 | 2 | 251 | 2 | 251.9 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 2 | 1 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 4901.7 | 2.2531 | 34.8809 | 2 | 34.8815 | 2 | 257.7 | 2 | 259.3 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 3 | 2 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 4601.9 | 2.2821 | 34.8884 | 2 | 34.8883 | 2 | 259.8 | 2 | 262.7 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 4 | 3 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 4249.1 | 2.3044 | 34.894 | 2 | 34.8938 | 2 | 260.2 | 2 | 265.1 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 5 | 4 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 3899.2 | 2.3507 | 34.9004 | 2 | 34.8999 | 2 | 260.4 | 2 | 267.2 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 6 | 5 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 3548.3 | 2.4825 | 34.9103 | 2 | 34.9104 | 2 | 260 | 2 | 267.6 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 7 | 6 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 3250.8 | 2.6684 | 34.9224 | 2 | 34.9243 | 2 | 257.1 | 2 | 265.7 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 8 | 7 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 2950 | 2.8746 | 34.936 | 2 | 34.9359 | 2 | 254.5 | 2 | 263.6 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 9 | 8 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 2651.3 | 3.102 | 34.9511 | 2 | 34.9507 | 2 | 251.7 | 2 | 261.6 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 10 | 9 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 2348.6 | 3.4022 | 34.9653 | 2 | 34.9659 | 2 | 251.2 | 2 | 261.2 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 11 | 10 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 2049.7 | 3.6547 | 34.969 | 2 | 34.9684 | 2 | 251.8 | 2 | 262 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 12 | 11 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 1751.3 | 3.9495 | 34.9774 | 2 | 34.9773 | 2 | 249.8 | 2 | 260 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 13 | 12 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 1426.7 | 4.6133 | 35.0296 | 2 | 35.0308 | 2 | 235.8 | 2 | 245 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 14 | 13 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 1102.9 | 6.3165 | 35.0855 | 2 | 35.0843 | 2 | 184.7 | 2 | 190.9 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 15 | 14 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 877.7 | 9.0908 | 35.2176 | 2 | 35.219 | 2 | 144.6 | 2 | 146.3 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 16 | 15 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 727.6 | 12.3998 | 35.6327 | 2 | 35.6339 | 2 | 161.9 | 2 | 159.3 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 17 | 16 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 575.7 | 15.3939 | 36.077 | 2 | 36.0779 | 2 | 178.4 | 2 | 173.9 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 18 | 17 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 424.8 | 17.7512 | 36.4708 | 2 | 36.4706 | 2 | 193.2 | 2 | 189 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 19 | 18 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 276.5 | 18.8396 | 36.6119 | 2 | 36.6165 | 2 | 211.1 | 2 | 208.7 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 20 | 19 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 211 | 19.0863 | 36.6443 | 2 | 36.643 | 2 | 214.4 | 2 | 213.3 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 21 | 20 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 148.6 | 19.5674 | 36.6827 | 2 | 36.6696 | 3 | 220 | 2 | 219.2 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 22 | 21 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 99.8 | 20.3247 | 36.7343 | 2 | 36.7348 | 2 | 202.4 | 2 | 202.9 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 23 | 22 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 51.5 | 21.5469 | 36.8121 | 2 | 36.814 | 2 | 220.1 | 2 | 210.7 | 2 |
| 31WBTSOC | AB01 | 24 | 1 | 24 | 23 | 2 | 20010501 | 1924 | 26.5002 | -73.2195 | 5059 | 2.8 | 22.0238 | 36.8226 | 2 | 36.8219 | 2 | 218.4 | 2 | 216.5 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 1 | 24 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 5163.8 | 2.0469 | 34.8516 | 2 | 34.8516 | 2 | 247.9 | 2 | 252.3 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 2 | 1 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 4998.2 | 2.1333 | 34.8652 | 2 | 34.8643 | 2 | 250.9 | 2 | 255.6 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 3 | 2 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 4698.9 | 2.2646 | 34.886 | 2 | 34.8852 | 2 | 258.9 | 2 | 263.3 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 4 | 3 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 4299.8 | 2.2956 | 34.8934 | 2 | 34.8921 | 2 | 261.3 | 2 | 270.4 | 4 |
| 31WBTSOC | AB01 | 25 | 1 | 5 | 4 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 3948.7 | 2.3382 | 34.8998 | 2 | 34.8984 | 2 | 261.4 | 2 | 268.6 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 6 | 5 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 3596.6 | 2.4571 | 34.9093 | 2 | 34.9086 | 2 | 260.9 | 2 | 269.7 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 7 | 6 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 3199 | 2.6619 | 34.9229 | 2 | 34.9226 | 2 | 258.3 | 2 | 268.3 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 8 | 7 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 2900.1 | 2.9053 | 34.9385 | 2 | 34.9385 | 2 | 254.8 | 2 | 265.7 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 9 | 8 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 2600.7 | 3.17 | 34.9541 | 2 | 34.9537 | 2 | 252.7 | 2 | 263.2 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 10 | 9 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 2302.7 | 3.4167 | 34.966 | 2 | 34.9653 | 2 | 251.6 | 2 | 264.7 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 11 | 10 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 2000.6 | 3.6577 | 34.9681 | 2 | 34.967 | 2 | 252.9 | 2 | 263.9 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 12 | 11 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 1700 | 3.9804 | 34.9807 | 2 | 34.9802 | 2 | 249.7 | 2 | 261.4 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 13 | 12 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 1400.4 | 4.6333 | 35.0292 | 2 | 35.0288 | 2 | 235.5 | 2 | 247.1 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 14 | 13 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 1148.1 | 6.0048 | 35.0875 | 2 | 35.0883 | 2 | 195.6 | 2 | 203.8 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 15 | 14 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 900.2 | 8.4357 | 35.1615 | 2 | 35.1615 | 2 | 147.7 | 2 | 150.4 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 16 | 15 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 748.8 | 11.5168 | 35.5034 | 2 | 35.5041 | 2 | 152.3 | 2 | 151.5 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 17 | 16 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 601.7 | 15.1291 | 36.0375 | 2 | 36.0373 | 2 | 174.7 | 2 | 169.9 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 18 | 17 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 452.9 | 17.5164 | 36.4293 | 2 | 36.4278 | 2 | 191.5 | 2 | 186.8 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 19 | 18 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 301 | 18.8084 | 36.608 | 2 | 36.6067 | 2 | 206.4 | 2 | 204 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 20 | 19 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 202.1 | 19.3573 | 36.6659 | 2 | 36.665 | 2 | 208.9 | 2 | 208.2 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 21 | 20 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 155.2 | 19.9467 | 36.7043 | 2 | 36.7079 | 2 | 197.2 | 2 | 196.8 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 22 | 21 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 111.6 | 20.878 | 36.7799 | 2 | 36.7805 | 2 | 206.3 | 2 | 205 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 23 | 22 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 60.5 | 21.8242 | 36.8031 | 2 | 36.8041 | 2 | 217.6 | 2 | 215.9 | 2 |
| 31WBTSOC | AB01 | 25 | 1 | 24 | 23 | 2 | 20010502 | 1544 | 26.5595 | -72.8512 | 5089 | 3.1 | 23.0434 | 36.8381 | 2 | 36.8352 | 2 | 213.8 | 2 | 213.6 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 1 | 24 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 5234.9 | 2.1077 | 34.8588 | 2 | 34.8584 | 2 | 251.7 | 2 | 251.6 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 2 | 1 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 5098.9 | 2.2676 | 34.8804 | 2 | 34.8793 | 2 | 259.2 | 2 | 258.7 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 3 | 2 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 4799.4 | 2.2762 | 34.8859 | 2 | -999 | 9 | 260.7 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 26 | 1 | 4 | 3 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 4498.8 | 2.289 | 34.8905 | 2 | 34.8895 | 2 | 260.8 | 2 | 263.5 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 5 | 4 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 4195.5 | 2.2913 | 34.8936 | 2 | 34.8933 | 2 | 260.5 | 2 | 265 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 6 | 5 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 3799.2 | 2.3741 | 34.903 | 2 | 34.9015 | 2 | 261.2 | 2 | 267.5 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 7 | 6 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 3400.6 | 2.5358 | 34.9149 | 2 | 34.9147 | 2 | 259.7 | 2 | 267.4 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 8 | 7 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 2998.4 | 2.8031 | 34.9317 | 2 | 34.9308 | 2 | 256.8 | 2 | 265.4 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 9 | 8 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 2692.9 | 3.0411 | 34.9471 | 2 | 34.946 | 2 | 253.2 | 2 | 262.6 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 10 | 9 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 2400 | 3.3216 | 34.9642 | 2 | 34.9629 | 2 | 251.2 | 2 | 261.1 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 11 | 10 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 2100.8 | 3.5739 | 34.973 | 2 | 34.9718 | 2 | 250.8 | 2 | 261 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 12 | 11 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 1798.7 | 3.8959 | 34.9776 | 2 | 34.9775 | 2 | 250.4 | 2 | 260.6 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 13 | 12 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 1449.5 | 4.603 | 35.0346 | 2 | 35.0343 | 2 | 236.3 | 2 | 245.5 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 14 | 13 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 1099.5 | 6.1941 | 35.078 | 2 | -999 | 9 | 185.7 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 26 | 1 | 15 | 14 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 902 | 8.391 | 35.1522 | 2 | 35.1507 | 2 | 146.3 | 2 | 148.9 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 16 | 15 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 698.6 | 12.2903 | 35.6075 | 2 | 35.6124 | 2 | 153.4 | 2 | 153.6 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 17 | 16 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 549.3 | 15.5899 | 36.1026 | 2 | 36.1008 | 2 | 176.7 | 2 | 171.8 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 18 | 17 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 400.6 | 17.798 | 36.4816 | 2 | 36.4807 | 2 | 195.8 | 2 | 190.7 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 19 | 18 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 311.4 | 18.4087 | 36.5656 | 2 | 36.5651 | 2 | 194.9 | 2 | 191.8 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 20 | 19 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 242.1 | 18.7464 | 36.5944 | 2 | 36.5923 | 2 | 200.1 | 2 | 198.3 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 21 | 20 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 178.6 | 19.1002 | 36.6195 | 2 | 36.6165 | 2 | 197.8 | 2 | 196.5 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 22 | 21 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 119.6 | 20.0368 | 36.7146 | 2 | 36.7083 | 2 | 197.3 | 2 | 194.9 | 2 |
| 31WBTSOC | AB01 | 26 | 1 | 23 | 22 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 55.5 | 21.594 | 36.8046 | 2 | -999 | 9 | 218.7 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 26 | 1 | 24 | 23 | 2 | 20010502 | 2227 | 26.5015 | -72.4652 | 5158 | 4.4 | 22.408 | 36.8603 | 2 | 36.859 | 2 | 215.7 | 2 | 215.6 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 1 | 24 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 5365.2 | 2.0638 | 34.8505 | 2 | 34.8514 | 2 | 248.8 | 2 | 248.7 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 2 | 1 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 5199.2 | 2.1699 | 34.8665 | 2 | 34.8659 | 2 | 251.7 | 2 | 253.6 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 3 | 2 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 4903.6 | 2.2757 | 34.8851 | 2 | 34.884 | 2 | 258.6 | 2 | 260.5 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 4 | 3 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 4597.1 | 2.2908 | 34.8899 | 2 | 34.8893 | 2 | 260.2 | 2 | 263.7 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 5 | 4 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 4298 | 2.3153 | 34.8954 | 2 | 34.894 | 2 | 261 | 2 | 265.6 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 6 | 5 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 3899.4 | 2.3675 | 34.9019 | 2 | 34.9017 | 2 | 260.9 | 2 | 267.7 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 7 | 6 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 3497.9 | 2.495 | 34.9121 | 2 | 34.9115 | 2 | 259.7 | 2 | 267.9 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 8 | 7 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 3098.5 | 2.7608 | 34.9296 | 2 | 34.9287 | 2 | 255.6 | 2 | 264.6 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 9 | 8 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 2801.3 | 2.9684 | 34.9426 | 2 | 34.9422 | 2 | 253.4 | 2 | 262.7 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 10 | 9 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 2499.1 | 3.2388 | 34.9598 | 2 | 34.9596 | 2 | 251.4 | 2 | 261 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 11 | 10 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 2199.9 | 3.4758 | 34.9678 | 2 | 34.9678 | 2 | 251.7 | 2 | 261.6 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 12 | 11 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 1898.5 | 3.773 | 34.9783 | 2 | 34.9781 | 2 | 250.3 | 2 | 260.6 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 13 | 12 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 1496.2 | 4.3828 | 35.0161 | 2 | 35.0151 | 2 | 240.9 | 2 | 250.7 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 14 | 13 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 1199.7 | 5.6398 | 35.0878 | 2 | 35.0865 | 2 | 209.4 | 2 | 216.2 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 15 | 14 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 997 | 7.1423 | 35.0992 | 2 | 35.0999 | 2 | 167.3 | 2 | 171.3 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 16 | 15 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 803.8 | 10.112 | 35.3319 | 2 | 35.3314 | 2 | 145.7 | 2 | 146.9 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 17 | 16 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 614.3 | 14.2118 | 35.8951 | 2 | 35.8984 | 2 | 170.2 | 2 | 169.2 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 18 | 17 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 501.9 | 16.3419 | 36.2392 | 2 | 36.2385 | 2 | 181.6 | 2 | 178.2 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 19 | 18 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 354.5 | 18.0022 | 36.5132 | 2 | 36.5136 | 2 | 193.6 | 2 | 191.3 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 20 | 19 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 249.8 | 18.8033 | 36.6083 | 2 | 36.6048 | 2 | 199.4 | 2 | 197.9 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 21 | 20 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 149.5 | 20.2531 | 36.7269 | 2 | 36.7241 | 2 | 201 | 2 | 199.4 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 22 | 21 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 101 | 21.3891 | 36.8011 | 2 | 36.7989 | 2 | 215.1 | 2 | 213.6 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 23 | 22 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 49.3 | 23.9096 | 36.7836 | 2 | 36.7912 | 2 | 213.7 | 2 | 211.6 | 2 |
| 31WBTSOC | AB01 | 27 | 1 | 24 | 23 | 2 | 20010503 | 427 | 26.5002 | -72.1017 | 5284 | 3.3 | 23.9602 | 36.7886 | 2 | 36.787 | 2 | 208.6 | 2 | 209.4 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 1 | 24 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 5467.4 | 2.0726 | 34.8501 | 2 | 34.8508 | 2 | 248 | 2 | 248.8 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 2 | 1 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 5300.5 | 2.2102 | 34.87 | 2 | 34.8708 | 2 | 254.3 | 2 | 254.7 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 3 | 2 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 4996.7 | 2.2946 | 34.8847 | 2 | -999 | 9 | 260 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 28 | 1 | 4 | 3 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 4698.4 | 2.3092 | 34.8901 | 2 | 34.8907 | 2 | 260.9 | 2 | 263.3 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 5 | 4 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 4395.7 | 2.3118 | 34.8932 | 2 | 34.894 | 2 | 260.8 | 2 | 265.2 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 6 | 5 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 4092.8 | 2.3359 | 34.8986 | 2 | 34.899 | 2 | 261.2 | 2 | 267.1 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 7 | 6 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 3697.2 | 2.4274 | 34.9064 | 2 | 34.9064 | 2 | 260.6 | 2 | 268.4 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 8 | 7 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 3297.4 | 2.6178 | 34.9203 | 2 | 34.9212 | 2 | 257.3 | 2 | 265.8 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 9 | 8 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 2899.5 | 2.8718 | 34.9371 | 2 | 34.9385 | 2 | 253.9 | 2 | 263.1 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 10 | 9 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 2600.2 | 3.0935 | 34.9516 | 2 | 34.9523 | 2 | 252 | 2 | 261.8 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 11 | 10 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 2300.6 | 3.3722 | 34.9639 | 2 | 34.9649 | 2 | 251.4 | 2 | 261.8 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 12 | 11 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 2000.4 | 3.6035 | 34.9697 | 2 | 34.9706 | 2 | 251.7 | 2 | 262.4 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 13 | 12 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 1594.9 | 4.1646 | 35.0035 | 2 | 35.0771 | 2 | 244.8 | 2 | 230.8 | 3 |
| 31WBTSOC | AB01 | 28 | 1 | 14 | 13 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 1299.8 | 5.1225 | 35.0762 | 2 | 35.0775 | 2 | 223.1 | 2 | 231.3 | 3 |
| 31WBTSOC | AB01 | 28 | 1 | 15 | 14 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 1052.4 | 6.6495 | 35.1044 | 2 | 35.1048 | 2 | 179.3 | 2 | 184.3 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 16 | 15 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 850.5 | 9.2174 | 35.2324 | 2 | 35.2322 | 2 | 144.6 | 2 | 138 | 4 |
| 31WBTSOC | AB01 | 28 | 1 | 17 | 16 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 599.7 | 14.6423 | 35.9636 | 2 | 35.9614 | 2 | 172 | 2 | 167.1 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 18 | 17 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 450.9 | 17.0385 | 36.3535 | 2 | 36.3535 | 2 | 190 | 2 | 185.5 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 19 | 18 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 299.7 | 18.4036 | 36.5692 | 2 | 36.5695 | 2 | 190.3 | 2 | 188.3 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 20 | 19 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 250.8 | 18.8043 | 36.6022 | 2 | 36.6024 | 2 | 194.5 | 2 | 192.9 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 21 | 20 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 174 | 19.7963 | 36.6902 | 2 | 36.6926 | 2 | 200.1 | 2 | 199.7 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 22 | 21 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 117.8 | 21.4463 | 36.8332 | 2 | 36.8329 | 2 | 210.5 | 2 | 207.9 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 23 | 22 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 59.1 | 23.9901 | 36.8164 | 2 | 36.8257 | 2 | 214.1 | 2 | 210.8 | 2 |
| 31WBTSOC | AB01 | 28 | 1 | 24 | 23 | 2 | 20010503 | 1121 | 26.5033 | -71.7317 | 5383 | 2.6 | 23.9792 | 36.8206 | 2 | 36.8202 | 2 | 208.4 | 2 | 210.5 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 1 | 24 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 5582.8 | 2.0701 | 34.848 | 2 | 34.848 | 2 | 246 | 2 | 249.5 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 2 | 1 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 5346.9 | 2.1844 | 34.8664 | 2 | 34.8651 | 2 | 251.6 | 2 | 254.2 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 3 | 2 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 5102.4 | 2.2629 | 34.8799 | 2 | 34.8785 | 2 | 256.2 | 2 | 259.1 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 4 | 3 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 4799.2 | 2.2898 | 34.8871 | 2 | 34.8872 | 2 | 258.3 | 2 | 262.7 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 5 | 4 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 4498.2 | 2.301 | 34.8922 | 2 | 34.8907 | 2 | 259.7 | 2 | 264.5 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 6 | 5 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 4198.8 | 2.3091 | 34.8955 | 2 | 34.8942 | 2 | 259.6 | 2 | 266 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 7 | 6 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 3798.6 | 2.3603 | 34.9023 | 2 | 34.9007 | 2 | 259.4 | 2 | 267.5 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 8 | 7 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 3397.9 | 2.4865 | 34.9129 | 2 | 34.9117 | 2 | 257.9 | 2 | 267.4 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 9 | 8 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 3000.8 | 2.7079 | 34.9268 | 2 | 34.9237 | 3 | 255.2 | 2 | 265.5 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 10 | 9 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 2698 | 2.937 | 34.9416 | 2 | 34.9417 | 2 | 252.7 | 2 | 263.5 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 11 | 10 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 2398.5 | 3.2317 | 34.9611 | 2 | 34.9607 | 2 | 249.8 | 2 | 260.2 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 12 | 11 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 2051.7 | 3.5317 | 34.9719 | 2 | 34.971 | 2 | 250.1 | 2 | 261.3 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 13 | 12 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 1699.6 | 4.0095 | 35.0024 | 2 | 35.0025 | 2 | 245.8 | 2 | 256.1 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 14 | 13 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 1400.4 | 4.6906 | 35.0408 | 2 | 35.0406 | 2 | 234.1 | 2 | 243.8 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 15 | 14 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 1100.6 | 6.3383 | 35.1039 | 2 | 35.1036 | 2 | 190 | 2 | 196.8 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 16 | 15 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 923.9 | 8.026 | 35.1173 | 2 | 35.1149 | 2 | 145.7 | 2 | 149.2 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 17 | 16 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 674.4 | 12.897 | 35.7119 | 2 | 35.7109 | 2 | 166.3 | 2 | 163.5 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 18 | 17 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 526 | 15.8412 | 36.1573 | 2 | 36.1586 | 2 | 184.7 | 2 | 181.1 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 19 | 18 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 375.8 | 17.8833 | 36.4949 | 2 | 36.4944 | 2 | 194.6 | 2 | 192 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 20 | 19 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 275.2 | 18.645 | 36.585 | 2 | 36.5833 | 2 | 199.5 | 2 | 197.7 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 21 | 20 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 166.6 | 20.4421 | 36.7488 | 2 | 36.7497 | 2 | 211.8 | 2 | 208.5 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 22 | 21 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 110.3 | 22.3801 | 36.9194 | 2 | 41.6036 | 4 | 211.8 | 2 | 209.8 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 23 | 22 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 54.6 | 23.7272 | 36.8611 | 2 | 36.8606 | 2 | 210.5 | 2 | 210.7 | 2 |
| 31WBTSOC | AB01 | 29 | 1 | 24 | 23 | 2 | 20010503 | 1808 | 26.5027 | -71.3513 | 5495 | 4.1 | 23.7326 | 36.8566 | 2 | 36.8547 | 2 | 208.2 | 2 | 211.1 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 1 | 24 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 5583.3 | 2.0749 | 34.8484 | 2 | 34.849 | 2 | 246.7 | 2 | 248.6 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 2 | 1 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 5197.2 | 2.22 | 34.8729 | 2 | 34.8728 | 2 | 253.3 | 2 | 256.4 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 3 | 2 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 4897.2 | 2.2735 | 34.8842 | 2 | 34.8852 | 2 | 256.9 | 2 | 260.6 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 4 | 3 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 4548.7 | 2.2932 | 34.8908 | 2 | 34.8901 | 2 | 258.6 | 2 | 263.9 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 5 | 4 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 4198.1 | 2.3103 | 34.896 | 2 | 34.8956 | 2 | 259.1 | 2 | 265.6 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 6 | 5 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 3850.7 | 2.3613 | 34.9024 | 2 | 34.9021 | 2 | 257.9 | 2 | 266.1 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 7 | 6 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 3496.9 | 2.4756 | 34.9119 | 2 | 34.9121 | 2 | 256 | 2 | 265.3 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 8 | 7 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 3148.9 | 2.6579 | 34.9249 | 2 | 34.9241 | 2 | 252 | 2 | 262.3 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 9 | 8 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 2802.8 | 2.8931 | 34.9398 | 2 | 34.9391 | 2 | 251.8 | 2 | 262 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 10 | 9 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 2499.4 | 3.1228 | 34.9526 | 2 | 34.9511 | 2 | 251.2 | 2 | 262.3 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 11 | 10 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 2200.3 | 3.4397 | 34.9737 | 2 | 34.9729 | 2 | 248.6 | 2 | 259.5 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 12 | 11 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 1903.8 | 3.679 | 34.9705 | 2 | 34.9714 | 2 | 251.3 | 2 | 262.8 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 13 | 12 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 1653.2 | 4.0014 | 34.9893 | 2 | 34.9893 | 2 | 247.7 | 2 | 258.6 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 14 | 13 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 1401.6 | 4.6699 | 35.0437 | 2 | 35.0446 | 2 | 234.9 | 2 | 244.2 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 15 | 14 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 1151.9 | 5.9387 | 35.1004 | 2 | 35.0999 | 2 | 199.4 | 2 | 206 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 16 | 15 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 898.9 | 8.6001 | 35.1695 | 2 | 35.172 | 2 | 144.3 | 2 | 146.9 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 17 | 16 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 704.2 | 12.2812 | 35.6225 | 2 | 35.6331 | 2 | 161.4 | 2 | 160.4 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 18 | 17 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 553.9 | 15.3555 | 36.079 | 2 | 36.0842 | 2 | 179.9 | 2 | 177.2 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 19 | 18 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 327.6 | 18.36 | 36.5544 | 2 | 36.5546 | 2 | 195.8 | 2 | 194.4 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 20 | 19 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 225.2 | 19.0179 | 36.6162 | 2 | 36.6143 | 2 | 198.9 | 2 | 197.7 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 21 | 20 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 148.5 | 20.8987 | 36.8446 | 2 | 36.84 | 2 | 197 | 2 | 194.8 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 22 | 21 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 89.5 | 22.93 | 36.9052 | 2 | 36.9069 | 2 | 216.2 | 2 | 212.5 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 23 | 22 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 43.5 | 23.6607 | 36.8615 | 2 | 36.8604 | 2 | 212.3 | 2 | 211.7 | 2 |
| 31WBTSOC | AB01 | 30 | 1 | 24 | 23 | 2 | 20010504 | 38 | 26.5042 | -71.002 | 5496 | 3.1 | 23.6569 | 36.8616 | 2 | 36.8602 | 2 | 209.4 | 2 | 211.6 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 1 | 24 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 5591.5 | 2.0823 | 34.849 | 2 | 34.85 | 2 | 247.1 | 2 | 248.8 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 2 | 1 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 5297.3 | 2.2457 | 34.8743 | 2 | 34.8744 | 2 | 255.4 | 2 | 257.2 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 3 | 2 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 4948 | 2.296 | 34.8859 | 2 | -999 | 9 | 259.7 | 2 | -999 | 9 |
| 31WBTSOC | AB01 | 31 | 1 | 4 | 3 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 4599.1 | 2.3036 | 34.8911 | 2 | 34.8913 | 2 | 259.8 | 2 | 264.1 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 5 | 4 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 4297.6 | 2.3137 | 34.8949 | 2 | 34.8952 | 2 | 259.9 | 2 | 266.2 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 6 | 5 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 3898.4 | 2.3475 | 34.9007 | 2 | 34.9017 | 2 | 259.6 | 2 | 267.2 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 7 | 6 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 3548 | 2.4436 | 34.9092 | 2 | 34.9094 | 2 | 258.7 | 2 | 267.6 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 8 | 7 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 3199.8 | 2.6084 | 34.9202 | 2 | 34.9208 | 2 | 256.1 | 2 | 266.2 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 9 | 8 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 2851.3 | 2.8514 | 34.9373 | 2 | 34.9385 | 2 | 251.8 | 2 | 262.4 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 10 | 9 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 2449.5 | 3.1971 | 34.9584 | 2 | 34.9588 | 2 | 250.1 | 2 | 261.2 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 11 | 10 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 2048.7 | 3.5672 | 34.9738 | 2 | 34.9755 | 2 | 249.7 | 2 | 260.8 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 12 | 11 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 1696.5 | 4.0129 | 34.9935 | 2 | 34.9942 | 2 | 246.9 | 2 | 258 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 13 | 12 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 1374.1 | 4.8357 | 35.0521 | 2 | 35.0523 | 2 | 230.9 | 2 | 240.3 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 14 | 13 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 1101.9 | 6.5159 | 35.1062 | 2 | 35.1058 | 2 | 183 | 2 | 189 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 15 | 14 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 949.7 | 7.6595 | 35.0846 | 2 | 35.0839 | 2 | 147.2 | 2 | 151.3 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 16 | 15 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 755.4 | 11.6233 | 35.5348 | 2 | 35.5414 | 2 | 157.4 | 2 | 156.5 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 17 | 16 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 651.3 | 13.7362 | 35.8281 | 2 | 35.8334 | 2 | 169.1 | 2 | 167.9 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 18 | 17 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 502.3 | 16.7449 | 36.3068 | 2 | 36.3054 | 2 | 190.4 | 2 | 185.6 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 19 | 18 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 349.1 | 18.3446 | 36.5533 | 2 | 36.5501 | 2 | 194.1 | 2 | 192.2 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 20 | 19 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 248.3 | 18.9733 | 36.6115 | 2 | 37.2716 | 4 | 198 | 2 | 197.3 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 21 | 20 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 160.7 | 20.7026 | 36.7695 | 2 | 36.7719 | 2 | 207.5 | 2 | 205.7 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 22 | 21 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 110 | 21.9796 | 36.8294 | 2 | 36.8275 | 2 | 214.6 | 2 | 212.7 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 23 | 22 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 57.7 | 23.9592 | 36.804 | 2 | 36.8136 | 3 | 213 | 2 | 210.3 | 2 |
| 31WBTSOC | AB01 | 31 | 1 | 24 | 23 | 2 | 20010504 | 929 | 26.5052 | -70.6832 | 5504 | 3.4 | 23.9509 | 36.8063 | 2 | 36.8053 | 2 | 207.8 | 2 | 209.9 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 1 | 24 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 5591.5 | 2.0755 | 34.8484 | 2 | 34.8492 | 2 | 246.2 | 2 | 248.6 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 2 | 1 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 5397.1 | 2.1656 | 34.8628 | 2 | 34.8622 | 2 | 249.6 | 2 | 253.2 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 3 | 2 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 5097.5 | 2.2553 | 34.879 | 2 | 34.8793 | 2 | 255.3 | 2 | 258.5 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 4 | 3 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 4800.9 | 2.286 | 34.8869 | 2 | 34.8866 | 2 | 257.6 | 2 | 265.8 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 5 | 4 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 4450.6 | 2.3002 | 34.893 | 2 | 34.8933 | 2 | 259 | 2 | 265.1 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 6 | 5 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 4104.5 | 2.3213 | 34.8974 | 2 | 34.898 | 2 | 258.9 | 2 | 266.8 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 7 | 6 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 3673.4 | 2.3959 | 34.9052 | 2 | 34.9056 | 2 | 258.8 | 2 | 267.9 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 8 | 7 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 3251.3 | 2.5795 | 34.9187 | 2 | 34.9188 | 2 | 255.8 | 2 | 266.3 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 9 | 8 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 2851.6 | 2.8369 | 34.9366 | 2 | 34.9361 | 2 | 251.4 | 2 | 262.4 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 10 | 9 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 2498.5 | 3.121 | 34.9523 | 2 | 34.9531 | 2 | 251 | 2 | 262.4 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 11 | 10 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 2151.6 | 3.5577 | 34.9877 | 2 | 34.9867 | 2 | 245 | 2 | 256.8 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 12 | 11 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 1799.8 | 3.9006 | 34.9851 | 2 | 34.9847 | 2 | 248.6 | 2 | 259.5 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 13 | 12 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 1553.9 | 4.3596 | 35.0159 | 2 | 35.0202 | 3 | 241.1 | 2 | 250.8 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 14 | 13 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 1297.6 | 5.3064 | 35.0783 | 2 | 35.079 | 2 | 219.1 | 2 | 226.5 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 15 | 14 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 1050 | 6.9913 | 35.089 | 2 | 35.0883 | 2 | 168.3 | 2 | 173 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 16 | 15 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 798.3 | 11.3044 | 35.4888 | 2 | 35.4881 | 2 | 154.3 | 2 | 153.9 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 17 | 16 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 650 | 14.124 | 35.8871 | 2 | 35.8895 | 2 | 174.5 | 2 | 170.8 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 18 | 17 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 527.3 | 16.482 | 36.2654 | 2 | 36.2672 | 2 | 190.2 | 2 | 186.5 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 19 | 18 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 402.2 | 18.0738 | 36.5194 | 2 | 36.5193 | 2 | 194.8 | 2 | 192.2 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 20 | 19 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 280.6 | 19.0305 | 36.6168 | 2 | 36.6153 | 2 | 198.9 | 2 | 197.1 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 21 | 20 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 151.4 | 21.2787 | 36.8062 | 2 | 36.8061 | 2 | 210.8 | 2 | 210 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 22 | 21 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 98.8 | 22.6087 | 36.8654 | 2 | 36.8638 | 2 | 215.9 | 2 | 215.7 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 23 | 22 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 50.2 | 23.9202 | 36.8011 | 2 | 36.8011 | 2 | 211 | 2 | 210.5 | 2 |
| 31WBTSOC | AB01 | 32 | 1 | 24 | 23 | 2 | 20010504 | 1607 | 26.5003 | -70.2323 | 5503 | 3.8 | 24.0186 | 36.7699 | 2 | 36.7682 | 2 | 207.4 | 2 | 209.9 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 1 | 24 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 5592.4 | 2.0808 | 34.8485 | 2 | 34.8498 | 2 | 246.5 | 2 | 248.5 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 2 | 1 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 5199.4 | 2.2083 | 34.8714 | 2 | 34.8714 | 2 | 252.2 | 2 | 256.4 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 3 | 2 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 4847.6 | 2.2717 | 34.8842 | 2 | 34.8838 | 2 | 256.6 | 2 | 260.7 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 4 | 3 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 4499.1 | 2.2957 | 34.892 | 2 | 34.8909 | 2 | 257.1 | 2 | 247.7 | 3 |
| 31WBTSOC | AB01 | 33 | 1 | 5 | 4 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 4199.7 | 2.3239 | 34.8972 | 2 | 34.8966 | 2 | 256.8 | 2 | 263.8 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 6 | 5 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 3900.7 | 2.361 | 34.9019 | 2 | 34.9011 | 2 | 258.9 | 2 | 267.4 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 7 | 6 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 3599.9 | 2.445 | 34.9088 | 2 | 34.9092 | 2 | 258.5 | 2 | 268 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 8 | 7 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 3299.1 | 2.5804 | 34.9193 | 2 | 34.9182 | 2 | 252.2 | 2 | 261.7 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 9 | 8 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 3000.2 | 2.7735 | 34.9349 | 2 | 34.9359 | 2 | 244.7 | 2 | 255.6 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 10 | 9 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 2652.2 | 3.0292 | 34.9551 | 2 | 34.956 | 2 | 243.1 | 2 | 254.5 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 11 | 10 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 2300.4 | 3.2791 | 34.9661 | 2 | 34.9664 | 2 | 248.2 | 2 | 259.2 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 12 | 11 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 1949.9 | 3.7891 | 35.0023 | 2 | 35.0023 | 2 | 244 | 2 | 255.3 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 13 | 12 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 1601.7 | 4.4149 | 35.0358 | 2 | 35.0658 | 4 | 238.3 | 2 | 238.7 | 3 |
| 31WBTSOC | AB01 | 33 | 1 | 14 | 13 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 1252.7 | 5.5898 | 35.0885 | 2 | 35.0887 | 2 | 210 | 2 | 217.6 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 15 | 14 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 1051.7 | 7.2134 | 35.1049 | 2 | 35.1107 | 3 | 165 | 2 | 169.6 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 16 | 15 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 900.9 | 9.1024 | 35.2175 | 2 | 35.2175 | 2 | 144.2 | 2 | 146.3 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 17 | 16 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 751.1 | 12.1297 | 35.5997 | 2 | 35.6014 | 2 | 158.9 | 2 | 157.3 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 18 | 17 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 602.4 | 15.221 | 36.0572 | 2 | 36.0658 | 2 | 182.1 | 2 | 177.4 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 19 | 18 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 451.6 | 17.5255 | 36.4341 | 2 | 36.4351 | 2 | 193 | 2 | 188.9 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 20 | 19 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 302.4 | 18.7867 | 36.6004 | 2 | 36.6014 | 2 | 202.3 | 2 | 200.2 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 21 | 20 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 183.1 | 20.7992 | 36.7904 | 2 | 36.789 | 2 | 207.3 | 2 | 203.4 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 22 | 21 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 108.8 | 22.614 | 36.8631 | 2 | 36.8618 | 2 | 214.2 | 2 | 212.7 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 23 | 22 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 54.6 | 23.7377 | 36.8438 | 2 | 36.8432 | 2 | 211.5 | 2 | 211 | 2 |
| 31WBTSOC | AB01 | 33 | 1 | 24 | 23 | 2 | 20010504 | 2241 | 26.5007 | -69.8323 | 5504 | 2.9 | 23.7214 | 36.8453 | 2 | 36.8438 | 2 | 208.4 | 2 | 210.9 | 2 |

# Appendix C - Hydrographic - CTD Data

(This page is intentionally left blank)

ABACO-01 R.V. Oceanus

CTD Station 1 (CTD001)

Latitude 27.024N Longitude 79.919W

26-Apr-2001 20:31Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  26.019		  26.018		  36.228		 209.6			   0.004		  23.954

10			  26.020		  26.017		  36.229		 210.2			   0.039		  23.955

20			  25.987		  25.983		  36.234		 209.8			   0.079		  23.970

30			  25.497		  25.490		  36.298		 209.7			   0.117		  24.172

50			  23.451		  23.440		  36.456		 207.0			   0.185		  24.910

75			  18.003		  17.990		  36.233		 152.7			   0.243		  26.220

100			  15.361		  15.345		  36.007		 129.3			   0.280		  26.672

125			  11.994		  11.978		  35.537		 125.2			   0.311		  27.011

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

140			   24			  10.433		  10.416		  35.320		     NaN

127			    1			  11.874		  11.858		  35.532		     NaN

112			    3			  13.927		  13.911		  35.826		     NaN

77			    4			  18.088		  18.075		  36.249		     NaN

49			    6			  23.865		  23.855		  36.441		     NaN

25			   13			  25.998		  25.992		  36.240		     NaN

ABACO-01 R.V. Oceanus

CTD Station 2 (CTD002)

Latitude 26.442N Longitude 78.668W

26-Apr-2001 12:00Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  25.109		  25.109		  36.304		 213.3			   0.004		  24.294

10			  25.111		  25.109		  36.304		 213.7			   0.036		  24.294

20			  25.124		  25.120		  36.305		 213.5			   0.073		  24.291

30			  25.129		  25.123		  36.305		 211.9			   0.109		  24.291

50			  25.135		  25.124		  36.306		 211.9			   0.182		  24.291

75			  24.197		  24.181		  36.547		 199.2			   0.269		  24.759

100			  23.423		  23.403		  36.838		 200.4			   0.343		  25.211

125			  22.850		  22.824		  36.877		 196.9			   0.411		  25.409

150			  22.038		  22.008		  36.866		 197.0			   0.474		  25.634

200			  20.699		  20.661		  36.805		 187.3			   0.587		  25.961

250			  19.389		  19.343		  36.683		 182.3			   0.687		  26.220

300			  18.391		  18.338		  36.564		 186.6			   0.777		  26.386

400			  16.855		  16.789		  36.331		 178.9			   0.944		  26.587

500			  14.722		  14.646		  35.984		 163.3			   1.093		  26.809

600			  12.542		  12.459		  35.654		 150.9			   1.224		  27.008

700			  10.228		  10.143		  35.351		 144.2			   1.336		  27.202

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

749			   24			   9.475		   9.388		  35.275		   152.0

650			    1			  11.449		  11.365		  35.498		   154.4

450			    3			  15.989		  15.916		  36.183		   175.6

450			    3			  15.989		  15.916		  36.184		   175.6

375			    4			  17.308		  17.245		  36.402		   188.1

301			    5			  18.280		  18.227		  36.548		   193.6

225			    6			  20.026		  19.983		  36.745		   190.0

125			   13			  22.965		  22.940		  36.840		   196.1

74			   14			  24.049		  24.033		  36.576		   203.1

49			   15			  25.135		  25.124		  36.320		   205.0

4			   16			  25.111		  25.110		  36.309		   205.6

ABACO-01 R.V. Oceanus

CTD Station 3 (CTD003)

Latitude 26.325N Longitude 78.713W

27-Apr-2001 13:21Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  25.735		  25.735		  36.217		 208.0			   0.004		  24.035

10			  25.740		  25.737		  36.219		 208.4			   0.039		  24.035

20			  25.670		  25.665		  36.224		 208.0			   0.077		  24.061

30			  25.522		  25.516		  36.258		 206.7			   0.115		  24.134

50			  25.227		  25.216		  36.299		 207.1			   0.190		  24.257

75			  24.719		  24.703		  36.537		 189.5			   0.277		  24.594

100			  24.093		  24.071		  36.642		 187.0			   0.359		  24.863

125			  23.115		  23.089		  36.894		 195.4			   0.431		  25.345

150			  22.261		  22.231		  36.867		 196.0			   0.495		  25.572

200			  21.286		  21.247		  36.845		 186.6			   0.612		  25.831

250			  19.762		  19.716		  36.702		 183.0			   0.718		  26.136

300			  18.496		  18.443		  36.578		 183.5			   0.811		  26.370

400			  16.944		  16.877		  36.346		 179.6			   0.978		  26.577

500			  14.504		  14.429		  35.948		 164.2			   1.125		  26.829

600			  12.204		  12.123		  35.604		 155.2			   1.252		  27.034

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

676			   24			   9.099		   9.022		  35.268		   156.9

548			    1			  13.420		  13.341		  35.776		   156.0

451			    2			  15.710		  15.638		  36.138		   170.9

352			    3			  17.617		  17.556		  36.453		   185.4

251			    4			  19.750		  19.704		  36.711		   186.1

174			    5			  21.791		  21.756		  36.854		   194.6

124			    6			  23.398		  23.373		  36.914		   197.5

49			   12			  25.246		  25.235		  36.298		   203.6

3			   13			  25.731		  25.731		  36.220		   203.5

ABACO-01 R.V. Oceanus

CTD Station 4 (CTD004)

Latitude 26.240N Longitude 78.772W

27-Apr-2001 14:30Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  25.813		  25.812		  36.212		 208.2			   0.004		  24.007

10			  25.814		  25.812		  36.213		 207.5			   0.039		  24.008

20			  25.814		  25.810		  36.213		 207.6			   0.078		  24.008

30			  25.817		  25.810		  36.213		 207.6			   0.117		  24.008

50			  25.457		  25.446		  36.339		 199.3			   0.194		  24.216

75			  24.929		  24.913		  36.474		 198.3			   0.283		  24.482

100			  24.578		  24.556		  36.509		 191.3			   0.369		  24.617

125			  23.795		  23.769		  36.717		 175.5			   0.448		  25.011

150			  22.636		  22.606		  36.866		 195.8			   0.516		  25.464

200			  21.261		  21.222		  36.845		 185.2			   0.640		  25.838

250			  19.988		  19.941		  36.736		 184.0			   0.745		  26.103

300			  18.371		  18.318		  36.566		 184.1			   0.837		  26.392

400			  16.943		  16.877		  36.348		 179.2			   1.002		  26.579

500			  14.983		  14.906		  36.025		 167.6			   1.152		  26.783

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

510			   24			  14.725		  14.648		  35.987		   164.3

434			    1			  16.199		  16.128		  36.217		   174.2

359			    2			  17.473		  17.412		  36.430		   183.9

319			    3			  18.022		  17.966		  36.514		   186.9

280			    4			  18.864		  18.814		  36.616		   188.6

250			    5			  19.961		  19.915		  36.725		   186.2

180			    6			  22.196		  22.160		  36.852		   200.3

139			   12			  23.177		  23.148		  36.885		   196.9

49			   13			  25.468		  25.458		  36.354		   200.3

3			   14			  25.816		  25.815		  36.216		   203.5

ABACO-01 R.V. Oceanus

CTD Station 5 (CTD005)

Latitude 26.161N Longitude 78.806W

27-Apr-2001 15:44Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  25.789		  25.789		  36.221		 207.2			   0.004		  24.021

10			  25.788		  25.786		  36.222		 207.2			   0.039		  24.022

20			  25.787		  25.783		  36.221		 207.0			   0.078		  24.023

30			  25.784		  25.777		  36.221		 207.0			   0.117		  24.024

50			  25.781		  25.769		  36.223		 206.4			   0.195		  24.028

75			  25.123		  25.107		  36.416		 201.1			   0.286		  24.379

100			  24.863		  24.841		  36.437		 200.6			   0.376		  24.476

125			  23.756		  23.730		  36.671		 201.9			   0.457		  24.988

150			  23.386		  23.355		  36.927		 195.5			   0.529		  25.292

200			  20.865		  20.826		  36.803		 186.0			   0.650		  25.915

250			  19.218		  19.173		  36.661		 182.3			   0.750		  26.246

300			  18.453		  18.400		  36.577		 183.8			   0.839		  26.380

400			  17.435		  17.367		  36.426		 184.9			   1.009		  26.521

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

445			   24			  16.660		  16.586		  36.284		   168.8

415			    1			  17.289		  17.219		  36.396		   183.3

349			    2			  18.052		  17.991		  36.520		   187.5

299			    3			  18.516		  18.463		  36.580		   187.4

249			    4			  19.254		  19.209		  36.659		   187.2

199			    5			  20.838		  20.800		  36.786		   190.1

150			    6			  23.349		  23.318		  36.914		   196.3

101			   12			  24.649		  24.627		  36.437		   196.1

50			   13			  25.775		  25.764		  36.236		   202.3

2			   14			  25.804		  25.804		  36.221		   202.7

ABACO-01 R.V. Oceanus

CTD Station 6 (CTD006)

Latitude 26.065N Longitude 78.858W

27-Apr-2001 16:59Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  25.685		  25.684		  36.190		 208.9			   0.004		  24.030

10			  25.686		  25.684		  36.193		 209.2			   0.039		  24.032

20			  25.639		  25.634		  36.186		 209.1			   0.077		  24.043

30			  25.517		  25.510		  36.191		 209.4			   0.116		  24.085

50			  25.184		  25.173		  36.310		 208.5			   0.190		  24.279

75			  24.875		  24.859		  36.342		 209.0			   0.280		  24.399

100			  24.247		  24.225		  36.484		 208.1			   0.367		  24.698

125			  23.698		  23.672		  36.835		 201.8			   0.443		  25.129

150			  22.240		  22.210		  36.836		 185.3			   0.511		  25.554

200			  19.988		  19.951		  36.728		 183.4			   0.621		  26.093

250			  19.163		  19.118		  36.647		 177.0			   0.717		  26.250

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

283			   24			  18.688		  18.638		  36.589		   177.0

248			    1			  19.275		  19.230		  36.646		   179.8

248			    2			  19.273		  19.228		  36.649		   179.6

199			    3			  19.990		  19.953		  36.724		   186.8

151			    4			  22.163		  22.133		  36.829		   186.1

99			    5			  24.295		  24.273		  36.498		   205.7

49			    6			  25.179		  25.168		  36.314		   205.4

2			   12			  25.672		  25.671		  36.189		   204.5

ABACO-01 R.V. Oceanus

CTD Station 7 (CTD007)

Latitude 26.501N Longitude 76.914W

28-Apr-2001 06:49Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.196		  24.196		  36.482		 214.0			   0.003		  24.705

10			  24.209		  24.207		  36.486		 214.2			   0.032		  24.705

20			  24.235		  24.231		  36.490		 214.0			   0.065		  24.701

30			  24.230		  24.223		  36.490		 213.8			   0.097		  24.703

50			  24.243		  24.233		  36.491		 213.9			   0.162		  24.701

75			  24.299		  24.282		  36.540		 213.0			   0.243		  24.723

100			  24.138		  24.117		  36.589		 211.5			   0.324		  24.810

125			  23.555		  23.528		  36.730		 208.8			   0.400		  25.092

150			  22.704		  22.673		  36.864		 203.5			   0.468		  25.443

200			  21.831		  21.791		  36.888		 193.1			   0.591		  25.712

250			  20.858		  20.810		  36.807		 193.6			   0.703		  25.922

300			  19.703		  19.648		  36.692		 193.2			   0.806		  26.147

400			  17.686		  17.617		  36.464		 190.2			   0.988		  26.488

500			  14.462		  14.386		  35.945		 166.0			   1.144		  26.835

600			  12.833		  12.749		  35.696		 157.9			   1.274		  26.982

700			   9.895		   9.812		  35.314		 147.8			   1.386		  27.229

800			   8.322		   8.236		  35.161		 152.4			   1.481		  27.365

900			   6.740		   6.653		  35.092		 180.3			   1.556		  27.539

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

969			   24			   6.111		   6.022		  35.083		   200.5

799			    1			   8.075		   7.990		  35.139		   158.2

648			    2			  11.499		  11.415		  35.506		   148.5

550			    3			  13.683		  13.604		  35.814		   159.3

449			    4			  16.985		  16.910		  36.336		   181.2

347			    5			  18.779		  18.717		  36.604		   190.7

306			    6			  19.483		  19.427		  36.669		     NaN

248			    7			  20.530		  20.483		  36.771		   181.0

200			    8			  21.865		  21.825		  36.874		   191.7

149			    9			  22.790		  22.759		  36.849		   207.0

99			   10			  24.071		  24.050		  36.636		   207.7

48			   11			  24.203		  24.193		  36.478		     NaN

ABACO-01 R.V. Oceanus

CTD Station 8 (CTD008)

Latitude 26.512N Longitude 76.843W

28-Apr-2001 10:48Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.698		  24.698		  36.560		 210.3			   0.003		  24.613

10			  24.700		  24.698		  36.560		 210.6			   0.033		  24.613

20			  24.704		  24.700		  36.569		 210.7			   0.066		  24.619

30			  24.608		  24.601		  36.550		 210.4			   0.100		  24.635

50			  24.391		  24.380		  36.565		 211.0			   0.165		  24.713

75			  24.051		  24.035		  36.616		 209.8			   0.244		  24.855

100			  23.621		  23.600		  36.731		 208.2			   0.320		  25.072

125			  23.098		  23.072		  36.824		 208.7			   0.391		  25.296

150			  22.047		  22.017		  36.850		 206.0			   0.455		  25.620

200			  21.148		  21.109		  36.810		 204.0			   0.571		  25.843

250			  20.328		  20.281		  36.760		 193.0			   0.678		  26.030

300			  19.442		  19.387		  36.681		 189.3			   0.777		  26.206

400			  17.947		  17.878		  36.511		 187.8			   0.956		  26.460

500			  16.075		  15.994		  36.198		 181.8			   1.120		  26.671

600			  13.386		  13.300		  35.776		 162.3			   1.261		  26.933

700			  11.053		  10.964		  35.449		 149.2			   1.379		  27.132

800			   9.196		   9.105		  35.232		 147.1			   1.481		  27.283

900			   7.127		   7.038		  35.099		 172.4			   1.560		  27.491

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

996			   24			   5.777		   5.688		  35.075		   209.4

900			    1			   7.128		   7.038		  35.096		   171.2

751			    2			  10.143		  10.053		  35.331		   145.4

626			    3			  12.874		  12.787		  35.695		   151.8

501			    4			  16.148		  16.066		  36.202		   178.1

399			    5			  17.953		  17.883		  36.508		   186.7

310			    6			  19.083		  19.027		  36.643		   188.0

224			    7			  20.665		  20.622		  36.774		   204.0

160			    8			  21.849		  21.817		  36.816		   209.0

119			    9			  23.231		  23.207		  36.788		   208.3

61			   10			  24.282		  24.269		  36.590		   208.4

3			   11			  24.695		  24.694		  36.560		   206.5

ABACO-01 R.V. Oceanus

CTD Station 9 (CTD009)

Latitude 26.507N Longitude 76.754W

28-Apr-2001 13:19Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.440		  24.439		  36.656		 213.4			   0.003		  24.764

10			  24.441		  24.439		  36.656		 213.7			   0.032		  24.764

20			  24.448		  24.444		  36.657		 213.6			   0.064		  24.764

30			  24.449		  24.443		  36.657		 213.3			   0.095		  24.764

50			  24.307		  24.296		  36.709		 213.0			   0.159		  24.847

75			  24.100		  24.084		  36.760		 212.6			   0.235		  24.950

100			  23.078		  23.057		  36.929		 201.1			   0.306		  25.381

125			  22.112		  22.087		  36.835		 211.3			   0.369		  25.588

150			  21.285		  21.255		  36.795		 210.4			   0.427		  25.791

200			  20.311		  20.274		  36.745		 206.3			   0.535		  26.020

250			  19.310		  19.265		  36.646		 200.5			   0.633		  26.212

300			  18.828		  18.774		  36.613		 205.6			   0.726		  26.313

400			  17.834		  17.765		  36.487		 195.8			   0.901		  26.470

500			  16.277		  16.195		  36.235		 180.6			   1.065		  26.653

600			  13.668		  13.581		  35.818		 158.0			   1.210		  26.907

700			  11.214		  11.125		  35.465		 146.8			   1.331		  27.115

800			   9.191		   9.100		  35.230		 148.1			   1.434		  27.282

900			   7.408		   7.316		  35.092		 161.4			   1.520		  27.447

1000			   6.293		   6.199		  35.078		 190.1			   1.591		  27.589

1100			   5.535		   5.437		  35.077		 220.6			   1.650		  27.684

1200			   5.032		   4.929		  35.058		 237.0			   1.703		  27.730

1300			   4.537		   4.429		  35.022		 250.3			   1.753		  27.757

1400			   4.336		   4.221		  35.013		 255.7			   1.800		  27.773

1500			   4.125		   4.003		  34.993		 259.6			   1.847		  27.781

1750			   3.857		   3.715		  34.980		 263.6			   1.963		  27.799

2000			   3.647		   3.484		  34.967		 264.6			   2.079		  27.812

2500			   3.281		   3.076		  34.959		 263.9			   2.309		  27.845

3000			   2.877		   2.629		  34.936		 267.0			   2.533		  27.867

3500			   2.557		   2.263		  34.915		 269.9			   2.753		  27.882

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

3902			   24			   2.365		   2.032		  34.901		   266.6

3297			    2			   2.677		   2.403		  34.923		   267.0

3001			    3			   2.880		   2.631		  34.934		   265.0

2698			    4			   3.115		   2.893		  34.949		   263.2

2398			    5			   3.367		   3.171		  34.962		   262.0

2101			    6			   3.619		   3.448		  34.970		   261.9

1802			    7			   3.744		   3.599		  34.968		   262.4

1487			    8			   4.059		   3.940		  34.985		   257.8

1201			    9			   4.981		   4.878		  35.055		   235.2

1005			   10			   6.275		   6.181		  35.078		   188.5

854			   11			   7.906		   7.816		  35.114		   151.7

702			   12			  11.149		  11.059		  35.470		   151.5

546			   13			  15.138		  15.053		  36.047		   164.7

451			   14			  17.305		  17.229		  36.399		   184.6

350			   15			  18.449		  18.387		  36.569		   189.3

250			   16			  19.301		  19.255		  36.648		   202.2

150			   17			  21.363		  21.333		  36.784		   213.0

100			   18			  22.943		  22.923		  36.870		   210.8

49			   19			  24.551		  24.540		  36.642		   207.7

3			   20			  24.645		  24.644		  36.602		   207.0

ABACO-01 R.V. Oceanus

CTD Station 10 (CTD010)

Latitude 26.513N Longitude 76.614W

28-Apr-2001 17:09Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.183		  24.182		  36.684		 211.4				   0.003		  24.862

10			  24.179		  24.177		  36.685		 211.7				   0.031		  24.865

20			  24.168		  24.164		  36.684		 210.8				   0.062		  24.868

30			  24.174		  24.168		  36.703		 210.9				   0.092		  24.882

50			  23.488		  23.478		  36.866		 206.2				   0.151		  25.210

75			  22.746		  22.730		  36.870		 212.7				   0.218		  25.431

100			  22.077		  22.057		  36.873		 205.7				   0.280		  25.625

125			  21.371		  21.346		  36.802		 211.3				   0.338		  25.771

150			  20.597		  20.568		  36.756		 200.0				   0.392		  25.949

200			  19.784		  19.747		  36.690		 199.7				   0.494		  26.119

250			  19.022		  18.977		  36.624		 200.7				   0.589		  26.269

300			  18.697		  18.644		  36.599		 200.8				   0.680		  26.335

400			  17.827		  17.758		  36.488		 195.7				   0.854		  26.473

500			  16.107		  16.026		  36.200		 181.4				   1.017		  26.665

600			  14.175		  14.086		  35.893		 166.1				   1.162		  26.859

700			  11.756		  11.663		  35.541		 153.5				   1.289		  27.074

800			   9.465			   9.373			  35.260		 147.3				   1.397		  27.261

900			   7.679			   7.586			  35.112		 153.7				   1.486		  27.424

1000			   6.494			   6.398			  35.079		 180.2				   1.561		  27.563

1100			   5.749			   5.649			  35.084		 213.7				   1.623		  27.663

1200			   5.282			   5.177			  35.075		 228.5				   1.678		  27.714

1300			   4.760			   4.650			  35.038		 244.7				   1.729		  27.745

1400			   4.442			   4.326			  35.013		 252.1				   1.778		  27.762

1500			   4.202			   4.079			  34.991		 257.9				   1.826		  27.771

1750			   3.780			   3.639			  34.964		 265.3				   1.944		  27.795

2000			   3.595			   3.433			  34.962		 264.8				   2.059		  27.814

2500			   3.233			   3.029			  34.953		 264.5				   2.287		  27.845

3000			   2.830			   2.583			  34.933		 266.8				   2.509		  27.869

3500			   2.507			   2.214			  34.913		 269.4				   2.726		  27.885

4000			   2.336			   1.993			  34.900		 268.6				   2.942		  27.892

4500			   2.279			   1.878			  34.890		 264.3				   3.164		  27.893

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4790			   24			   2.204			   1.770			  34.876		   256.8

4500			    1				   2.277			   1.877			  34.892		   263.0

4247			    2				   2.288			   1.917			  34.888		   264.7

3870			    3				   2.370			   2.040			  34.902		   266.9

3497			    4				   2.510			   2.218			  34.913		   267.5

3120			    5				   2.741			   2.483			  34.927		   265.9

2747			    6				   3.045			   2.820			  34.943		   263.4

2447			    7				   3.256			   3.057			  34.952		   263.1

2147			    8				   3.448			   3.275			  34.954		   264.3

1850			    9				   3.697			   3.549			  34.960		   264.0

1551			   10			   4.030			   3.905			  34.977		   258.9

1247			   11			   4.980			   4.873			  35.050		   226.7

1046			   12			   6.254			   6.156			  35.087		   194.3

897			   13			   7.734			   7.641			  35.105		   154.8

750			   14			  10.358		  10.266		  35.368		   148.2

601			   15			  14.061		  13.972		  35.877		   163.9

485			   16			  16.402		  16.323		  36.260		   179.5

356			   17			  18.164		  18.102		  36.543		   189.7

277			   18			  18.780		  18.731		  36.604		   201.1

201			   19			  19.727		  19.690		  36.679		   200.1

151			   20			  20.635		  20.606		  36.758		   205.9

101			   21			  22.379		  22.358		  36.921		   194.8

49			   22			  23.676		  23.665		  36.865		   205.6

4				   23			  24.239		  24.238		  36.695		   208.7

ABACO-01 R.V. Oceanus

CTD Station 11 (CTD011)

Latitude 26.527N Longitude 76.492W

28-Apr-2001 23:55Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1				  24.518		  24.518		  36.637		 209.5			   0.003			  24.726

10			  24.516		  24.514		  36.638		 209.9			   0.032			  24.728

20			  24.515		  24.511		  36.640		 209.5			   0.064			  24.730

30			  24.448		  24.441		  36.643		 208.8			   0.096			  24.753

50			  23.405		  23.395		  36.716		 212.3			   0.158			  25.120

75			  22.920		  22.905		  36.842		 211.0			   0.226			  25.359

100			  22.238		  22.218		  36.831		 211.5			   0.290			  25.548

125			  21.661		  21.636		  36.883		 189.1			   0.349			  25.752

150			  20.680		  20.651		  36.773		 211.2			   0.404			  25.940

200			  19.214		  19.178		  36.635		 197.1			   0.502			  26.226

250			  18.825		  18.780		  36.614		 202.3			   0.593			  26.312

300			  18.557		  18.504		  36.581		 200.3			   0.683			  26.357

400			  17.785		  17.716		  36.484		 193.7			   0.856			  26.479

500			  16.127		  16.046		  36.199		 180.5			   1.018			  26.660

600			  14.009		  13.921		  35.867		 167.7			   1.164			  26.874

700			  11.888		  11.795		  35.561		 155.1			   1.290			  27.064

800			   9.700			   9.606			  35.283		 146.3			   1.398			  27.240

900			   7.961			   7.866			  35.130		 156.8			   1.490			  27.396

1000			   6.682			   6.585			  35.070		 174.5			   1.568			  27.531

1100			   5.891			   5.791			  35.084		 208.5			   1.632			  27.646

1200			   5.279			   5.173			  35.062		 228.4			   1.690			  27.704

1300			   4.857			   4.746			  35.044		 241.4			   1.742			  27.740

1400			   4.365			   4.249			  35.004		 252.8			   1.791			  27.763

1500			   4.147			   4.025			  34.991		 258.0			   1.838			  27.776

1750			   3.797			   3.656			  34.968		 262.3			   1.955			  27.796

2000			   3.590			   3.429			  34.962		 263.4			   2.070			  27.814

2500			   3.146			   2.944			  34.948		 263.4			   2.295			  27.849

3000			   2.756			   2.511			  34.927		 266.4			   2.514			  27.871

3500			   2.451			   2.160			  34.908		 267.6			   2.727			  27.885

4000			   2.320			   1.977			  34.897		 266.4			   2.942			  27.891

4500			   2.296			   1.895			  34.891		 264.0			   3.165			  27.892

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4923			   24			   2.303			   1.849			  34.885		   261.6

3801			    3				   2.368			   2.046			  34.901		   266.9

3449			    4				   2.460			   2.178			  34.319		   268.2

3149			    5				   2.677			   2.417			  34.923		   267.1

2801			    6				   2.922			   2.693			  34.938		   264.3

2501			    7				   3.168			   2.965			  34.947		   263.8

2200			    8				   3.362			   3.185			  34.956		   263.3

1901			    9				   3.664			   3.511			  34.964		   263.8

1603			   10			   4.000			   3.870			  34.980		   259.4

1375			   11			   4.498			   4.384			  35.015		   247.9

1174			   12			   5.285			   5.183			  35.060		   227.1

979			   13			   6.810			   6.714			  35.155		   158.5

804			   14			   9.632			   9.538			  35.278		   146.0

652			   15			  13.072		  12.980		  35.740		   160.0

503			   16			  16.214		  16.132		  36.213		   177.1

402			   17			  17.782		  17.713		  36.482		   189.7

302			   18			  18.634		  18.580		  36.586		   200.7

224			   19			  19.188		  19.147		  36.634		   199.9

160			   20			  20.726		  20.696		  36.774		   213.7

110			   21			  21.923		  21.901		  36.878		   196.7

54			   22			  23.456		  23.445		  36.822		   212.8

3				   23			  24.455		  24.454		  36.638		   208.2

ABACO-01 R.V. Oceanus

CTD Station 12 (CTD012)

Latitude 26.529N Longitude 76.316W

29-Apr-2001 04:20Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.180		  24.180		  36.717		 213.4			   0.003	  24.888

10			  24.187		  24.185		  36.718		 213.3			   0.031	  24.887

20			  24.190		  24.186		  36.718		 213.3			   0.061	  24.887

30			  24.192		  24.186		  36.719		 213.0			   0.092	  24.888

50			  23.998		  23.988		  36.717		 211.9			   0.153	  24.946

75			  22.723		  22.708		  36.853		 209.3			   0.220	  25.425

100			  22.039		  22.019		  36.815		 213.1			   0.283	  25.592

125			  21.624		  21.599		  36.837		 196.9			   0.342	  25.727

150			  20.877		  20.849		  36.832		 181.4			   0.397	  25.931

200			  19.380		  19.343		  36.657		 192.1			   0.496	  26.199

250			  18.869		  18.824		  36.617		 199.4			   0.588	  26.303

300			  18.554		  18.501		  36.581		 200.1			   0.678	  26.357

400			  17.816		  17.747		  36.485		 193.1			   0.851	  26.473

500			  16.174		  16.093		  36.211		 180.1			   1.014	  26.658

600			  14.397		  14.307		  35.924		 169.0			   1.162	  26.836

700			  12.066		  11.972		  35.585		 155.5			   1.291	  27.049

800			   9.625			   9.531			  35.277		 146.4			   1.400	  27.248

900			   7.775			   7.681			  35.104		 151.3			   1.492	  27.403

1000			   6.784			   6.686			  35.093		 180.3			   1.570	  27.535

1100			   5.992			   5.890			  35.088		 204.8			   1.635	  27.636

1200			   5.210			   5.105			  35.064		 229.3			   1.692	  27.714

1300			   4.643			   4.534			  35.028		 245.4			   1.743	  27.751

1400			   4.376			   4.261			  35.008		 252.0			   1.792	  27.765

1500			   4.094			   3.973			  34.980		 258.5			   1.839	  27.773

1750			   3.742			   3.602			  34.967		 262.3			   1.955	  27.801

2000			   3.542			   3.382			  34.963		 262.6			   2.068	  27.819

2500			   3.113			   2.911			  34.947		 263.2			   2.291	  27.851

3000			   2.708			   2.463			  34.924		 266.3			   2.507	  27.873

3500			   2.469			   2.177			  34.909		 267.7			   2.722	  27.884

4000			   2.340			   1.996			  34.898		 266.4			   2.937	  27.890

4500			   2.347			   1.945			  34.895		 266.2			   3.162	  27.892

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4908			   24			   2.317			   1.865			  34.888		   262.1

4795			    1				   2.333			   1.895			  34.891		   263.7

4498			    3				   2.348			   1.946			  34.894		   264.8

4047			    4				   2.336			   1.987			  34.897		   265.7

3600			    5				   2.434			   2.132			  34.907		   267.9

3199			    6				   2.639			   2.375			  34.920		   267.3

2797			    7				   2.860			   2.633			  34.933		   265.5

2450			    8				   3.131			   2.934			  34.950		   262.9

2099			    9				   3.404			   3.236			  34.955		   263.9

1800			   10			   3.645			   3.501			  34.960		   264.2

1500			   11			   4.177			   4.055			  34.992		   255.6

1198			   12			   5.109			   5.005			  35.050		   230.7

950			   13			   7.367			   7.271			  35.100		   165.6

825			   14			   9.469			   9.373			  35.257		   145.4

678			   15			  12.460		  12.367		  35.656		   148.7

552			   16			  15.331		  15.244		  36.078		   172.9

461			   17			  17.009		  16.932		  36.343		   182.3

371			   18			  17.995		  17.931		  36.510		   191.0

230			   19			  19.077		  19.035		  36.623		   199.0

176			   20			  20.041		  20.009		  36.715		   199.4

120			   21			  21.859		  21.835		  36.867		   195.8

60			   22			  23.107		  23.095		  36.827		   212.8

3				   23			  24.252		  24.251		  36.716		   209.1

ABACO-01 R.V. Oceanus

CTD Station 13 (CTD013)

Latitude 26.528N Longitude 76.127W

29-Apr-2001 08:53Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.908		  24.908		  36.545		 208.3			   0.003			  24.538

10			  24.910		  24.907		  36.547		 208.1			   0.034			  24.539

20			  24.910		  24.905		  36.546		 208.5			   0.068			  24.539

30			  24.910		  24.904		  36.554		 207.2			   0.102			  24.545

50			  24.354		  24.344		  36.721		 207.3			   0.168			  24.842

75			  23.460		  23.445		  36.832		 203.9			   0.240			  25.193

100			  22.878		  22.858		  36.928		 189.6			   0.307			  25.438

125			  21.970		  21.945		  36.852		 205.0			   0.369			  25.641

150			  21.506		  21.477		  36.855		 192.2			   0.427			  25.775

200			  20.014		  19.976		  36.722		 193.4			   0.533			  26.082

250			  19.007		  18.962		  36.640		 183.7			   0.628			  26.285

300			  18.433		  18.380		  36.579		 187.4			   0.717			  26.386

400			  17.602		  17.534		  36.455		 189.7			   0.888			  26.502

500			  16.218		  16.136		  36.225		 176.9			   1.049			  26.659

600			  14.309		  14.220		  35.915		 170.9			   1.196			  26.848

700			  12.170		  12.075		  35.600		 156.7			   1.326			  27.041

800			  10.055		   9.958			  35.326		 151.3			   1.438			  27.214

900			   8.125			   8.029			  35.140		 151.0			   1.534			  27.379

1000			   6.720			   6.623			  35.068		 170.1			   1.612			  27.524

1100			   5.920			   5.819			  35.083		 206.4			   1.677			  27.641

1200			   5.060			   4.957			  35.047		 233.1			   1.732			  27.718

1300			   4.667			   4.558			  35.029		 245.4			   1.783			  27.749

1400			   4.256			   4.142			  35.000		 254.5			   1.831			  27.771

1500			   3.935			   3.816			  34.971		 260.9			   1.877			  27.782

1750			   3.667			   3.528			  34.953		 264.6			   1.991			  27.797

2000			   3.493			   3.333			  34	.953		 264.0			   2.105			  27.816

2500			   3.152			   2.949			  34.947		 263.4			   2.330			  27.847

3000			   2.722			   2.477			  34.925		 266.4			   2.549			  27.872

3500			   2.521			   2.228			  34.912		 268.0			   2.765			  27.882

4000			   2.336			   1.992			  34.898		 266.2			   2.982			  27.890

4500			   2.319			   1.917			  34.892		 265.0			   3.206			  27.892

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4888			   24			   2.308			   1.859			  34.887		   262.0

4848			    1				   2.303			   1.859			  34.887		   261.9

4299			    3				   2.314			   1.937			  34.894		   265.1

3634			    5				   2.461			   2.155			  34.907		   267.7

3249			    6				   2.620			   2.351			  34.918		   267.6

2898			    7				   2.792			   2.556			  34.929		   266.3

2550			    8				   3.130			   2.923			  34.947		   263.8

2200			    9				   3.373			   3.195			  34.953		   264.1

1902			   10			   3.571			   3.420			  34.952		   265.2

1600			   11			   3.732			   3.606			  34.961		   263.6

1378			   12			   4.219			   4.108			  34.999		   253.8

1151			   13			   5.347			   5.246			  35.068		   225.0

92	8			   14			   7.777			   7.680			  35.112		   155.1

725			   15			  11.418		  11.324		  35.490		   152.0

560			   16			  15.318		  15.230		  36.063		   171.6

450			   17			  17.049		  16.974		  36.361		   180.3

349			   18			  18.091		  18.030		  36.530		   188.7

219			   19			  19.725		  19.684		  36.721		   182.8

160			   20			  21.025		  20.994		  36.780		   200.4

109			   21			  22.582		  22.560		  36.904		   193.6

48			   22			  24.839		  24.828		  36.650		   206.0

2				   23			  24.878		  24.878		  36.542		   206.3

ABACO-01 R.V. Oceanus

CTD Station 14 (CTD014)

Latitude 26.531N Longitude 75.928W

29-Apr-2001 13:43Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.878		  24.878		  36.502		 207.7			   0.003			  24.514

10			  24.889		  24.887		  36.503		 207.5			   0.034			  24.512

20			  24.895		  24.891		  36.503		 207.0			   0.068			  24.511

30			  24.902		  24.895		  36.503		 207.9			   0.103			  24.510

50			  24.903		  24.892		  36.504		 206.3			   0.171			  24.512

75			  23.636		  23.620		  36.851		 208.9			   0.248			  25.156

100			  22.672		  22.652		  36.880		 205.4			   0.315			  25.461

125			  21.831		  21.806		  36.857		 201.8			   0.376			  25.685

150			  21.329		  21.300		  36.871		 184.0			   0.433			  25.836

200			  20.159		  20.122		  36.762		 181.0			   0.539			  26.074

250			  19.318		  19.273		  36.674		 185.2			   0.636			  26.230

300			  18.816		  18.762		  36.619		 186.3			   0.728			  26.320

400			  17.913		  17.843		  36.504		 187.5			   0.905			  26.463

500			  16.369		  16.287		  36.253		 176.9			   1.068			  26.645

600			  14.422		  14.332		  35.934		 163.5			   1.217			  26.838

700			  12.565		  12.468		  35.655		 160.3			   1.349			  27.007

800			  10.427		  10.328		  35.369		 149.3			   1.464			  27.183

900			   8.415			   8.317			  35.150		 147.5			   1.562			  27.343

1000			   7.165			   7.065			  35.089		 166.0			   1.645			  27.480

1100			   6.201			   6.098			  35.095		 196.6			   1.715			  27.615

1200			   5.444			   5.337			  35.076		 222.3			   1.774			  27.695

1300			   4.897			   4.786			  35.048		 239.8			   1.827			  27.738

1400			   4.512			   4.395			  35.018		 249.3			   1.877			  27.758

1500			   4.200			   4.078			  34.994		 256.2			   1.925			  27.774

1750			   3.825			   3.683			  34.969		 262.2			   2.043			  27.794

2000			   3.559			   3.398			  34.954		 263.9			   2.158			  27.811

2500			   3.219			   3.015			  34.953		 261.4			   2.387			  27.846

3000			   2.819			   2.572			  34.932		 265.2			   2.609			  27.869

3500			   2.506			   2.214			  34.911		 267.9			   2.826			  27.883

4000			   2.350			   2.006			  34.899		 267.1			   3.044			  27.890

4500			   2.281			   1.881			  34.890		 263.6			   3.267			  27.893

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4832			   24			   2.246			   1.806			  34.882		   258.6

4618			    1				   2.266			   1.851			  34.887		   261.5

4318			    2				   2.290			   1.911			  34.892		   264.7

3922			    3				   2.370			   2.034			  34.902		   267.3

3624			    4				   2.481			   2.175			  34.909		   267.9

3224			    5				   2.668			   2.401			  34.922		   267.2

2923			    6				   2.881			   2.641			  34.935		   265.5

2599			    7				   3.151			   2.939			  34.951		   263.0

2225			    8				   3.418			   3.238			  34.951		   264.3

1924			    9				   3.581			   3.427			  34.957		   264.6

1627			   10			   3.947			   3.816			  34.974		   260.6

1401			   11			   4.450			   4.334			  35.016		   248.8

1204			   12			   5.343			   5.236			  35.074		   223.5

1051			   13			   6.617			   6.515			  35.087		   182.8

953			   14			   7.582			   7.483			  35.097		   160.3

751			   15			  11.436		  11.338		  35.508		   156.5

577			   16			  15.286		  15.196		  36.069		   173.0

460			   17			  17.127		  17.049		  36.376		   185.1

362			   18			  18.337		  18.273		  36.565		   190.0

229			   19			  19.748		  19.706		  36.715		   189.9

229			   20			  19.745		  19.702		  36.715		   187.7

173			   21			  20.681		  20.648		  36.809		   185.9

4				   23			  24.902		  24.901		  36.497		   215.4

ABACO-01 R.V. Oceanus

CTD Station 15 (CTD015)

Latitude 26.551N Longitude 75.728W

29-Apr-2001 21:11Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.714		  24.714		  36.585		 206.5			   0.003			  24.627

10			  24.707		  24.705		  36.587		 207.0			   0.033			  24.631

20			  24.700		  24.695		  36.590		 206.5			   0.066			  24.636

30			  24.383		  24.377		  36.689		 207.4			   0.098			  24.808

50			  23.717		  23.706		  36.853		 210.3			   0.158			  25.132

75			  22.937		  22.922		  36.850		 211.8			   0.226			  25.361

100			  22.460		  22.440		  36.867		 208.3			   0.290			  25.512

125			  21.696		  21.672		  36.805		 209.9			   0.351			  25.682

150			  21.096		  21.067		  36.778		 209.3			   0.408			  25.830

200			  20.050		  20.013		  36.706		 200.8			   0.515			  26.060

250			  19.119		  19.074		  36.634		 197.0			   0.611			  26.251

300			  18.794		  18.740		  36.602		 202.8			   0.702			  26.313

400			  17.972		  17.903		  36.510		 193.3			   0.879			  26.453

500			  16.813		  16.729		  36.322		 184.6			   1.046			  26.594

600			  15.250		  15.157		  36.067		 168.6			   1.201			  26.760

700			  13.174		  13.075		  35.740		 156.9			   1.340			  26.951

800			  10.990		  10.889		  35.435		 144.8			   1.462			  27.135

900			   9.043			   8.941			  35.212		 146.2			   1.566			  27.294

1000			   7.612			   7.508			  35.101		 157.2			   1.655			  27.426

1100			   6.520			   6.414			  35.079		 183.7			   1.730			  27.561

1200			   5.823			   5.712			  35.088		 210.7			   1.795			  27.659

1300			   5.141			   5.028			  35.064		 232.5			   1.851			  27.723

1400			   4.736			   4.617			  35.036		 243.0			   1.903			  27.748

1500			   4.466			   4.340			  35.018		 250.1			   1.953			  27.764

1750			   3.981			   3.838			  34.983		 259.7			   2.074			  27.789

2000			   3.702			   3.539			  34.973		 260.2			   2.192			  27.812

2500			   3.268			   3.064			  34.957		 261.3			   2.422			  27.845

3000			   2.748			   2.503			  34.926		 265.8			   2.643			  27.871

3500			   2.430			   2.139			  34.908		 266.9			   2.855			  27.886

4000			   2.295			   1.952			  34.895		 265.1			   3.068			  27.892

4500			   2.264			   1.864			  34.888		 261.9			   3.290			  27.893

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4770			   24			   2.194			   1.764			  34.877		   260.2

4395			    1				   2.271			   1.883			  34.890		   265.7

3997			    2				   2.300			   1.958			  34.896		   274.0

3696			    3				   2.361			   2.050			  34.903		   280.0

3399			    4				   2.476			   2.195			  34.911		   274.3

3101			    5				   2.673			   2.419			  34.922		   280.1

2802			    6				   2.910			   2.682			  34.937		   265.6

2501			    7				   3.226			   3.022			  34.951		   263.9

2203			    8				   3.488			   3.309			  34.953		   265.0

2003			    9				   3.684			   3.521			  34.966		   262.8

1804			   10			   3.875			   3.729			  34.973		   261.2

1604			   11			   4.226			   4.093			  34.998		   254.5

1410			   12			   4.678			   4.559			  35.036		   243.5

1208			   13			   5.685			   5.575			  35.088		   199.6

1006			   14			   7.415			   7.312			  35.092		   160.2

806			   15			  10.946		  10.844		  35.444		   150.3

605			   16			  15.034		  14.940		  36.042		   163.7

483			   17			  17.095		  17.013		  36.371		   181.0

355			   18			  18.382		  18.319		  36.557		   190.0

223			   19			  19.783		  19.741		  36.690		   201.7

154			   20			  21.102		  21.072		  36.776		   211.5

90			   21			  22.543		  22.525		  36.831		   219.1

45			   22			  23.378		  23.369		  36.813		   213.7

3				   23			  24.487		  24.486		  36.620		   207.8

ABACO-01 R.V. Oceanus

CTD Station 16 (CTD016)

Latitude 26.531N Longitude 75.515W

29-Apr-2001 02:37Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.068		  24.068		  36.686		 212.8			   0.003			  24.898

10			  24.067		  24.065		  36.688		 213.1			   0.030			  24.900

20			  24.075		  24.070		  36.687		 212.1			   0.061			  24.898

30			  23.383		  23.377		  36.781		 214.8			   0.090			  25.175

50			  23.241		  23.230		  36.780		 213.8			   0.146			  25.217

75			  22.352		  22.337		  36.822		 213.7			   0.212			  25.507

100			  21.819		  21.799		  36.817		 208.1			   0.272			  25.656

125			  21.296		  21.271		  36.804		 202.9			   0.330			  25.793

150			  20.864		  20.835		  36.782		 208.8			   0.385			  25.897

200			  20.246		  20.209		  36.730		 209.9			   0.490			  26.026

250			  19.322		  19.276		  36.643		 199.6			   0.590			  26.206

300			  18.937		  18.883		  36.619		 201.5			   0.683			  26.289

400			  18.279		  18.208		  36.551		 194.5			   0.864			  26.409

500			  17.320		  17.235		  36.398		 188.1			   1.036			  26.531

600			  15.699		  15.604		  36.129		 177.3			   1.197			  26.707

700			  13.716		  13.614		  35.823		 155.8			   1.340			  26.905

800			  11.755		  11.649		  35.536		 147.3			   1.466			  27.072

900			   9.680			   9.574			  35.283		 146.1			   1.577			  27.246

1000			   8.010			   7.903			  35.126		 150.3			   1.671			  27.387

1100			   6.768			   6.660			  35.079		 174.8			   1.751			  27.528

1200			   6.018			   5.906			  35.087		 201.8			   1.818			  27.633

1300			   5.358			   5.242			  35.073		 225.0			   1.877			  27.704

1400			   4.882			   4.761			  35.047		 239.7			   1.931			  27.740

1500			   4.512			   4.386			  35.019		 248.9			   1.982			  27.760

1750			   3.993			   3.849			  34.982		 259.4			   2.104			  27.787

2000			   3.712			   3.549			  34.969		 262.2			   2.222			  27.808

2500			   3.247			   3.042			  34.958		 260.9			   2.452			  27.848

3000			   2.750			   2.505			  34.928		 265.4			   2.672			  27.872

3500			   2.381			   2.091			  34.905		 267.5			   2.883			  27.888

4000			   2.278			   1.936			  34.894		 265.7			   3.094			  27.892

4500			   2.252			   1.853			  34.887		 262.3			   3.315			  27.893

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4760			   24			   2.173			   1.744			  34.874		   255.2

4497			    1				   2.255			   1.855			  34.887		   261.7

4200			    2				   2.274			   1.909			  34.891		   264.2

3899			    3				   2.295			   1.964			  34.896		   266.0

3600			    4				   2.360			   2.061			  34.902		   267.4

3301			    5				   2.531			   2.259			  34.916		   267.5

3000			    6				   2.734			   2.489			  34.927		   265.9

2699			    7				   3.046			   2.825			  34.947		   263.0

2451			    8				   3.297			   3.096			  34.961		   261.2

2202			    9				   3.529			   3.349			  34.968		   261.3

1951			   10			   3.765			   3.606			  34.972		   261.6

1701			   11			   4.114			   3.974			  34.995		   256.3

1453			   12			   4.676			   4.553			  35.029		   243.4

1176			   13			   6.132			   6.022			  35.085		   197.7

1052			   14			   7.297			   7.190			  35.097		   165.4

851			   15			  10.627		  10.521		  35.391		   147.6

649			   16			  14.600		  14.501		  35.958		   161.5

458			   17			  17.806		  17.726		  36.482		   190.1

344			   18			  18.649		  18.588		  36.586		   201.1

260			   19			  19.210		  19.163		  36.628		   199.8

180			   20			  20.614		  20.579		  36.769		   210.4

110			   21			  21.592		  21.570		  36.806		   210.7

ABACO-01 R.V. Oceanus CTD

Station 17 (CTD017)

Latitude 26.515N Longitude 75.304W

30-Apr-2001 07:24Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.438		  24.437		  36.650		 207.9			   0.003			  24.760

10			  24.445		  24.443		  36.651		 207.6			   0.032			  24.759

20			  24.456		  24.452		  36.649		 208.2			   0.064			  24.755

30			  23.941		  23.935		  36.804		 209.4			   0.095			  25.027

50			  23.361		  23.350		  36.919		 207.7			   0.151			  25.287

75			  22.403		  22.388		  36.829		 209.9			   0.216			  25.498

100			  21.842		  21.822		  36.809		 208.0			   0.277			  25.643

125			  21.309		  21.284		  36.777		 208.7			   0.335			  25.769

150			  20.967		  20.938		  36.779		 208.2			   0.391			  25.866

200			  20.410		  20.372		  36.740		 208.4			   0.497			  25.991

250			  19.441		  19.395		  36.650		 198.5			   0.597			  26.181

300			  18.942		  18.888		  36.620		 199.6			   0.691			  26.289

400			  18.373		  18.302		  36.557		 198.2			   0.872			  26.389

500			  17.576		  17.490		  36.442		 192.4			   1.046			  26.503

600			  16.190		  16.093		  36.210		 180.4			   1.210			  26.657

700			  14.230		  14.125		  35.898		 166.6			   1.359			  26.855

800			  12.089		  11.981		  35.585		 151.5			   1.491			  27.047

900			  10.033		   9.925			  35.319		 146.3			   1.604			  27.214

1000			   8.298			   8.189			  35.139		 147.8			   1.702			  27.354

1100			   6.955			   6.845			  35.085		 171.6			   1.785			  27.507

1200			   6.147			   6.034			  35.087		 200.2			   1.854			  27.617

1300			   5.436			   5.320			  35.075		 223.6			   1.914			  27.697

1400			   4.975			   4.854			  35.053		 237.8			   1.969			  27.734

1500			   4.575			   4.448			  35.025		 248.6			   2.020			  27.758

1750			   4.008			   3.864			  34.981		 259.5			   2.143			  27.785

2000			   3.701			   3.538			  34.970		 262.4			   2.262			  27.809

2500			   3.256			   3.051			  34.959		 261.0			   2.492			  27.848

3000			   2.743			   2.498			  34.929		 265.0			   2.712			  27.873

3500			   2.395			   2.105			  34.905		 267.7			   2.922			  27.887

4000			   2.278			   1.936			  34.895		 265.6			   3.133			  27.892

4500			   2.222			   1.824			  34.884		 260.6			   3.353			  27.893

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4716			   24			   2.221			   1.796			  34.880		   258.3

4298			    1				   2.245			   1.869			  34.889		   262.9

3921			    2				   2.291			   1.957			  34.896		   266.0

3571			    3				   2.373			   2.076			  34.904		   267.9

3225			    4				   2.535			   2.271			  34.934		   267.9

2902			    5				   2.830			   2.593			  34.952		   264.8

2622			    6				   3.137			   2.922			  34.962		   262.1

2425			    7				   3.351			   3.152			  34.969		   261.0

2223			    8				   3.541			   3.358			  34.970		   261.1

2025			    9				   3.714			   3.548			  34.971		   262.0

1824			   10			   3.901			   3.752			  34.976		   260.6

1623			   11			   4.244			   4.109			  35.000		   254.2

1224			   13			   5.974			   5.860			  35.086		   205.1

1224			   13			   5.974			   5.860			  35.086		   205.1

1028			   14			   7.824			   7.716			  35.106		   153.6

825			   15			  11.461		  11.353		  35.498		   152.5

623			   16			  15.636		  15.537		  36.116		   173.3

449			   17			  18.000		  17.922		  36.503		   191.5

326			   18			  18.873		  18.814		  36.611		   203.9

225			   19			  19.976		  19.934		  36.692		   193.2

151			   20			  20.912		  20.883		  36.777		   212.9

100			   21			  21.934		  21.914		  36.799		   213.1

3				   22			  24.276		  24.276		  36.688		   208.5

ABACO-01 R.V. Oceanus

CTD Station 18 (CTD018)

Latitude 26.529N Longitude 75.091W

30-Apr-2001 12:28Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  23.301		  23.300		  36.789		 213.5			   0.003			  25.203

10			  23.309		  23.307		  36.790		 212.6			   0.028			  25.202

20			  23.311		  23.307		  36.791		 211.9			   0.055			  25.203

30			  23.308		  23.302		  36.790		 212.7			   0.083			  25.204

50			  23.108		  23.097		  36.827		 213.2			   0.138			  25.292

75			  22.535		  22.520		  36.824		 212.3			   0.203			  25.457

100			  22.065		  22.045		  36.823		 211.2			   0.265			  25.591

125			  21.451		  21.426		  36.767		 210.1			   0.324			  25.722

150			  21.105		  21.075		  36.776		 207.6			   0.381			  25.826

200			  20.396		  20.358		  36.745		 208.9			   0.489			  25.997

250			  19.377		  19.332		  36.651		 202.6			   0.588			  26.198

300			  18.957		  18.902		  36.618		 203.9			   0.682			  26.284

400			  18.530		  18.459		  36.575		 202.4			   0.864			  26.364

500			  17.624		  17.538		  36.450		 193.9			   1.040			  26.497

600			  16.486		  16.387		  36.258		 184.8			   1.207			  26.626

700			  14.796		  14.689		  35.984		 172.9			   1.361			  26.800

800			  12.565		  12.454		  35.650		 159.8			   1.497			  27.006

900			  10.414		  10.302		  35.370		 150.8			   1.615			  27.189

1000			   8.447			   8.337			  35.145		 146.0			   1.716			  27.337

1100			   7.138			   7.027			  35.085		 168.0			   1.800			  27.482

1200			   6.259			   6.145			  35.088		 195.0			   1.873			  27.603

1300			   5.536			   5.419			  35	.078		 220.9			   1.934			  27.687

1400			   5.006			   4.884			  35.052		 236.9			   1.989			  27.730

1500			   4.601			   4.474			  35.027		 246.9			   2.041			  27.757

1750			   4.020			   3.876			  34.983		 258.1			   2.164			  27.786

2000			   3.702			   3.539			  34.964		 262.3			   2.284			  27.805

2500			   3.197			   2.993			  34.952		 262.1			   2.515			  27.847

3000			   2.746			   2.500			  34.929		 265.3			   2.733			  27.873

3500			   2.380			   2.090			  34.905		 267.3			   2.943			  27.888

4000			   2.271			   1.929			  34.894		 265.6			   3.154			  27.892

4500			   2.238			   1.839			  34.886		 261.8			   3.374			  27.893

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4685			   24			   2.204			   1.783			  34.877		   257.6

4400			    1				   2.246			   1.858			  34.888		   261.9

4249			    2				   2.253			   1.883			  34.890		   264.1

3873			    3				   2.275			   1.948			  34.895		   265.8

3499			    4				   2.372			   2.083			  34.905		   268.2

3127			    5				   2.608			   2.353			  34.919		   267.6

2749			    6				   2.961			   2.736			  34.942		   264.1

2448			    7				   3.297			   3.097			  34.960		   262.2

2153			    8				   3.598			   3.421			  34.971		   261.6

1850			    9				   3.878			   3.727			  34.976		   261.1

1549			   10			   4.459			   4.329			  35.018		   249.1

1252			   11			   5.867			   5.752			  35.087		   208.5

1050			   12			   7.731			   7.621			  35.097		   153.7

900			   13			  10.438		  10.327		  35.371		   147.5

750			   14			  13.661		  13.552		  35.809		   162.7

601			   15			  16.220		  16.122		  36.212		   179.5

475			   16			  17.888		  17.805		  36.493		   190.4

351			   17			  18.702		  18.639		  36.597		   200.2

276			   18			  19.026		  18.976		  36.620		   203.2

184			   19			  20.548		  20.513		  36.765		   211.6

148			   20			  20.911		  20.882		  36.780		   211.8

100			   21			  22.058		  22.038		  36.818		   215.6

48			   22			  23.130		  23.120		  36.864		   214.0

4				   23			  23.326		  23.325		  36.789		   212.4

ABACO-01 R.V. Oceanus

CTD Station 19 (CTD019)

Latitude 26.502N Longitude 74.797W

30-Apr-2001 17:28Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  23.413		  23.413		  36.775		 211.8			   0.003			  25.160

10			  23.416		  23.414		  36.777		 211.6			   0.028			  25.161

20			  23.418		  23.414		  36.777		 212.4			   0.056			  25.161

30			  23.412		  23.406		  36.779		 211.8			   0.084			  25.165

50			  23.323		  23.313		  36.814		 211.7			   0.140			  25.219

75			  22.769		  22.753		  36.803		 212.3			   0.208			  25.373

100			  22.368		  22.348		  36.823		 211.5			   0.272			  25.505

125			  21.789		  21.764		  36.817		 206.8			   0.333			  25.666

150			  21.310		  21.281		  36.801		 202.1			   0.391			  25.788

200			  20.263		  20.226		  36.746		 193.1			   0.499			  26.034

250			  19.375		  19.330		  36.658		 191.2			   0.598			  26.204

300			  18.911		  18.857		  36.619		 195.7			   0.691			  26.296

400			  18.267		  18.197		  36.551		 191.7			   0.871			  26.412

500			  17.581		  17.495		  36.444		 190.6			   1.045			  26.503

600			  16.312		  16.214		  36.234		 181.4			   1.210			  26.648

700			  14.385		  14.279		  35.922		 171.8			   1.361			  26.841

800			  12.519		  12.409		  35.644		 156.2			   1.495			  27.010

900			  10.510		  10.398		  35.378		 148.3			   1.613			  27.178

1000			   8.457			   8.347			  35.152		 145.6			   1.714			  27.341

1100			   7.094			   6.983			  35.080		 164.6			   1.799			  27.484

1200			   6.158			   6.044			  35.089		 197.6			   1.870			  27.617

1300			   5.473			   5.356			  35.075		 221.5			   1.930			  27.693

1400			   4.954			   4.833			  35.048		 237.7			   1.985			  27.733

1500			   4.493			   4.367			  35.017		 249.3			   2.036			  27.760

1750			   3.989			   3.846			  34.979		 260.2			   2.158			  27.786

2000			   3.643			   3.481			  34.957		 264.8			   2.277			  27.805

2500			   3.206			   3.002			  34.953		 262.6			   2.505			  27.847

3000			   2.719			   2.474			  34.927		 265.6			   2.723			  27.874

3500			   2.384			   2.094			  34.906		 267.5			   2.933			  27.889

4000			   2.246			   1.905			  34.892		 264.8			   3.142			  27.893

4500			   2.205			   1.807			  34.882		 259.4			   3.361			  27.892

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4609			   24			   2.195			   1.784			  34.879		   257.6

4345			    1				   2.218			   1.838			  34.885		   260.9

4020			    2				   2.245			   1.901			  34.891		   264.1

3695			    3				   2.303			   1.994			  34.899		   266.6

3370			    4				   2.445			   2.168			  34.910		   268.0

3050			    5				   2.669			   2.421			  34.923		   266.7

2701			    6				   2.972			   2.753			  34.939		   264.5

2343			    7				   3.338			   3.148			  34.956		   262.7

2123			    8				   3.569			   3.396			  34.956		   263.5

1922			    9				   3.747			   3.591			  34.966		   263.0

1722			   10			   4.037			   3.895			  34.981		   258.6

1522			   11			   4.442			   4.315			  35.012		   249.6

1322			   12			   5.283			   5.166			  35.064		   227.6

1126			   13			   6.778			   6.667			  35.081		   176.3

921			   14			   9.803			   9.693			  35.293		   145.4

724			   15			  13.914		  13.807		  35.845		   163.9

549			   16			  17.108		  17.016		  36.361		   183.4

381			   17			  18.440		  18.373		  36.568		   191.2

263			   18			  19.258		  19.210		  36.646		   194.8

191			   19			  20.392		  20.356		  36.750		   198.3

145			   20			  21.583		  21.554		  36.824		   204.5

92			   21			  22.508		  22.489		  36.815		   213.1

46			   22			  23.430		  23.420		  36.782		   212.1

3				   23			  23.500		  23.500		  36.777		   211.7

ABACO-01 R.V. Oceanus

CTD Station 20 (CTD020)

Latitude 26.512N Longitude 74.518W

30-Apr-2001 22:57Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  23.488		  23.488		  36.793		 211.9			   0.003			  25.152

10			  23.496		  23.494		  36.795		 212.3			   0.028			  25.151

20			  23.500		  23.496		  36.795		 212.2			   0.056			  25.151

30			  23.502		  23.496		  36.795		 211.4			   0.084			  25.151

50			  23.502		  23.491		  36.796		 210.5			   0.141			  25.152

75			  22.844		  22.829		  36.818		 210.7			   0.211			  25.363

100			  22.183		  22.163		  36.834		 205.1			   0.274			  25.566

125			  21.878		  21.853		  36.834		 202.2			   0.334			  25.653

150			  21.297		  21.268		  36.800		 201.7			   0.392			  25.791

200			  20.316		  20.278		  36.737		 195.4			   0.499			  26.013

250			  19.407		  19.361		  36.655		 194.4			   0.599			  26.194

300			  18.992		  18.938		  36.623		 200.8			   0.692			  26.279

400			  18.332		  18.262		  36.557		 193.6			   0.874			  26.400

500			  17.461		  17.376		  36.423		 189.2			   1.047			  26.516

600			  16.009		  15.912		  36.180		 176.9			   1.210			  26.676

700			  14.274		  14.169		  35.903		 168.3			   1.359			  26.849

800			  11.861		  11.754		  35.553		 153.7			   1.489			  27.066

900			   9.964			   9.856			  35.313		 146.4			   1.602			  27.221

1000			   8.176			   8.068			  35.140		 151.9			   1.699			  27.374

1100			   6.869			   6.761			  35.067		 168.9			   1.782			  27.505

1200			   6.100			   5.987			  35.088		 199.3			   1.851			  27.624

1300			   5.386			   5.270			  35.072		 223.4			   1.911			  27.701

1400			   4.891			   4.770			  35.046		 238.9			   1.965			  27.738

1500			   4.519			   4.392			  35.022		 248.4			   2.015			  27.762

1750			   3.934			   3.792			  34.975		 260.3			   2.137			  27.788

2000			   3.708			   3.545			  34.972		 261.2			   2.255			  27.811

2500			   3.185			   2.982			  34.955		 260.8			   2.483			  27.851

3000			   2.695			   2.451			  34.926		 265.5			   2.699			  27.875

3500			   2.334			   2.046			  34.901		 266.8			   2.907			  27.889

4000			   2.251			   1.910			  34.892		 264.7			   3.116			  27.893

4500			   2.186			   1.788			  34.879		 258.2			   3.335			  27.892

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4557			   24			   2.193			   1.788			  34.879		   258.3

4236			    1				   2.234			   1.866			  34.888		   262.7

3874			    2				   2.267			   1.939			  34.895		   265.7

3600			    3				   2.311			   2.012			  34.899		   271.0

3348			    4				   2.435			   2.161			  34.910		   267.9

3098			    5				   2.623			   2.371			  34.921		   267.1

2848			    6				   2.827			   2.596			  34.934		   264.7

2599			    7				   3.088			   2.877			  34.949		   262.5

2373			    8				   3.326			   3.133			  34.964		   260.8

2123			    9				   3.584			   3.410			  34.959		   261.4

1974			   10			   3.731			   3.570			  34.972		   262.0

1621			   11			   4.210			   4.076			  35.000		   255.1

1400			   12			   4.843			   4.722			  35.041		   240.0

1174			   13			   6.301			   6.189			  35.089		   194.9

976			   14			   8.705			   8.596			  35.178		   146.6

778			   15			  12.420		  12.313		  35.638		   156.1

629			   16			  15.466		  15.367		  36.091		   171.9

477			   17			  17.742		  17.659		  36.469		   189.1

386			   18			  18.458		  18.390		  36.567		   193.7

295			   19			  18.945		  18.892		  36.619		   202.1

213			   20			  20.120		  20.080		  36.721		   197.7

162			   21			  21.228		  21.197		  36.799		   203.7

110			   22			  22.215		  22.193		  36.838		   209.3

60			   23			  23.406		  23.394		  36.795		   211.4

ABACO-01 R.V. Oceanus

CTD Station 21 (CTD021)

Latitude 26.519N Longitude 74.243W

01-May-2001 04:33Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  23.509		  23.509		  36.782		 212.4			   0.003			  25.137

10			  23.516		  23.514		  36.783		 211.5			   0.028			  25.136

20			  23.516		  23.512		  36.783		 211.1			   0.056			  25.137

30			  23.519		  23.512		  36.784		 212.0			   0.085			  25.138

50			  23.521		  23.511		  36.784		 210.4			   0.141			  25.138

75			  22.966		  22.950		  36.782		 213.8			   0.212			  25.300

100			  22.439		  22.419		  36.852		 205.9			   0.276			  25.507

125			  21.921		  21.896		  36.846		 203.3			   0.337			  25.650

150			  21.252		  21.222		  36.799		 205.5			   0.395			  25.803

200			  20.021		  19.983		  36.711		 204.1			   0.501			  26.072

250			  19.205		  19.160		  36.640		 194.9			   0.597			  26.234

300			  18.852		  18.799		  36.616		 200.4			   0.689			  26.309

400			  18.056		  17.986		  36.521		 191.3			   0.867			  26.441

500			  17.179		  17.095		  36.375		 187.5			   1.037			  26.547

600			  15.684		  15.589		  36.128		 177.6			   1.197			  26.710

700			  13.562		  13.461		  35.795		 164.1			   1.342			  26.915

800			  11.454		  11.350		  35.502		 152.8			   1.467			  27.102

900			   9.486			   9.381			  35.254		 144.7			   1.576			  27.255

1000			   7.827			   7.722			  35.108		 151.8			   1.668			  27.400

1100			   6.774			   6.666			  35.090		 176.2			   1.747			  27.536

1200			   5.829			   5.719			  35.073		 210.9			   1.814			  27.646

1300			   5.183			   5.069			  35.064		 230.2			   1.871			  27.718

1400			   4.769			   4.650			  35.038		 241.8			   1.923			  27.746

1500			   4.467			   4.341			  35.017		 248.5			   1.973			  27.763

1750			   3.935			   3.792			  34.978		 260.2			   2.094			  27.790

2000			   3.658			   3.495			  34.968		 261.7			   2.212			  27.812

2500			   3.198			   2.994			  34.955		 261.3			   2.438			  27.850

3000			   2.707			   2.463			  34.925		 266.4			   2.656			  27.873

3500			   2.361			   2.072			  34.903		 267.8			   2.866			  27.888

4000			   2.266			   1.924			  34.893		 266.2			   3.076			  27.892

4500			   2.223			   1.824			  34.884		 261.6			   3.296			  27.892

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4608			   24			   2.206			   1.795			  34.881		   258.5

4397			    1				   2.238			   1.851			  34.887		   261.6

4097			    2				   2.263			   1.910			  34.892		   264.6

3795			    3				   2.289			   1.970			  34.897		   265.9

3499			    4				   2.362			   2.073			  34.904		   267.9

3198			    5				   2.555			   2.293			  34.918		   267.4

2901			    6				   2.799			   2.563			  34.932		   265.9

2598			    7				   3.109			   2.897			  34.952		   261.7

2388			    8				   3.327			   3.133			  34.964		   260.7

2187			    9				   3.497			   3.319			  34.968		   261.7

1989			   10			   3.688			   3.526			  34.972		   261.7

1791			   11			   3.881			   3.735			  34.975		   261.0

1488			   12			   4.514			   4.389			  35.024		   247.9

1289			   13			   5.275			   5.161			  35.069		   227.6

990			   14			   7.933			   7.828			  35.113		   151.8

788			   15			  11.784		  11.679		  35.683		   158.5

662			   16			  14.484		  14.384		  35.945		   168.3

529			   17			  16.824		  16.736		  36.316		   180.7

401			   18			  18.179		  18.109		  36.534		   190.5

273			   19			  19.014		  18.965		  36.623		   200.1

170			   20			  20.748		  20.715		  36.781		   205.2

171			   21			  20.713		  20.681		  36.778		   204.8

113			   22			  22.080		  22.058		  36.842		   207.9

55			   23			  23.512		  23.500		  36.748		   211.2

ABACO-01 R.V. Oceanus

CTD Station 22 (CTD022)

Latitude 26.518N Longitude 73.963W

01-May-2001 09:58Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  23.243		  23.242		  36.726		 213.2			   0.003			  25.172

10			  23.254		  23.251		  36.727		 213.7			   0.028			  25.170

20			  23.262		  23.258		  36.726		 212.9			   0.056			  25.168

30			  23.264		  23.258		  36.726		 212.2			   0.084			  25.168

50			  23.264		  23.253		  36.727		 212.3			   0.140			  25.170

75			  22.825		  22.810		  36.827		 210.2			   0.208			  25.376

100			  22.404		  22.383		  36.848		 210.8			   0.272			  25.514

125			  21.836		  21.811		  36.829		 201.7			   0.333			  25.662

150			  21.446		  21.416		  36.816		 200.0			   0.391			  25.762

200			  20.284		  20.246		  36.742		 196.3			   0.500			  26.025

250			  19.226		  19.180		  36.648		 193.5			   0.597			  26.235

300			  18.876		  18.822		  36.618		 197.7			   0.690			  26.305

400			  18.074		  18.004		  36.528		 191.9			   0.868			  26.442

500			  17.201		  17.116		  36.383		 187.4			   1.038			  26.548

600			  15.482		  15.387		  36.095		 177.0			   1.196			  26.730

700			  13.366		  13.266		  35.764		 162.4			   1.337			  26.931

800			  11.211		  11.108		  35.467		 151.8			   1.460			  27.120

900			   9.142			   9.039			  35.217		 144.1			   1.566			  27.282

1000			   7.717			   7.612			  35.108		 156.6			   1.656			  27.416

1100			   6.545			   6.438			  35.091		 186.0			   1.732			  27.567

1200			   5.753			   5.643			  35.085		 212.7			   1.796			  27.665

1300			   5.159			   5.045			  35.068		 229.9			   1.851			  27.724

1400			   4.817			   4.697			  35.048		 240.3			   1.903			  27.749

1500			   4.471			   4.345			  35.021		 248.7			   1.953			  27.766

1750			   3.928			   3.786			  34.977		 259.1			   2.073			  27.791

2000			   3.653			   3.490			  34.970		 261.0			   2.190			  27.814

2500			   3.177			   2.973			  34.953		 261.7			   2.416			  27.850

3000			   2.741			   2.496			  34.928		 265.6			   2.635			  27.873

3500			   2.399			   2.109			  34.906		 267.7			   2.846			  27.888

4000			   2.276			   1.934			  34.895		 266.4			   3.058			  27.892

4500			   2.238			   1.839			  34.886		 262.7			   3.278			  27.893

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4717			   24			   2.229			   1.804			  34.882		   258.9

4448			    1				   2.243			   1.850			  34.886		   261.8

4198			    2				   2.260			   1.896			  34.890		   264.0

3898			    3				   2.291			   1.961			  34.896		   266.1

3548			    4				   2.389			   2.095			  34.905		   267.6

3125			    5				   2.637			   2.381			  34.921		   267.3

2747			    6				   2.946			   2.723			  34.940		   264.1

2448			    7				   3.256			   3.056			  34.957		   261.7

2149			    8				   3.554			   3.378			  34.969		   261.7

1842			    9				   3.786			   3.637			  34.972		   261.6

1550			   10			   4.345			   4.216			  35.012		   251.4

1252			   11			   5.338			   5.227			  35.073		   224.2

1050			   12			   7.105			   7.000			  35.093		   170.6

900			   13			   9.172			   9.069			  35.220		   144.8

750			   14			  12.219		  12.117		  35.593		   153.2

599			   15			  15.352		  15.258		  36.070		   173.1

351			   17			  18.441		  18.379		  36.567		   185.7

248			   18			  19.163		  19.118		  36.637		   191.3

202			   19			  20.189		  20.151		  36.732		   196.0

151			   20			  21.272		  21.243		  36.812		   183.2

151			   20			  21.272		  21.243		  36.812		   198.8

101			   21			  22.340		  22.319		  36.836		   211.5

50			   22			  23.250		  23.240		  36.726		   212.1

4				   23			  23.233		  23.232		  36.150		   211.8

ABACO-01 R.V. Oceanus

CTD Station 23 (CTD023)

Latitude 26.519N Longitude 73.588W

01-May-2001 15:43Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			 	  22.381		  22.381		  36.837		 213.9			   0.002			  25.506

10			  22.387		  22.385		  36.839		 213.8			   0.025			  25.506

20			  22.389		  22.385		  36.839		 214.1			   0.049			  25.506

30			  22.392		  22.386		  36.839		 213.2			   0.074			  25.506

50			  22.387		  22.377		  36.838		 215.0			   0.124			  25.508

75			  22.039		  22.024		  36.813		 214.6			   0.185			  25.589

100			  21.857		  21.837		  36.823		 215.9			   0.245			  25.650

125			  21.482		  21.457		  36.807		 210.6			   0.304			  25.744

150			  20.882		  20.854		  36.793		 197.2			   0.359			  25.900

200			  19.764		  19.727		  36.700		 188.2			   0.462			  26.132

250			  19.091		  19.046		  36.634		 196.9			   0.557			  26.259

300			  18.873		  18.819		  36.620		 203.8			   0.648			  26.307

400			  18.109		  18.039		  36.531		 192.6			   0.827			  26.435

500			  17.109		  17.024		  36.371		 183.6			   0.997			  26.561

600			  15.209		  15.115		  36.057		 169.2			   1.154			  26.761

700			  13.101		  13.002		  35.728		 157.2			   1.293			  26.957

800			  11.015		  10.913		  35.442		 148.6			   1.414			  27.136

900			   8.965			   8.863			  35.205		 146.5			   1.517			  27.301

1000			   7.445			   7.342			  35.097		 158.5			   1.605			  27.447

1100			   6.240			   6.137			  35.071		 186.4			   1.678			  27.592

1200			   5.722			   5.612			  35.086		 209.1			   1.740			  27.669

1300			   5.274			   5.159			  35.078		 225.8			   1.795			  27.718

1400			   4.682			   4.563			  35.032		 242.7			   1.848			  27.751

1500			   4.377			   4.252			  35.009		 250.8			   1.897			  27.766

1750			   3.911			   3.769			  34.976		 260.6			   2.017			  27.791

2000			   3.659			   3.497			  34.969		 261.8			   2.134			  27.813

2500			   3.250			   3.046			  34.957		 261.3			   2.364			  27.846

3000			   2.825			   2.578			  34.933		 264.4			   2.586			  27.870

3500			   2.498			   2.205			  34.912		 268.7			   2.802			  27.885

4000			   2.317			   1.974			  34.898		 267.9			   3.017			  27.892

4500			   2.268			   1.868			  34.889		 265.3			   3.239			  27.893

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

4936			   24			   2.209			   1.758			  34.875		   256.2

4800			    1				   2.243			   1.807			  34.881		   259.1

4550			    2				   2.268			   1.862			  34.888		   262.5

4146			    3				   2.297			   1.938			  34.894		   265.5

3800			    4				   2.382			   2.059			  34.903		   267.8

3498			    5				   2.499			   2.207			  34.912		   267.3

3150			    6				   2.724			   2.463			  34.927		   265.6

2800			    7				   2.979			   2.750			  34.941		   263.6

2551			    8				   3.187			   2.978			  34.953		   253.4

2251			    9				   3.464			   3.280			  34.965		   262.0

1899			   10			   3.768			   3.613			  34.972		   261.7

1584			   11			   4.191			   4.061			  34.995		   255.2

1273			   12			   5.348			   5.235			  35.082		   223.0

851			   14			   9.885			   9.783			  35.298		   144.7

699			   15			  13.168		  13.069		  35.733		   156.1

551			   16			  16.135		  16.045		  36.206		   175.5

400			   17			  18.122		  18.052		  36.530		   190.5

300			   18			  18.911		  18.857		  36.622		   203.6

226			   19			  19.383		  19.341		  36.658		   195.7

161			   20			  20.818		  20.787		  36.788		   196.9

110			   21			  21.696		  21.674		  36.814		   216.0

51			   22			  22.555		  22.544		  36.855		   214.7

2				   23			  22.562		  22.561		  36.858		   214.7

ABACO-01 R.V. Oceanus

CTD Station 24 (CTD024)

Latitude 26.517N Longitude 73.234W

01-May-2001 20:56Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  21.608		  21.607		  36.805		 216.5			   0.002			  25.701

10			  21.608		  21.606		  36.807		 216.7			   0.023			  25.702

20			  21.610		  21.606		  36.807		 217.4			   0.046			  25.702

30			  21.592		  21.586		  36.808		 217.3			   0.069			  25.709

50			  21.228		  21.218		  36.797		 211.2			   0.114			  25.802

75			  20.672		  20.658		  36.764		 204.2			   0.167			  25.931

100			  20.101		  20.082		  36.719		 207.9			   0.218			  26.052

125			  19.706		  19.683		  36.694		 214.7			   0.267			  26.139

150			  19.383		  19.355		  36.659		 207.9			   0.314			  26.197

200			  19.090		  19.054		  36.652		 215.9			   0.406			  26.271

250			  18.967		  18.922		  36.638		 216.1			   0.497			  26.294

300			  18.731		  18.677		  36.605		 205.2			   0.588			  26.331

400			  17.992		  17.922		  36.514		 193.0			   0.764			  26.451

500			  17.008		  16.924		  36.348		 185.0			   0.932			  26.568

600			  15.130		  15.037		  36.038		 173.5			   1.087			  26.764

700			  12.767		  12.670		  35.680		 158.1			   1.224			  26.986

800			  10.597		  10.498		  35.392		 149.8			   1.342			  27.171

900			   8.534			   8.435			  35.170		 147.7			   1.441			  27.341

1000			   7.193			   7.092			  35.082		 162.4			   1.525			  27.470

1100			   6.379			   6.275			  35.087		 189.1			   1.597			  27.585

1200			   5.625			   5.516			  35.081		 215.6			   1.659			  27.677

1300			   5.136			   5.022			  35.065		 231.5			   1.714			  27.724

1400			   4.710			   4.591			  35.039		 243.3			   1.765			  27.753

1500			   4.371			   4.247			  35.009		 251.3			   1.815			  27.767

1750			   3.929			   3.786			  34.976		 260.3			   1.935			  27.789

2000			   3.706			   3.543			  34.970		 261.7			   2.053			  27.809

2500			   3.273			   3.068			  34.960		 260.5			   2.283			  27.847

3000			   2.852			   2.604			  34.935		 263.8			   2.506			  27.869

3500			   2.516			   2.223			  34.913		 268.3			   2.725			  27.883

4000			   2.329			   1.986			  34.898		 268.0			   2.941			  27.891

4500			   2.288			   1.887			  34.890		 265.5			   3.164			  27.893

5000			   2.229			   1.769			  34.877		 259.4			   3.397			  27.892

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

5133			   24			   2.123			   1.650			  34.862		   251.9

4902			    1				   2.253			   1.805			  34.881		   259.3

4602			    2				   2.282			   1.869			  34.888		   262.7

4249			    3				   2.305			   1.933			  34.894		   265.1

3899			    4				   2.351			   2.018			  34.900		   267.2

3548			    5				   2.483			   2.185			  34.910		   267.6

3251			    6				   2.668			   2.398			  34.924		   265.7

2950			    7				   2.875			   2.632			  34.936		   263.6

2651			    8				   3.102			   2.885			  34.951		   261.6

2349			    9				   3.402			   3.210			  34.966		   261.2

2050			   10			   3.655			   3.487			  34.968		   262.0

1751			   11			   3.949			   3.806			  34.977		   260.0

1427			   12			   4.613			   4.492			  35.031		   245.0

1103			   13			   6.316			   6.212			  35.084		   190.9

878			   14			   9.090			   8.990			  35.219		   146.3

728			   15			  12.399		  12.299		  35.634		   159.3

576			   16			  15.393		  15.303		  36.078		   173.9

425			   17			  17.751		  17.678		  36.471		   189.0

277			   18			  18.839		  18.789		  36.616		   208.7

211			   19			  19.086		  19.047		  36.643		   213.3

149			   20			  19.567		  19.539		  36.670		   219.2

100			   21			  20.323		  20.305		  36.735		   202.9

51			   22			  21.545		  21.535		  36.814		   210.7

3				   23			  22.024		  22.023		  36.822		   216.5

ABACO-01 R.V. Oceanus

CTD Station 25 (CTD025)

Latitude 26.580N Longitude 72.858W

02-May-2001 17:23Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  22.934		  22.933		  36.843		 212.7			   0.003			  25.351

10			  22.933		  22.931		  36.844		 212.6			   0.026			  25.353

20			  22.935		  22.931		  36.844		 212.5			   0.052			  25.353

30			  22.932		  22.926		  36.845		 212.5			   0.079			  25.355

50			  22.054		  22.044		  36.809		 213.4			   0.129			  25.581

75			  21.574		  21.559		  36.804		 209.8			   0.188			  25.713

100			  21.259		  21.239		  36.812		 210.6			   0.244			  25.808

125			  20.732		  20.709		  36.776		 199.2			   0.299			  25.926

150			  20.052		  20.024		  36.716		 205.6			   0.350			  26.065

200			  19.478		  19.442		  36.673		 202.6			   0.447			  26.186

250			  19.040		  18.994		  36.632		 202.5			   0.540			  26.270

300			  18.885		  18.831		  36.629		 211.0			   0.632			  26.310

400			  18.013		  17.944		  36.511		 193.5			   0.809			  26.444

500			  16.983		  16.899		  36.340		 186.1			   0.979			  26.567

600			  14.999		  14.907		  36.020		 173.0			   1.134			  26.779

700			  12.636		  12.539		  35.660		 154.8			   1.270			  26.997

800			  10.471		  10.372		  35.377		 147.8			   1.386			  27.182

900			   8.553			   8.454			  35.173		 148.6			   1.485			  27.340

1000			   7.235			   7.134			  35.088		 162.5			   1.569			  27.469

1100			   6.338			   6.234			  35.082		 188.8			   1.642			  27.587

1200			   5.760			   5.650			  35.087		 212.3			   1.704			  27.666

1300			   5.103			   4.990			  35.058		 232.9			   1.759			  27.723

1400			   4.667			   4.549			  35.031		 244.9			   1.810			  27.751

1500			   4.366			   4.242			  35.009		 251.9			   1.860			  27.767

1750			   3.910			   3.767			  34.976		 261.0			   1.980			  27.791

2000			   3.681			   3.518			  34.971		 262.5			   2.097			  27.812

2500			   3.254			   3.049			  34.959		 262.0			   2.325			  27.848

3000			   2.808			   2.562			  34.932		 266.5			   2.547			  27.870

3500			   2.500			   2.207			  34.913		 269.1			   2.763			  27.885

4000			   2.334			   1.990			  34.899		 269.3			   2.979			  27.892

4500			   2.276			   1.876			  34.890		 266.5			   3.201			  27.893

5000			   2.136			   1.679			  34.866		 255.2			   3.433			  27.889

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

5164			   24			   2.047			   1.573			  34.852		   252.3

4998			    1				   2.133			   1.677			  34.864		   255.6

4699			    2				   2.265			   1.841			  34.885		   263.3

4300			    3				   2.295			   1.918			  34.892		   270.4

3949			    4				   2.338			   2.000			  34.898		   268.6

3597			    5				   2.457			   2.155			  34.909		   269.7

3199			    6				   2.662			   2.398			  34.923		   268.3

2900			    7				   2.905			   2.667			  34.939		   265.7

2601			    8				   3.170			   2.957			  34.954		   263.2

2303			    9				   3.416			   3.228			  34.965		   264.7

2001			   10			   3.658			   3.496			  34.967		   263.9

1700			   11			   3.980			   3.842			  34.980		   261.4

1400			   12			   4.633			   4.515			  35.029		   247.1

1148			   13			   6.003			   5.897			  35.088		   203.8

900			   14			   8.435			   8.337			  35.162		   150.4

749			   15			  11.504		  11.406		  35.504		   151.5

602			   16			  15.128		  15.034		  36.037		   169.9

453			   17			  17.517		  17.439		  36.428		   186.8

301			   18			  18.808		  18.754		  36.607		   204.0

202			   19			  19.356		  19.319		  36.665		   208.2

155			   20			  19.948		  19.919		  36.708		   196.8

112			   21			  20.878		  20.856		  36.780		   205.0

60			   22			  21.823		  21.811		  36.804		   215.9

3				   23			  23.043		  23.042		  36.835		   213.6

ABACO-01 R.V. Oceanus

CTD Station 26 (CTD026)

Latitude 26.515N Longitude 72.462W

02-May-2001 00:02Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  22.574		  22.573		  36.849		 214.5			   0.003			  25.460

10			  22.579		  22.577		  36.851		 213.6			   0.025			  25.461

20			  22.574		  22.570		  36.851		 214.9			   0.050			  25.463

30			  22.547		  22.541		  36.852		 214.1			   0.075			  25.472

50			  21.736		  21.726		  36.810		 214.7			   0.124			  25.671

75			  21.108		  21.093		  36.788		 208.0			   0.180			  25.830

100			  20.421		  20.402		  36.746		 198.5			   0.233			  25.986

125			  19.903		  19.879		  36.706		 194.2			   0.283			  26.096

150			  19.446		  19.418		  36.662		 187.7			   0.331			  26.184

200			  18.889		  18.853		  36.614		 199.1			   0.422			  26.293

250			  18.642		  18.597		  36.594		 201.8			   0.511			  26.343

300			  18.326		  18.273		  36.560		 192.9	  		   0.599			  26.399

400			  17.586		  17.517		  36.448		 192.7			   0.769			  26.501

500			  16.534		  16.452		  36.272		 181.2			   0.933			  26.621

600			  14.350		  14.260		  35.906		 167.0			   1.082			  26.832

700			  12.119		  12.025		  35.590		 154.5			   1.213			  27.043

800			  10.518		  10.419		  35.387		 149.8			   1.327			  27.182

900			   8.516			   8.417			  35.164		 147.5			   1.426			  27.339

1000			   7.054			   6.955			  35.086		 166.8			   1.509			  27.493

1100			   6.292			   6.188			  35.077		 189.4			   1.579			  27.589

1200			   5.666			   5.558			  35.089		 214.0			   1.640			  27.679

1300			   5.146			   5.033			  35.066		 231.2			   1.695			  27.724

1400			   4.739			   4.620			  35.040		 242.6			   1.747			  27.751

1500			   4.453			   4.328			  35.021		 249.5			   1.797			  27.768

1750			   3.945			   3.802			  34.980		 259.9			   1.917			  27.791

2000			   3.748			   3.584			  34.989		 258.9			   2.034			  27.820

2500			   3.210			   3.006			  34.957		 261.7			   2.262			  27.851

3000			   2.794			   2.548			  34.931		 266.3			   2.482			  27.871

3500			   2.481			   2.190			  34.912		 269.4			   2.697			  27.886

4000			   2.321			   1.977			  34.898		 269.0			   2.912			  27.892

4500			   2.287			   1.887			  34.891		 267.2			   3.134			  27.893

5000			   2.273			   1.811			  34.883		 263.2			   3.367			  27.893

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

5235			   24			   2.106			   1.620			  34.858		   251.6

5099			    1				   2.268			   1.794			  34.879		   258.7

4499			    3				   2.289			   1.888			  34.890		   263.5

4195			    4				   2.291			   1.926			  34.893		   265.0

3799			    5				   2.374			   2.052			  34.902		   267.5

3401			    6				   2.535			   2.253			  34.915		   267.4

2998			    7				   2.802			   2.556			  34.931		   265.4

2693			    8				   3.041			   2.821			  34.946		   262.6

2400			    9				   3.321			   3.125			  34.963		   261.1

2101			   10			   3.574			   3.403			  34.972		   261.0

1799			   11			   3.895			   3.749			  34.977		   260.6

1449			   12			   4.602			   4.480			  35.034		   245.5

902			   14			   8.391			   8.293			  35.151		   148.9

699			   15			  12.290		  12.195		  35.612		   153.6

549			   16			  15.588		  15.501		  36.101		   171.8

401			   17			  17.797		  17.728		  36.481		   190.7

311			   18			  18.408		  18.353		  36.565		   191.8

242			   19			  18.746		  18.703		  36.592		   198.3

179			   20			  19.100		  19.068		  36.616		   196.5

120			   21			  20.034		  20.012		  36.708		   194.9

4				   23			  22.407		  22.407		  36.859		   215.6

ABACO-01 R.V. Oceanus

CTD Station 27 (CTD027)

Latitude 26.531N Longitude 72.098W

03-May-2001 06:13Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.018		  24.018		  36.758		 206.7			   0.003			  24.968

10			  24.021		  24.019		  36.767		 207.6			   0.030			  24.974

20			  24.020		  24.016		  36.768		 207.7			   0.060			  24.976

30			  24.024		  24.018		  36.767		 207.8			   0.089			  24.975

50			  23.182		  23.171		  36.803		 210.9			   0.148			  25.252

75			  21.775		  21.760		  36.818		 212.4			   0.211			  25.667

100			  21.205		  21.185		  36.793		 212.4			   0.267			  25.809

125			  20.726		  20.702		  36.771		 201.8			   0.321			  25.924

150			  20.361		  20.332		  36.745		 198.8			   0.374			  26.004

200			  19.244		  19.208		  36.645		 192.7			   0.471			  26.225

250			  18.768		  18.723		  36.610		 199.2			   0.562			  26.323

300			  18.433		  18.380		  36.571		 195.6			   0.651			  26.380

400			  17.655		  17.586		  36.460		 191.8			   0.822			  26.493

500			  16.311		  16.229		  36.236		 180.5			   0.985			  26.645

600			  14.435		  14.345		  35.933		 170.0			   1.133			  26.835

700			  12.433		  12.338		  35.637		 160.3			   1.265			  27.019

800			  10.186		  10.089		  35.343		 148.3			   1.377			  27.205

900			   8.413			   8.315			  35.152		 146.8			   1.473			  27.345

1000			   7.128			   7.027			  35.096		 168.6			   1.556			  27.491

1100			   6.235			   6.131			  35.103		 196.9			   1.625			  27.617

1200			   5.555			   5.447			  35.084		 216.7			   1.684			  27.688

1300			   5.158			   5.044			  35.073		 230.3			   1.738			  27.728

1400			   4.714			   4.595			  35.045		 242.8			   1.789			  27.757

1500			   4.413			   4.288			  35.021		 249.5			   1.839			  27.772

1750			   3.910			   3.768			  34.977		 260.0			   1.957			  27.792

2000			   3.693			   3.530			  34.978		 259.3			   2.074			  27.817

2500			   3.240			   3.035			  34.959		 261.0			   2.302			  27.849

3000			   2.825			   2.578			  34.934		 265.3			   2.522			  27.871

3500			   2.519			   2.226			  34.914		 269.5			   2.739			  27.884

4000			   2.354			   2.010			  34.901		 269.5			   2.956			  27.891

4500			   2.302			   1.900			  34.892		 267.8			   3.179			  27.893

5000			   2.253			   1.792			  34.880		 262.2			   3.413			  27.892

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

5365			   24			   2.064			   1.563			  34.851		   248.7

5199			    1				   2.170			   1.687			  34.866		   253.6

4904			    2				   2.276			   1.826			  34.884		   260.5

4597			    3				   2.291			   1.878			  34.889		   263.7

4298			    4				   2.315			   1.938			  34.894		   265.6

3899			    5				   2.367			   2.034			  34.902		   267.7

3498			    6				   2.495			   2.203			  34.912		   267.9

3098			    7				   2.761			   2.505			  34.929		   264.6

2801			    8				   2.968			   2.739			  34.942		   262.7

2499			    9				   3.239			   3.035			  34.960		   261.0

2200			   10			   3.476			   3.297			  34.968		   261.6

1898			   11			   3.773			   3.619			  34.978		   260.6

1496			   12			   4.383			   4.259			  35.015		   250.7

1200			   13			   5.640			   5.531			  35.087		   216.2

997			   14			   7.141			   7.041			  35.100		   171.3

804			   15			  10.112		  10.015		  35.331		   146.9

614			   16			  14.217		  14.125		  35.898		   169.2

502			   17			  16.341		  16.259		  36.239		   178.2

354			   18			  18.002		  17.940		  36.514		   191.3

250			   19			  18.803		  18.758		  36.605		   197.9

149			   20			  20.251		  20.222		  36.724		   199.4

101			   21			  21.389		  21.369		  36.799		   213.6

49			   22			  23.909		  23.898		  36.791		   211.6

3				   23			  23.960		  23.959		  36.787		   209.4

ABACO-01 R.V. Oceanus

CTD Station 28 (CTD028)

Latitude 26.529N Longitude 71.729W

03-May-2001 12:59Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.021		  24.021		  36.820		 209.9			   0.003			  25.014

10			  24.022		  24.020		  36.822		 209.8			   0.029			  25.015

20			  24.022		  24.018		  36.822		 209.5			   0.059			  25.016

30			  24.024		  24.018		  36.822		 208.7			   0.088			  25.016

50			  24.028		  24.017		  36.823		 209.7			   0.147			  25.017

75			  22.993		  22.978		  36.890		 212.3			   0.217			  25.375

100			  22.392		  22.371		  36.949		 202.0			   0.280			  25.594

125			  21.289		  21.265		  36.810		 204.0			   0.339			  25.799

150			  20.586		  20.558		  36.756		 197.1			   0.393			  25.952

200			  19.417		  19.380		  36.658		 188.9			   0.492			  26.191

250			  18.916		  18.871		  36.611		 197.5			   0.585			  26.286

300			  18.537		  18.484		  36.583		 189.6			   0.675			  26.363

400			  17.657		  17.588		  36.460		 192.8			   0.847			  26.493

500			  16.262		  16.181		  36.229		 184.1			   1.009			  26.651

600			  14.305		  14.216		  35.914		 166.6			   1.157			  26.848

700			  12.105		  12.011		  35.589		 151.4			   1.286			  27.045

800			  10.137		  10.041		  35.332		 146.0			   1.398			  27.205

900			   8.345			   8.247			  35.156		 148.7			   1.495			  27.358

1000			   7.098			   6.998			  35.105		 169.6			   1.576			  27.502

1100			   6.204			   6.101			  35.103		 197.4			   1.644			  27.622

1200			   5.487			   5.380			  35.083		 219.6			   1.703			  27.696

1300			   5.107			   4.993			  35.075		 231.8			   1.756			  27.735

1400			   4.731			   4.612			  35.049		 241.8			   1.807			  27.759

1500			   4.434			   4.308			  35.026		 249.7			   1.856			  27.774

1750			   3.856			   3.715			  34.975		 261.1			   1.974			  27.796

2000			   3.612			   3.450			  34.970		 262.0			   2.090			  27.818

2500			   3.176			   2.973			  34.956		 261.1			   2.314			  27.853

3000			   2.777			   2.531			  34.932		 264.5			   2.532			  27.873

3500			   2.512			   2.220			  34.913		 269.3			   2.748			  27.884

4000			   2.348			   2.004			  34.900		 269.7			   2.964			  27.891

4500			   2.313			   1.912			  34.893		 268.7			   3.188			  27.893

5000			   2.295			   1.832			  34.885		 264.8			   3.423			  27.893

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

5467			   24			   2.073			   1.558			  34.851		   248.8

5300			    1				   2.210			   1.712			  34.871		   254.7

4698			    3				   2.309			   1.884			  34.891		   263.3

4396			    4				   2.312			   1.923			  34.894		   265.2

4093			    5				   2.336			   1.982			  34.899		   267.1

3697			    6				   2.427			   2.115			  34.906		   268.4

3297			    7				   2.618			   2.344			  34.921		   265.8

2900			    8				   2.872			   2.634			  34.939		   263.1

2600			    9				   3.093			   2.882			  34.952		   261.8

2301			   10			   3.372			   3.185			  34.965		   261.8

2000			   11			   3.604			   3.442			  34.971		   262.4

1300			   13			   5.122			   5.009			  35.077		   230.8

1300			   13			   5.122			   5.009			  35.077		   231.3

1052			   14			   6.649			   6.547			  35.105		   184.3

850			   15			   9.220			   9.122			  35.232		   138.0

600			   16			  14.642		  14.551		  35.961		   167.1

451			   17			  17.038		  16.963		  36.354		   185.5

300			   18			  18.403		  18.350		  36.569		   188.3

251			   19			  18.803		  18.758		  36.602		   192.9

174			   20			  19.796		  19.763		  36.693		   199.7

118			   21			  21.441		  21.418		  36.833		   207.9

59			   22			  23.989		  23.977		  36.826		   210.8

3 			   23			  23.979		  23.979		  36.820		   210.5

ABACO-01 R.V. Oceanus

CTD Station 29 (CTD029)

Latitude 26.534N Longitude 71.337W

03-May-2001 19:57Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  23.843		  23.842		  36.841		 209.0			   0.003			  25.083

10			  23.841		  23.839		  36.842		 208.2			   0.029			  25.085

20			  23.836		  23.831		  36.842		 208.3			   0.057			  25.087

30			  23.831		  23.825		  36.841		 208.8			   0.086			  25.088

50			  23.809		  23.799		  36.844		 209.1			   0.144			  25.098

75			  23.214		  23.198		  36.873		 211.7			   0.213			  25.297

100			  22.498		  22.478		  36.938		 206.1			   0.278			  25.555

125			  21.776		  21.751		  36.937		 195.6			   0.337			  25.761

150			  20.484		  20.455		  36.766		 196.3			   0.392			  25.987

200			  19.265		  19.228		  36.638		 190.3			   0.490			  26.214

250			  18.780		  18.736		  36.610		 198.1			   0.581			  26.320

300			  18.420		  18.367		  36.566		 192.3			   0.669			  26.380

400			  17.622		  17.554		  36.457		 193.4			   0.840			  26.499

500			  16.323		  16.241		  36.240		 184.6			   1.002			  26.646

600			  14.357		  14.267		  35.926		 174.0			   1.149			  26.846

700			  12.433		  12.337		  35.652		 163.8			   1.279			  27.030

800			  10.299		  10.201		  35.358		 148.0			   1.392			  27.197

900			   8.421			   8.323			  35.151		 144.3			   1.490			  27.343

1000			   7.156			   7.056			  35.082		 163.1			   1.574			  27.475

1100			   6.372			   6.267			  35.104		 195.1			   1.645			  27.600

1200			   5.562			   5.454			  35.085		 221.5			   1.705			  27.688

1300			   5.079			   4.966			  35.062		 234.7			   1.759			  27.728

1400			   4.689			   4.571			  35.043		 243.3			   1.810			  27.759

1500			   4.411			   4.286			  35.025		 249.1			   1.859			  27.776

1750			   3.943			   3.800			  34.995		 256.6			   1.976			  27.803

2000			   3.573			   3.411			  34.971		 260.3			   2.091			  27.823

2500			   3.088			   2.887			  34.951		 260.9			   2.311			  27.857

3000			   2.709			   2.464			  34.928		 263.7			   2.526			  27.876

3500			   2.441			   2.150			  34.910		 268.3			   2.737			  27.887

4000			   2.326			   1.982			  34.899		 268.4			   2.951			  27.892

4500			   2.297			   1.896			  34.892		 267.0			   3.173			  27.893

5000			   2.274			   1.812			  34.883		 262.5			   3.407			  27.893

5500			   2.104			   1.584			  34.854		 250.4			   3.650			  27.887

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

5583			   24			   2.070			   1.541			  34.848		   249.5

5347			    1				   2.184			   1.682			  34.865		   254.2

5102			    2				   2.263			   1.789			  34.879		   259.1

4799			    3				   2.289			   1.852			  34.887		   262.7

4498			    4				   2.301			   1.900			  34.891		   264.5

4199			    5				   2.309			   1.943			  34.894		   266.0

3799			    6				   2.360			   2.038			  34.901		   267.5

3398			    7				   2.486			   2.205			  34.912		   267.4

3001			    8				   2.707			   2.463			  34.924		   265.5

2698			    9				   2.937			   2.718			  34.942		   263.5

2398			   10			   3.231			   3.037			  34.961		   260.2

2052			   11			   3.532			   3.366			  34.971		   261.3

1700			   12			   4.008			   3.869			  35.002		   256.1

1400			   13			   4.690			   4.572			  35.041		   243.8

1101			   14			   6.339			   6.234			  35.104		   196.8

924			   15			   8.026			   7.928			  35.115		   149.2

674			   16			  12.896		  12.802		  35.711		   163.5

526			   17			  15.841		  15.757		  36.159		   181.1

376			   18			  17.883		  17.818		  36.494		   192.0

275			   19			  18.644		  18.595		  36.583		   197.7

167			   20			  20.441		  20.410		  36.750		   208.5

110			   21			  22.379		  22.357		  41.604		   209.8

55			   22			  23.727		  23.715		  36.861		   210.7

4				   23			  23.732		  23.731		  36.855		   211.1

ABACO-01 R.V. Oceanus

CTD Station 30 (CTD030)

Latitude 26.537N Longitude 71.009W

04-May-2001 02:10Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  23.632		  23.632		  36.855		 211.3			   0.003			  25.156

10			  23.636		  23.634		  36.857		 211.7			   0.028			  25.157

20			  23.640		  23.636		  36.856		 210.4			   0.056			  25.156

30			  23.640		  23.634		  36.857		 210.3			   0.084			  25.157

50			  23.588		  23.578		  36.871		 211.3			   0.140			  25.184

75			  23.042		  23.026		  36.875		 211.6			   0.208			  25.348

100			  22.642		  22.621		  36.892		 210.9			   0.273			  25.479

125			  21.747		  21.722		  36.915		 199.0			   0.333			  25.752

150			  21.075		  21.046		  36.867		 188.5			   0.388			  25.903

200			  19.343		  19.306		  36.647		 191.2			   0.488			  26.201

250			  18.773		  18.728		  36.599		 198.7			   0.579			  26.314

300			  18.394		  18.341		  36.562		 199.8			   0.668			  26.384

400			  17.671		  17.603		  36.464		 192.0			   0.839			  26.492

500			  16.461		  16.379		  36.262		 185.9			   1.003			  26.631

600			  14.490		  14.399		  35.947		 177.4			   1.152			  26.834

700			  12.539		  12.442		  35.659		 162.4			   1.283			  27.015

800			  10.529		  10.430		  35.399		 153.1			   1.397			  27.189

900			   8.608			   8.509			  35.170		 145.8			   1.496			  27.329

1000			   7.132			   7.032			  35.069		 160.1			   1.581			  27.468

1100			   6.345			   6.241			  35.074		 183.8			   1.653			  27.580

1200			   5.690			   5.581			  35.085		 213.0			   1.715			  27.673

1300			   5.255			   5.140			  35.081		 228.5			   1.771			  27.723

1400			   4.834			   4.713			  35.061		 239.3			   1.823			  27.757

1500			   4.393			   4.269			  35.016		 250.6			   1.872			  27.771

1750			   3.853			   3.711			  34.975		 260.3			   1.991			  27.796

2000			   3.591			   3.430			  34.970		 261.0			   2.106			  27.820

2500			   3.119			   2.917			  34.953		 261.0			   2.327			  27.856

3000			   2.735			   2.490			  34.930		 262.6			   2.542			  27.875

3500			   2.477			   2.186			  34.912		 265.5			   2.755			  27.886

4000			   2.336			   1.992			  34.900		 268.3			   2.970			  27.892

4500			   2.297			   1.896			  34.892		 266.8			   3.193			  27.894

5000			   2.264			   1.802			  34.882		 262.7			   3.427			  27.893

5500			   2.098			   1.579			  34.853		 250.1			   3.669			  27.887

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

5583			   24			   2.075			   1.545			  34.849		   248.6

5197			    1				   2.220			   1.735			  34.873		   256.4

4897			    2				   2.273			   1.825			  34.885		   260.6

4549			    3				   2.293			   1.886			  34.890		   263.9

4198			    4				   2.310			   1.944			  34.896		   265.6

3851			    5				   2.361			   2.034			  34.902		   266.1

3497			    6				   2.476			   2.184			  34.912		   265.3

3149			    7				   2.658			   2.399			  34.924		   262.3

2803			    8				   2.892			   2.664			  34.939		   262.0

2499			    9				   3.123			   2.920			  34.951		   262.3

2200			   10			   3.440			   3.261			  34.973		   259.5

1904			   11			   3.679			   3.525			  34.971		   262.8

1653			   12			   4.001			   3.867			  34.989		   258.6

1402			   13			   4.670			   4.551			  35.045		   244.2

1152			   14			   5.935			   5.828			  35.100		   206.0

899			   15			   8.601			   8.502			  35.172		   146.9

704			   16			  12.280		  12.185		  35.633		   160.4

554			   17			  15.356		  15.269		  36.084		   177.2

328			   18			  18.362		  18.304		  36.555		   194.4

225			   19			  19.018		  18.977		  36.614		   197.7

149			   20			  20.897		  20.868		  36.840		   194.8

90			   21			  22.930		  22.912		  36.907		   212.5

44			   22			  23.660		  23.650		  36.860		   211.7

3				   23			  23.656		  23.656		  36.860		   211.6

ABACO-01 R.V. Oceanus

CTD Station 31 (CTD031)

Latitude 26.540N Longitude 70.681W

04-May-2001 11:18Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.018		  24.018		  36.795		 206.7			   0.003			  24.996

10			  24.015		  24.013		  36.797		 207.2			   0.030			  24.999

20			  24.024		  24.020		  36.797		 207.4			   0.059			  24.997

30			  24.023		  24.016		  36.797		 208.3			   0.089			  24.998

50			  24.026		  24.015		  36.798		 206.9			   0.148			  24.999

75			  23.173		  23.157		  36.903		 211.6			   0.219			  25.332

100			  22.084		  22.064		  36.847		 208.7			   0.281			  25.604

125			  21.561		  21.536		  36.829		 204.2			   0.340			  25.738

150			  20.810		  20.781		  36.810		 198.1			   0.395			  25.933

200			  19.551		  19.515		  36.666		 191.7			   0.497			  26.161

250			  18.876		  18.831		  36.611		 199.8			   0.590			  26.296

300			  18.556		  18.503		  36.580		 195.4			   0.679			  26.357

400			  17.820		  17.751		  36.487		 192.7			   0.853			  26.473

500			  16.565		  16.483		  36.274		 181.3			   1.018			  26.615

600			  14.757		  14.665		  35.985		 173.8			   1.169			  26.805

700			  12.852		  12.754		  35.703		 164.0			   1.303			  26.987

800			  10.592		  10.493		  35.403		 151.6			   1.420			  27.181

900			   8.659	 	  	   8.559			  35.175		 145.4			   1.520			  27.325

1000			   7.192			   7.091			  35.061		 156.3			   1.606			  27.454

1100			   6.453			   6.347			  35.080		 185.3			   1.679			  27.571

1200			   5.795			   5.685			  35.092		 210.4			   1.742			  27.666

1300			   5.180			   5.066			  35.068		 229.2			   1.798			  27.721

1400			   4.756			   4.637			  35.044		 242.2			   1.850			  27.752

1500			   4.509			   4.383			  35.031		 246.8			   1.900			  27.770

1750			   3.950			   3.807			  34.989		 257.4			   2.019			  27.797

2000			   3.716			   3.553			  34.987		 257.9			   2.135			  27.821

2500			   3.134			   2.932			  34.954		 260.5			   2.358			  27.855

3000			   2.736			   2.491			  34.929		 264.1			   2.574			  27.874

3500			   2.460			   2.169			  34.910		 267.9			   2.786			  27.886

4000			   2.332			   1.989			  34.899		 268.7			   3.001			  27.892

4500			   2.307			   1.906			  34.893		 267.5			   3.224			  27.893

5000			   2.294			   1.832			  34.885		 264.0			   3.459			  27.893

5500			   2.152			   1.630			  34.860		 253.3			   3.704			  27.888

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

5592			   24			   2.082			   1.551			  34.850		   248.8

5297			    1				   2.246			   1.747			  34.874		   257.2

4599			    3				   2.304			   1.890			  34.891		   264.1

4298			    4				   2.313			   1.936			  34.895		   266.2

3898			    5				   2.347			   2.015			  34.902		   267.2

3548			    6				   2.444			   2.147			  34.909		   267.6

3200			    7				   2.608			   2.345			  34.921		   266.2

2851			    8				   2.851			   2.619			  34.939		   262.4

2449			    9				   3.197			   2.998			  34.959		   261.2

2049			   10			   3.566			   3.400			  34.975		   260.8

1696			   11			   4.012			   3.874			  34.994		   258.0

1374			   12			   4.836			   4.718			  35.052		   240.3

1102			   13			   6.516			   6.410			  35.106		   189.0

950			   14			   7.659			   7.561			  35.084		   151.3

755			   15			  11.623		  11.523		  35.541		   156.5

651			   16			  13.735		  13.640		  35.833		   167.9

502			   17			  16.746		  16.662		  36.305		   185.6

349			   18			  18.344		  18.282		  36.550		   192.2

248			   19			  18.973		  18.928		  37.272		   197.3

161			   20			  20.702		  20.672		  36.772		   205.7

110			   21			  21.979		  21.957		  36.827		   212.7

58			   22			  23.959		  23.946		  36.814		   210.3

3				   23			  23.951		  23.950		  36.805		   209.9

ABACO-01 R.V. Oceanus

CTD Station 32 (CTD032)

Latitude 26.521N Longitude 70.200W

04-May-2001 18:03Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  24.017		  24.016		  36.773		 205.8			   0.003			  24.979

10			  24.021		  24.019		  36.774		 207.1			   0.030			  24.980

20			  24.019		  24.015		  36.775		 207.8			   0.059			  24.981

30			  24.010		  24.004		  36.775		 207.9			   0.089			  24.985

50			  23.983		  23.972		  36.800		 207.8			   0.149			  25.013

75			  23.189		  23.173		  36.891		 211.2			   0.220			  25.318

100			  22.599		  22.579		  36.877		 209.9			   0.285			  25.480

125			  21.769		  21.744		  36.846		 206.6			   0.346			  25.693

150			  21.156		  21.127		  36.797		 204.9			   0.403			  25.827

200			  20.532		  20.494		  36.747		 208.6			   0.511			  25.963

250			  19.243		  19.197		  36.640		 194.9			   0.611			  26.224

300			  18.751		  18.697		  36.603		 198.5			   0.703			  26.325

400			  18.002		  17.932		  36.514		 193.8			   0.880			  26.449

500			  16.850		  16.766		  36.326		 187.6			   1.048			  26.589

600			  14.939		  14.846		  36.014		 176.0			   1.201			  26.788

700			  12.765		  12.667		  35.691		 164.0			   1.336			  26.995

800			  10.877		  10.776		  35.438		 153.0			   1.453			  27.157

900			   8.967			   8.866			  35.205		 145.3			   1.555			  27.301

1000			   7.550			   7.447			  35.090		 155.4			   1.644			  27.426

1100			   6.487			   6.382			  35.093		 188.9			   1.719			  27.576

1200			   5.800			   5.690			  35.096		 212.1			   1.782			  27.668

1300			   5.248			   5.133			  35.078		 229.7			   1.838			  27.722

1400			   4.854			   4.733			  35.055		 240.2			   1.890			  27.750

1500			   4.572			   4.445			  35.039		 246.7			   1.940			  27.769

1750			   3.953			   3.811			  34.984		 258.5			   2.061			  27.793

2000			   3.673			   3.510			  34.979		 259.5			   2.177			  27.819

2500			   3.130			   2.927			  34.955		 259.6			   2.400			  27.856

3000			   2.728			   2.483			  34.929		 263.5			   2.615			  27.874

3500			   2.444			   2.153			  34.909		 267.5			   2.827			  27.887

4000			   2.329			   1.986			  34.899		 267.9			   3.042			  27.892

4500			   2.302			   1.901			  34.893		 266.8			   3.264			  27.894

5000			   2.275			   1.813			  34.883		 262.6			   3.499			  27.893

5500			   2.111			   1.591			  34.854		 250.3			   3.742			  27.887

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

5591			   24			   2.076			   1.545			  34.849		   248.6

5397			    1				   2.165			   1.657			  34.862		   253.2

5098			    2				   2.255			   1.782			  34.879		   258.5

4801			    3				   2.286			   1.849			  34.887		   265.8

4451			    4				   2.300			   1.905			  34.893		   265.1

4104			    5				   2.321			   1.966			  34.898		   266.8

3673			    6				   2.396			   2.087			  34.906		   267.9

3251			    7				   2.579			   2.312			  34.919		   266.3

2852			    8				   2.837			   2.605			  34.936		   262.4

2499			    9				   3.121			   2.919			  34.953		   262.4

2152			   10			   3.558			   3.383			  34.987		   256.8

1800			   11			   3.900			   3.753			  34.985		   259.5

1554			   12			   4.360			   4.230			  35.020		   250.8

1298			   13			   5.309			   5.194			  35.079		   226.5

1050			   14			   6.992			   6.887			  35.088		   173.0

798			   15			  11.305		  11.202		  35.488		   153.9

650			   16			  14.123		  14.026		  35.889		   170.8

527			   17			  16.481		  16.394		  36.267		   186.5

402			   18			  18.072		  18.002		  36.519		   192.2

281			   19			  19.029		  18.978		  36.615		   197.1

151			   20			  21.277		  21.247		  36.806		   210.0

99			   21			  22.612		  22.592		  36.864		   215.7

50		   	   22			  23.920		  23.909		  36.801		   210.5

4				   23			  24.018		  24.018		  36.768		   209.9

ABACO-01 R.V. Oceanus

CTD Station 33 (CTD033)

Latitude 26.520N Longitude 69.806W

04-May-2001 00:27Z

Pressure		Temp90		PoTemp90	Salinity		Oxygen		   DynHt		   SigTh

dbar			     oC		           oC		      PSS-78		μmol/kg		    m2/s2		   kg/m3

1			  23.814		  23.813		  36.837		 208.7			   0.003			  25.089

10			  23.813		  23.810		  36.839		 207.9			   0.029			  25.091

20			  23.818		  23.814		  36.839		 209.0			   0.057			  25.090

30			  23.820		  23.813		  36.839		 209.5			   0.086			  25.090

50			  23.824		  23.813		  36.839		 208.8			   0.144			  25.090

75			  23.558		  23.542		  36.895		 210.7			   0.215			  25.213

100			  23.073		  23.052		  36.884		 211.7			   0.284			  25.349

125			  22.309		  22.284		  36.860		 210.7			   0.348			  25.551

150			  21.601		  21.571		  36.810		 205.3			   0.408			  25.714

200			  20.597		  20.559		  36.763		 208.1			   0.519			  25.957

250			  19.388		  19.342		  36.649		 191.9			   0.619			  26.194

300			  18.872		  18.818		  36.606		 199.2			   0.712			  26.296

400			  18.046		  17.977		  36.520		 193.1			   0.890			  26.443

500			  16.971		  16.887		  36.343		 186.4			   1.059			  26.573

600			  15.371		  15.277		  36.085		 181.0			   1.215			  26.747

700			  13.384		  13.283		  35.778		 167.8			   1.355			  26.938

800			  11.130		  11.027		  35.458		 149.2			   1.478			  27.127

900			   9.167			   9.064			  35.226		 145.4			   1.583			  27.285

1000			   7.609			   7.505			  35.091		 153.2			   1.673			  27.418

1100			   6.728			   6.620			  35.095		 179.4			   1.750			  27.546

1200			   5.869			   5.758			  35.092		 208.1			   1.816			  27.657

1300			   5.379			   5.263			  35.083		 225.9			   1.873			  27.710

1400			   5.026			   4.904			  35.076		 234.9			   1.927			  27.746

1500			   4.673			   4.545			  35.055		 243.0			   1.977			  27.771

1750			   4.048			   3.903			  35.009		 253.6			   2.097			  27.804

2000			   3.720			   3.556			  34.996		 254.6			   2.212			  27.828

2500			   3.130			   2.928			  34.961		 254.8			   2.432			  27.861

3000			   2.779			   2.533			  34.935		 254.3			   2.647			  27.875

3500			   2.483			   2.191			  34.912		 267.7			   2.862			  27.886

4000			   2.350			   2.006			  34.901		 268.7			   3.077			  27.892

4500			   2.294			   1.893			  34.892		 265.0			   3.300			  27.894

5000			   2.255			   1.794			  34.880		 261.6			   3.534			  27.892

5500			   2.100			   1.580			  34.853		 249.4			   3.775			  27.886

Pressure		 Niskin		Temp90		PoTemp90	Salinity		Oxygen

dbar			 				    oC		           oC		      PSS-78		μmol/kg

5592			   24			   2.081			   1.550			  34.850		   248.5

5199			    1				   2.208			   1.724			  34.871		   256.4

4848			    2				   2.272			   1.829			  34.884		   260.7

4499			    3				   2.295			   1.895			  34.891		   247.7

4200			    4				   2.324			   1.957			  34.897		   263.8

3901			    5				   2.361			   2.028			  34.901		   267.4

3600			    6				   2.445			   2.143			  34.909		   268.0

3299			    7				   2.580			   2.307			  34.918		   261.7

3000			    8				   2.773			   2.527			  34.936		   255.6

2652			    9				   3.029			   2.813			  34.956		   254.5

2300			   10			   3.279			   3.094			  34.966		   259.2

1950			   11			   3.790			   3.630			  35.002		   255.3

1602			   12			   4.414			   4.280			  35.066		   238.7

1253			   13			   5.590			   5.476			  35.089		   217.6

1052			   14			   7.213			   7.106			  35.111		   169.6

901			   15			   9.102			   8.999			  35.217		   146.3

751			   16			  12.129		  12.028		  35.601		   157.3

602			   17			  15.220		  15.126		  36.066		   177.4

452			   18			  17.524		  17.447		  36.435		   188.9

302			   19			  18.786		  18.732		  36.601		   200.2

183			   20			  20.795		  20.760		  36.789		   203.4

109			   21			  22.613		  22.590		  36.862		   212.7

55			   22			  23.737		  23.726		  36.843		   211.0

3				   23			  23.721		  23.720		  36.844		   210.9