USING LANDSAT FOR COASTAL MANAGEMENT-1: 
MAPPING AND DETERMINING MAJOR SUBSTRATE CATEGORIES.

Aim of Lesson

The lesson seeks to show how various image processing techniques can aid in the interpretation and optimal display of a multispectral satellite image.  It illustrates the initial stages of mapping a remote and rarely visited, inaccessible coastline.  This mapping includes estimating the locations of important features such as coral reefs, the exact shoreline and major terrain types.

Objectives

After completing this lesson you should be able to:-

1) Make a true colour composite image;

2) Identify the precise shoreline and create binary masks to separate land from sea;

3) Apply a binary mask to distinguish shallow submerged features from land;

4) Use filters and stretches to explore and help interpret images.

Contents

This lesson has the following sections:-

Background Information

Exercise 1: Making a colour composite

Exercise 2: Defining the precise boundary between land and sea

Exercise 3: Further use of filters and stretches to enhance images

Answers

After Using Landsat for Coastal Management-2

Information about the source of this Lesson

References

Glossary

Appendix 1: Landsat satellites

Appendix 2: Landsat Thematic Mapper (TM) sensor

Appendix 3: Landsat Multi-Spectral Scanner (MSS)

Background Information

Before you start to work through the lesson, you will find it helpful to read through the topics which follow below, in particular the Lesson Notes and Image Information.

Using Bilko for Windows

This lesson has been written for use in conjunction with the UNESCO image processing software Bilko for Windows version 1.00.

If you have not already done so you should launch the Bilko for Windows application software. Remember that you can easily switch between the Window containing this lesson, and the Window containing Bilko for Windows by pressing <ALT>+<TAB> on the keyboard.

Bilko for Windows has its own Help system which you should consult for information about how to manipulate the image processing operations.  If you have not used Bilko for Windows before, you should work through the Introduction to Bilko for Windows before proceeding further with this lesson.

Lesson notes

The lesson is split into 3 exercises.

Exercise 1: Making a colour composite image  A good way of getting to understand a Thematic Mapper image, which, unlike SPOT HRV or Landsat MSS, has the advantage of wavebands taken in the visible blue, green and red parts of the electromagnetic spectrum, is to display these wavebands on the computer's screen through the blue, green and red guns. If carried out carefully this process results in a view similar to a colour photograph of the area. This can be very helpful in becoming familiarised with an image. Exercise 1 introduces you to the making of colour composite images.

Exercise 2: Defining the precise boundary between land and sea  This exercise is split into three parts. In the first part the difficulties of determining the precise land/sea boundary is explored on a Landsat TM1 (blue waveband) image. In the second part a Landsat TM4 (near infra-red waveband) image is used to define the land-sea boundary and, having done this, to create a binary mask which can be used to mask out the land. In the third part this mask is applied to the TM1 image to allow optimal stretching of the sea pixels.

Rationale: It is helpful to separate the land from the sea during image processing because pixel values in each area tend to be very different so that procedures which work well for land may be suboptimal for sea and vice-versa. This exercise demonstrates how to isolate land and sea pixels on separate image files.  Since infra-red (TM band 4) is totally absorbed by even a few cm of water, but is strongly reflected from most land surfaces it provides the best image for separating land and sea.

Exercise 3: Filters and stretches provide useful means of enhancing images. The final exercise explores their use for various purposes. Both low-pass (smoothing) and high-pass (sharpening) filters are examined.

Coral reefs, including fringing, barrier and patch reefs, are present extensively on the image. Coral reefs grow such that they have steep slopes facing into deep water, and have flat tops which lie more or less at the low tide level.  They may be exposed at extreme low tide, and if they have seaweed (e.g. Sargassum or Turbinaria) which float, or have coral boulders which stick out of the water they may even be exposed at mid-tide. On the landward side of the reefs may be extensive shallow lagoonal areas.

Images used in this lesson

The part-scene provided is of a desert coastline along the Red Sea. It is an image from the Thematic Mapper sensor (TM) of Landsat, whose pixels are 30 x 30 m. It is already geometrically "corrected", that is, it corresponds to a vertical view with Mercator projection.  The region concerned is rarely visited and is inaccessible, but some ground observations have been obtained and photographs taken on land. The scene is taken at mid tide, but the area has a total tidal range of less than 0.5 m. General information on Red Sea habitats and resources is given in Edwards and Head (1987) and Sheppard, Price and Roberts (1992).

The scene would lie conformally over a map or chart of the area of the same scale, were one available.  However, such a map is not available to civilians, to this scale or any other; naval charts of the area are over 50 years old and not accurate, and terrestrial maps are mere sketches which bear little resemblance to the actual terrain.

The satellite image is thus the only accurate "map" of this region, a situation which is not uncommon in tropical countries.

There is a requirement for a map of the area, and for identifying the major habitat types, including offshore reefs and onshore vegetation stands, if any, for coastal management requirements.

Bands 1 to 4 of the Landsat TM image of the study area are provided in image files named RASTM1.GIF, RASTM2.GIF, RASTM3.GIF and RASTM4.GIF, respectively.  The size of the image portions is 512 pixels × 480 scan lines, corresponding to an area of size 15.36 × 14.40 km on the ground (less than 1% of the area of a full Landsat scene).

Exercise 1: Making a colour composite

Colour composites are formed by displaying the different channels or bands of multispectral images on the red, green and blue (RGB) guns of the computer's monitor. Initial inspection of such composite images, which for appropriate sensors can look very similar to colour aerial photographs or infra-red photographs, may provide a much better idea of what is in the image than just looking at individual bands in isolation.

The first task in this exercise is to make a true colour composite which will give you a colour photograph like view of the image from space, making it fairly clear what is land and what is sea. The second task is to explore this colour composite to find out more about the spectral signatures of major features of the image.

Task 1: Making a 'true' colour composite

1. Open and display the blue (RASTM1), green (RASTM2) and red (RASTM3) bands of the Thematic Mapper image.

2. Before we combine these three images to make a 'true' colour composite (displaying the red band on the red gun of the computer monitor, the green band on the green gun, and the blue band on the blue gun), we need to display each of them optimally. To see how well RASTM1 is displayed let us first examine a histogram of the DN values of its pixels.

Make RASTM1 the active window and then display the histogram of the image. Note that no pixels in the image have a value (brightness) less than 100, which is why the image is rather bright and a little lacking in contrast. Close the histogram document.

3. To make sure all the images have good contrast we will now perform an automatic linear stretch on each one in turn. Click on each image in turn, making sure all of each image is selected (<SHIFT>+<SPACEBAR>) before selecting Stretch, Auto Linear from the menu. Note how the contrast improves on the images.

4. The underlying values of the three images have not actually changed but the way they are displayed has changed as a result of the stretches. Before we can combine the stretched images in a composite, we need to create 3 new images which actually have pixels with the new (post contrast stretch) values.

Click on the stretched image of RASTM1 to make it the active window and select File, New and in the New dialog box select IMAGE Document. Click on OK and a new window (e.g. Image1) with the stretched pixel values will be displayed.

5. Close RASTM1 and then create new images based on the stretched RASTM2 and RASTM3 images. Close RASTM2 and RASTM3 images so that you are left with the three new images on the workspace (these will normally be in windows with titles Image1, Image2 and Image3 and these names will be used to refer to these stretched images for the rest of this exercise).

[Hint: If you need to, you can check which original image each of the new images was derived from by pulling down the grey border at the top of the image (below the Title Bar) and reading the revealed text.]

6. These stretched images can now be combined into a composite image. Firstly, select Image, Connect and in the Connect dialog box click on Image1 (blue band, TM1) and holding down the <CONTROL> key click on Image2 (green band, TM2) and Image3 (red band, TM3). When all three of the new images are selected, click on OK. A connected image window (with 3 subwindows) entitled Image1 Image2 Image3 (or similar) appears. Minimise the unconnected originals.

7. The computer screen is an RGB (Red, Green, Blue) display; thus we want Image3 (red band, TM3) displayed on the first gun, Image2 (green band, TM2) on the second gun, and Image1 (blue band, TM1) on the third gun. So when the three button connect toolbar appears at the top left of the UNESCO BILKO window, click on the right hand button first to make Image3 the first image, click on the middle button second, and the left hand button third. The toolbar should look as follows {bmc connect.bmp}.

8. Now select Image, Multi, Composite to make the true colour composite.

Study the image. Note the desert to the left of the image with an escarpment bordering the coastal plain and the coral reefs off the coast on the right hand side. The composite image is very similar to how the area would look from a high flying aircraft.

Although the colour composite image is very useful in allowing us to become acquainted with the area, we need to remember that the original pixel values have been changed by the automatic linear stretch carried out earlier.

9. Minimise the true colour composite image, close the stretched images, and close the connected images window.

Task 2: Exploring a colour composite

The easiest way to to inspect the spectral signatures (reflectances in several wavebands) of various habitats is to make a colour composite which retains the original pixel values. Unfortunately, this will not look as nice as the colour composite you have just created.

1. Connect the RASTM1, RASTM2 and RASTM3 images using the Image, Connect option, and use the connect toolbar buttons to make sure the images are displayed on the blue(3), green (2) and red (1) guns respectively. Then select Image, Multi, Composite to create a colour composite.

Note how poor the composite of raw unstretched images looks.

2. If you click on the image the status bar shows not only the coordinates of the pixel under the cursor but also the pixel brightness values in each of the three images making up the composite in RGB order (in this case TM3, TM2, TM1).

Investigate deep water pixels, shallow water pixels and land pixels. Note the relative brightnesses in the three bands. If you need to, double-click to zoom in on the parts of the image in which you are interested.

Question 1.1. What are the relative brightnesses (DN values) of:

a) deep water pixels (around coordinates 490, 460),

b) shallow water pixels (around coordinates 425, 420),

c) coastal desert plain pixels (around coordinates 300, 430),

in each of 3 wavebands (note absolute values and express blue and green waveband reflectances as approximate ratio (use median DN values) relative to red waveband values).

When you have answered Q. 1.1, close the colour composite image, close the connected images window, and minimise the images.

3. For completeness you need also to examine the values of deep water, shallow water and coastal desert plain pixels in the infra-red waveband image (TM4).

Open and display the RASTM4 image.

Investigate deep water pixels, shallow water pixels and land pixels using the cursor and status bar. If you need to, double-click to zoom in on the parts of the image in which you are interested.

Question 1.2. What are the brightnesses (DN values) of:

a) deep water pixels (around coordinates 490, 460),

b) shallow water pixels (around coordinates 425, 420),

c) coastal desert plain pixels (around coordinates 300, 430),

in the infra-red (note absolute values and express reflectances as approximate ratio (use median DN values) relative to red waveband values noted above).

Exercise 2: Defining the precise boundary between land and sea on an image

The precise boundary between land and sea may not always be clear on an image particularly those taken in the visible wavebands. A good way to define the land-sea boundary is to use images taken in the infra-red where reflectance from water will be low but that from land high creating a reasonably sharp boundary. Landsat Thematic Mapper Bands 5 and 7, where sea pixels will be almost zero in reflectance (in clear water), would be most suitable for Landsat TM images but we can manage reasonably well with the near infra-red Band 4.

Once the land-sea boundary has been defined on one image (usually an infra-red one), i.e. all pixels which are either land or sea can be separated on that image, the image can be used to produce a binary mask for images in visible bands where the boundary is less distinct. Masks allow either the land or sea parts of an image to be processed separately or displayed separately to better effect.

This exercise is divided into three tasks. For the first task, you will examine the TM1 (blue band) image to see for yourself how difficult it is to determine precisely where the land/water boundary is, using this image alone. In the second task, you will use the TM4 (infra-red band) to create a binary mask in which all sea pixels are set to 1 and all land pixels are set to 0. For the third task, you will use this mask image to mask out either land or sea in the visible waveband images (TM bands 1, 2 and 3).

Task 1: Identifying the land/sea boundary on a Landsat TM Band 1 image

1. Display the Thematic Mapper Band 1 (blue) image RASTM1 and explore it using the mouse cursor and Status Bar information to see whether you can detect the precise shoreline using the RASTM1 image by itself.

2. In particular, inspect the shallow embayment in the south central part of the image around coordinates (235, 460) and note the very gradual change in reflectance between sea and land. Similarly examine the coral reefs around coordinates (475, 340) and the sand banks, island and reefs around coordinates (420, 35). Can you tell if and where the reefs emerge?

Use zoom and transecting capabilities of Bilko for Windows if necessary to aid your study.

Note how difficult it is to determine where land begins and sea ends on the TM1 (blue band) image.

Question 2.1.1. Why is the coastline so poorly defined in RASTM1?

Question 2.1.2. What are the highest pixel values in shallow water?

Question 2.1.3. What are the values of pixels along the bottom of the escarpment around coordinates (125,225) and (65, 80)?

3. Display a histogram of the unstretched RASTM1 image. [If you have a stretched image, select Stretch, Options and de-select the default option which is to Apply stretches to charts, clipboards etc. Remember to reselect this option later.].

Note that the histogram has three modes and that the lowest pixel value is 100.

Question 2.1.4. What pixels make up the lowest mode and highest mode of the histogram?

Question 2.1.5. Referring to your answer to Question 2.1.3, why is so difficult to separate land from sea in RASTM1?

4. When you have finished, close the histogram and minimise the RASTM1 image.

The next task is to create a binary mask to allow you to define the coastline sharply.

Task 2: Creating a binary mask using an infra-red image (Landsat TM Band 4).

Having seen how difficult, if not impossible, it is to separate land from sea pixels on the basis of their reflectance in TM Band 1 (blue), we will proceed to create a mask defining the land-sea boundary using TM Band 4 (near-IR).

1. Display the Thematic Mapper Band 4 (near infra-red) image (RASTM4). Note that the image is overall rather dark with the sea being particularly dark. Display the histogram of the image. Examine the histogram.

Question 2.2.1. What are the lowest and highest values taken by any pixels in the image?

Question 2.2.2. Why is the histogram bimodal?

When you have answered the questions, close the histogram window.

2. Returning to the RASTM1 image window, use the cursor and Status Bar information to note the pixel values on land and in deepwater and then study their values across boundaries between land and water in several parts of the image (use the View, Zoom option if necessary to see individual pixels at the boundaries).

Question 2.2.3. What values do deepwater pixels have?

Question 2.2.4. If clear seawater absorbs all the infra-red radiation, why is the reflectance not zero for pixels in the deep sea?

Question 2.2.5. What are the highest values of pixels in water covered areas?

Note that even with this near infra-red image the land-sea boundary is not entirely clear because along the boundary many pixels will lie partly on land and partly on water (so-called 'mixels').

3. We will use the near-IR image RASTM4 to create a binary mask which can be used to mask out either the sea or land when combined with other images such as the blue band RASTM1 image using Formula documents. We will start by creating a binary mask which has all land pixels set to 0 and all sea pixels set to 1. This can be readily manipulated in a Formula document to make a mask with all land pixels set to 1 and all sea pixels set to 0.

Dry land pixels seem to have a minimum DN of approximately 60-65. To be on the safe side we will regard pixels with DN < 65 as sea. Consequently to create a mask from RASTM4 we will:

1) divide the pixel values by (65x2)+1 to set all sea pixels to zero (nearest integer),

2) multiply by -1 to make all land pixels negative, and finally

3) add one to the final result making all sea pixels 1.

The negative land pixels will all be set to the nearest positive integer value which is zero.

4. Firstly, use Image, Connect to connect the two images RASTM1 and RASTM4. On the connect toolbar make RASTM4 image no. 1 (i.e. click the right hand button first) and RASTM1 image no. 2.

5. Use File, New menu option to open a new Formula document. Enter the following formula to create the binary mask:

@1/131*-1+1;

Copy and paste the formula to the connected images.

6. Perform an automatic linear stretch of the resultant image (which will be more or less black all over) and you will see that the land is black and sea is white. Use the cursor and Status Bar information to check the underlying pixel values and verify that the land pixel DNs = 0 and the sea pixel DNs = 1.

7. Save the mask image as LANDMASK.GIF. Close the connected images window and minimise the Formula document.

The next task in this exercise is to apply the binary mask.

Task 3: Applying a binary mask image.

1. To apply the stretched mask to RASTM1 where the boundary between land and sea is rather indistinct, connect the RASTM1 and LANDMASK images, making LANDMASK image No. 1 and RASTM1 image no. 2 on the connnect toolbar.

[Hint: You may wish to carry out an automatic linear stretch of the LANDMASK image in order to see the land-sea boundary].

2. Restore the Formula document and edit the formula until it is as follows:

@1*@2; 			[= Mask image x RASTM1]

Copy and paste this formula to the connected images. The resultant image has the land clearly defined in black (pixel values = 0) but the sea unchanged from that in the original RASTM1 image.

3. Display a histogram of the masked RASTM1 image. You may need to select Options, Scale and click on Ignore zero to display this properly.

Question 2.3.1. What approximately are the lowest and highest pixels values?

4. Use these values to construct a manual stretch of the masked image to display the sea pixels over the full display scale. Apply this stretch to the masked RASTM1 image. Note the great enhancement of the shallow water detail so obtained.

Close the connected images window.

5. To apply the same mask to RASTM1 but to use it to mask out the sea one needs to multiply the binary mask by -1 and then add 1. Thus you need to edit the Formula document as follows:

(@1*-1+1)*@2;

Try this. The resultant image should have all the sea black and the land unchanged.

6. Display a histogram of this image (see 2. above for note on display). The lowest pixel values in the land area are approximately 130.

Experiment with manual stretches to display the land to best advantage.

7. When you have finished, close the histogram, stretches and connected images and minimise other images.

Another way of seeing what effect a mask will have and of creating one is to use a vertically stepped manual stretch.

8. Display the unstretched RASTM4 (near-IR) image. Select all of it and then select Stretch, Manual. By creating a stretch to set all pixel values below a certain value to zero and all above that value to 255, and then applying that stretch to an image, and saving the stretched image as a new image document, one can create a binary mask with all pixels either 0 or 255.

Here we will just investigate the effect of where you place the step.

[Note: If one divides the mask pixels by 255 before applying the mask, it will act just the same as a mask with all pixels set to either 0 or 1.]

Experiment with the effect of setting all pixels less than 50, 65 or 80 to zero and all pixels above these values to 255 with manual stretches.

9. The resultant image will have the sea displayed as black and the land white.

Question 2.3.2. What is the most obvious effect of setting the land-sea boundary at 80?

Question 2.3.3. What is the most obvious effect of setting the land-sea boundary at 50?

Exercise 3: Further use of filters and stretches to enhance images

This exercise is divided into two main tasks. The first task involves looking at the effects of some high-pass (sharpening or edge-enhancing) and low-pass (smoothing) filters. The second task involves the use of a Gaussian stretch and some advanced use of filters in combination with Formula documents and stretches.

Task 1: Applying high-pass and low-pass filters.

An alternative way to identify the land-sea boundary might be to apply an edge enhancing or high-pass filter which will enhance sharp changes in pixel values (Mather, 1987: 252-264) such as might be expected at the land/water interface.

1. Restore or open and display the RASTM4 image (infra-red waveband), select all the image and apply either a Sobel or Roberts high-pass (sharpening) filter from the Image, Filter menu.  If necessary apply an Auto Linear stretch to the resultant image.  Notice the bright lines.  The lines represents the boundaries between pixels with markedly different values and are brighter the more the difference in DN between adjacent pixels.

Question 3.1.1. Is it possible to tell where the land-sea boundary is after applying these high-pass filters?

2. When you have seen the effects of the Sobel/Roberts filters, close the filtered images and minimise RASTM4.

3. Either restore or open and display the RASTM1 image and the LANDMASK mask image. Connect the two images (Image, Connect) and then open a Formula document and multiply the two images together to mask out the land.

4. Study of a histogram of the resultant masked image reveals that 69.5% of the pixels are land (set to zero). To stretch the sea optimally, enter Stretch, Options and set the AutoLinear Min% to 70 (the nearest integer to 69.5%) and Max% to 99. Apply an automatic linear stretch.

5. Open the colour palette RASTMSEA.PAL and apply this palette to the stretched masked RASTM1 image. Examine the resultant image and note the wave speckling on parts of the sea. This speckling can be removed with a low-pass or smoothing filter.

6. Two predefined 3x3 smoothing filters are provided by Bilko for Windows; these are the Median and Mean filters. We will use the Mean filter which moves through the image replacing the central pixel in every 3x3 array by the mean (average) of the nine pixels in that array. This filter is equivalent to applying the following Custom filter where all pixels in the array have equal weighting:

1    1    1

1    1    1

1    1    1

Apply this filter and note the removal of speckling on the sea. Examine also the effect on the southern part of the flask shaped embayment by comparing the smoothed image with the original.

7. Now look at the effects of the following customised filter on the same masked RASTM1 image (Image, Filter, Custom). Enter the following values in the filter grid:

1    3    1

3    9    3

1    3    1

This is a more gentle filter which "weights" the replaced pixel such that its own value is worth 9 times the value of the corner pixels. Apply this filter and compare the results achieved with those of the harsher smoothing filter. Look in particular at the coral reefs and southern part of the flask shaped bay.

Question 3.1.2. What are the differences in the effect of each filter?  Which filter removes the wave "speckle" with less loss of other information?  Why?

Task 2: Advanced use of stretches and filters.

1. We have generally been using automatic linear stretches but the other predefined stretches can reveal things not shown by this stretch.

Display the RASTM4 image. Apply a Gaussian stretch and examine the sea carefully. Note the banding of the sea. Note that this banding is not revealed by an automatic linear stretch but is just visible using a histogram equalization stretch. Use the cursor and transect capabilities to determine approximately how many pixels wide the bands are?

Question 3.2.1. How wide are the bands? And what might cause this banding?

Minimise RASTM4.

2. We will now explore the use of filters to enhance the land part of the image using the red waveband image (TM3) with the sea masked out.

If necessary, open and display the RASTM3 image. Then connect this image with the LANDMASK masking image. Make the LANDMASK image no. 1 and RASTM3 image no. 2 on the connect toolbar. Open a Formula document and use the following formula to mask out the sea.

(@1*-1+1)*@2;

3. Apply the Laplacian high-pass filter to the resultant masked image (Image, Filter, Laplace). In the filtered image which results note how clearly the land-sea boundary is defined by a white line. This is not surprising as all the sea pixels are now set to zero so there is very sudden change in pixel values at the boundary which the high-pass filter will enhance.

4. For best results it is suggested that the output of a Laplace filter be subtracted from the original image.

Connect your Laplace filtered image to the masked RASTM3 image and then use a Formula document to subtract the Laplace filtered image from it.

Note the considerable amount of detail revealed on the resultant image particularly around the shoreline and along the escarpment to the left of the image.

Close the connected images window and the Laplace filtered image. Retain the final image.

5. For comparison with this image, apply a simple N-S edge enhancement filter to the masked RASTM3 image.

1    0   -1

2    0   -2

1    0   -1

Question 3.2.2. What is the primary effect generated by this filter?

This effect may be further emphasised by applying a histogram equalization stretch to the image. Compare the resultant image to the previous one generated using the Laplacian filter.

6. Close the various files before exiting Bilko for Windows.

Answers to Lesson

Answer to question 1.1.

|  | Red (R) |  | Green(G) |  | Blue (B) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Habitat | Values | Ratio:Red | Values | Ratio:Red | Values | Ratio:Red |
| Deep water | 27-32 | 1 | 33-36 | 1.2 | 100-107 | 3.5 |
| Shallow water | 81-105 | 1 | 76-102 | 0.96 | 150-189 | 1.8 |
| Coastal plain | 178-190 | 1 | 119-129 | 0.67 | 208-224 | 1.2 |

i) Note the much higher reflectances in the blue (TM1) than in the red (TM3) of water pixels and similar reflectances in shallow water (< 3 m deep) in the green (TM2) and in the red. This is because water absorbs red light much more strongly than green light, which in turn is absorbed more than blue light. Because the reflectances in shallow water area are so similar in the red and green wavebands, we know that the sea there must be very shallow.

Answer to question 1.2.

|  | Infra-red |  |
| --- | --- | --- |
| Habitat | Values | Ratio:Red |
| Deep water | 8-11 | 0.52 |
| Shallow water | 25-45 | 0.45 |
| Coastal plain | 200-210 | 0.76 |

i) Note particularly the relatively low reflectance of deep and shallow water. This is because infra-red radiation is very strongly absorbed by water, penetrating only a few cm.

Answer to question 2.1.1. The original image RASTM1 does not show the coastline at all clearly because the blue-green light (450-520 nm) which is measured in this waveband reflects nearly as strongly from the sea bed through shallow water as it does from the surrounding dry land.

Answer to question 2.1.2. The highest values of pixels in shallow water are approximately 145-170.

Answer to question 2.1.3. Pixels along the bottom of the escarpment (in shadow) have values around 130-150. Thus some land pixels have lower reflectances in TM band 1 than some shallow water pixels.

Answer to question 2.1.4. Pixels in the sea (deeper water) make up the lowest mode and pixels on the land make up the highest mode.

Answer to question 2.1.5. Because although most sea pixels have lower reflectances than land pixels, some land pixels have lower reflectances than some shallow water areas (see answers to Questions 2.1.2 and 2.1.3).

Answer to question 2.2.1. The lowest and highest values taken by any pixels in the image are 10 and 171; 99.9% of pixel values lie between 14 and 162.

Answer to question 2.2.2. The histogram for Band 4 is bimodal because part of the image is sea, part is land, and these two substrates have major differences in reflectance characteristics in the near infra-red.

Answer to question 2.2.3. The deepwater pixels have values of 14-18. Note also that shallower water pixels have values around 25-35, and very shallow water pixels have values around 40-55.

Answer to question 2.2.4. The reflectance is non-zero over the deep sea because there is dust in the air which reflects infra-red in the pixels over the sea, rather than the sea itself.  Nearer the coast other substances in the seawater, such as sediment, will tend to produce significant reflectance in the near-IR. The lower mode of the bimodal distribution is comprised of pixels over the sea.

Answer to question 2.2.5. Pixels with values 50-55 seem to lie on the water side of some land/sea boundaries; pixels with values 60-65 seem to be on the land side of these boundaries. A reasonable boundary value is somewhere in the DN range of 55-65. So as not to risk masking out shallow reefs we will select 65 as the boundary value.

Answer to question 2.3.1. The lowest sea pixel values are about 100 and the highest approximately 200. Thus an optimal stretch will map 100 to zero on the display scale and map 200 to 255 on the display scale.

Answer to question 2.3.2. Too much of the land is masked out with the shadowed areas below the escarpment being classed as sea.

Answer to question 2.3.3. The offshore coral reefs to the bottom right of the image are classed as land.

Answer to question 3.1.1. It is not really possible to tell exactly where the land-sea boundary is. Although the land-sea boundary stands out as a very bright line (particularly with the unstretched Sobel filtered image), many other linear features are also enhanced making it difficult to tell exactly where the land-sea boundary is. Overall the land part of the image is fairly cluttered with bright lines indicating one sort of boundary or another.

Answer to question 3.1.2. The weighted filter is more "gentle", removing speckle with little loss of other information.  In many instances small scale details remain after applying the weighted 3×3 filter which are removed by the application of the stronger smoothing filter.

Answer to question 3.2.1. The bands average 16 pixels wide. The banding is caused by the scanning system of the Landsat Thematic Mapper. For each waveband the sensors have to scan 16 lines on each scan because of the speed of the satellite over the ground (thus scanning 480 m along track on each scan). There is often a slight difference (in this case of only 1 or 2 DN values) between the brightnesses recorded when scanning in one direction than in the other and this has been picked out by the Gaussian stretch. The banding is clearest on the infra-red image (TM4), also visible on the red image (TM3) and just visible on the green waveband image (TM2).

Answer to question 3.2.2. This filter gives the land a three-dimensional aspect so that you feel you can see the relief of the terrain.

USING LANDSAT FOR COASTAL MANAGEMENT-2 
VEGETATION MAPPING.

Aim of Lesson.

The lesson develops simple classification procedures to illustrate how vegetation can be identified on a Landsat TM image, and how the major areas of vegetation in the image can be quantified.

Objectives

After completing this lesson, you should be able to:

1) identify the principal characteristics shown by coastal vegetation in satellite images

2) show vegetation on an image in a highlighted form by assigning the vegetation to a particular colour

3) measure the area of ground covered by stands of vegetation.

Contents

This lesson has the following sections:-

Background Information

Exercise 1:  Creating a false colour composite image

Exercise 2:  Identifying and mapping coastal vegetation

Exercise 3:  Estimating the amount of vegetation cover

Answers

Information about the source of this Lesson

References

Glossary

Appendix 1: Landsat satellites

Appendix 2: Landsat Thematic Mapper (TM) sensor

Appendix 3: Landsat Multi-Spectral Scanner (MSS)

Background Information

Before you start to work through the lesson, you will find it helpful to read through the topics which follow below, in particular the Lesson Notes and Image Information.

Using Bilko for Windows

This lesson has been written for use in conjunction with the UNESCO image processing software Bilko for Windows version 1.00.

If you have not already done so you should launch the Bilko for Windows application software. Remember that you can easily switch between the Window containing this lesson, and the Window containing Bilko for Windows by pressing <ALT>+<TAB> on the keyboard.

Bilko for Windows has its own Help system which you should consult for information about how to manipulate the image processing operations.  If you have not used Bilko for Windows before, you should work through the Introduction to Bilko for Windows before proceeding further with this lesson.

Lesson Notes

This lesson is a continuation of the previous lesson on Mapping and determining major substrate categories.

The lesson is split into 3 exercises.

Exercise 1. Creating a false colour composite image. A good way of seeing where there are significant areas of vegetation on an image is to create a false colour composite image from the green, red and near infra-red waveband images. On such an image vegetated areas will tend to stand out as red patches if the near-IR image is displayed on the red gun, the red image on the green gun and the green image on the blue gun. The first exercise involves creating and exploring such a colour composite.

Exercise 2. Identifying and mapping coastal vegetation. In this exercise you create a Normalised Difference Vegetation Index (NDVI) image based on the red and near-IR images to enhance well-vegetated areas and create a palette to display these areas clearly.

Exercise 3. Estimating the amount of vegetation cover. Finally you will use your NDVI image to calculate the area of the image which is mangrove habitat or has significant vegetation cover.

Vegetation contains the green, photosynthetic chemical compound chlorophyll.  This substance absorbs red wavelengths but reflects infra-red.  Use is made of this in identifying which pixels have high vegetation cover.

Strong infra-red reflectance is also a characteristic of hot sand and rock, even if covered by no chlorophyll at all.  However, sand and rocks tend not to absorb red as much as chlorophyll does.

Vegetation distribution  Ground-truthing site visits over part of the area revealed two small mangrove stands, in addition to extensive mud flats which commonly support rich photosynthetic "blue-green" algae (Cyanobacteria) as well (Friedman and Krumbein, 1985).  The edge of the reef crest in this southern Red Sea site supports thick stands of the brown macroalga (seaweed) called Sargassum.

It is not practical to provide the habitat photographs taken by the ground-truthing team with this lesson, but if this were an operational research project, you would benefit from having access to photographs, preferably taken close to the time of the satellite overpass.

The Images in this Lesson

This lesson uses the same image data as the previous lesson on Mapping and determining major substrate categories.

The part-scene provided is of a desert coastline along the Red Sea. It is an image from the Thematic Mapper sensor (TM) of Landsat, whose pixels are 30 x 30 m. It is already geometrically "corrected", that is, it corresponds to a vertical view with Mercator projection.  The region concerned is rarely visited and is inaccessible, but some ground observations have been obtained and photographs taken on land. The scene is taken at mid tide, but the area has a total tidal range of less than 0.5 m.

The scene would lie conformally over a map or chart of the area of the same scale, were one available.  However, such a map is not available to civilians, to this scale or any other; naval charts of the area are over 50 years old and not accurate, and terrestrial maps are mere sketches which bear little resemblance to the actual terrain.

The satellite image is thus the only accurate "map" of this region, a situation which is not uncommon in tropical countries.

There is a requirement for a map of the area, and for identifying the major habitat types, including offshore reefs and onshore vegetation stands, if any, for coastal management requirements.

Bands 1 to 4 of the Landsat TM image of the study area are provided in image files named RASTM1.GIF, RASTM2.GIF, RASTM3.GIF and RASTM4.GIF, respectively.  The size of the image portions is 512 pixels × 480 scan lines, corresponding to an area of size 15.36 × 14.40 km on the ground (less than 1% of the area of a full Landsat scene).

Exercise 1: Task 1: Making a false colour composite

The first task in this exercise is to make a false colour composite designed to show where there are patches of vegetation, which as in infra-red photographs will be displayed in red.

We will create a False Colour Composite using bands 2, 3 and 4 of the TM scene. In this case we will display TM Band 2 (green) on the blue gun, Band 3 (red) on the green gun, and Band 4 (near infra-red) on the red gun. Vegetation stands should be visible as dark red patches because vegetation reflects strongly in the near-infra red (displayed as red) but is poorly reflective in the red (displayed as green).

1. Open and display the green (RASTM2), red (RASTM3) and near infra-red (RASTM4) bands of the Thematic Mapper image.

Before we combine these three images to make a false colour composite, we need to display each of them optimally.

2. To make sure all the images have good contrast we will now perform an automatic linear stretch on each one in turn. Click on each image in turn, making sure all of each image is selected (<SHIFT>+<SPACEBAR>) before selecting Stretch, Auto Linear from the menu. Note how the contrast improves on the images.

3. The underlying values of the three images have not actually changed but the way they are displayed has changed as a result of the stretches (i.e. only the Look-Up Table - LUT - has changed). Before we can combine the stretched images in a composite, we need to create 3 new images which actually have pixels with the new (post contrast stretch) values.

Click on the stretched image of RASTM2 to make it the active window and select File, New and in the New dialog box select IMAGE Document. Click on OK and a new window (e.g. Imagen where n is an integer) with the stretched pixel values will be displayed.

4. Minimise RASTM2 and then create new images based on the stretched RASTM3 and RASTM4 images. Minimise RASTM3 and RASTM4 images so that you are left with the three new images on the workspace.

[Hint: If you forget which stretched image derived from which waveband, you can check by pulling down the grey border at the top of the image (below the Title Bar) and reading the revealed text.]

5. With the mouse select Image, Connect and in the Connect dialog box select the stretched green (TM2), red (TM3) and near infra-red (TM4) images. Click on the right button of the three button connect toolbar to make sure the near-IR (TM4) image is displayed on the red gun, the red (TM3) image is displayed on the green gun, and the green (TM2) image is displayed on the blue gun.

6. Make sure the connected images window is the active one and select Image, Multi, Composite to create the false colour composite.

7. Areas with dense vegetation appear red and areas with some photosynthetic pigment (e.g. blue-green algae (Cyanobacteria), other algae (red or brown), or salt marsh plants) appear with a magenta tinge. Some patches of dense vegetation are noticeable on the edge of the large flask-shaped bay (around coordinates 175, 225) which are presumed to be mangroves.

This was later confirmed by a ground-truthing visit. The extensive light magenta areas on the coast are largely sabkha (or sebkha) – periodically inundated flat areas near the high intertidal with salt crusts and blue-green algal (Cyanobacteria) mats (Friedman and Krumbein, 1985).

8. When you have finished studying this image and have made a mental note of the major vegetation areas (red), we will study the spectral signatures more closely.

Although the colour composite image is very useful in allowing us to pinpoint potential areas of vegetation, we need to remember that the original pixel values have been changed by the automatic linear stretch carried out earlier.

Save the false colour composite image as RASTMFCC.GIF, close the stretched images, and close the connected images window.

Task 2: Exploring a false colour composite

The easiest way to to inspect the spectral signatures (reflectances in several wavebands) of various habitats is to make a colour composite which retains the original pixel values. Unfortunately, this will not look as nice as the false colour composite you have just created.

1. Connect the RASTM2, RASTM3 and RASTM4 images using the Image, Connect option, and use the connect toolbar buttons to make sure the images are displayed on the blue(3), green (2) and red (1) guns respectively.

2. Select Image, Multi, Composite to create a colour composite.

Note how dark the composite of raw unstretched images looks. The image can be improved by turning up the screen brightness. [If you do this, remember to turn it down again when you are finished!]

3. If you click on the image the status bar shows not only the coordinates of the pixel under the cursor but also the pixel brightness values in each of the three images making up the composite in RGB order (in this case TM4, TM3, TM2).

Investigate shallow water pixels, land pixels and vegetation. Note the relative brightnesses in the three bands. If you need to, double-click to zoom in on the parts of the image in which you are interested.

Question 1.1. What are the relative brightnesses (DN values) of:

a) shallow water pixels (around coordinates 425, 420),

b) coastal desert plain pixels (around coordinates 300, 430),

c) presumed vegetation pixels (around coordinates 175, 225),

in each of 3 wavebands (note absolute values and express green and red waveband reflectances as approximate ratio (use median DN values) relative to infra-red waveband values).

[Hint: You can refer to your answers to Questions 1.1 and 1.2 of the previous lesson for some of the absolute values for green (TM2), red (TM3), and near infra-red (TM4) wavebands].

4. Minimise the false colour composite image (which you will need in the next exercise), minimise the original images, and close the connected images window.

Exercise 2: Mapping vegetation

Having used a False Colour Composite image to gain an idea of where vegetation may be present, you will now use the red (TM3) and infra-red (TM4) waveband images to see whether we can map this vegetation.

1. From our ground-truthing visits, we know roughly where the mangroves, algal flats and Sargassum are located.  It has therefore been possible to identify four positions, in terms of x and y co-ordinates on the image, where the ground cover is known.  These points are representative of their surrounding area, and are defined as follows:

|  | x coordinate | y coordinate | Notes |
| --- | --- | --- | --- |
| Mangroves 1 | 175 | 228 | Centre of mangrove stand 1 |
| Mangroves 2 | 323 | 123 | Centre of mangrove stand 2 |
| Algal mats | 433 | 136 | large area |
| Sargassum | 491 | 229 | in long strip on reef edge |

Restore the false colour composite image made with the unstretched wavebands TM2, TM3 and TM4.

2. Study each of the areas listed above (Step 1) using the View, Zoom option so you can see individual pixels. Confirm to yourself the following points.

i.	In the central parts of the mangrove areas note that the infra-red waveband (TM4) brightnesses in each pixel tend to be greater than the red waveband (TM3) brightnesses. Outside the vegetated areas the reverse tends to be true.

ii.	In the algal mat area note that the infra-red waveband (TM4) brightnesses tend to be more or less equal but slightly less than the red waveband (TM3) brightnesses.

iii.	In the strip of Sargassum along the shore note that the infra-red waveband (TM4) brightnesses in each pixel (DNs ranging from 106-136) tend to be less than the red waveband (TM3) brightnesses (DNs ranging from 101-133). This is reversed in pixels to both seaward and landward.

Question 2.1. What characteristic distinguishes pixels with mangrove or Sargassum from neighbouring pixels?

3. Rather than making a simple ratio of the RASTM4 and RASTM3 images to enhance the vegetated areas one can calculate the more sophisticated Normalised Difference Vegetation Index (NDVI) which is equivalent to:

µ §

Connect the RASTM3 and RASTM4 images and calculate the NDVI using Image, Multi, NDVI.

4. To better see the result carry out a histogram equalization stretch of the resultant image.

The brightest areas should represent vegetation as it is generally only in vegetated areas that pixels are more reflective in the near infra-red than in the red in this image, whilst in unvegetated areas pixels tend to have higher reflectances in the red than infra-red and thus will appear dark in an IR/red ratio.

5. Display the histogram of the stretched NDVI image. Note the cluster of pixels with DN values of 250-255. To see clearly where these pixels are on the image you need to edit the palette and assign bright colours to the brightest DN values.

6. Open the palette for the image using File, New and selecting a PALETTE document. Set pixels with DN value 255 to red (Red=255, Green=0, Blue=0), those with DN values 250-254 to magenta (Red=255, Green=0, Blue=255), and those with DN values 240-249 to yellow (Red=255, Green=255, Blue=0). Copy the palette and paste it to the stretched NDVI image.

7. The central mangrove areas and other dense vegetation (including the Sargassum fringe) should now be coloured red, areas of sparser vegetation and blue-green algal mats will largely be magenta, and areas which may contain very sparse vegetation will be coloured yellow.

8. Open and display the false colour composite image RASTMFCC which you saved earlier and connect this to your coloured NDVI image. Now compare the two to satisfy yourself that the vegetated areas have been classified correctly.

Exercise 3: Calculating the area of vegetation

If you make the assumption that the red areas (DN = 255) on the NDVI image provide an estimate of the total area of mangrove, you can now use the histogram of the image to calculate the percentage of the image which is mangrove habitat. This is not an inappropriate assumption as there will be areas of sparse mangrove which classify among the magenta pixels, which should balance out areas of dense Sargassum and blue-green algal mat areas which may be classified among the red pixels. Similarly, you can use the number of red and magenta pixels (DN = 250-255) to estimate the area of the image which has significant vegetation.

1. Open a histogram of the NDVI image. Remembering that each pixel is 30x30m and thus has an area of 900 m², use the histogram and status bar to answer the following questions.

Question 3.1. Given the assumption above (click on « if necessary to refresh your memory), what is the area of mangroves on the image?

Question 3.2. What area of the image has significant vegetation cover?

2. Note the indications of emergent (or barely submerged) vegetation (algae, coral zooxanthellae) along the top of the barrier reef. In particular inspect the NDVI image around coordinates (403, 475) and (422, 446). Note that although one can deduce the presence of photosynthetic organisms, it is very difficult to distinguish different types of vegetation from each other.

Close the connected images and other images when you have finished.

3. To find out how much of the total land area is vegetated you need firstly to calculate the area of land. The easiest way to do this is to look at a histogram of the mask image made in the previous lesson.

Open and display this image (saved as LANDMASK.GIF). Then display the histogram. All land pixels are set to zero in the mask image so position the cursor on DN zero to find out how many land pixels there are.

Question 3.3. How many pixels are land and what area does this represent in hectares (ha)? What percentage of the land has significant vegetation cover?

Answers to Lesson

Answer to question 1.1.

|  | Infra-red (R) |  | Red (G) |  | Green (B) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Habitat | Values | Ratio:IR | Values | Ratio:IR | Values | Ratio:IR |
| Shallow water | 24-59 | 1 | 81-105 | 2.2 | 76-102 | 2.2 |
| Coastal plain | 136-145 | 1 | 178-190 | 1.3 | 119-129 | 0.88 |
| Vegetation | 78-90 | 1 | 70-91 | 0.96 | 61-74 | 0.80 |

i) Note particularly the relatively low reflectance of shallow water and relatively high reflectance of vegetation in the infra-red (TM4).

ii) Note also how the bright desert sand reflects all wavebands fairly well.

iii) Note that in the middle of the presumed mangrove, pixel DN values are ranked as follows: Green < Red < IR, whilst around the edge of the vegetation Green < Red = IR. In the surrounding area Red > IR > Green.

Answer to question 2.1. Unlike neigbouring pixels which tend to have greater reflectance in the red than near-IR, pixels dominated by mangrove or Sargassum tend to have greater reflectance in the infra-red than in the red. This is less clear cut in blue-green algal mat pixels in the sebkha areas where red reflectances are almost equal but slightly less than near-IR reflectances. A possible way of enhancing vegetation areas is to take advantage of this anomalous ratio.

Answer to question 3.1. Inspecting the histogram of the stretched NDVI image you will find that there are 919 pixels which have a value of 255 and which are thus assumed to be mangrove. 919 x 900 =  827,100 m² or approximately 83 ha.

(1 ha = 10,000 m²).

Answer to question 3.2. Inspecting the histogram of the stretched NDVI image you will find that there are 919+745+835+969+668+1097 = 5233 pixels with values from 250-255 and which are thus assumed to have significant vegetative cover. 5233 x 900 =  4,709,700 m² or approximately 471 ha.

Answer to question 3.3. There are 170,851 pixels which are land. This is equivalent to 15,377 ha. The area of land with significant vegetation cover represents 3.1% of the total land area. This seems reasonable for a coastal desert area.

Information about the lesson authors

The basic layout of much of this lesson was originally developed for the text based Third Computer-based Learning Module for Bilko (Applications of Marine and Coastal Image Data) by Charles Sheppard. It was modified and rewritten as an on-line lesson for Bilko for Windows by Alasdair Edwards.

Comments or suggestions relating to this lesson should be sent to:

Dr A.J. Edwards

Centre for Tropical Coastal Management Studies

Department of Marine Sciences and Coastal Management

University of Newcastle

| Newcastle upon Tyne | Fax: | +44 (0)191 222 7891 |
| --- | --- | --- |
| NE1 7RU, UNITED KINGDOM | E-mail: | a.j.edwards@ncl.ac.uk |

Dr C.R.C. Sheppard

Ecosystems Analysis and Management Group

Department of Biological Sciences

University of Warwick

| Coventry | Fax: | +44 (0)120 352 4619 |
| --- | --- | --- |
| CV4 7AL, UNITED KINGDOM | E-mail: | SH@dna.bio.warwick.ac.uk |

References

Edwards, A.J. and Head, S.M. (eds.) (1987). Red Sea. Pergamon Press. 441 pp.

Friedman, G.M. and Krumbein, W.E. (eds.) (1985). Hypersaline Ecoystems. The Gavish Sabkha. Springer-Verlag. 484 pp.

Mather, P.M. (1987). Computer Processing of Remotely-Sensed Images. Wiley. 352 pp.

Sheppard, C., Price, A. and Roberts, C. (1992). Marine Ecology of the Arabian Region. Patterns and Processes in Extreme Tropical Environments. Academic Press. 359 pp.

Glossary

Binary Mask: A binary mask is one where all pixels in an area you are not interested in are set to 0, and all pixels in the area you are interested in are set to 1. Thus if the pixels in another image are multiplied by the binary mask then all those in the area in which you are interested remain unchanged in value whilst those you are not interested in are set to zero.

ETM+: Enhanced Thematic Mapper+; sensor with 7 wavebands (3 visible, 4 infra-red) which are more or less identical to those of the Thematic MapperTM_Topic but with improved 60 m resolution for thermal IR band 6, plus a panchromatic band (520-900 nm) with 15 m spatial resolution. Due to be carried on Landsat 7.

MSS: Multi-Spectral Scanner, 4 waveband (2 visible, 2 infra-red) sensor carried on Landsats 1-5 with a spatial resolution of 79 m.

Normalised Difference Vegetation Index: this is a ratio which enhances vegetation. It involves ratioing a near infra-red waveband image (where vegetation reflects well) with a visible red waveband image (where vegetation absorbs). The ratio is calculated as follows:

µ §

where Image1 is the near-IR and Image2 is the red waveband image. The division by the sum of the reflectances normalises the difference and produces a ratio which can range from -1 to +1. Adding 1 to this makes a ratio which ranges from 0 to +2. Division by 2 reduces the range to 0 to +1 and multiplication by 255 stretches this ratio across the full 0-255 display range.

Spatial Resolution: this is the smallest area on the ground which can be resolved by a sensor and is equivalent to the Instantaneous Field Of View (IFOV) of a sensor. The term "pixel size" is often used loosely as alternative to spatial resolution although pixel size is not always the same as IFOV.

SPOT HRV: SPOT is the French Système Probatoire d'Observation de la Terre (also: Satellite Pour l'Observation de la Terre). HRV is its Haute Resolution Visible (High Resolution Visible) sensor which in multispectral mode operates in 3 wavebands (green, red and near-IR) at 20 m spatial resolution. It can also operate in panchromatic mode at a spatial resolution of 10 m.

Swath width: The average width of ground scanned by a sensor on a single pass overhead. This is 185 km for Landsat MSS and TM and 60 km for SPOT HRV.

TM: Thematic Mapper, 7 waveband (3 visible, 4 infra-red) sensor carried on Landsats 4-5 with a spatial resolution of 30 m (apart from Band 6 [thermal IR] which has 120 m resolution).

Appendix 1: Landsat satellites

The US Landsat satellite series represents the longest running of the civilian Earth resources satellites carrying sensors with better than 100 m ground resolution. The first satellite ERTS-1 (Earth Resources Technology Satellite) subsequently renamed Landsat-1 was launched by NASA (National Aeronautics and Space Administration) in 1972. The first generation Landsat satellites (Landsat-1 to Landsat-3) had a higher orbit (907-915 km) and thus longer time between repeat coverage of a given area (18 days) than the second generation satellites (Landsat-4 and Landsat-5) which are in 705 km high orbits giving repeat coverage every 16 days.

All of the Landsat series have carried the Multi-Spectral Scanner (MSS) sensor and since the launch of the second generation satellites with Landsat-4 they have also carried the Thematic Mapper (TM) sensor.

| Satellite | Launch | Altitude | End of operation | Repeat coverage |  | Key sensors |
| --- | --- | --- | --- | --- | --- | --- |
| Landsat-1 | July 1972 | 907 km | 1978 | 18 days |  | MSS |
| Landsat-2 | January 1975 | 908 km | 1983 | 18 days |  | MSS |
| Landsat-3 | March 1978 | 915 km | 1983 | 18 days |  | MSS |
| Landsat-4 | July 1982 | 705 km | in operation | 16 days |  | MSS, TM |
| Landsat-5 | March 1985 | 705 km | in operation | 16 days |  | MSS, TM |
| Landsat-6 | October 1993 | *** Failed to achieve orbit *** |  |  | MSS, ETM |  |
| Landsat-7 | Due December 1998 | 705 km |  | 16 days |  | MSS, ETM+ |

Note: Landsat 4 Thematic Mapper sensor failed in August 1993.

For additional details of the Landsat programme and sensors browse the following World Wide Web address:

http://geo.arc.nasa.gov/esdstaff/landsat

Appendix 2: Landsat Thematic Mapper (TM) sensor

The Landsat Thematic Mapper (TM) has been operational since 1984 following the launch of Landsat-4. The spatial resolution for the sensor is 30 m except for Band 6 which measures emitted thermal infra-red radiation and has a resolution of 120 m. The swath width for the sensor is 185 km. At present (1995) the Thematic Mapper is only operational on Landsat-5 having failed on Landsat-4 in August 1993.

The spectral characteristics of the Landsat TM bands are as follows:-

| TM Band | Colour | Wavelengths (nm) |
| --- | --- | --- |
| 1 | Blue | 450 - 520 |
| 2 | Green | 520 - 600 |
| 3 | Red | 630 - 690 |
| 4 | Near Infra-red | 760 - 900 |
| 5 | Short Wave Infra-red | 1550 - 1750 |
| 6 | Thermal Infra-red | 10,400 - 12,500 |
| 7 | Short Wave Infra-red | 2080 - 2350 |

The image size is 185 km across by 172 km along-track; equivalent to 5760 lines by 6928 pixels. Each scene is 246 Mbytes.

Appendix 3: Landsat Multi-Spectral Scanner (MSS)

The Landsat Thematic Mapper (MSS) has been operational since 1972 following the launch of Landsat-1. The spatial resolution for the sensor is 79 m. The swath width for the sensor is 185 km. At present (1995) the Multi-Spectral Scanner is operational on both Landsat-4 and Landsat-5.

The spectral characteristics of the Landsat MSS bands are as follows:-

| MSS Band |  | Colour | Wavelengths (nm) |
| --- | --- | --- | --- |
| Landsat 1-3 | Landsat 4-5 |  |  |
| 4 | 1 | Green | 500-600 |
| 5 | 2 | Red | 600-700 |
| 6 | 3 | Near Infra-red | 700-800 |
| 7 | 4 | Near Infra-red | 800-1100 |