1

N42RF ERROR SUMMARY

DISTURBANCE OVER LEEWARD ISLANDS

Flight ID: 080814H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSFX

Dynamic Pressure						PQF1

Vert. Wind						           RA159X

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There was one data gap:  151341Z – 151349Z.  No lag was observed between the two inertial outputs.

There were no liquid water sensors available for the flight.

Due to a data system failure onboard the aircraft at take-off, all horizontal and vertical wind output is erroneous until 154200Z.

The fuselage measured static pressure (PSF)  and fueslage measured dynamic pressure (PQF1) outputs had erroneous data from 164100Z – 164400Z. The erroneous data for both sensors was removed using statistical techniques. From 194400Z – 194600Z

PSF displayed erroneous data and it was removed using statistical techniques.

Measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited numerous time periods of erroneous data throughout the flight. During the following time frames: 173624Z – 173703Z, 173749Z – 173910Z and 210820Z – 210928Z the erroneous data was removed by substituting TDM3(Maycomm/TDL) output,

TDM2 = TDM3

For 201542Z – 201617Z erroneous TDM2 data was removed and replaced using the following equation                   TDM2 = TDM3 + 1.5

For the following time frames erroneous TDM2 data was removed and replaced via using statistical techniques: 182600Z – 182615Z, 182624Z – 182635Z, 183247Z – 183255Z, 194104Z – 194130Z, 195118Z – 195126Z, 200536Z – 200554Z, 200634Z – 200701Z,

201155Z – 201203Z, 201211Z – 201219Z and 201336Z – 201352Z.

All other instruments worked optimally during the flight.

Seventeen (17) GPS dropsondes were deployed with 16 being good.

No tempdrop messages were sent to NHC.

Takeoff		Landing

1502Z		 2242Z

Aircraft Static Pressure                   1007.7mb	1006.3mb

Corrected Tower Pressure               1007.5mb	           1006.4mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073