1

N42RF ERROR SUMMARY

DISTURBANCE OVER PUERTO RICO

Flight ID: 080815H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           RA159X

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There were no data gaps.  No lag was observed between the two inertial outputs.

There were no liquid water sensors available for the flight.

Measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited several time periods of erroneous data throughout the flight. During the following time frames:

164843Z – 164906Z, 180224Z – 180259Z and 181038Z – 181057Z the erroneous data was removed by substituting TDM3(Maycomm/TDL) output,

TDM2 = TDM3

For the following time frames erroneous TDM2 data was removed and replaced using statistical techniques: 164945Z – 164959Z, 165003Z – 165020Z, 173249Z – 173306Z, 182031Z – 182036Z and 182131Z – 182134Z.

All other instruments worked optimally during the flight.

Fourteen (14) GPS dropsondes were deployed with 12 being good. Twelve tempdrop messages were sent to NHC.

Takeoff		Landing

1456Z		2259Z

Aircraft Static Pressure                   1007.8mb	1006.3mb

Corrected Tower Pressure               1008.5mb	           1007.4mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073