1

N42RF ERROR SUMMARY

FERRY TO MACDILL FROM BARBADOS

Flight ID: 080817H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           RA159X

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There were no data gaps.  No lag was observed between the two inertial outputs.

Approaching the Western Bahamas the aircraft flew along and through thunderstorms. Also there was windshift at flight level from SE to SW along 81.3W indicating a mid-level ridge axis.

There were no liquid water sensors available for the flight.

All other instruments worked optimally during the flight.

No GPS dropsondes were deployed.

Takeoff		Landing

1508Z		2028Z

Aircraft Static Pressure                   1006.3mb	           1011.6mb

Corrected Tower Pressure               1006.8mb	           1012.4mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073