1

N42RF ERROR SUMMARY

TS FAY OVER FLORIDA STRAITS

FERRY TO NEW ORLEANS

Flight ID: 080818H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           RA159X

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There were no data gaps.  No lag was observed between the two inertial outputs.

There were no liquid water sensors available for the flight.

The APN-159 (RA159X) radar altimeter output displayed numerous time periods of erroneous values. For the time periods 201507Z – 201638Z, 202055Z – 202256Z and 202524Z – 202531Z, the RA159 output was removed and directly replaced with output from inertial #1's (INE1) blended GPS altitude (ALTI1), RA159 = ALTI1.

The remainder of the RA159 data glitches were fixed using statistical techniques during the following time periods: 175200Z – 175400Z, 183222Z – 183227Z,

183335Z – 183336Z, 190200Z – 190400Z, 193450Z – 193650Z, 195600Z – 195800Z, 200336Z – 200502Z, 201000Z – 201200Z, 201226Z – 201232Z, 202320Z – 202520Z,

202742Z - 203457Z and 203800Z – 204000Z. continued until the end of the mission.

Measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited several time periods of erroneous data during the flight. During the following time frames:

154907Z – 1155017Z, 160524Z – 160619Z, 163753Z – 163831Z, 163945Z – 163950Z, 164050Z – 164114Z, 180314Z - 181410Z and 193817Z – 193653Z the erroneous data was removed by substituting TDM3(Maycomm/TDL) output,

TDM2 = TDM3

All other instruments worked optimally during the flight.

Fifteen (15) GPS dropsondes were deployed with 12 being good. Twelve (12) tempdrop messages were sent to NHC.

Takeoff		Landing

1501Z		2140Z

Aircraft Static Pressure                   1012.7mb	1009.9mb

Corrected Tower Pressure               1012.9mb	           1010.2mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073