1

N42RF ERROR SUMMARY

H. GUSTAV IN WESTERN CARIBBEAN

Flight ID: 080829H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           RA159X

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There were no data gaps.  However beginning with this flight and continuing through the H080912 H. Ike flight, there was observed a 3 to 4 second lag between all INE1 and INE2 outputs. INE1was selected since it best corresponded with the analog output.

There were no liquid water sensors available for the flight.

The APN-159 (RA159X) radar altimeter output displayed numerous time periods of erroneous values. The RA159 data glitches were fixed using statistical techniques during the following time periods: 211600Z – 211800Z, 215200Z – 215400Z,

220100Z – 220300Z, 220400Z – 220600Z, 221000Z – 221200Z, 222500Z – 222700Z, 224300Z – 224500Z, 225200Z – 225400Z, 230100Z – 230300Z, 230400Z – 230600Z,

233500Z – 233700Z, 233800Z – 234000Z, 234400Z – 234600Z, 003800Z – 004000Z, and 014500Z - 014700Z.

For the time period 022441Z – 022531Z the RA159 output was removed and directly replaced with output from inertial #1's (INE1) blended GPS altitude (ALTI1),

RA159 = ALTI1.

Measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited several time periods of erroneous data during the flight. During the following time frames:

210306Z – 210352Z, 211601Z – 211622Z, 211705Z – 211713Z, 212252Z – 212300Z, 212853Z – 212906Z, 214912Z – 215821Z, 220601Z – 220610Z, 000248Z - 000505Z and 003420Z – 003427Z the erroneous data was removed by substituting TDM3(Maycomm/TDL) output,

TDM2 = TDM3

The x-direction groundspeed output from inertial #1 (GSUI1) spiked from

014700Z – 014900Z. The spike was removed and patched using statistical techniques.

All other instruments worked optimally during the flight.

Fifteen (15) GPS dropsondes were deployed with 8 being good and 7 being fastfalls. Four (4) tempdrop messages were sent to NHC.

Fifteen (15) AXBTs were deployed all being good.

Takeoff		Landing

2019Z		0410Z

Aircraft Static Pressure                   1009.9mb	1011.3mb

Corrected Tower Pressure               1009.9mb	           1011.8mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073