1

N42RF ERROR SUMMARY

H. GUSTAV IN GULF OF MEXICO

Flight ID: 080831H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           RA159X

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There were three (3) data gaps: 230641Z – 230730Z, 230851Z – 230900Z and

014801Z – 014900Z. Beginning with the H080829 H. Gustav flight and continuing through the H080912 H. Ike flight, there was observed a 3 to 4 second lag between all INE1 and INE2 outputs. INE1was selected since it best corresponded with the analog output.

There were no liquid water sensors available for the flight.

The APN-159 (RA159X) radar altimeter output had several time periods of erroneous values. The RA159 data glitches were fixed using statistical techniques during the following time periods: 194940Z – 195140Z, 203700Z – 203900Z, 212500Z – 212700Z, 001100Z – 001300Z, 004500Z – 004700Z, 011300Z – 011500Z, 011600Z – 011800Z, 022830Z – 023030Z, and 031940Z - 032140Z.

Measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited several  time periods of erroneous data during the flight.  For 030253Z – 030338Z, and 031114Z - 031127Z

the erroneous data was removed by substituting TDM3(Maycomm/TDL) output,

TDM2 = TDM3

For 000237Z – 000510Z, and 002120Z – 002141Z the erroneous TDM2 data was removed by substituting TDM1(Buck) output,

TDM2 = TDM1

All other instruments worked optimally during the flight.

Seventeen (17) GPS dropsondes were deployed with 15 being good. Fifteen (15) tempdrop messages were sent to NHC.

Sixteen (16) AXBTs were deployed with 15 being good.

Takeoff		Landing

1922Z		0349Z

Aircraft Static Pressure                   1008.6mb	1010.9mb

Corrected Tower Pressure               1008.8mb	           1011.1mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073