1

N42RF ERROR SUMMARY

TS HANNA RECCO MISSION

Flight ID: 080904H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           RA159X

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There were no data gaps. Beginning with the H080829 H. Gustav flight and continuing through the H080912 H. Ike flight, there was observed a 3 to 4 second lag between all INE1 and INE2 outputs. INE1was selected since it best corresponded with the analog output.

There were no liquid water sensors available for the flight.

The inertial #1 data was erroneous from 064023Z – 064108Z. This will produce  erroneous horizontal and vertical wind values.

The APN-159 (RA159X) radar altimeter output had several time periods of erroneous values. The RA159 data glitches were fixed using statistical techniques during the following time periods: 035901Z – 040100Z, 044700Z – 044900Z, 063500Z – 063700Z, 065300Z – 065500Z, 070200Z – 070400Z, 073800Z – 074000Z, 075300Z – 075500Z, 080000Z – 080200Z, 091800Z – 092000Z, 092700Z – 092900Z and 101500Z – 101700Z.

NOTE: The RA159X output was very noisy between 115635Z – 121100Z so vertical wind (WZ) output should be viewed very carefully.

Measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited several  time periods of erroneous data during the flight.  For 112310Z – 112511Z, and 122818Z - 123019Z

the erroneous data was removed by substituting TDM3(Maycomm/TDL) output,

TDM2 = TDM3

For 120253Z – 120648Z the erroneous TDM2 data was removed by substituting TDM1(Buck) output,

TDM2 = TDM1

All other instruments worked optimally during the flight.

Four (4) GPS dropsondes were deployed with 3 being good. Three (3) tempdrop messages were sent to NHC.

Takeoff		Landing

0327Z		1243Z

Aircraft Static Pressure                   1010.9mb	1010.9mb

Corrected Tower Pressure               1011.0mb	           1011.6mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073