1

N42RF ERROR SUMMARY

H. IKE MISSION/FERRY TO ST. CROIX

Flight ID: 080906H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           RA159X

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There was one data gap: 150731Z – 150737Z. Beginning with the H080829 H. Gustav flight and continuing through the H080912 H. Ike flight, there was observed a 3 to 4 second lag between all INE1 and INE2 outputs. INE1was selected since it best corresponded with the analog output.

There were no liquid water sensors available for the flight.

The APN-159 (RA159X) radar altimeter output had two time periods of erroneous values. The RA159 data glitches were fixed using statistical techniques during the following time periods: 110430Z – 110630Z and 1110Z – 111200Z.

Measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited several  time periods of erroneous data during the flight.  For 120253Z – 120606Z, 140600Z – 140800Z and 141100Z – 141300Z the erroneous data was removed by substituting TDM3(Maycomm/TDL) output,

TDM2 = TDM3

For 150000Z – 150400Z  the erroneous TDM2 data was removed and patched using statistical techniques.

Fuselage dynamic attack pressure (PQAF) had a data spike between 174300Z – 174500Z.

The spike was removed and patched using statistical techniques.

All other instruments worked optimally during the flight.

Four (4) GPS dropsondes were deployed with 3 being good. Three (3) tempdrop messages were sent to NHC.

Takeoff		Landing

1053Z		2056Z

Aircraft Static Pressure                   1011.7mb	1010.0mb

Corrected Tower Pressure               1012.4mb	           1010.6mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073