1

N42RF ERROR SUMMARY

H. IKE OVER SE GULF OF MEXICO

Flight ID: 080910H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSFX

Dynamic Pressure						PQF1

Vert. Wind						           GPS(Blended)

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There were two data gaps: 200931Z – 200939Z and 005451Z – 005458Z.

Beginning with the H080829 H. Gustav flight and continuing through the H080912 H. Ike flight, there was observed a 3 to 4 second lag between all INE1 and INE2 outputs. INE1was selected since it best corresponded with the analog output.

There were no liquid water sensors available for the flight.

The APN-159 (RA159X) was inop for the entire flight. The blended GPS altitude from INE1 (ALTI1) was used.

Measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited a couple of  time periods of erroneous data during the flight.  For 203834Z – 205655Z the erroneous data was removed by substituting TDM3(Maycomm/TDL) output,

TDM2 = TDM3

For 000249Z – 000529Z  the erroneous TDM2 data was removed by substituting TDM1(Buck) output,

TDM2 = TDM1

Fuselage static pressure (PSF) exhibited several time periods of erroneous data during the flight.  For 211400Z – 211800Z the erroneous data was removed and patched using  statistical techniques.

All other instruments worked optimally during the flight.

Eighteen (18) GPS dropsondes were deployed with 17 being good. Five (5) tempdrop messages were sent to NHC.

Ten (10) AXBTs were deployed with eight (8) being good.

Takeoff		Landing

2003Z		0330Z

Aircraft Static Pressure                   1010.1mb	1013.3mb

Corrected Tower Pressure               1010.5mb	           1013.8mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073