NOAA P-3 N43RF

Tail Doppler Radar Ike

Flight ID: 080910I

Sensor or system						Number or Name

INE								1

Accelerometer				1

Temperature Probe						1

Dew Point Probe						2 (EdgeTech)

Altitude (for vertical wind)					Blended GPS (alti1)

Static Pressure							Rosemount Fuselage PSF

Dynamic Pressure						Rosemount Fuselage PQF1

Time Source							Micro99

Constants File		n43_hur08.adc

Notes:

There were two time/data gaps during this flight. These occurred from 132430Z-132432Z and 150649Z-150708Z.

Due to hardware changes in some of the navigational equipment, a number of parameters have been adjusted in the netCDF file. RA159 and ALTI1 both display the blended altitude solution of the GPS/RINU system, RA232 displays the corrected HG9550 radar altitude, ALTI2 displays the uncorrected HG9550 radar altitude, and ALTCLN displays the NOVATEL radar altitude data.

Temperature data from sensor number 3 is erroneous.

There was an ~40 mb difference in differential attack pressure between the fuselage and radome.

There was an ~12 mb difference in differential sideslip pressure between the fuselage and radome.

No Collins or Ashtech GPS data exist for this flight.

The King Liquid Water Sensor was inoperative throughout the flight.

Data from dew point sensor number 2 was edited in order to bring relative humidity values down below ~120% during times when oversaturation occurred. These corrections occurred from 132457Z-132510Z and from 135521Z-135549Z. Also, dewpoint sensor number 2 was erroneous from 151710Z-151946Z. Data from dewpoint sensor number 3 was substituted into sensor number 2 values during this time.

There was a data spike in the NOVATEL data from 130631Z-130632Z.

Dewpoint sensor number one displayed erroneous spikes during the following times: 103010Z-103352Z, 130135Z-130430Z, and 151520Z to the end of the flight. No corrections were made to the data.

SPECIAL NOTE!!!	Vertical ground, vertical air and vertical speeds, computed

using Dave Jorgensen’s vertical wind algorithm should be used for vertical

wind analysis.

Takeoff                           Land

0808Z                            1526Z

Aircraft Static Pressure 	1011.3 mb     		1012.1 mb

Corrected Tower Pressure	1011.1 mb		1013.2 mb

Flight Director:			Martin Mayeaux   (813) 828-3310 ext. 3086