1

N42RF ERROR SUMMARY

H. IKE NEAR THE TEXAS COAST

Flight ID: 080912H

Sensor or system						Number or Name

Inertial  							INE1

Accelerometer						ACC1

Temperature Probe				                      TT1

Dew Point Probe						TDM2X (EdgeTech)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						           GPS(Blended)

Constants File                                                                  n42_hur08v3.adc

Project Directory						/acdata/2008/hur08

Notes:

There was one data gap: 210741Z – 210746Z.

Beginning with the H080829 H. Gustav flight and continuing through the H080912

H. Ike flight, there was observed a 3 to 4 second lag between all INE1 and INE2 outputs. INE1was selected since it best corresponded with the analog output.

There were no liquid water sensors available for the flight.

The APN-159 (RA159X) was inop for the entire flight. The blended GPS altitude from INE1 (ALTI1) was used.

Measured dewpoint temperature #2 (EdgeTech)...TDM2...exhibited a time period of erroneous data during the flight.  For 000253Z – 000516Z the erroneous data was removed by substituting TDM1(Buck) output,

TDM2 = TDM1

All other instruments worked optimally during the flight.

Twenty-one (21) GPS dropsondes were deployed with 20 being good. Twenty (20) tempdrop messages were sent to NHC.

Ten (10) AXBTs were deployed with seven (7) being good.

Takeoff		Landing

2104Z		0526Z

Aircraft Static Pressure                   1013.8mb	1016.3mb

Corrected Tower Pressure               1014.0mb	           1016.6mb

Flight Director:	A. Barry Damiano       (813) 828-3310 ext. 3073