Flight ID: 20110929I1

Sensor or system						Variable Name

Inertial	(for wind derivation)					ine1

Accelerometer							acci1

Temperature Probe						tt1

Dew Point Probe						tdm2

Altitude (for vertical wind)					alti1

Static Pressure							psf

Dynamic Pressure						pqf2

Constants file							/acdata/adc/2011/43_11v3.adc

Flight Directory						/acdata/2011/MET/20110929I1

Local Met Data:			Takeoff (1337Z)		Landing (2136Z)

Aircraft Static Pressure		 1013.6  mb			1012.7  mb

Tower Pressure			 1013.3  mb			1012.9  mb

Notes

Dynamic pressure pqf1 started spiking just before landing at 213610Z until block-in.  Pqf2 contained no spikes and was selected as default.

There were periods where the dewpoint temperature exceeded the ambient temperature in the storm environment, resulting in RH values > 100%. This was likely due to heavy rain and/or an artificial warming of the dewpoint sensor as it tried to burn off excess moisture. Corrections were not made to the data.

Psf and psw were within 0.5mb of station pressure on takeoff and landing.

All other instruments appeared to work optimally.

20 Dropsondes launched, 19 good 1 bad

11 AXBTs were launched, 9 good 2 bad