Flight ID: 20110924I1

Sensor or system						Variable Name

Inertial	(for wind derivation)					ine1

Accelerometer							acci1

Temperature Probe						tt1

Dew Point Probe						tdm2

Altitude (for vertical wind)					alti1

Static Pressure							psf

Dynamic Pressure						pqf2

Constants file							/acdata/adc/2011/43_11v3.adc

Flight Directory						/acdata/2011/MET/20110924I1

Local Met Data:			Takeoff (1403Z)		Landing (2203Z)

Aircraft Static Pressure		 1012.7 mb			1011.3  mb

Tower Pressure			 1012.8  mb			1010.8  mb

Notes

Dynamic pressure pqf1 contained several downward spikes of about 8mb compared to other dynamic pressure sources from 181313Z – 181344Z; because of this pqf2 was selected as default.

The Novatel GPS position and altitude data were erroneous with several spikes from 192211Z-193656Z.  Alti1 is default.

There were periods where the dewpoint temperature exceeded the ambient temperature resulting in RH values > 100%. This was likely due to heavy rain, a wet-bulb effect on the total temperature sensor, and/or an artificial warming of the dewpoint sensor as it tried to burn off excess moisture. Corrections were not made to the data.

AXBT channels 1 and 3 worked properly, but channel 2 displayed several spikes in the BT data.

Psf and psw were within 0.5mb of station pressure on takeoff and landing.

All other instruments appeared to work optimally.

7 Dropsondes and 1 AXBT were launched from the aircraft.