Flight ID: 20120823H2

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.2

Dew Point Probe						TDM.2

Static Pressure							PSM.2

Dynamic Pressure						PQM.2

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2012/20120823H2

Constants File	              20120823H2/AAMPSConfig/core/n42.xml

Local Met Data:			Takeoff (2002Z)		Landing (0403Z)

Aircraft Static Pressure (PSM.2)	1007.7 mb			1007.8 mb

Tower Pressure (corrected)		1007.0 mb			1007.0 mb

Notes:

There was a data gap in all parameters from 00:00:09Z – 00:00:51Z.

The Edgetech dewpoint, TDM.2, performed best and was used as default.  Dew point values intermittently exceeded ambient temperature values during portions of flight where the aircraft was in precipitation, causing RH values greater than 100%.

The Novatel Alt, Lat and Lon (GPS.3) all spiked for one second at 20:58:01Z and 00:59:16Z.  The blended inertial-GPS solution (Alt, Lat and Lon I-GPS.1) is the default position source.

SPECIAL NOTE!!!    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds, vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 22 GPS dropsondes deployed from the aircraft internally, 21 were sent, 1 was bad.

For a period of 3 minutes, from 03:28:31Z to 03:31:20Z, during the transit from the storm, 8 one second data gaps were observed in only some derived and reference parameters (with a .d or ref extension).