Flight ID: 20120826N2

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3x

Attack Angle		PDAlpha.2

Dew Point Probe						TDM.1

Dynamic Pressure						PQM.2

Geopotential Altitude						ALTGA.d (AltGPS.3x)

Inertial Selected						INE.1

Static Pressure							PSM.2

Slip Angle		PDBeta.2

Temperature Probe						TTM.1

Constants File		49cal102

Flight Directory		acdata/MET/2012/20120826N2

Local Met Data:			Takeoff (1723Z)		Landing (0155Z)

Aircraft Static Pressure		1010.4mb			1012.2mb

Tower Pressure (corrected)		1012.7mb			1014.5mb

Notes:

At 00z there was a system wide data gap that appeared from 00:00:07z to 00:00:53z.

TDM.1 and TDM.2 are both not rated for use under -50 deg C.  Cannot be considered reliable for dewpoints colder than -50C.

PQM.1 Malfunctioned throughout the flight and PQM.2 should be used in derived parameters.

There was a downward spike at 20:09:45z with amplitude of approximately 700m in the AltGPS.3(Novatel unit) data.  The .AXC file was created to filter out the spike of erroneous data in AltGPS.3 and AltGPS.3x is used as the default altitude.

From 21:15z to 22:09z an example can be seen of a disagreement in our GPS altitude sources.  GPS.3 (Novatel) seems to read about a 20m difference compared to GPS.1 and GPS.1 altitudes.  Still with this disagreement it is advised to use GPS.3.