Flight ID: 20120828H2

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.2

Dew Point Probe						TDM.2X

Static Pressure							PSM.2

Dynamic Pressure						PQM.2

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2012/20120828H2

Constants File	              20120828H2/AAMPSConfig/core/n42.xml

Local Met Data:			Takeoff (1943Z)		Landing (0414Z)

Aircraft Static Pressure (PSM.2)	1012.3 mb			1013.2 mb

Tower Pressure (corrected)		1011.5 mb			1014.4 mb

Notes:

There was a data gap in all parameters from 00:00:07Z – 00:00:50Z.

The Edgetech dewpoint, TDM.2, performed best and was used as default.   However, it spiked erroneously when the aircraft climbed through the freezing level, from 03:18:18:Z – 03:21:41Z, when TDM.1 did not.  During this time the following substitution was made:  TDM.2X = TDM.1  Dew point values intermittently exceeded ambient temperature values during portions of flight where the aircraft was in precipitation, causing RH values greater than 100%.

The Novatel Alt, Lat and Lon (GPS.3) had one data spikes at 02:58:05Z.  The blended inertial-GPS solution Alt, Lat and Lon (I-GPS.1) is the default position source.

SPECIAL NOTE!!!    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds, vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 35 GPS dropsondes (34 good) and 22 AXBT’s released from the aircraft.  There were 6 hurricane penetrations.

During the last hour of the flight, the transit back to KMCF, from 03:02:17Z to 04:05:22Z, 15 one second data gaps were observed in only some derived and reference parameters (with a .d or ref extension).