DEPLOYMENT  INSTRUCTIONS

# Read Carefully

----------------------------------------------------------------------------------------------

FOLD

| 1. Remove plastic wrap | 2. DO NOT REMOVE paper tape, cardboard, or anything BUT plastic. |
| --- | --- |
| 3. Throw buoy in water. |  |

DEPLOYMENT INSTRUCTIONS for

## SVP AND SVP-BAROMETER DRIFTERS

Remove the buoys from the shipping container.  REMOVE ONLY the plastic shrink-wrap.  DO NOT REMOVE paper tape securing the drogue and tether.  DO NOT REMOVE cardboard surrounding the float.

DANGER: DO NOT REMOVE the paper tape securing the tether and drogue.  If you do, the drogue and/or tether can unfurl during deployment and cause injury!

2) Record the five digit ID number of the drifter.  This number can be found on the shipping container, the plastic shrink-wrap or the protective cardboard box.  It is also inscribed on the surface float.

3) If testing the buoy is desired prior to deployment, the magnet can be removed from the buoy by separating it from the float through a hole in the box surrounding the float.  This action will start the ARGOS transmitter for testing. Re-attaching the magnet in the same position will turn off the transmitter and reset the program starting point. The transmitter will restart on its original program when the magnet is again removed.

4) Throw the buoy from the stern, lowest possible deck (preferably less than 10 meters including heave), into the sea.  The ship may be traveling between 2 - 25 knots.  The tether and drogue are secured with paper tape that will dissolve in the water.

5) Record the date, time (GMT) and location of deployment as well as the five digit ID, and send this information to the Global Drifter Program.

Thank you very much for your help!

CONTACT PERSON:

Shaun Dolk, Global Drifter Program

NOAA/AOML/PhOD, 4301 Rickenbacker Cswy, Miami, FL 33149, USA

Tel: 305-361-4546    Fax: 305-361-4366    Telex:  6507457601

email: Shaun.Dolk@noaa.gov

Web site: http://www.aoml.noaa.gov/phod/dac/dep_form.html.

**************************************************************************

Sample log sheet

ID          Date       		Time (GMT)      	Latitude         		Longitude

xxxxx      mm/dd/yy       	hh:mm        		DD mm.mm NS     	 DDD mm.mm EW

_______  ____/____/____  	____:____   		____ _______ __  	____ ________ __