Hurricane Field Program Plan

Part II Appendices

Table of Contents

APPENDIX A…………………………………………………………………………………………. 1

Decision and Notification Process………………………………………………………………. 1

APPENDIX B: CALIBRATION; DATA BUOYS………….……………......................................... 5

B.1 En-route Calibration of Aircraft Systems…………………………………………………... 5

B.2 Tables Buoy/C-MAN/NOS Platform Over Flight Locations……………….……………… 6

Table B-2.3 Automated Surface Observing System (ASOS)/METAR Sites………….…... 13

APPENDIX C: DOD/NWS RAWIN/RAOB AND NWS COASTAL LAND-BASED RADARS... 22

APPENDIX D: PRINCIPAL DUTIES OF THE NOAA SCIENTIFIC PERSONNEL………….. 24

APPENDIX E: NOAA RESEARCH OPERATIONAL PROCEDURES AND CHECK LISTS... 28

E.1 “Conditions-of-Flight” Commands…………………………………………………………. 29

E.2 Lead Project Scientist………………………………………………………………………. 30

E.3 Cloud Physics Scientist……………………………………………………………………... 37

E.4 Boundary Layer Scientist…………………………………………………………………… 39

E.5 Radar Scientist……………………………………………………………………………… 42

E.6 Dropsonde Scientist………………………………………………………………………… 45

APPENDIX F: SYSTEMS OF MEASURE AND UNIT CONVERSION FACTORS…………… 48

APPENDIX G: AIRCRAFT SCIENTIFIC INSTRUMENTATION……………………………… 49

APPENDIX H: NOAA EXPENDABLES AND RECORDING MEDIA…………………………. 53

ACRONYMS AND ABBREVIATIONS……………………………………………………………. 55

Hurricane Field Program Plan

Part II

Appendix A

DECISION AND NOTIFICATION PROCESS

The decision and notification process is illustrated in Figs. A-1, A-2, and A-3.  This process occurs in four steps:

1) A research mission is determined to be probable within 72 h [field program director]. Consultation with the director of HRD, and the AOC Project Manager determines: flight platform availability, crew and equipment status, and the type of mission(s) likely to be requested.

2) The Field Program Advisory Panel [F. Marks (Director, HRD), R. Rogers (Director, Hurricane Field Program), J. Dunion, E. Uhlhorn, S. Aberson, M. Black, J. Cione, P. Dodge, J. Gamache, J. Kaplan, S. Murillo and J. McFadden (or AOC designee) meets to discuss possible missions and operational modes. Probable mission determination and approval to proceed is given by the HRD director (or designee).

3) Primary personnel are notified by the Hurricane Field Program Director [R. Rogers].

4) Secondary personnel are notified by their primary affiliate (Table A-2).

General information, including updates of program status, are provided continuously by tape. Call (305) 221-3679 to listen to the recorded message. During normal business hours, callers should use (305) 361-4400 for other official inquiries and contacts. During operational periods, an MGOC team member is available by phone at (305) 229-4407 or (305) 221-4381. The MGOC team leader and the HRD field program director will be available by cell phone. (Appropriate telepager phone numbers will be provided to program participants before the start of the field program.)

**Note: Time of briefings, conference calls, decisions, and deployments are dictated by timing limitations imposed by the AOC crew.

Fig. A-1: Decision and notification process for Day “-2”.

**Note: Time of briefings, conference calls, decisions, and deployments are dictated by timing limitations imposed by the AOC crew.

Fig. A-2: Decision and notification process for Day “-1”

**Note: Time of briefings, conference calls, decisions, and deployments are dictated by timing limitations imposed by the AOC crew.

Fig. A-3: Decision and notification process for Day “0”

Appendix B: Calibration; Data Buoys

B.1 En-Route Calibration of Aircraft Systems

Instrument calibrations are checked by flying aircraft intercomparison patterns whenever possible during the hurricane field program or when the need for calibration checks is suggested by a review of the data. In addition, an over flight of a surface pressure reference is advisable en route or while on station when practicable. Finally, all flights enroute to and from the storm are required to execute a true airspeed (TAS) calibration pattern. This pattern is illustrated in Fig. B-1.

Fig. B-1 En-Route TAS calibration pattern.

| Station | Lat | Lon | Height | Provider | # Station Location | # Payload Comments |
| --- | --- | --- | --- | --- | --- | --- |
| RSJT2 | 26.801 | -97.47 | 10 (m) | TCOON | RINCON DEL SAN JOSE POTRERO LOPENO SW |  |
| BABT2 | 27.3 | -97.42 | 10 | TCOON | BAFFIN BAY POINT OF ROCKS |  |
| PTIT2 | 26.06 | -97.26 | 10 | NOS-NWLON | PORT ISABEL, TX, 8779770 |  |
| MQTT2 | 27.58 | -97.22 | 10 | NOS-NWLON | 8775870 - MALAQUITE BEACH, CORPUS CHRISTI, TX |  |
| PTAT2 | 27.83 | -97.05 | 14.9 | NDBC | PORT ARANSAS, TX |  |
| 42044 | 26.11 | -97.03 | 4 | TABS | PS-1126 TABS J | TABS II |
| RTOT2 | 27.76 | -96.98 | 20 | TCOON | RTNS OFFSHORE |  |
| 42020 | 26.916 | -96.7 | 5 | NDBC | 50 NM SOUTHEAST OF CORPUS CHRISTI, TX | 3-meter discus buoy, DACT payload |
| PCNT2 | 28.45 | -96.4 | 9 | TCOON | MATAGORDA BAY PORT O'CONNOR |  |
| 42045 | 26.13 | -96.31 | 4 | TABS | PI-745 TABS K | TABS II |
| 42019 | 27.916 | -95.36 | 5 | NDBC | FREEPORT, TX 60 NM SOUTH OF FREEPORT, TX | 3-meter discus buoy, DACT payload |
| FPTT2 | 28.95 | -95.31 | 10 | NOS-NWLON | 8772440 - FREEPORT, TX |  |
| MGPT2 | 29.68 | -94.99 | 10 | NOS-NWLON | 8770613 – MORGAN’S POINT, TX |  |
| EPTT2 | 29.48 | -94.92 | 10 | NOS-NWLON | 8771013 - EAGLE POINT, TX |  |
| 42043 | 28.99 | -94.9 | 4 | TABS | GA-252 TABS B | TABS II |
| GTOT2 | 29.31 | -94.79 | 10 | NOS-NWLON | 8771450 - GALVESTON PIER 21, TX |  |
| GPST2 | 29.29 | -94.79 | 7 | NOS-NWLON | 8771510 - GALVESTON PLEASURE PIER, TX |  |
| GLPT2 | 29.285 | -94.788 | 15 | TCOON | GALVESTON PLEASURE PIER |  |
| GNJT2 | 29.3583 | -94.725 | 10 | NOS-NWLON | 8771341 - GALVESTON BAY (NORTH JETTY), TX |  |
| 42035 | 29.246 | -94.41 | 5 | NDBC | GALVESTON 22 NM EAST OF GALVESTON, TX | 3-meter discus buoy, DACT payload |
| SRST2 | 29.67 | -94.05 | 12.5 | NDBC | SABINE, TX |  |
| 42055 | 22.02 | -94.05 | 10 | NDBC | BAY OF CAMPECHE | 12-meter discus buoy, ARES payload |
| 42046 | 27.53 | -94.02 | 4 | TABS | HI-A595 TABS N | TABS II |
| FGBL1 | 28.118 | -93.67 | 22.9 | ESSI Corp | FOREST OIL - HIGH ISLAND - HI-334B |  |
| 42002 | 25.892 | -93.568 | 10 | NDBC | W GULF 240 NM SOUTH-SOUTHEAST OF SABINE, TX | 10-meter discus buoy, MARS payload |
| 42047 | 27.54 | -93.36 | 4 | TABS | HI-A389 TABS V | TABS II |
| 42038 | 27.42 | -92.57 | 5 | NDBC | NORTH MID GULF OF MEXICO | 3-meter discus buoy, ARES payload |
| MRSL1 | 29.441 | -92.061 | 23.42 | LSU WAVCIS | MARSH ISLAND, CSI03 |  |
| SLPL1 | 29.516 | -91.557 | 5.5 | LSU WAVCIS | SALT POINT, CSI14 |  |
| 42362 | 27.8 | -90.67 | 122 | Shell Intl E&P | BRUTUS - GREEN CANYON 158 | Fixed Drilling Platform |
| TAML1 | 29.187 | -90.665 | 10 | LUMCON | TAMBOUR BAY |  |
| LUML1 | 29.253 | -90.663 | 13.2 | LUMCON | LUMCON MARINE CENTER |  |
| ILDL1 | 29.053 | -90.533 | 19.2 | LSU WAVCIS | ISLE CERNIERES, CSI05 |  |
| SPLL1 | 28.867 | -90.483 | 40.34 | LSU WAVCIS | SOUTH TIMBALIER BLOCK 52,CSI06 |  |
| BYGL1 | 29.78 | -90.42 | 9.1 | NOS-NWLON | 8762483 - BAYOU GAUCHE, LA |  |
| LABL1 | 30.05 | -90.37 | 9.1 | NOS-NWLON | 8762482 - BAYOU LABRANCH, LA |  |
| LKPL1 | 30.31 | -90.28 | 13 | LUMCON | WESTERN LAKE PONCHARTRAIN |  |
| GISL1 | 29.263 | -89.957 | 9.5 | NOS-NWLON | 8761724 - GRAND ISLE, LA |  |
| 42001 | 25.928 | -89.653 | 10 | NDBC | MID GULF 180 NM SOUTH OF SOUTHWEST PASS, LA | 12-meter discus buoy, ARES payload |
| OSTF1 | 30.3567 | -89.6125 | 10 | NDBC | STENNIS TEST FACILITY |  |
| BURL1 | 28.906 | -89.429 | 30.5 | NDBC | SOUTHWEST PASS, LA |  |
| SIPM6 | 30.266 | -89.007 | 10.98 | LSU WAVCIS | SHIP ISLAND PASS,CSI13 |  |
| 42007 | 30.091 | -88.772 | 5 | NDBC | BILOXI 22 NM SOUTH-SOUTHEAST OF BILOXI, MS | 3-meter discus buoy, DACT payload |
| 42040 | 29.195 | -88.253 | 5 | NDBC | 64 NM SOUTH OF DAUPHIN ISLAND, AL | 3-meter discus buoy, DACT payload |
| DPIA1 | 30.25 | -88.07 | 17.4 | NDBC | 8735180 - DAUPHIN ISLAND, AL |  |

B-2 Buoy, C-MAN, and NOS Platform Locations

Gulf of Mexico

| Station | Lat | Lon | Height | Provider | Station Location | Payload Comments |
| --- | --- | --- | --- | --- | --- | --- |
| PCLF1 | 30.40 | -87.21 | 10 (m) | NOS-NWLON | 8729840 - PENSACOLA, FL |  |
| 42039 | 28.80 | -86.04 | 5 | NDBC | PENSACOLA - 115NM EAST SOUTHEAST OF PENSACOLA, FL | 3-meter discus buoy, DACT payload |
| 42003 | 25.94 | -85.92 | 10 | NDBC | E GULF 260 NM SOUTH OF PANAMA CITY, FL | 10-meter discus buoy, ARES payload |
| PCBF1 | 30.21 | -85.88 | 6.1 | NOS-NWLON | 8729210 - PANAMA CITY BEACH, FL |  |
| APCF1 | 29.73 | -84.98 | 10 | NOS-NWLON | 8728690 - APALACHICOLA, FL |  |
| SGOF1 | 29.41 | -84.86 | 35.1 | NDBC | TYNDALL AFB TOWER C (N4), FL |  |
| 42036 | 28.51 | -84.51 | 5 | NDBC | W. TAMPA 106 NM WEST NORTHWEST OF TAMPA, FL | 3-meter discus buoy, ARES payload |
| SHPF1 | 30.06 | -84.29 | 10 | COMPS | SHELL POINT, FL |  |
| 42024 | 27.46 | -84.22 | 3.2 | COMPS | W. FL MERHAB |  |
| 42022 | 27.50 | -83.74 | 3.2 | COMPS | CMP24 - WEST FLORIDA CENTRAL BUOY |  |
| KTNF1 | 29.82 | -83.59 | 10 | NDBC | KEATON BCH, FL |  |
| 42021 | 28.30 | -83.30 | 2.8 | COMPS | CMP4 - PASCO COUNTY BUOY, FL |  |
| 42023 | 26.05 | -83.07 | 3.2 | COMPS | CM3 - WEST FLORIDA SOUTH BUOY |  |
| CDRF1 | 29.14 | -83.03 | 10 | NDBC | CEDAR KEY, FL |  |
| 42013 | 27.16 | -82.95 | 2.8 | COMPS | NA2 - NAVY-2 |  |
| CWBF1 | 27.98 | -82.83 | 6.4 | NOS-NWLON | 8726724 - CLEARWATER BEACH, FL |  |
| FHPF1 | 28.15 | -82.80 | 10 | COMPS | FRED HOWARD PARK, FL |  |
| ANCF1 | 28.19 | -82.79 | 10 | COMPS | ANCLOTE GULF PARK, FL |  |
| PLSF1 | 24.69 | -82.77 | 10 | NDBC | PULASKI SHOAL LIGHT, FL |  |
| EGKF1 | 27.60 | -82.76 | 10 | COMPS | EGMONT KEY, FL |  |
| TARF1 | 28.16 | -82.76 | 7 | COMPS | TARPON SPRINGS, FL |  |
| ANMF1 | 27.54 | -82.74 | 10.8 | COMPS | ANNA MARIA, FL |  |
| PTRF1 | 28.29 | -82.73 | 10.1 | COMPS | PORT RICHEY, FL |  |
| HSSF1 | 28.77 | -82.71 | 6.6 | COMPS | HOMOSASSA, FL |  |
| ARPF1 | 28.44 | -82.67 | 10.3 | COMPS | ARIPEKA, FL |  |
| SAPF1 | 27.76 | -82.63 | 6.1 | NOS-NWLON | 8726520 - ST. PETERSBURG, FL |  |
| PMAF1 | 27.64 | -82.56 | 10 | NOS-NWLON | 8726384 - PORT MANATEE, FL |  |
| OPTF1 | 27.86 | -82.55 | 10 | NOS-NWLON | 8726607 - OLD PORT TAMPA, FL |  |
| VENF1 | 27.07 | -82.45 | 11.6 | NDBC | VENICE, FL |  |
| MCYF1 | 27.91 | -82.42 | 10 | NOS-NWLON | 8726667 - MCKAY BAY ENTRANCE, FL |  |
| 42014 | 25.25 | -82.21 | 2.8 | COMPS | W. FL SEA-COOS |  |
| BGCF1 | 26.40 | -81.88 | 12 | COMPS | BIG CARLOS PASS, FL |  |
| SANF1 | 24.46 | -81.88 | 13.1 | NDBC | SAND KEY, FL |  |
| FMRF1 | 26.65 | -81.87 | 6.1 | NOS-NWLON | 8725520 - FORT MYERS, FL |  |
| KYWF1 | 24.55 | -81.81 | 6.4 | NOS-NWLON | 8724580 - KEY WEST, FL |  |
| FRDF1 | 30.67 | -81.47 | 6.4 | NOS-NWLON | 8720030 - FERNANDINA BEACH, FL |  |
| MYPF1 | 30.40 | -81.43 | 10 | NOS-NWLON | 8720218 - MAYPORT (BAR PILOTS DOCK), FL |  |
| SAUF1 | 29.86 | -81.26 | 16.5 | NDBC | ST. AUGUSTINE |  |
| SMKF1 | 24.63 | -81.11 | 48.5 | NDBC | SOMBRERO KEY, FL |  |
| VCAF1 | 24.71 | -81.11 | 6.4 | NOS-NWLON | 8723970 - VACA KEY, FL |  |
| LONF1 | 24.84 | -80.86 | 7 | NDBC | LONG KEY, FL |  |
| ALRF1 | 24.90 | -80.60 | 47.5 | NDBC | ALLIGATOR REEF |  |
| TRDF1 | 28.42 | -80.59 | 6.4 | NOS-NWLON | 8721604 - TRIDENT PIER, FL |  |
| 41012 | 30.00 | -80.50 | 5 | NDBC | ST. AUGUSTINE, FL 40NM ENE OF ST AUGUSTINE, FL | 3-meter discus buoy, ARES payload |
| MLRF1 | 25.01 | -80.38 | 15.8 | NDBC | MOLASSES REEF, FL |  |
| 41009 | 28.50 | -80.18 | 5 | NDBC | CANAVERAL 20 NM EAST OF CAPE CANAVERAL, FL | 6-meter NOMAD buoy, ARES payload |
| VAKF1 | 25.73 | -80.16 | 9.1 | NOS-NWLON | 8723214 - VIRGINIA KEY, FL |  |
| FWYF1 | 25.59 | -80.10 | 43.9 | NDBC | FOWEY ROCKS, FL |  |
| LKWF1 | 26.61 | -80.03 | 13.7 | NDBC | LAKE WORTH, FL |  |
| SPGF1 | 26.69 | -79.00 | 9.8 | NDBC | SETTLEMENT POINT, GBI |  |
| 41010 | 28.91 | -78.55 | 5 | NDBC | CANAVERAL EAST 120NM EAST OF CAPE CANAVERAL | 6-meter NOMAD buoy, ARES payload |
| CMRC3 | 23.80 | -76.10 | 10 | CREWS/AOML | NORTH NORMAN'S REEF, BAHAMAS |  |

B-2 Buoy, C-MAN, and NOS Platform Locations (cont’d)

| Station | Lat | Lon | Height (m) | Station Provider | Station Location | Payload Comments |
| --- | --- | --- | --- | --- | --- | --- |
| FPKG1 | 32.03 | -80.9 | 6.7 | NOS-NWLON | 8670870 - FORT PULASKI, FL |  |
| 41008 | 31.40 | -80.87 | 5 | NDBC | GRAY’S REEF, SAVANNAH, GA | 3-meter discus buoy, ARES payload |
| SPAG1 | 31.38 | -80.57 | 50 | SIO | U.S. NAVY TOWER R2 GA |  |
| FRPS1 | 32.34 | -80.46 | 10 | NOS-NWLON | 8668498 - FRIPPS INLET, SC |  |
| 41033 | 32.28 | -80.41 | 3 | Caro-COOPS | FRIPP NEARSHORE (FRP 2) |  |
| SECG1 | 30.80 | -80.32 | 35 | NC-COOS | U.S. NAVY TOWER R4 GA |  |
| SKMG1 | 31.53 | -80.24 | 50 | SIO | U.S. NAVY TOWER M2R6 GA |  |
| TYBG1 | 31.63 | -79.92 | 34 | SIO | U.S. NAVY TOWER R8 GA |  |
| FBIS1 | 32.68 | -79.89 | 9.8 | NDBC | FOLLY ISLAND, SC |  |
| SCIS1 | 32.86 | -79.71 | 10 | NOS-NWLON | 8664941 - S CAPERS ISLAND, SC |  |
| 41029 | 32.81 | -79.63 | 3 | Caro-COOPS | CAPERS NEARSHORE (CAP 2) |  |
| 41030 | 32.52 | -79.34 | 3 | Caro-COOPS | CAPERS MID-SHELF (CAP 3) |  |
| 41004 | 32.51 | -79.10 | 5 | NDBC | EDISTO, CHARLESTON, SC | 3-meter discus buoy, VEEP payload |
| MROS1 | 33.66 | -78.92 | 10 | NOS-NWLON | 8661070 - SPRINGMAID PIER, SC |  |
| SNSN7 | 33.87 | -78.51 | 10 | Caro-COOPS | 8659897 - SUNSET BEACH, NC |  |
| 41024 | 33.83 | -78.48 | 3 | Caro-COOPS | SUNSET NEARSHORE (SUN 2) |  |
| JMPN7 | 34.21 | -77.80 | 10 | CORMP | 8658163 - JOHNNY MERCER PIER, WRIGHTSVILLE BEACH, NC |  |
| 41038 | 34.14 | -77.71 | 3 | CORMP | ILM2, WRIGHTSVILLE BEACH, NC |  |
| FPSN7 | 33.49 | -77.59 | 44.2 | NDBC | FRYING PAN SHOALS, NC |  |
| 41013 | 33.48 | -77.58 | 5 | NDBC | FRYING PAN SHOALS, NC BUOY | 3-meter discus buoy, DACT payload |
| 41037 | 33.99 | -77.36 | 3 | CORMP | ILM3, WRIGHTSVILLE BEACH, NC |  |
| 41035 | 34.48 | -77.28 | 5 | NDBC | ONSLOW BAY, NC | 3-meter discus buoy, ARES payload |
| CLKN7 | 34.62 | -76.52 | 9.8 | NDBC | CAPE LOOKOUT, NC |  |
| 41002 | 32.28 | -75.19 | 5 | NDBC | S HATTERAS - CHARLESTON, SC | 6-meter NOMAD buoy, VEEP payload |

B-2 Buoy, C-MAN, and NOS Platform Locations (cont’d)

| Station | Lat | Lon | Height | Station Provider | Station Location | Payload Comments |
| --- | --- | --- | --- | --- | --- | --- |
| BLTM2 | 39.27 | -76.58 | 10(m) | NOS-NWLON | 8574680 - BALTIMORE, MD |  |
| FSKM2 | 39.22 | -76.53 | 10 | NOS-NWLON | 8574728 - FRANCIS SCOTT KEY BRIDGE, MD |  |
| YKTV2 | 37.23 | -76.48 | 10 | NOS-NWLON | 8637689 - YORKTOWN, VA |  |
| LWTV2 | 38.00 | -76.47 | 10 | NOS-NWLON | 8635750 - LEWISETTA, VA |  |
| CBIM2 | 38.32 | -76.45 | 10 | CBOS | CHESAPEAKE BIO LAB WEATHER STATION, MD |  |
| SLIM2 | 38.32 | -76.45 | 10 | NOS-NWLON | 8577330 - SOLOMON’S ISLAND, MD |  |
| TPLM2 | 38.90 | -76.44 | 18 | NDBC | THOMAS POINT, MD |  |
| CRYV2 | 36.89 | -76.34 | 10 | NOS-NWLON | 8638595 - SOUTH CRANEY ISLAND, VA |  |
| YKRV2 | 37.25 | -76.33 | 10 | NOS-NWLON | 8637611 - YORK RIVER E. REAR RANGE LIGHT, VA |  |
| MNPV2 | 36.78 | -76.30 | 10 | NOS-NWLON | 8639348 - MONEY POINT, VA |  |
| TCBM2 | 39.12 | -76.25 | 10 | NOS-NWLON | 8573364 - TOLCHESTER BEACH, MD |  |
| HPLM2 | 38.59 | -76.13 | 10 | CBOS | HORN POINT WEATHER STATION MD |  |
| CBBV2 | 36.97 | -76.11 | 13 | NOS-NWLON | 8638863 - CHESAPEAKE BAY BRIDGE TUNNEL, VA |  |
| CAMM2 | 38.57 | -76.07 | 6.1 | NOS-NWLON | 8571892 - CAMBRIDGE, MD |  |
| BISM2 | 38.22 | -76.04 | 10 | NOS-NWLON | 8571421 - BISHOPS HEAD, MD |  |
| RPLV2 | 37.54 | -76.02 | 10 | NOS-NWLON | 8632837 - RAPPAHANNOCK LIGHT, VA |  |
| CHYV2 | 36.93 | -76.01 | 10 | NOS-NWLON | 8638999 - CAPE HENRY, VA |  |
| KPTV2 | 37.17 | -75.99 | 6.4 | NOS-NWLON | 8632200 - KIPTOPEKE, VA |  |
| CHCM2 | 39.53 | -75.81 | 10 | NOS-NWLON | 8573927 - CHESAPEAKE CITY, MD |  |
| DUCN7 | 36.18 | -75.75 | 20.4 | NDBC | DUCK PIER, NC |  |
| CHLV2 | 36.90 | -75.71 | 43.3 | NDBC | CHESAPEAKE LIGHT, VA |  |
| WAHV2 | 37.61 | -75.69 | 10 | NOS-NWLON | 8631044 - WACHAPREAGUE, VA |  |
| DELD1 | 39.58 | -75.59 | 10 | NOS-NWLON | 8551762 - DELAWARE CITY, DE |  |
| RDYD1 | 39.56 | -75.57 | 10 | NOS-NWLON | 8551910 - REEDY POINT, DE |  |
| 41025 | 35.15 | -75.29 | 5 | NDBC | DIAMOND SHOALS | 3-meter discus buoy, ARES payload |
| PHBP1 | 39.93 | -75.14 | 6.4 | NOS-NWLON | 8545240 - PHILADELPHIA, PA |  |
| LWSD1 | 38.78 | -75.12 | 10 | NOS-NWLON | 8557380 - LEWES, DE |  |
| BRND1 | 38.99 | -75.11 | 10 | NOS-NWLON | 8555889 - BRANDYWINE SHOAL LIGHT, DE |  |
| CMAN4 | 38.97 | -74.96 | 12.2 | NOS-NWLON | 8536110 - CAPE MAY, NJ |  |
| 44014 | 36.58 | -74.83 | 5 | NDBC | 64 NM EAST OF VIRGINIA BEACH, VA | 3-meter discus buoy, DACT payload |
| AVAN4 | 39.09 | -74.72 | 10 | NDBC | STEVENS INST |  |
| 44009 | 38.46 | -74.70 | 5 | NDBC | 26 NM SOUTHEAST OF CAPE MAY, NJ | 3-meter discus buoy, VEEP payload |
| ACMN4 | 39.38 | -74.42 | 15 | Stevens Inst | ATLANTIC CITY MARINA, NJ |  |
| ACYN4 | 39.36 | -74.42 | 10 | NOS-NWLON | 8534720 - ATLANTIC CITY, NJ |  |
| BRBN4 | 39.61 | -74.20 | 10 | Stevens Inst | AVALON, NJ |  |
| 44004 | 38.46 | -70.69 | 5 | NDBC | HOTEL 200 NM EAST OF CAPE MAY,NJ | 6-meter NOMAD buoy, ARES payload |

B-2 Buoy, C-MAN, and NOS Platform Locations (cont’d)

| Station | Lat | Lon | Height (m) | Provider | Station Location | Payload Comments |
| --- | --- | --- | --- | --- | --- | --- |
| BDRN4 | 40.08 | -74.87 | 10 | NOS-NWLON | 8539094 - BURLINGTON, DELAWARE RIVER, NJ |  |
| NBLP1 | 40.14 | -74.75 | 10 | NOS-NWLON | 8548989 – NEW BOLD, PA |  |
| BGNN4 | 40.64 | -74.15 | 9.1 | NOS-NWLON | 8519483 - BERGEN POINT WEST REACH, NJ |  |
| NCSC3 | 41.10 | -74.15 | 16 | Long Island Ferry | NORTH CENTRAL SOUND, CT |  |
| BATN6 | 40.70 | -74.02 | 10 | NOS-NWLON | 8518750 - THE BATTERY, NY |  |
| SDHN4 | 40.47 | -74.01 | 10 | NOS-NWLON | 8531680 - SANDY HOOK, NJ |  |
| ALSN6 | 40.46 | -73.83 | 49.1 | NDBC | AMBROSE LIGHT, NY |  |
| KPTN6 | 40.81 | -73.78 | 10 | NOS-NWLON | 8516945 - KING’S POINT, NY |  |
| 44040 | 40.96 | -73.58 | 3.5 | MYSOUND | WESTERN LONG ISLAND SOUND | buoy |
| BHRC3 | 41.18 | -73.19 | 16 | Long Island Ferry | BRIDGEPORT TERMINAL, CT |  |
| FWIC3 | 41.15 | -73.17 | 16 | Long Island Ferry | FAYER WEATHER ISLAND, CT |  |
| 44025 | 40.25 | -73.17 | 5 | NDBC | LONG ISLAND 33 NM SOUTH OF ISLIP, NY | 3-meter discus buoy, DACT payload |
| NOSC3 | 41.12 | -73.16 | 16 | Long Island Ferry | NORTHERN OPEN SOUND, CT |  |
| MISC3 | 41.07 | -73.13 | 16 | Long Island Ferry | NORTH MIDDLE SOUND, CT |  |
| MISN6 | 41.05 | -73.12 | 16 | Long Island Ferry | SOUTH MIDDLE SOUND, NY |  |
| SCSN6 | 41.02 | -73.11 | 16 | Long Island Ferry | SOUTH CENTRAL SOUND, NY |  |
| SOSN6 | 41.00 | -73.10 | 16 | Long Island Ferry | SOUTHERN OPEN SOUND, NY |  |
| OFPN6 | 40.97 | -73.08 | 16 | Long Island Ferry | OLD FIELD ISLAND, NY |  |
| PTJN6 | 40.95 | -73.07 | 16 | Long Island Ferry | PORT JEFFERSON, NY |  |
| NWHC3 | 41.28 | -72.91 | 6.4 | NOS-NWLON | 8465705 - NEW HAVEN, CT |  |
| 44039 | 41.14 | -72.66 | 3.5 | MYSOUND | CENTRAL LONG ISLAND SOUND | 2.4-meter foam hull buoy |
| NLNC3 | 41.36 | -72.09 | 10 | NOS-NWLON | 8461490 - NEW LONDON, CT |  |
| LDLC3 | 41.31 | -72.08 | 20 | MySound | LEDGE LIGHT WEATHER STATION, CT |  |
| 44017 | 40.70 | -72.00 | 5 | NDBC | 23 NM SOUTHWEST OF MONTAUK POINT, NY | 3-meter discus buoy, ARES payload |
| QPTR1 | 41.59 | -71.41 | 6.4 | NOS-NWLON | 8454049 - QUONSET POINT, RI |  |
| FOXR1 | 41.81 | -71.35 | 4.2 | NOS-NWLON | 8454000 - PROVIDENCE, RI |  |
| PTCR1 | 41.64 | -71.34 | 10 | NOS-NWLON | 8452951 - POTTER COVE, PRUDENCE ISLAND, RI |  |
| CPTR1 | 41.72 | -71.34 | 4.3 | NOS-NWLON | 8452944 - CONIMICUT LIGHT, RI |  |
| NWPR1 | 41.51 | -71.33 | 6.4 | NOS-NWLON | 8452660 - NEWPORT, RI |  |
| BLTM3 | 41.71 | -71.17 | 10 | NOS-NWLON | 8447387 - FALL RIVER, MA |  |
| FRVM3 | 41.71 | -71.16 | 3.6 | NOS-NWLON | 8447386 - FALL RIVER, MA |  |
| BUZM3 | 41.40 | -71.03 | 24.8 | NDBC | BUZZARDS BAY, MA |  |
| 44013 | 42.35 | -70.69 | 5 | NDBC | BOSTON 16 NM EAST OF BOSTON, MA | 3-meter discus buoy, DACT payload |
| BZBM3 | 41.52 | -70.67 | 10 | NOS-NWLON | 8447930 - WOODS HOLE, MA |  |
| IOSN3 | 42.97 | -70.62 | 19.2 | NDBC | ISLE OF SHOALS, NH |  |
| 44029 | 42.52 | -70.57 | 4 | GoMOOS | BUOY A0102 - MASS. BAY/STELLWAGEN | 2-meter discus buoy |
| 44030 | 43.18 | -70.43 | 4 | GoMOOS | BUOY B0102 - WESTERN MAINE SHELF | 2-meter discus buoy |
| CASM1 | 43.66 | -70.25 | 10 | NOS-NWLON | PORTLAND, ME, 8418150 |  |
| 44007 | 43.53 | -70.14 | 5 | NDBC | PORTLAND 12 NM SOUTHEAST OF PORTLAND, ME | 3-meter discus buoy, ARES payload |
| 44031 | 43.57 | -70.06 | 4 | GoMOOS | BUOY C0201 - CASCO BAY | 2-meter discus buoy |
| 44008 | 40.50 | -69.43 | 5 | NDBC | NANTUCKET 54 NM SOUTHEAST OF NANTUCKET | 3-meter discus buoy, ARES payload |
| 44032 | 43.72 | -69.36 | 4 | GoMOOS | BUOY E0104 - CENTRAL MAINE SHELF | 2-meter discus buoy |
| 44018 | 41.30 | -69.20 | 5 | NDBC | SE CAPE COD 30 NM EAST OF NANTUCKET, MA | 3-meter discus buoy, ARES payload |
| 44005 | 43.17 | -69.18 | 5 | NDBC | GULF OF MAINE, NH | 6-meter NOMAD buoy, DACT payload |
| 44033 | 44.06 | -69.00 | 4 | GoMOOS | BUOY F0103 - WEST PENOBSCOT BAY | 2-meter discus buoy |
| MISM1 | 43.78 | -68.85 | 16.5 | NDBC | MATINICUS ROCK, ME |  |
| MDRM1 | 43.97 | -68.13 | 22.6 | NDBC | MT DESERT ROCK, ME |  |
| 44037 | 43.49 | -67.88 | 4 | GoMOOS | BUOY M0102 - JORDAN BASIN | 2-meter discus buoy |
| 44585 | 43.35 | -67.64 | 10 | NDBC |  | Drifting Buoy |
| 44027 | 44.27 | -67.31 | 5 | NDBC | JONESPORT, MAINE | 3-meter discus buoy, ARES payload |
| 44035 | 44.89 | -67.02 | 4 | GoMOOS | BUOY J0201 - COBSCOOK BAY | 2-meter discus buoy |
| PSBM1 | 44.90 | -66.99 | 6.4 | NOS-NWLON | 8410140 - EASTPORT, ME |  |

B-2 Buoy, C-MAN, and NOS Platform Locations (cont’d)

| Station | Lat | Lon | Height | Provider | Station Location | Payload Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 44137 | 41.83 | -60.94 | 5(m) | Env Canada | EAST SCOTIA SLOPE |  |
| 44141 | 42.10 | -56.22 | 5 | Env Canada | LAURENTIAN FAN |  |
| 44024 | 42.31 | -65.93 | 4 | GoMOOS | BUOY N - NORTHEAST CHANNEL | 2-meter discus buoy |
| 44142 | 42.50 | -64.02 | 5 | Env Canada | LA HAVE BANK |  |
| 44038 | 43.62 | -66.55 | 4 | GoMOOS | BUOY L0102 - SCOTIAN SHELF | 2-meter discus buoy |
| 44140 | 43.75 | -51.74 | 5 | Env Canada | TAIL OF THE BANK |  |
| 44034 | 44.11 | -38.11 | 4 | GoMOOS | BUOY I0103 - EASTERN MAINE SHELF | 2-meter discus buoy |
| 44139 | 44.26 | -57.39 | 5 | Env Canada | BANQUREAU BANKS |  |
| 44258 | 44.54 | -63.35 | 5 | Env Canada | HALIFAX HARBOR |  |
| 44036 | 45.20 | -66.02 | 4 | GoMOOS | BUOY K0102 - SAINT JOHN | 2-meter discus buoy |
| 44251 | 46.44 | -53.39 | 5 | Env Canada | NICKERSON BANK |  |
| 44150 | 46.85 | -64.64 | 5 | Env Canada | LA HAVE BANK | 3-meter discus buoy |
| 44255 | 47.28 | -57.35 | 5 | Env Canada | NE BURGEO BANK |  |

B-2 Buoy, C-MAN, and NOS Platform Locations (cont’d)

| Station | Lat | Lon | Height | Station Provider | Station Location | Payload Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 51003 | 19.16 | -160.74 | 5 (m) | NDBC | 205 NM SOUTHWEST OF HONOLULU, HI | 6-meter NOMAD buoy, VEEP payload |
| 51002 | 17.15 | -157.79 | 5 | NDBC | 215 NM SOUTH SOUTHWEST OF HILO, HI | 6-meter NOMAD buoy, ARES payload |
| 51004 | 17.52 | -152.51 | 5 | NDBC | SE HAWAII 185 NM SOUTHEAST OF HILO, HI | 6-meter NOMAD buoy, ARES payload |
| 42056 | 19.87 | -85.06 | 10 | NDBC | YUCATAN BASIN | 12-meter discus buoy, ARES payload |
| 42057 | 14.99 | -79.99 | 10 | NDBC | WESTERN CARIBBEAN | 10-meter discus buoy, ARES payload |
| 42058 | 14.98 | -74.99 | 10 | NDBC | CENTRAL CARIBBEAN | 10-meter discus buoy, ARES payload |
| LPPR1 | 17.94 | -67.05 | 10 | CREWS/AOML | LA PARGUERA, PR |  |
| SRVI1 | 17.78 | -64.76 | 10 | CREWS/AOML | ST. CROIX, USVI |  |
| 41100 | 15.90 | -57.90 | 3.6 | Meteo France | LESSER ANTILLES |  |
| 41101 | 14.60 | -56.20 | 3.6 | Meteo France | EAST OF MARTINIQUE |  |
| 41040 | 14.50 | -53.02 | 5 | NDBC | WEST ATLANTIC | 6-meter NOMAD buoy, ARES payload |
| 41041 | 14.53 | -46.00 | 5 | NDBC | MIDDLE ATLANTIC | 6-meter NOMAD buoy, ARES payload |
| 41026 | 12.00 | -38.00 | 5 | Caro-COOPS | SUNSET MID-SHELF (SUN 3) |  |

B-2 Buoy, C-MAN, and NOS Platform Locations (cont’d)

B-3.ASOS/METAR Platform Locations

| # | Station | Lat | Lon | Provider |  |
| --- | --- | --- | --- | --- | --- |
| 1 | KRND | 29.53 | -98.28 | NWS |  |
| 2 | KMFE | 26.18 | -98.24 | FAA |  |
| 3 | KBMQ | 30.74 | -98.24 | FAA |  |
| 4 | KSEP | 32.22 | -98.18 | NWS |  |
| 5 | K25R | 26.44 | -98.13 | NWS |  |
| 6 | KEBG | 26.43 | -98.12 | NWS |  |
| 7 | KBKS | 27.21 | -98.12 | NWS |  |
| 8 | KMWL | 32.78 | -98.07 | FAA |  |
| 9 | KBAZ | 29.71 | -98.05 | FAA |  |
| 10 | KNOG | 27.89 | -98.04 | NWS |  |
| 11 | KEWX | 29.70 | -98.03 | NWS |  |
| 12 | KALI | 27.74 | -98.03 | FAA |  |
| 13 | KT65 | 26.18 | -97.97 | NWS |  |
| 14 | KHYI | 29.90 | -97.87 | NWS |  |
| 15 | KGRK | 31.07 | -97.83 | NWS |  |
| 16 KNQI | 27.50 | -97.82 | DODn |  |  |
| 17 | KGDJ | 32.43 | -97.82 | NWS |  |
| 18 | KGOP | 31.42 | -97.80 | NWS |  |
| 19 | KATT | 30.32 | -97.77 | NWS |  |
| 20 | KHLR | 31.15 | -97.72 | NWS |  |
| 21 | KRBO | 27.78 | -97.69 | NWS |  |
| 22 | KT53 | 27.78 | -97.69 | NWS |  |
| 23 | KAUS | 30.18 | -97.68 | FAA |  |
| 24 | KBSM | 30.20 | -97.68 | NWS |  |
| 25 | KGTU | 30.68 | -97.68 | NWS |  |
| 26 | KILE | 31.08 | -97.68 | NWS |  |
| 27 | KHRL | 26.23 | -97.66 | FAA |  |
| 28 | KCRP | 27.79 | -97.51 | NWS |  |
| 29 | KNGW | 27.72 | -97.45 | DODn |  |
| 30 | KNFW | 32.77 | -97.45 | DODn |  |
| 31 | KCPT | 32.35 | -97.43 | NWS |  |
| 32 | KBRO | 25.91 | -97.42 | NWS |  |
| 33 | KTPL | 31.15 | -97.40 | NWS |  |
| 34 | KPIL | 26.16 | -97.34 | FAA |  |
| 35 | KPWG | 31.48 | -97.32 | NWS |  |
| 36 | KNVT | 27.63 | -97.31 | NWS |  |
| 37 | KFWS | 32.56 | -97.31 | NWS |  |
| 38 | KFWD | 32.79 | -97.30 | NWS |  |
| 39 | KNGP | 27.68 | -97.28 | DODn |  |
| 40 | KACT | 31.62 | -97.22 | NWS |  |
| 41 | K5T5 | 32.08 | -97.10 | NWS |  |
| 42 | KRAS | 27.81 | -97.09 | NWS |  |
| 43 | KGKY | 32.66 | -97.09 | FAA |  |
| 44 | KINJ | 32.08 | -97.08 | NWS |  |
| 45 | KGPM | 32.70 | -97.05 | NWS |  |
| 46 | KRKP | 28.08 | -97.05 | FAA |  |
| 47 | KGYB | 30.17 | -96.98 | NWS |  |
| 48 | KNBE | 32.73 | -96.97 | NWS |  |
| 49 | K3T5 | 29.91 | -96.95 | NWS |  |
| 50 | KVCT | 28.86 | -96.93 | NWS |  |
| 51 | K4T6 | 32.46 | -96.91 | NWS |  |
| 52 | KJWY | 32.46 | -96.91 | NWS |  |
| 53 | KRBD | 32.68 | -96.87 | FAA |  |
| 54 | KLNC | 32.57 | -96.72 | NWS |  |
| 55 | KRWV | 30.52 | -96.70 | NWS |  |
| 56 | KPKV | 28.65 | -96.68 | NWS |  |
| 57 | KT97 | 28.65 | -96.68 | NWS |  |
| 58 | KLHB | 30.87 | -96.62 | NWS |  |
| 59 | KHQZ | 32.75 | -96.53 | NWS |  |
| 60 | KCRS | 32.03 | -96.40 | FAA |  |
| 61 | K11R | 30.22 | -96.37 | NWS |  |
| 62 | KCLL | 30.58 | -96.37 | FAA |  |
| 63 | KTRL | 32.71 | -96.27 | FAA |  |
| 64 | KPSX | 28.73 | -96.25 | FAA |  |
| 65 | KARM | 29.25 | -96.15 | NWS |  |
| 66 | KT46 | 28.02 | -95.87 | NWS |  |
| 67 | K3R1 | 28.97 | -95.86 | NWS |  |
| 68 | KBYY | 28.97 | -95.86 | NWS |  |
| 69 | KPSN | 31.77 | -95.70 | NWS |  |
| 70 | KSGR | 29.62 | -95.66 | FAA |  |
| 71 | KUTS | 30.74 | -95.59 | FAA |  |
| 72 | KDWH | 30.07 | -95.56 | FAA |  |
| 73 | KJDD | 32.74 | -95.50 | NWS |  |
| 74 | KLBX | 29.12 | -95.46 | FAA |  |
| 75 | KCXO | 30.36 | -95.41 | FAA |  |
| 76 | KXCO | 30.36 | -95.41 | NWS |  |
| 77 | KDKR | 31.31 | -95.40 | NWS |  |
| 78 | KTYR | 32.35 | -95.40 | FAA |  |
| 79 | KIAH | 29.99 | -95.36 | NWS |  |
| 80 | KHOU | 29.64 | -95.28 | FAA |  |
| 81 | KLVJ | 29.52 | -95.24 | FAA |  |
| 82 | KJSO | 31.87 | -95.22 | NWS |  |
| 83 | KEFD | 29.60 | -95.17 | NWS |  |
| 84 | KHGX | 29.47 | -95.08 | NWS |  |
| 85 | KJXI | 32.70 | -94.95 | NWS |  |
| # | Station | Lat | Lon | Provider |  |
| 86 | KGLS | 29.27 | -94.87 | FAA |  |
| 87 | KLFK | 31.24 | -94.75 | FAA |  |
| 88 | KOCH | 31.58 | -94.72 | NWS |  |
| 89 | K25T | 26.93 | -94.68 | NWS |  |
| 90 | KGGG | 32.35 | -94.65 | FAA |  |
| 91 | K01T | 28.13 | -94.40 | NWS |  |
| 92 | KBPT | 29.95 | -94.36 | NWS |  |
| 93 | KJAS | 30.89 | -94.03 | NWS |  |
| 94 | KLCH | 29.95 | -94.03 | NWS |  |
| 95 | KH39 | 27.95 | -93.98 | NWS |  |
| 96 | KRPE | 29.70 | -93.95 | NWS |  |
| 97 | KSHV | 32.45 | -93.82 | NWS |  |
| 98 | KORG | 30.07 | -93.80 | NWS |  |
| 99 | K5R0 | 28.22 | -93.75 | NWS |  |
| 100 | KDTN | 32.54 | -93.75 | FAA |  |
| 101 | KH08 | 28.47 | -93.73 | NWS |  |
| 102 | KBAD | 32.50 | -93.67 | NWS |  |
| 103 | KW60 | 28.15 | -93.35 | NWS |  |
| 104 | KDRI | 30.83 | -93.34 | NWS |  |
| 105 | K7R5 | 29.78 | -93.30 | NWS |  |
| 106 | KAQV | 31.40 | -93.28 | NWS |  |
| 107 | KPOE | 31.05 | -93.20 | NWS |  |
| 108 | K4C0 | 29.78 | -93.18 | NWS |  |
| 109 | KCWF | 30.22 | -93.15 | NWS |  |
| 110 | KIER | 31.73 | -93.10 | NWS |  |
| 111 | KDNK | 31.10 | -93.00 | NWS |  |
| 112 | KBKB | 31.00 | -92.97 | NWS |  |
| 113 | KE12 | 28.08 | -92.70 | NWS |  |
| 114 | KACP | 30.75 | -92.69 | NWS |  |
| 115 | K3B6 | 28.22 | -92.68 | NWS |  |
| 116 | KRSN | 32.51 | -92.59 | NWS |  |
| 117 | K01R | 31.13 | -92.57 | NWS |  |
| 118 | KAEX | 31.33 | -92.57 | FAA |  |
| 119 | KESF | 31.40 | -92.30 | FAA |  |
| 120 | K7R4 | 29.78 | -92.13 | NWS |  |
| 121 | KMLU | 32.52 | -92.03 | FAA |  |
| 122 | K7R8 | 28.30 | -91.98 | NWS |  |
| 123 | KARA | 30.03 | -91.89 | FAA |  |
| 124 | K2F8 | 32.76 | -91.88 | NWS |  |
| 125 | KSRN | 29.12 | -91.87 | NWS |  |
| 126 | K41I | 28.47 | -91.78 | NWS |  |
| 127 | KP92 | 29.55 | -91.53 | NWS |  |
| 128 | KXCN | 27.77 | -91.52 | NWS |  |
| 129 | K28K | 27.88 | -91.50 | NWS |  |
| 130 | KPTN | 29.72 | -91.33 | NWS |  |
| 131 | KS02 | 28.50 | -91.30 | NWS |  |
| 132 | KHEZ | 31.62 | -91.30 | NWS |  |
| 133 | KS65 | 28.50 | -91.20 | NWS |  |
| 134 | KBTR | 30.53 | -91.14 | NWS |  |
| 135 | K7R3 | 29.70 | -91.10 | NWS |  |
| 136 | KTVR | 32.35 | -91.03 | FAA |  |
| 137 | KGSM | 28.53 | -90.98 | NWS |  |
| 138 | KHUM | 29.57 | -90.67 | NWS |  |
| 139 | KS58 | 28.53 | -90.58 | NWS |  |
| 140 | KMCB | 31.18 | -90.47 | FAA |  |
| 141 | KHDC | 30.52 | -90.42 | NWS |  |
| 142 | K2GL | 29.42 | -90.30 | NWS |  |
| 143 | KGAO | 29.44 | -90.26 | NWS |  |
| 144 | KMSY | 29.99 | -90.25 | NWS |  |
| 145 | KHKS | 32.33 | -90.22 | FAA |  |
| 146 | K9F2 | 29.10 | -90.20 | NWS |  |
| 147 | KJAN | 32.32 | -90.08 | NWS |  |
| 148 | KNBG | 29.83 | -90.03 | DODn |  |
| 149 | KNEW | 30.05 | -90.03 | FAA |  |
| 150 | KAXO | 29.25 | -89.97 | NWS |  |
| 151 | KASD | 30.34 | -89.82 | FAA |  |
| 152 | KLIX | 30.33 | -89.82 | NWS |  |
| 153 | KHSA | 30.37 | -89.45 | NWS |  |
| 154 | K1B7 | 29.35 | -89.43 | NWS |  |
| 155 | K7R1 | 29.26 | -89.36 | FAA |  |
| 156 | KBVE | 29.26 | -89.36 | NWS |  |
| 157 | KPIB | 31.47 | -89.33 | NWS |  |
| 158 | KHBG | 31.28 | -89.25 | FAA |  |
| 159 | KGPT | 30.41 | -89.08 | FAA |  |
| 160 | K1G7 | 28.78 | -89.05 | NWS |  |
| 161 | KBIX | 30.42 | -88.92 | NWS |  |
| 162 | KMEI | 32.33 | -88.75 | NWS |  |
| 163 | KNMM | 32.55 | -88.57 | DODn |  |
| 164 | KPQL | 30.46 | -88.53 | FAA |  |
| 165 | KMOB | 30.69 | -88.25 | NWS |  |
| 166 | KRAM | 29.05 | -88.08 | NWS |  |
| 167 | KBFM | 30.62 | -88.06 | FAA |  |
| 168 | KJKA | 30.29 | -87.67 | NWS |  |
| 169 | KNBJ | 30.39 | -87.63 | NWS |  |

Station Location

RANDOLPH AFB TX

MCALLEN MILLER INTL ARPT, MCALLEN, TX

BURNET MUNI ARPT, BURNET, TX

STEPHENVILLE/CLARK TX

EDINBURG INTL ARPT, TX

EDINBURG INTL ARPT, TX

BROOKS CO ARPT, TX

MINERAL WELLS MUNI TX

NEW BRAUNFELS MUNI ARPT, NEW BRAUNFELS, TX

ORANGE GROVE NALF, TX

AUSTIN/SAN ANTONIO NEXRAD & SCD, TX

ALICE INTL ARPT, ALICE, TX

MID VALLEY ARPT, TX

SAN MARCOS (AWOS) TX

FORT HOOD/GRAY AAF TX

KINGSVILLE, KINGSVILLE, TX

GRANBURY MUNI ARPT, TX

GATESVILLE/CITY-CO ARPT, TX

AUSTIN CITY/CAMP MABRY, TX

FT HOOD AAF/KILLEEN TX

ROBSTOWN, NUECES CO ARPT, TX

NUECES CO ARPT, TX

AUSTIN-BERGSTROM INTL ARPT, TX

BERGSTROM AFB/AUSTI TX

GEORGETOWN (AWOS) TX

KILLEEN MUNI (AWOS) TX

HARLINGEN RIO GRANDE VALLEY ARPT, TX

CORPUS CHRISTI INTL ARPT, CORPUS CHRISTI, TX

CABANISS NAVY AUXILIARY LANDING FLD,,TX

FORT WORTH NAS TX

CLEBURNE MUNI ARPT, TX

BROWNSVILLE S PADRE ISL INTL ARPT, TX

TEMPLE/MILLER (AWOS) TX

PORT ISABEL CAMERON CO ARPT, PORT ISABEL, TX

MC GREGOR (AWOS) TX

CORPUS CHRISTI/WALDRON FLD, TX

FORT WORTH SPINKS/DFW NEXRAD, TX

FORT WORTH, TX

CORPUS CHRISTI NAVAL AIR STATION, TX

WACO-MADISON COOPER TX

HILLSBORO MUNI ARPT, TX

MUSTANG BEACH ARPT, TX

ARLINGTON MUNI ARPT, TX

HILLSBORO MUNI ARPT, TX

GRAND PRAIRIE, TX

ROCKPORT ARANSAS MUNI ARPT, ROCKPORT, TX

GIDDINGS-LEE CO ARPT, TX

DALLAS NAS/HENSLEY TX

FAYETTE RGNL AIR CTR ARPT, TX

VICTORIA RGNL ARPT, VICTORIA, TX

MID-WAY RGNL ARPT, TX

MID-WAY RGNL ARPT, TX

DALLAS/REDBIRD ARPT TX

LANCASTER ARPT, TX

CALDWELL MUNI ARPT, TX

CALHOUN CO ARPT, TX

CALHOUN CO ARPT, TX

HEARNE MUNI ARPT, TX

MESQUITE METRO ARPT, TX

CORSICANA/CAMPBELL FLD, TX

BRENHAM/BRENHAM MUNI APT, TX

COLLEGE STATION TX

TERRELL MUNI ARPT, TX

PALACIOS MUNI TX

WHARTON RGNL ARPT, TX

SOUTH BRAZOS A70 TX

BAY CITY MUNI ARPT, TX

BAY CITY MUNI ARPT, TX

PALESTINE MUNI ARPT, TX

HOUSTON/HULL FLD, TX

HUNTSVILLE MUNI ARPT, HUNTSVILLE, AL

HOUSTON HOOKS MEMORIAL ARPT, HOUSTON, TX

WOOD CO ARPT, TX

ANGLETON / LAKE JACKSON BRAZORI ARPT,, TX

CONROE MONTGOMERY CO ARPT, CONROE, AL

CONROE ARPT, TX

CROCKET/HOUSTON CO ARPT, TX

TYLER/POUNDS FLD TX

HOUSTON BUSH INTERCONTINENTAL ARPT, TX

HOUSTON WILLIAM P HOBBY ARPT, HOUSTON, TX

HOUSTON CLOVER FLD, HOUSTON, TX

CHEROKEE CO ARPT, TX

HOUSTON/ELLINGTON TX

HOUSTON/GALVESTON NEXRAD & SCD, TX

FOX STEPHENS FLD/GILMER MUNI, TX

Station Location

GALVESTON SCHOLES ARPT, GALVESTON, TX

LUFKIN ANGELINA CTY ARPT, LUFKIN, TX

NACOGDOCHES (AWOS) TX

PHILLIPS OIL PLATFORM, TX

LONGVIEW/GREGG CO. TX

HIGH ISLAND LA

PORT ARTHUR JEFFERSON CO, BEAUMONT, TX

JASPER CO/BELL FLD ARPT, TX

LAKE CHARLES RGNL ARPT, LAKE CHARLES, LA

HIGH ISLAND A572C LA

SABINE PASS TX

SHREVEPORT RGNL ARPT, SHREVEPORT, LA

ORANGE CO ARPT, TX

EAST ADDITION B323 TX

SHREVEPORT DOWNTOWN ARPT, SHREVEPORT, LA

HIGH ISLAND 264C (SAWRS), LA

BARKSDALE AFB LA

WEST CAMERON 560, LA

BEAUREGARD RGNL ARPT, LA

CAMERON HELIPORT LA

PEASON RIDGE FT POL, LA

FORT POLK (ARMY) LA

OIL PLATFORM, LA

CHENAULT AIRPARK LA

NATCHITOCHES, LA

SELF LANDING STRIP, LA

FULLERTON LANDING, LA

EAST CAMERON 345 (SAWRS), LA

ALLEN PARISH ARPT, LA

EUGENE ISLAND 330 (SAWRS), LA

RUSTON RGNL ARPT, LA

CLAIBORNE RNG (AFS) LA

ALEXANDRIA INTL ARPT, ALEXANDRIA, LA

ALEXANDRIA ESLER RGNL, ALEXANDRIA, LA

INTRACOASTAL CITY LA

MONROE RGNL LA

SOUTH MARSH ISLAND LA

NEW IBERIA ACADIANA RGNL, NEW IBERIA, LA

MOREHOUSE MEM ARPT/BASTROP LA US

SOUTH MARSH 268A (SAWRS), FL

EUGENE ISLAND B266C LA

SALT POINT, SALT POINT, LA

GREEN CANYON 184 (SAWRS), LA

GRNCYN BLK52 (SAWRS) LA

PATTERSON MEMORIAL LA

SHIP SHOAL B224A LA

NATCHEZ/HARDY (AWOS) MS

SHIP SHOAL 198G LA

BATON ROUGE RYAN ARPT, BATON ROUGE, LA

AMELIA/LAKE PALOURD LA

VICKSBURG/TALLULAH RGNL (ASOS), MS

SHIP SHOAL 207A (SAWRS), LA

HOUMA-TERREBONNE LA

SOUTH TIMBALIER LA

MCCOMB PIKE CO ARPT, MCCOMB, MS

HAMMOND MUNI ARPT, LA

GALLIANO HELIPORT L, LA

GALLIANO/SOUTH LAFOURCHE ARPT, LA

NEW ORLEANS INTL ARPT, NEW ORLEANS, LA

JACKSON HAWKINS FLD JACKSON, MS

FOURCHON (SAWRS) LA

JACKSON/THOMPSON MS

NEW ORLEANS NAS LA

NEW ORLEANS LAKEFRONT ARPT, NEW ORLEANS, LA

GRAND ISLE (SAWRS) LA

SLIDELL ARPT, SLIDELL, LA

SLIDELL WSFO & NEXRAD & SCD, LA

STENNIS INTL ARPT, MS

BOOTHVILLE HELIPORT, LA

VENICE PHI HELIPORT LA

VENICE PHI HELIPORT, VENICE, LA

PINE BELT RGNL AWOS MS

HATTIESBURG CHAIN MUNI A HATTIESBURG, MS

GULFPORT - BILOXI RGNL ARPT, GULFPORT, MS

MISSISSIPPI CANYON LA

KEESLER AFB/BILOXI MS

MERIDIAN/KEY FLD MS

MERIDIAN NAS/MCCAIN MS

PASCAGOULA LOTT INTL ARPT, PASCAGOULA, MS

MOBILE RGNL ARPT, MOBILE, AL

PHILLIPS PLATFORM, AL

MOBILE DOWNTOWN ARPT, MOBILE, AL

JACK EDWARDS ARPT/GULF SHORES AL US

BARIN FLD NAF, AL

|  | # | Station | Lat | Lon | Height |  |
| --- | --- | --- | --- | --- | --- | --- |
| Station Location | 1 | KNDZ | 30.70 | -87.02 | 10 (m) |  |
| MILTON/WHITING FIELD S | 2 | KNSE | 30.72 | -87.02 | 10 |  |
| MILTON/WHITING FIELD NAS-N. | 3 | KNFJ | 30.51 | -86.95 | 10 |  |
| MILTON/PENSACOLA NALF | 4 | KHRT | 30.43 | -86.68 | 10 |  |
| HURLBURT FLD (AF) | 5 | KQBL | 30.42 | -86.68 | 10 |  |
| AFCWC NTFS | 6 | KVPS | 30.48 | -86.53 | 10 |  |
| EGLIN AFB | 7 | KEGI | 30.65 | -86.52 | 10 |  |
| DUKE FLD/EGLIN AUX | 8 | KCEW | 30.78 | -86.52 | 10 |  |
| BOB SIKES ARPT, CRESTVIEW | 9 | KDTS | 30.39 | -86.47 | 10 |  |
| FT. WALTON BCH ARPT, DESTIN | 10 | KPFN | 30.21 | -85.69 | 10 |  |
| BAY CO ARPT, PANAMA CITY | 11 | KPAM | 30.07 | -85.58 | 10 |  |
| TYNDALL AFB | 12 | KMAI | 30.84 | -85.18 | 10 |  |
| MARIANNA MUNI ARPT | 13 | KAAF | 29.73 | -85.03 | 9 |  |
| APALACHICOLA ARPT | 14 | KTLH | 30.39 | -84.35 | 10 |  |
| TALLAHASSEE RGNL ARPT | 15 | K40J | 30.07 | -83.57 | 10 |  |
| PERRY-FOLEY ARPT, | 16 | KCTY | 29.63 | -83.11 | 10 |  |
| CROSS CITY ARPT, CROSS CITY | 17 | KPIE | 27.91 | -82.69 | 10 |  |
| ST. PETERSBURG ARPT | 18 | KSPG | 27.77 | -82.63 | 8 |  |
| ALBERT-WHITTED ARPT | 19 | KRRF | 28.18 | -82.62 | 10 |  |
| PORT RICHEY (ASOS) | 20 | KSRQ | 27.40 | -82.56 | 10 |  |
| SARASOTA-BRADENTON ARPT | 21 | KTPA | 27.96 | -82.54 | 8 |  |
| TAMPA INTL ARPT, TAMPA | 22 | KMCF | 27.85 | -82.52 | 10 |  |
| MACDILL AFB, TAMPA | 23 | KBKV | 28.47 | -82.45 | 10 |  |
| HERNANDO CO. ARPT | 24 | KTPF | 27.92 | -82.45 | 10 |  |
| PETER O KNIGHT ARPT/TAMPA | 25 | KTBW | 27.70 | -82.40 | 10 |  |
| TAMPA BAY FL | 26 | KVDF | 28.01 | -82.35 | 10 |  |
| TAMPA VANDENBURG ARPT | 27 | KGNV | 29.69 | -82.28 | 10 |  |
| GAINESVILLE RGNL ARPT | 28 | KOCF | 29.17 | -82.22 | 10 |  |
| OCALA_MUNI_(AWOS) | 29 | KPCM | 28.00 | -82.16 | 10 |  |
| PLANT CITY MUNI ARPT | 30 | KLAL | 27.98 | -82.02 | 10 |  |
| LAKELAND RGNL | 31 | KPGD | 26.92 | -81.99 | 10 |  |
| PUNTA GORDA ARPT | 32 | KVVG | 28.97 | -81.97 | 10 |  |
| THE VILLAGES ,FL | 33 | KNZC | 30.22 | -81.88 | 10 |  |
| CECIL FLD NAS FL | 34 | KVQQ | 30.22 | -81.88 | 10 |  |
| CECIL FLD NAS,FL | 35 | KFMY | 26.59 | -81.86 | 8 |  |
| FT MYERS PAGE FIELD | 36 | KLEE | 28.82 | -81.81 | 10 |  |
| LEESBURG MUNI ARPT | 37 | KBOW | 27.95 | -81.78 | 10 |  |
| BARTOW MUNI ARPT | 38 | KAPF | 26.15 | -81.77 | 10 |  |
| NAPLES MUNI ARPT | 39 | KRSW | 26.53 | -81.77 | 10 |  |
| FT. MYERS SW RGNL ARPT | 40 | KGIF | 28.06 | -81.76 | 10 |  |
| WINTER HAVEN'S GILBERT ARPT |  |  |  |  |  |  |
|  | # | Station | Lat | Lon | Height |  |
| Station Location | 41 | KEYW | 24.55 | -81.75 | 10 (m) |  |
| KEY WEST INTL ARPT | 42 | KJAX | 30.50 | -81.69 | 10 |  |
| JACKSONVILLE INTL ARPT | 43 | KNIP | 30.23 | -81.68 | 10 |  |
| JACKSONVILLE NAS | 44 | KNQX | 24.57 | -81.68 | 10 |  |
| KEY WEST NAS | 45 | KNAE | 29.14 | -81.63 | 10 |  |
| ASTOR/BOMBING RANGE DET | 46 | K90J | 29.12 | -81.57 | 10 |  |
| ASTOR NAS | 47 | KCRG | 30.34 | -81.51 | 8 |  |
| JACKSONVILLE CRAIG MUNI ARPT | 48 | KISM | 28.29 | -81.44 | 10 |  |
| KISSIMMEE MUNI ARPT | 49 | KNRB | 30.40 | -81.42 | 10 |  |
| MAYPORT NS | 50 | KSGJ | 29.96 | -81.34 | 10 |  |
| ST AUGUSTINE ARPT | 51 | KORL | 28.55 | -81.34 | 10 |  |
| ORLANDO EXECUTIVE ARPT | 52 | KMCO | 28.42 | -81.33 | 8 |  |
| ORLANDO INTL ARPT, ORLANDO | 53 | KSFB | 28.78 | -81.24 | 10 |  |
| ORLANDO SANFORD ARPT | 54 | KOMN | 29.30 | -81.11 | 10 |  |
| ORMOND BCH MUNI ARPT | 55 | KDAB | 29.17 | -81.07 | 10 |  |
| DAYTONA BCH INTL ARPT | 56 | KMTH | 24.73 | -81.05 | 10 |  |
| MARATHON ARPT | 57 | KEVB | 29.06 | -80.95 | 10 |  |
| NEW SMYRNA BCH MUNI ARPT | 58 | KTIX | 28.52 | -80.80 | 10 |  |
| TITUSVILLE | 59 | KTTS | 28.62 | -80.72 | 10 |  |
| NASA SHUTTLE FCLTY | 60 | KMLB | 28.10 | -80.64 | 10 |  |
| MELBOURNE INTL ARPT | 61 | KCOF | 28.23 | -80.61 | 10 |  |
| COCOA BCH PATRICK AFB | 62 | KTMB | 25.64 | -80.44 | 10 |  |
| MIA/KENDALL-TAMIAMI ARPT | 63 | KVRB | 27.66 | -80.41 | 10 |  |
| VERO BEACH MUNI ARPT | 64 | KA | 25.61 | -80.41 | 10 |  |
| MIAMI NEXRAD | 65 | KHST | 25.48 | -80.38 | 10 |  |
| HOMESTEAD AFB | 66 | KFPR | 27.50 | -80.38 | 8 |  |
| FT. PIERCE/ST. LUCIE CO. INTL | 67 | KMIA | 25.78 | -80.32 | 10 |  |
| MIAMI INTL ARPT, MIAMI | 68 | KOPF | 25.91 | -80.28 | 10 |  |
| OPA LOCKA ARPT, MIAMI | 69 | KHWO | 26.00 | -80.23 | 10 |  |
| HOLLYWOOD N PERRY ARPT (ASOS) | 70 | KSUA | 27.18 | -80.22 | 10 |  |
| STUART/WITHAM FLD ARPT | 71 | KFXE | 26.20 | -80.13 | 10 |  |
| FT. LAUDERDALE EXECUTIVE ARPT | 72 | KFLL | 26.07 | -80.15 | 10 |  |
| FT LAUD/HOLLYWOOD INTL ART | 73 | KPBI | 26.68 | -80.12 | 10 |  |
| WEST PALM BCH | 74 | KPMP | 26.25 | -80.12 | 10 |  |
| POMPANO BEACH | 75 | KBCT | 26.38 | -80.11 | 10 |  |
| BOCA RATON ARPT | 76 | MYBS | 25.73 | -79.30 | 10 |  |
| ALICE TOWN/S BIMINI, BA | 77 | MYGW | 26.70 | -78.97 | 10 |  |
| WEST END INTL ARPT, BA | 78 | MYGF | 26.55 | -78.70 | 10 |  |
| FREEPORT INTL ARPT, BA | 79 | MYNN | 25.05 | -77.47 | 10 |  |

B-3. ASOS/METAR Platform Locations (cont’d)

Provider

NWS

NWS

NWS

NWS

NWS

NWS

NWS

FAA

FAA

FAA

NWS

FAA

NWS

NWS

NWS

NWS

FAA

FAA

NWS

FAA

NWS

NWS

FAA

NWS

NWS

NWS

FAA

NWS

NWS

NWS

FAA

NWS

DODn

NWS

FAA

FAA

NWS

NWS

FAA

FAA

Provider

NWS

NWS

DODn

DODn

NWS

NWS

FAA

NWS

DODn

NWS

FAA

NWS

FAA

NWS

NWS

FAA

NWS

NWS

NWS

FAA

DODa

FAA

FAA

NWS

NWS

FAA

NWS

FAA

FAA

NWS

FAA

FAA

NWS

FAA

NWS

INTL

INTL

INTL

INTL

NASSAU INTL ARPT, BA

|  |  | # | Station | Lat |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Provider | Station Location |  | 1 | KCKL | 32.90 |  |
| NWS | CENTREVILLE/BIBB CO AL |  | 2 | KGZH | 31.42 |  |
| FAA | EVERGREEN MIDDLETON FLD, EVERGREEN, AL |  | 3 | KDCU | 34.66 |  |
| FAA | DECATUR PRYOR FLD, DECATUR, AL |  | 4 | K3A1 | 34.27 |  |
| NWS | FOLSOM FLD ARPT, AL |  | 5 | KHSV | 34.64 |  |
| NWS | HUNTSVILLE INTL/ JONES FLD, HUNTSVILLE, AL |  | 6 | KEET | 33.18 |  |
| FAA | ALABASTER SHELBY CO ARPT, ALABASTER, AL |  | 7 | KBMX | 33.17 |  |
| NWS | BIRMINGHAM NEXRAD, AL |  | 8 | KBHM | 33.57 |  |
| FAA | BIRMINGHAM INTL ARPT, BIRMINGHAM, AL |  | 9 | KHUA | 34.68 |  |
| NWS | REDSTONE ARMY AIRFLD, AL |  | 10 | KM82 | 34.86 |  |
| NWS | MADISON CO EXEC ARPT, AL |  | 11 | KMDQ | 34.86 |  |
| NWS | MADISON CO EXEC ARPT, AL |  | 12 | KMQY | 36.02 |  |
| NWS | SMYRNA, TN |  | 13 | KMGM | 32.30 |  |
| NWS | MONTGOMERY DANNELLY ARPT, AL |  | 14 | K79J | 31.32 |  |
| NWS | ANDALUSIA/OPP ARPT AL |  | 15 | KMXF | 32.38 |  |
| NWS | MAXWELL AFB/MONTGOM AL |  | 16 | K8A0 | 34.23 |  |
| NWS | ALBERTVILLE MUNI ARPT, AL |  | 17 | KGAD | 33.97 |  |
| NWS | GADSEN MUNI (AWOS) AL |  | 18 | KALX | 32.91 |  |
| NWS | THOMAS C RUSSELL FLD ARPT, AL |  | 19 | KANB | 33.59 |  |
| FAA | ANNISTON METRO ARPT, ANNISTON, AL |  | 20 | KSXS | 31.35 |  |
| NWS | SCHELL AHP AL |  | 21 | K4A9 | 34.47 |  |
| NWS | ISBELL FLD ARPT, AL |  | 22 | KOZR | 31.28 |  |
| NWS | CAIRNS AAF/OZARK AL |  | 23 | KAUB | 32.60 |  |
| NWS | AUBURN UNIV. (AMOS) AL |  | 24 | KDHN | 31.32 |  |
| FAA | DOTHAN RGNL ARPT, DOTHAN, AL |  | 25 | KAUO | 32.62 |  |
| NWS | AUBURN-OPELIKA APT AL |  | 26 | KCHA | 35.03 |  |
| NWS | CHATTANOOGA/LOVELL, TN |  | 27 | KFFC | 34.35 |  |
| FAA | PEACHTREE CITY FLACON FLD, ATLANTA, GA |  | 28 | KRMG | 34.34 |  |
| NWS | ROME RB RUSSELL ARPT, ROME, GA |  | 29 | KCSV | 35.95 |  |
| FAA | CROSSVILLE MEMORIAL, TN |  | 30 | KLGC | 33.01 |  |
| NWS | LA GRANGE/CALLAWAY ARPT, GA |  | 31 | KLSF | 32.33 |  |
| NWS | FORT BENNING GA |  | 32 | KCSG | 32.52 |  |
| NWS | COLUMBUS METRO ARPT GA |  | 33 | K11J | 31.40 |  |
| NWS | EARLY CO ARPT, GA |  | 34 | KBIJ | 31.40 |  |
| NWS | EARLY CO ARPT GA |  | 35 | KDNN | 34.72 |  |
| NWS | DALTON MUNI ARPT, GA |  | 36 | KVPC | 34.12 |  |
| FAA | CARTERSVILLE ARPT, GA |  | 37 | KCCO | 33.31 |  |
| NWS | NEWNAN COWETA ARPT, GA |  | 38 | KBGE | 30.97 |  |
| NWS | DECATUR CO INDUS AIR PARK, GA |  | 39 | KRYY | 34.01 |  |
| NWS | MARIETTA/COBB CO ARPT, GA |  | 40 | KMGE | 33.92 |  |
| NWS | DOBBINS AFB/MARIETT GA |  | 41 | KATL | 33.62 |  |
| NWS | ATLANTA HARTSFLD INTL ARPT, ATLANTA, GA |  | 42 | K47A | 34.31 |  |
| NWS | CHEROKEE CO ARPT, GA |  | 43 | KPDK | 33.87 |  |
| FAA | ATLANTA DEKLAB-PEACHTREE ARPT, GA |  | 44 | KOPN | 32.95 |  |
| NWS | THOMASTON-UPSON CO ARPT, GA |  | 45 | KOQT | 36.02 |  |
| NWS | OAK RIDGE |  | 46 | KABY | 31.54 |  |
| FAA | ALBANY SW GA RGNL ARPT, ALBANY, GA |  | 47 | KMRX | 35.82 |  |
| NWS | KNOXVILLE |  | 48 | KTYS | 35.82 |  |
| NWS | KNOXVILLE MUNI |  | 49 | KLZU | 33.98 |  |
| NWS | LAWRENCEVILLE/GWINNETT CO, GA |  | 50 | K6A3 | 35.19 |  |
| NWS | ANDREWS, NC |  | 51 | KRHP | 35.19 |  |
| NWS | ANDREWS-MURPHY ARPT, NC |  | 52 | KGVL | 34.27 |  |
| FAA | GAINESVILLE LEE GLIMER MEM ARPT, GA |  | 53 | KMGR | 31.08 |  |
| NWS | MOULTRIE MUNI ARPT, GA |  | 54 | KWDR | 33.98 |  |
| NWS | WINDER-BARROW ARPT, GA |  | 55 | KMCN | 32.68 |  |
| NWS | MACON MIDDLE RGNL ARPT, MACON, GA |  | 56 | KWRB | 32.63 |  |
| NWS | WARNER ROBINS AFB GA |  | 57 | K1A5 | 35.22 |  |
| NWS | MACON CO ARPT, NC |  | 58 | KAHN | 33.95 |  |
| NWS | ATHENS BEN EPPS ARPT, ATHENS, GA |  | 59 | KVLD | 30.78 |  |
| FAA | VALDOSTA RGNL ARPT, VALDOSTA, GA |  | 60 | KFZG | 31.68 |  |
| NWS | FITZGERALD MUNI ARPT, GA |  | 61 | KMLJ | 33.14 |  |
| NWS | MILLEDGEVILLE/BALDWIN CO ARPT, GA |  | 62 | KVAD | 30.97 |  |
| NWS | MOODY AFB/VALDOSTA GA |  | 63 | K3J7 | 33.60 |  |
| NWS | GREENE CO RGNL ARPT, GA |  | 64 | KDBN | 32.57 |  |
| NWS | DUBLIN/W H BUD BARRON ARPT, GA |  | 65 | KCEU | 34.67 |  |
| FAA | CLEMSON-OCONEE CO ARPT, CLEMSON, SC |  | 66 | KDQH | 31.47 |  |
| NWS | DOUGLAS MUNI ARPT, GA |  | 67 | K27A | 34.10 |  |
| NWS | ELBERT CO ARPT/PATZ FLD, GA |  | 68 | KIIY | 33.78 |  |
| NWS | WASHINGTON-WILKES CO ARPT, GA |  | 69 | KAND | 34.50 |  |
| FAA | ANDERSON CO ARPT, ANDERSON, SC |  | 70 | KAVL | 35.43 |  |
| NWS | ASHVILLE RGNL ARPT, ASHVILLE, NC |  | 71 | KHQU | 33.53 |  |
| NWS | THOMSON, GA |  | 72 | KAMG | 31.54 |  |
| FAA | ALMA BACON CO ARPT, ALMA, GA |  | 73 | KAYS | 31.25 |  |
| NWS | WAYCROSS/WARE CO. GA |  | 74 | KBHC | 31.71 |  |
| NWS | BAXLEY MUNI ARPT, GA |  | 75 | KGYH | 34.76 |  |
| NWS | DONALDSON CENTER ARPT, SC |  | 76 | KVDI | 32.19 |  |
| NWS | VIDALIA MUNI APRT, GA |  | 77 | KGSP | 34.91 |  |
| NWS | GREER GREENVILLE-SPARTANBRG ARPT, SC |  | 78 | KGRD | 34.25 |  |
| FAA | GREENWOOD CO ARPT, GREENWOOD, SC |  | 79 | KDNL | 33.47 |  |
| FAA | AUGUSTA DANIEL FLD ARPT, AGUSTA, GA |  | 80 | KAGS | 33.37 |  |
| NWS | AUGUSTA BUSH FLD, AUGUSTA, GA |  | 81 | KFQD | 35.43 |  |
| NWS | RUTHERFORDTON/MARCHMAN FLD, NC |  | 82 | KTBR | 32.48 |  |
| NWS | STATESBORO-BULLOCH CO ARPT, GA |  | 83 | KMRN | 35.82 |  |
| NWS | MORGANTON-LENOIR ARPT, NC |  | # | Station | Lat |  |
| Provider | Station Location |  | 84 | KEHO | 35.25 |  |
| NWS | SHELBY MUNI ARPT, NC |  | 85 | KJYL | 32.65 |  |
| NWS | SYLVANIA/PLANTATION AIRPARK, GA |  | 86 | KLHW | 31.88 |  |
| NWS | FT STEWART/WRIGHT GA |  | 87 | KNBQ | 30.79 |  |
| NWS | KINGS BAY NAVAL STN, GA |  | 88 | KBQK | 31.25 |  |
| NWS | BRUNSWICK/GLYNCO GA |  | 89 | KSSI | 31.15 |  |
| FAA | BRUNSWICK MALCOLM MCKINNON ARPT, GA |  | 90 | KHKY | 35.74 |  |
| FAA | HICKORY RGNL ARPT, HICKORY, NC |  | 91 | KSAV | 32.13 |  |
| NWS | SAVANNAH INTL ARPT, SAVANNAH, GA |  | 92 | KIPJ | 35.48 |  |
| NWS | LINCOLNTON NC |  | 93 | KAKH | 35.20 |  |
| FAA | GASTONIA MUNI ARPT, GASTONIA, NC |  | 94 | KSVN | 32.02 |  |
| NWS | HUNTER (AAF) GA |  | 95 | K29J | 34.98 |  |
| NWS | ROCK HILL, SC |  | 96 | KUZA | 34.98 |  |
| FAA | ROCK HILL YORK CO ARPT, ROCK HILL, SC |  | 97 | KTOI | 31.96 |  |
| FAA | TROY MUNI ARPT, TROY, AL |  | 98 | KCUB | 33.97 |  |
| FAA | COLUMBIA OWENS FLD ARPT, COLUMBIA, SC |  | 99 | KSVH | 35.77 |  |
| NWS | STATESVILLE MUNI ARPT, NC |  | 100 | KCLT | 35.22 |  |
| NWS | CHARLOTTE DOUGLAS INTL ARPT, NC |  | 101 | KGMU | 35.22 |  |
| FAA | GREENVILLE DOWNTOWN ARPT, GREENVILLE, SC |  | 102 | KOGB | 33.46 |  |
| FAA | ORANGEBURG ARPT, ORANGEBURG, SC |  | 103 | KMMT | 33.92 |  |
| NWS | MCENTIRE ANG BASE SC |  | 104 | KNBC | 32.48 |  |
| NWS | BEAUFORT MCAS SC |  | 105 | KJQF | 35.38 |  |
| NWS | CONCORD NC |  | 106 | KHXD | 32.22 |  |
| NWS | HILTON HEAD (AWOS) SC |  | 107 | KEQY | 33.01 |  |
| FAA | MONROE ARPT, MONROE, NC |  | 108 | KRUQ | 35.65 |  |
| NWS | SALISBURY/ROWAN CO ARPT, NC |  | 109 | KFTY | 33.77 |  |
| FAA | ATLANTA FULTON CO ARPT, ATLANTA, GA |  | 110 | KSSC | 33.97 |  |
| NWS | SHAW AFB/SUMTER SC |  | 111 | KEXX | 35.78 |  |
| NWS | LEXINGTON DAVIDSON CO ARPT, NC |  | 112 | KVUJ | 35.42 |  |
| NWS | ALBEMARLE/STANLY CO ARPT, NC |  | 113 | KCAE | 33.94 |  |
| NWS | COLUMBIA METROPOLITAN ARPT, COLUMBIA, SC |  | 114 | KIGC | 32.90 |  |
| NWS | CHARLESTON AFB SC |  | 115 | KCHS | 32.90 |  |
| NWS | CHARLESTON INTL ARPT, CHARLESTON, SC |  | 116 | KNEX | 32.92 |  |
| NWS | NISE CHARLESTON SC |  | 117 | KHBI | 35.65 |  |
| NWS | ASHEBORO MUNI ARPT, NC |  | 118 | KW44 | 35.65 |  |
| NWS | ASHEBORO MUNI ARPT, NC |  | 119 | KUDG | 34.45 |  |
| NWS | DARLINGTON, SC |  | 120 | K45J | 34.89 |  |
| NWS | ROCKINGHAM-HAMLET ARPT, NC |  | 121 | KFLO | 34.19 |  |
| FAA | FLORENCE RGNL ARPT, FLORENCE, SC |  | 122 | KHFF | 35.03 |  |
| NWS | MACKALL AAF NC |  | 123 | KSOP | 35.23 |  |
| NWS | SOUTHERN PINES AWOS NC |  | 124 | KMEB | 34.78 |  |
| FAA | MAXTON/LAURINBURG MAXTON ARPT, NC |  | 125 | KTTA | 35.58 |  |
| NWS | SANFORD-LEE CO RGNL ARPT, NC |  | 126 | KIGX | 35.94 |  |
| FAA | CHAPEL HILL WILLIAMS ARPT, CHAPEL HILL, NC |  | 127 | KLBT | 34.61 |  |
| FAA | LUMBERTON MUNI ARPT, LUMBERTON, NC |  | 128 | KPOB | 35.17 |  |
| NWS | POPE AFB NC |  | 129 | KFBG | 35.13 |  |
| NWS | FORT BRAGG/SIMMONS NC |  | 130 | KMYR | 33.68 |  |
| NWS | MYRTLE BEACH (CIV) SC |  | 131 | KFAY | 34.98 |  |
| FAA | FAYETTEVILLE RGNL NC |  | 132 | KRDU | 35.89 |  |
| NWS | RALEIGH DURHAM INTL, RALEIGH, NC |  | 133 | K37W | 35.38 |  |
| NWS | ERWIN/HARNETT CO ARPT, NC |  | 134 | KHRJ | 35.38 |  |
| NWS | ERWIN/HARNETT CO ARPT, NC |  | 135 | KCRE | 33.82 |  |
| FAA | N. MYRTLE BCH GRAND STRAND ARPT, SC |  | 136 | KCPC | 34.27 |  |
| NWS | COLUMBUS CO MUNI ARPT, NC |  | 137 | KEYF | 34.60 |  |
| NWS | CURTIS L BROWN JR FLD ARPT, NC |  | 138 | KRAX | 35.67 |  |
| NWS | RALEIGH/DURHAM NEXRAD, NC |  | 139 | KJNX | 35.54 |  |
| NWS | SMITHFLD/JOHNSTON CO ARPT, NC |  | 140 | KCTZ | 34.98 |  |
| NWS | SAMPSON CO ARPT/CLINTON, NC |  | 141 | KLHZ | 36.02 |  |
| NWS | LOUISBURG/FRANKLIN CO ARPT, NC |  | 142 | KSUT | 33.93 |  |
| NWS | OAK ISL/BRUNSWICK CO ARPT, NC |  | 143 | KDPL | 35.00 |  |
| NWS | KENANSVILLE/DUPLIN CO, NC |  | 144 | KGSB | 35.33 |  |
| NWS | SEYMOUR-JOHNSON AFB NC |  | 145 | KGWW | 35.46 |  |
| NWS | GOLDSBORO-WAYNE MUNI ARPT, NC |  | 146 | KILM | 34.27 |  |
| NWS | WILMINGTON INTL ARPT, WILIMINGTON, NC |  | 147 | KRWI | 35.85 |  |
| FAA | ROCKY MOUNT WILSON ARPT, ROCKY MOUNT, NC |  | 148 | KISO | 35.33 |  |
| NWS | KINSTON/STALLINGS NC |  | 149 | KOAJ | 34.83 |  |
| NWS | JACKSONVILLE (AWOS) NC |  | 150 | KNCA | 34.72 |  |
| NWS | NEW RIVER MCAS NC |  | 151 | KPGV | 35.63 |  |
| NWS | PITT-GREENVILLE ARP NC |  | 152 | KOCW | 35.57 |  |
| NWS | WASHINGTON/WARREN FLD APRT, NC |  | 153 | KEWN | 35.07 |  |
| FAA | NEW BERN CRAVEN RGNL ARPT, NEW BERN, NC |  | 154 | KNJM | 34.68 |  |
| NWS | BOGUE FLD MCALF NC |  | 155 | KMHX | 34.78 |  |
| NWS | NEWPORT NC |  | 156 | KNKT | 34.90 |  |
| DODn | CHERRY POINT MCAS NC |  | 157 | KNIS | 34.89 |  |
| NWS | CHERRY POINT MCAS, NC |  | 158 | KMRH | 34.73 |  |
| FAA | BEAUFORT M SMITH FLD ARPT, BEAUFORT, NC |  | 159 | KEDE | 36.03 |  |
| NWS | EDENTON/NORTHEASTERN ARPT, NC |  | 160 | KBUY | 36.05 |  |
| FAA | BURLINGTON ALAMANCE RNGL ARPT, NC |  | 161 | KNBT | 35.02 |  |
| NWS | PINEY ISLAND BT-11 BOMBING RG, NC |  | 162 | KMQI | 35.92 |  |
| NWS | MANTEO/DARE CO RGNL NC |  | 163 | KFFA | 36.02 |  |
| NWS | KILL DEVIL HILLS/FIRST FLT ARPT, NC |  | 164 | KHSE | 35.23 |  |
| NWS | HATTERAS BILLY MITCHELL FLD, NC |  | 165 | KHAT | 35.27 |  |

B-3. ASOS/METAR Platform Locations (cont’d)

Lon

-87.25

-87.05

-86.94

-86.86

-86.77

-86.78

-86.77

-86.75

-86.68

-86.56

-86.56

-86.52

-86.41

-86.40

-86.37

-86.26

-86.08

-85.96

-85.86

-85.85

-85.72

-85.72

-85.50

-85.45

-85.43

-85.20

-85.15

-85.15

-85.08

-85.07

-85.00

-84.93

-84.90

-84.89

-84.87

-84.85

-84.77

-84.64

-84.60

-84.52

-84.44

-84.42

-84.27

-84.26

-84.23

-84.20

-83.98

-83.98

-83.96

-83.86

-83.86

-83.82

-83.80

-83.67

-83.65

-83.60

-83.42

-83.33

-83.27

-83.27

-83.24

-83.20

-83.14

-82.99

-82.88

-82.85

-82.82

-82.82

-82.71

-82.54

-82.52

-82.51

-82.40

-82.39

-82.38

-82.37

-82.21

-82.15

-82.04

-81.96

-81.94

-81.74

-81.61

Lon

-81.60

-81.60

-81.57

-81.56

-81.47

-81.39

-81.38

-81.20

-81.16

-81.16

-81.15

-81.06

-81.06

-81.01

-80.99

-80.96

-80.96

-80.96

-80.85

-80.80

-80.72

-80.71

-80.70

-80.62

-80.52

-80.52

-80.47

-80.30

-80.15

-80.12

-80.05

-80.04

-79.98

-79.90

-79.90

-79.88

-79.76

-79.73

-79.50

-79.40

-79.37

-79.10

-79.06

-79.06

-79.02

-78.93

-78.93

-78.88

-78.78

-78.73

-78.73

-78.72

-78.71

-78.58

-78.49

-78.39

-78.36

-78.33

-78.07

-77.98

-77.97

-77.96

-77.90

-77.90

-77.62

-77.62

-77.45

-77.4

-77.05

-77.05

-77.03

-76.88

-76.88

-76.86

-76.66

-76.57

-76.47

-76.46

-75.70

-75.67

-75.62

-75.55

NWS

CAPE HATTERAS NC

|  |  |  | # |  | Station |  | Lat |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Height |  | Provider |  | Station Location |  | 1 |  | KZZV |  | 39.95 |  |  |
| 10 (m) |  | NWS |  | ZANESVILLE MUNI, OH |  | 2 |  | KBJJ |  | 40.87 |  |  |
| 10 |  | NWS |  | WOOSTER/WAYNE CO ARPT, OH |  | 3 |  | KSLW |  | 40.88 |  |  |
| 10 |  | NWS |  | SMITHVILLE/WOOSTER, OH |  | 4 |  | K6V3 |  | 37.06 |  |  |
| 10 |  | NWS |  | TAZEWELL CO ARPT, VA |  | 5 |  | KRLX |  | 38.31 |  |  |
| 10 |  | NWS |  | CHARLESTON NEXRAD,WV |  | 6 |  | KTNB |  | 36.20 |  |  |
| 10 |  | NWS |  | BOONE/WATAUGA CO HOSP HELIPAD, NC |  | 7 |  | KCRW |  | 38.37 |  |  |
| 10 |  | NWS |  | CHARLESTON/KANAWHA, WV |  | 8 |  | KI16 |  | 37.60 |  |  |
| 10 |  | NWS |  | KEE FLD ARPT, WV |  | 9 |  | KAKR |  | 41.04 |  |  |
| 10 |  | NWS |  | AKRON FULTON INTL ARPT (ASOS), OH |  | 10 |  | KPKB |  | 39.35 |  |  |
| 10 |  | FAA |  | PARKERSBURG/WILSON, WV |  | 11 |  | KCAK |  | 40.92 |  |  |
| 10 |  | NWS |  | AKRON-CANTON RGNL, OH |  | 12 |  | KGEV |  | 36.43 |  |  |
| 10 |  | NWS |  | JEFFERSON/ASHE CO ARPT, NC |  | 13 |  | KPHD |  | 40.47 |  |  |
| 10 |  | NWS |  | NEW PHILA/HARRY CLEVER FLD, OH |  | 14 |  | KMKJ |  | 36.90 |  |  |
| 10 |  | NWS |  | MARION/WYTHEVILLE |  | 15 |  | KBLF |  | 37.30 |  |  |
| 8 |  | FAA |  | BLUEFIELD MERCER CO. ARPT, WV |  | 16 |  | KBKW |  | 37.78 |  |  |
| 10 |  | NWS |  | BECKLEY MEMORIAL, WV |  | 17 |  | KUKF |  | 36.22 |  |  |
| 10 |  | NWS |  | N WILKESBORO/WILKES CO ARPT, NC |  | 18 |  | KHLX |  | 36.77 |  |  |
| 10 |  | NWS |  | HILLSVILLE |  | 19 |  | KPSK |  | 37.13 |  |  |
| 10 |  | NWS |  | DUBLIN/NEW RIVER VALLEY, VA |  | 20 |  | K48I |  | 38.69 |  |  |
| 10 |  | NWS |  | BRAXTON CO ARPT, WV |  | 21 |  | KHLG |  | 40.18 |  |  |
| 10 |  | FAA |  | WHEELING/OHIO CO, WV |  | 22 |  | KMWK |  | 36.46 |  |  |
| 10 |  | NWS |  | MT AIRY/SCURRY CO ARPT, NC |  | 23 |  | KRNK |  | 37.20 |  |  |
| 10 |  | NWS |  | ROANOKE/BLACKBURG, VA |  | 24 |  | KBCB |  | 37.22 |  |  |
| 10 |  | NWS |  | VIRGINIA TECH ARPT, VA |  | 25 |  | KUCP |  | 41.03 |  |  |
| 10 |  | NWS |  | NEW CASTLE MUNI ARPT, PA |  | 26 |  | KLWB |  | 37.87 |  |  |
| 10 |  | NWS |  | LEWISBURG/GREENBRIE, WV |  | 27 |  | KBVI |  | 40.77 |  |  |
| 10 |  | NWS |  | BEAVER FALLS ARPT, PA |  | 28 |  | KAFJ |  | 40.13 |  |  |
| 10 |  | NWS |  | WASHINGTON (AWOS), PA |  | 29 |  | KW22 |  | 39.00 |  |  |
| 10 |  | NWS |  | UPSHUR CO RGNL ARPT, WV |  | 30 |  | KINT |  | 36.13 |  |  |
| 10 |  | FAA |  | WINSTON-SALEM/SMITH, NC |  | 31 |  | KCKB |  | 39.30 |  |  |
| 10 |  | FAA |  | CLARKSBURG/BENEDUM, WV |  | 32 |  | KPIT |  | 40.50 |  |  |
| 10 |  | NWS |  | PITTSBURG INTL, PA |  | 33 |  | KMTV |  | 36.63 |  |  |
| 10 |  | NWS |  | MARTINSVILLE |  | 34 |  | KROA |  | 37.32 |  |  |
| 10 |  | NWS |  | ROANOKE REGIONAL ARPT, VA |  | 35 |  | KBTP |  | 40.78 |  |  |
| 10 |  | NWS |  | BUTLER CO (AWOS), PA |  | 36 |  | KGSO |  | 36.10 |  |  |
| 10 |  | NWS |  | GREENSBORO PIEDMONT TRIAD INTL, NC |  | 37 |  | KAGC |  | 40.35 |  |  |
| 10 |  | FAA |  | PITTSBURG/ALLEGEN, PA |  | 38 |  | KMGW |  | 39.65 |  |  |
| 10 |  | FAA |  | MORGANTOWN/HART FLD, WV |  | 39 |  | K78N |  | 36.44 |  |  |
| 10 |  | NWS |  | ROCKINGHAM CO SHILOH ARPT, NC |  | 40 |  | KEKN |  | 38.88 |  |  |
| 10 |  | NWS |  | ELKINS/RANDOLPH FLD |  | 41 |  | KSIF |  | 36.44 |  |  |
| 10 |  | NWS |  | ROCKINGHAM CO SHILOH ARPT, NC |  | 42 |  | KHSP |  | 37.95 |  |  |
| 10 |  | NWS |  | HOT SPRINGS/INGALLS |  | 43 |  | KFWQ |  | 40.21 |  |  |
| 10 |  | NWS |  | MONONGAHELA/ROSTRAVER ARPT, PA |  | 44 |  | KLBE |  | 40.28 |  |  |
| 10 |  | NWS |  | LATROBE/WESTMORLAND, PA |  | 45 |  | KDAN |  | 36.57 |  |  |
| 10 |  | FAA |  | DANVILLE REGIONAL ARPT,DANVILLE, VA |  | 46 |  | KLYH |  | 37.32 |  |  |
| 10 |  | NWS |  | LYNCHBURG ARPT,LYNCHBURG, VA |  | 47 |  | KW99 |  | 38.98 |  |  |
| 10 |  | NWS |  | PETERSBURG/GRANT CO ARPT, WV |  | 48 |  | KIDI |  | 40.63 |  |  |
| 10 |  | NWS |  | INDIANA/STEWART FLD, PA |  | 49 |  | KTDF |  | 36.28 |  |  |
| 10 |  | NWS |  | ROXBORO/PERSON CO ARPT, NC |  | 50 |  | KSHD |  | 38.27 |  |  |
| 10 |  | NWS |  | STAUNTON/SHENANDOAH |  | 51 |  | KJST |  | 40.32 |  |  |
| 10 |  | FAA |  | JOHNSTOWN CAMBRIA ARPT, JOHNSTOWN, PA |  | 52 |  | KCBE |  | 39.52 |  |  |
| 10 |  | NWS |  | GREATER CUMBERLAND RGNL ARPT, MD |  | 53 |  | KHNZ |  | 36.36 |  |  |
| 10 |  | NWS |  | HENDERSON-OXFORD ARPT, NC |  | 54 |  | KCHO |  | 38.13 |  |  |
| 10 |  | FAA |  | CHARLOTTESVILLE ALBEMARLE ARPT, VA |  | 55 |  | KFVX |  | 37.35 |  |  |
| 10 |  | NWS |  | FARMVILLE |  | 56 |  | KFIG |  | 41.04 |  |  |
| 10 |  | FAA |  | CLEARFIELD LAWRENCE ARPT, PA |  | 57 |  | KN97 |  | 41.05 |  |  |
| 10 |  | NWS |  | CLEARFIELD, PA |  | 58 |  | KOKV |  | 39.15 |  |  |
| 10 |  | NWS |  | WINCHESTER RGNL |  | 59 |  | KAVC |  | 36.69 |  |  |
| 10 |  | NWS |  | S HILL/MECKLENBURG-BRUNSWICK |  | 60 |  | KOMH |  | 38.25 |  |  |
| 10 |  | NWS |  | ORANGE CO ARPT, VA |  | 61 |  | KMRB |  | 39.40 |  |  |
| 10 |  | FAA |  | MARTINSBURG RGNL |  | 62 |  | KLKU |  | 38.01 |  |  |
| 10 |  | NWS |  | LOUISA CO /FREEMAN FLD, VA |  | 63 |  | KCJR |  | 38.53 |  |  |
| 10 |  | NWS |  | CULPEPER RGNL ARPT, VA |  | 64 |  | KUNV |  | 40.85 |  |  |
| 10 |  | NWS |  | STATE COLLEGE, PA |  | 65 |  | KCTP |  | 40.85 |  |  |
| 10 |  | NWS |  | STATE COLLEGE, PA |  | 66 |  | KHGR |  | 39.70 |  |  |
| 10 |  | FAA |  | HAGERSTOWN WASHINGTON CO. RGNL, , MD |  | 67 |  | KRZZ |  | 36.44 |  |  |
| 10 |  | FAA |  | ROANOKE RAPIDS/HALIFAX ARPT, NC |  | 68 |  | KJYO |  | 39.08 |  |  |
| 10 |  | NWS |  | LEESBURG/GODFREY |  | 69 |  | KPTB |  | 37.18 |  |  |
| 10 |  | NWS |  | PETERSBURG (AWOS) |  | 70 |  | KHEF |  | 38.72 |  |  |
| 10 |  | NWS |  | MANASSAS MUNI (AWOS) |  | 71 |  | KEMV |  | 36.69 |  |  |
| 10 |  | NWS |  | EMPORIA-GREENSVILLE RGNL ARPT, VA |  | 72 |  | KLWX |  | 38.98 |  |  |
| 10 |  | NWS |  | STERLING, VA |  | 73 |  | KRMN |  | 38.40 |  |  |
| 10 |  | NWS |  | STAFFORD RGNL ARPT, VA |  | 74 |  | KEZF |  | 38.27 |  |  |
| 10 |  | NWS |  | SHANNON ARPT |  | 75 |  | KQBV |  | 39.63 |  |  |
| 10 |  | NWS |  | CAMP DAVID, MD |  | 76 |  | KIAD |  | 38.94 |  |  |
| 10 |  | NWS |  | DULLES INTL ARPT, WASHINGTON, DC |  | 77 |  | KOFP |  | 37.71 |  |  |
| 10 |  | FAA |  | RICHMOND ASHLAND HANOVER CO, VA |  | 78 |  | KQAH |  | 39.73 |  |  |
| 10 |  | NWS |  | SITE R, PA |  | 79 |  | KJWX |  | 39.73 |  |  |
| 10 |  | NWS |  | FT RITCHIE, MD |  | 80 |  | K43M |  | 39.73 |  |  |
| 10 |  | NWS |  | FT RITCHIE/SITE R, MD |  | 81 |  | KFDK |  | 39.42 |  |  |
| 10 |  | NWS |  | FREDERICK MUNI ARPT, MD |  | 82 |  | KRIC |  | 37.51 |  |  |
| 10 |  | NWS |  | RICHMOND INTL ARPT, RICHMOND, VA |  | 83 |  | KNYG |  | 38.50 |  |  |
| 10 |  | NWS |  | QUANTICO MCAF |  | 84 |  | KDAA |  | 38.72 |  |  |
| 10 |  | NWS |  | FT BELVOIR/DAVISO |  | # |  | Station |  | Lat |  |  |
| Height |  | Provider |  | Station Location |  | 85 |  | KASJ |  | 36.30 |  |  |
| 10 |  | NWS |  | AHOSKIE/TRI-CO ARPT, NC |  | 86 |  | KDCA |  | 38.85 |  |  |
| 10 |  | NWS |  | WASHINGTON REAGAN NATL ARPT |  | 87 |  | KAKQ |  | 36.98 |  |  |
| 8 |  | NWS |  | WAKEFIELD MUNI ARPT, WAKEFIELD, VA |  | 88 |  | KFKN |  | 36.70 |  |  |
| 10 |  | NWS |  | FRANKLIN/JB ROSE |  | 89 |  | KADW |  | 38.82 |  |  |
| 10 |  | NWS |  | ANDREWS AFB, MD |  | 90 |  | KSEG |  | 40.82 |  |  |
| 10 |  | FAA |  | SELINSGROVE PENN VALLEY ARPT |  | 91 |  | KCXY |  | 40.22 |  |  |
| 10 |  | FAA |  | HARRISBURG CAPITAL CITY ARPTPA |  | 92 |  | KTHV |  | 40.20 |  |  |
| 10 |  | FAA |  | YORK MUNI ARPT, YORK, PA |  | 93 |  | KMDT |  | 40.20 |  |  |
| 10 |  | FAA |  | MIDDLETOWN HARRISBURG INTL ARPT, PA |  | 94 |  | KFYJ |  | 37.52 |  |  |
| 10 |  | NWS |  | MID PENINSULA RGNL ARPT, VA |  | 95 |  | KFME |  | 39.09 |  |  |
| 10 |  | NWS |  | FT MEADE/TIPTON ARPT, MD |  | 96 |  | KJGG |  | 37.24 |  |  |
| 10 |  | NWS |  | WILLIAMSBURG-JAMESTOWN ARPT, VA |  | 97 |  | KBWI |  | 39.17 |  |  |
| 10 |  | NWS |  | BALTIMORE WASHINGTON INTL, MD |  | 98 |  | KFAF |  | 37.13 |  |  |
| 10 |  | NWS |  | FT EUSTIS/FELKER |  | 99 |  | KDMH |  | 39.28 |  |  |
| 10 |  | NWS |  | BALTIMORE/INNER HARBOR (ASOS), MD |  | 100 |  | KSFQ |  | 36.68 |  |  |
| 10 |  | NWS |  | SUFFOLK MUNI ARPT, VA |  | 101 |  | KMUI |  | 40.43 |  |  |
| 10 |  | NWS |  | MURI AAF/INDIANTOWN, PA |  | 102 |  | KPHF |  | 37.13 |  |  |
| 10 |  | FAA |  | NEWPORT NEWS INTL ARPT, VA |  | 103 |  | KNAK |  | 38.98 |  |  |
| 10 |  | NWS |  | US NAVAL ACAD ANNAPOLIS, MD |  | 104 |  | KBUY |  | 36.05 |  |  |
| 10 |  | FAA |  | BURLINGTON ALAMANCE RNGL ARPT, NC |  | 105 |  | KNUI |  | 38.15 |  |  |
| 10 |  | NWS |  | ST INIGOES/WEBSTER FLD, MD |  | 106 |  | KMTN |  | 39.33 |  |  |
| 10 |  | NWS |  | BALTIMORE/MARTIN, MD |  | 107 |  | KNHK |  | 38.28 |  |  |
| 10 |  | DODn |  | PATUXENT RIVER NAS, MD |  | 108 |  | KLFI |  | 37.08 |  |  |
| 10 |  | NWS |  | LANGLEY AFB |  | 109 |  | KCPK |  | 36.67 |  |  |
| 10 |  | NWS |  | CHESAPEAKE MUNI ARPT, VA |  | 110 |  | KLNS |  | 40.12 |  |  |
| 8 |  | FAA |  | ELIZABETHTOWN, LITITZ, PA |  | 111 |  | KNGU |  | 36.93 |  |  |
| 10 |  | DODn |  | NORFOLK NAS/CHAMBER |  | 112 |  | KORF |  | 36.90 |  |  |
| 10 |  | NWS |  | NORFOLK INTERNATIONAL ARPT, VA |  | 113 |  | KECG |  | 36.26 |  |  |
| 10 |  | FAA |  | ELIZABETH CITY CG ARPT, NC |  | 114 |  | KAPG |  | 39.47 |  |  |
| 10 |  | NWS |  | PHILLIPS AAF/ABERDN, MD |  | 115 |  | KNFE |  | 36.70 |  |  |
| 10 |  | NWS |  | FENTRESS/NAS AUX, VA |  | 116 |  | KESN |  | 38.80 |  |  |
| 10 |  | NWS |  | EASTON/NEWMAN FLD, MD |  | 117 |  | KNTU |  | 36.82 |  |  |
| 10 |  | NWS |  | OCEANA NAS/SOUCEK |  | 118 |  | KONX |  | 36.40 |  |  |
| 10 |  | NWS |  | CURRITUCK CO ARPT, NC |  | 119 |  | K9W7 |  | 36.40 |  |  |
| 10 |  | NWS |  | CURRITUCK CO ARPT, NC |  | 120 |  | KRDG |  | 40.38 |  |  |
| 10 |  | FAA |  | READING/SPAATZ FLD, PA |  | 121 |  | K40N |  | 39.98 |  |  |
| 10 |  | NWS |  | CHESTER CO ARPT, PA |  | 122 |  | KMFV |  | 37.65 |  |  |
| 10 |  | NWS |  | MELFA/ACCOMACK ARPT |  | 123 |  | KILG |  | 39.68 |  |  |
| 10 |  | NWS |  | WILMINGTON ARPT, DE |  | 124 |  | KPTW |  | 40.23 |  |  |
| 10 |  | FAA |  | POTTSTOWN LIMERICK ARPT, PA |  | 125 |  | KSBY |  | 38.34 |  |  |
| 8 |  | FAA |  | SALISBURY WICOMICO RGNL ARPT, MD |  | 126 |  | KWAL |  | 37.94 |  |  |
| 10 |  | NWS |  | WALLOPS ISLAND FLIGHT, VA |  | 127 |  | KDOV |  | 39.13 |  |  |
| 10 |  | NWS |  | DOVER AFB, DE |  | 128 |  | KABE |  | 40.65 |  |  |
| 10 |  | NWS |  | ALLENTOWN-BETHLEHEM, PA |  | 129 |  | KMPO |  | 41.13 |  |  |
| 10 |  | FAA |  | MT POCONO, MT POCONO, PA |  | 130 KUKT |  | 40.44 |  |  |  |  |
| 10 |  | NWS |  | QUAKERTOWN ARPT, PA |  | 131 |  | KGED |  | 38.68 |  |  |
| 10 |  | FAA |  | GEORGETOWN SUSSEX CO ARPT, DE |  | 132 |  | KLOM |  | 40.14 |  |  |
| 10 |  | NWS |  | WINGS FLD ARPT, PA |  | 133 |  | KPHL |  | 39.88 |  |  |
| 10 |  | NWS |  | PHILADELPHIA INTL PA |  | 134 |  | KPHI |  | 39.88 |  |  |
| 10 |  | NWS |  | PHILADELPHIA, PA |  | 135 |  | KNXX |  | 40.20 |  |  |
| 10 |  | DODn |  | WILLOW GROVE NAS |  | 136 |  | KOXB |  | 38.31 |  |  |
| 10 |  | FAA |  | OCEAN CITY MUNI ARPT, MD |  | 137 KDYL |  | 40.33 |  |  |  |  |
| 10 |  | FAA |  | DOYLESTOWN ARPT, PA |  | 138 |  | KMIV |  | 39.37 |  |  |
| 10 |  | FAA |  | MILLVILLE MUNI, NJ |  | 139 |  | KPNE |  | 40.08 |  |  |
| 10 |  | NWS |  | PHILADELPHIA NW, PA |  | 140 |  | KWWD |  | 39.02 |  |  |
| 10 |  | NWS |  | WILDWOOD (AWOS), NJ |  | 141 |  | KTYZ |  | 39.95 |  |  |
| 10 |  | NWS |  | MONMOUTH BCH (ASOS), NJ |  | 142 |  | KN91 |  | 38.95 |  |  |
| 10 |  | NWS |  | CAPE MAY (CGS), NJ |  | 143 |  | KVAY |  | 39.95 |  |  |
| 10 |  | FAA |  | MT HOLLY S JERSEY RGNL, NJ |  | 144 |  | KTTN |  | 40.28 |  |  |
| 10 |  | FAA |  | TRENTON/MERCER CO. NJ |  | 145 |  | K12N |  | 41.02 |  |  |
| 10 |  | NWS |  | ANDOVER AEROFLEX, NJ |  | 146 |  | KSJB |  | 40.62 |  |  |
| 10 |  | NWS |  | SOMERVILLE, NJ |  | 147 |  | KN52 |  | 40.62 |  |  |
| 10 |  | NWS |  | SOMERVILLE, NJ |  | 148 |  | KSMQ |  | 40.62 |  |  |
| 10 |  | FAA |  | SOMERVILLE SOMERSET ARPT, NJ |  | 149 |  | KWRI |  | 40.02 |  |  |
| 10 |  | NWS |  | MCGUIRE AFB, NJ |  | 150 |  | KACY |  | 39.45 |  |  |
| 10 |  | NWS |  | ATLANTIC CITY INTL, NJ |  | 151 |  | KMMU |  | 40.80 |  |  |
| 10 |  | NWS |  | MORRISTOWN MUNI, NJ |  | 152 |  | KDIX |  | 39.95 |  |  |
| 10 |  | NWS |  | PHILADELPHIA NEXRAD, PA |  | 153 |  | KNEL |  | 40.03 |  |  |
| 10 |  | NWS |  | LAKEHURST NAS, NJ |  | 154 |  | KCDW |  | 40.88 |  |  |
| 10 |  | NWS |  | CALDWELL/ESSEX CO., NJ |  | 155 |  | KEWR |  | 40.70 |  |  |
| 10 |  | NWS |  | NEWARK INTL ARPT, NJ |  | 156 |  | KBLM |  | 40.18 |  |  |
| 10 |  | NWS |  | BELMAR-FARMDALE, NJ |  | 157 |  | KTEB |  | 40.85 |  |  |
| 10 |  | NWS |  | TETERBORO ARPT, NJ |  | 158 |  | KNYC |  | 40.77 |  |  |
| 10 |  | NWS |  | NEW YORK CITY, NY |  | 159 |  | KLGA |  | 40.77 |  |  |
| 10 |  | NWS |  | NY LA GUARDIA, NY |  | 160 |  | KJFK |  | 40.63 |  |  |
| 10 |  | NWS |  | NY JFK INTL ARPT, NY |  | 161 |  | KHPN |  | 41.07 |  |  |
| 10 |  | FAA |  | WHITE PLAINS, NY |  | 162 |  | KFRG |  | 40.73 |  |  |
| 10 |  | FAA |  | FARMINGDALE/REPUBLIC, NY |  | 163 |  | KISP |  | 40.80 |  |  |
| 10 |  | FAA |  | ISLIP LONG ISLAND MCARTHUR ARPT, NY |  | 164 |  | KHWV |  | 40.82 |  |  |
| 10 |  | FAA |  | BROOKHAVEN ARPT, NY |  | 165 |  | KOKX |  | 40.87 |  |  |
| 10 |  | NWS |  | BROOKHAVEN, NY |  | 166 |  | KFOK |  | 40.85 |  |  |
| 10 |  | FAA |  | WEST HAMPTON BCH, NY |  | 167 |  | KHTO |  | 40.97 |  |  |
| 10 |  | NWS |  | EAST HAMPTON, NY |  | 168 |  | KMTP |  | 41.08 |  |  |

B-3 ASOS/METAR Platform Locations (cont’d)

Lon

-81.90

-81.88

-81.83

-81.80

-81.72

-81.65

-81.60

-81.56

-81.46

-81.43

-81.43

-81.42

-81.42

-81.35

-81.20

-81.12

-81.10

-80.82

-80.68

-80.65

-80.65

-80.55

-80.42

-80.42

-80.41

-80.40

-80.40

-80.28

-80.27

-80.22

-80.22

-80.22

-80.02

-79.97

-79.95

-79.94

-79.93

-79.92

-79.85

-79.85

-79.85

-79.83

-79.83

-79.40

-79.33

-79.21

-79.13

-79.11

-78.98

-78.90

-78.84

-78.76

-78.53

-78.45

-78.43

-78.42

-78.41

-78.15

-78.05

-78.05

-77.98

-77.97

-77.86

-77.85

-77.85

-77.73

-77.71

-77.57

-77.52

-77.52

-77.48

-77.48

-77.46

-77.45

-77.45

-77.45

-77.44

-77.43

-77.43

-77.43

-77.37

-77.32

-77.30

-77.18

Lon

-77.17

-77.03

-77.00

-76.90

-76.87

-76.86

-76.86

-76.77

-76.77

-76.76

-76.76

-76.72

-76.68

-76.62

-76.62

-76.60

-76.57

-76.49

-76.48

-76.47

-76.42

-76.42

-76.40

-76.37

-76.32

-76.30

-76.28

-76.19

-76.17

-76.17

-76.13

-76.07

-76.03

-76.02

-76.02

-75.97

-75.87

-75.77

-75.60

-75.55

-75.51

-75.47

-75.47

-75.43

-75.38

-75.38

-75.37

-75.27

-75.25

-75.25

-75.15

-75.13

-75.12

-75.07

-75.02

-74.92

-74.92

-74.88

-74.85

-74.82

-74.73

-74.67

-74.67

-74.67

-74.60

-74.57

-74.42

-74.41

-74.35

-74.28

-74.17

-74.13

-74.07

-73.98

-73.90

-73.77

-73.70

-73.42

-73.10

-72.86

-72.86

-72.63

-72.25

-71.92

10

NWS

MONTAUK ARPT, NY

|  |  | # |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lat | Lon | Provider | Station Location | 1 |  |
| 42.11 | -77.99 | FAA | WELLSVILLE MUNI ARPT, NY | 2 |  |
| 42.57 | -77.72 | FAA | DANSVILLE, NY | 3 |  |
| 43.12 | -77.67 | NWS | ROCHESTER/MONROE CO, NY | 4 |  |
| 42.64 | -77.05 | FAA | PENN YAN ARPT, PENN YAN, NY | 5 |  |
| 42.16 | -76.90 | FAA | ELMIRA CORNING RGNL ARPT, ELMIRA, NY | 6 |  |
| 42.48 | -76.45 | NWS | ITHACA/TOMPKINS CO., NY | 7 |  |
| 43.35 | -76.39 | FAA | FULTON/OSWEGO CTY ARPT, NY | 8 |  |
| 43.11 | -76.10 | NWS | SYRACUSE HANCOCK INTL ARPT | 9 |  |
| 44.00 | -76.02 | FAA | WATERTOWN INTL ARPT, NY | 10 |  |
| 42.21 | -75.98 | NWS | BINGHAMTON RGNL ARPT | 11 |  |
| 44.05 | -75.73 | NWS | FORT DRUM/WHEELER, NY | 12 |  |
| 41.33 | -75.72 | NWS | WILKES-BARRE SCRANTON INTL ARPT | 13 |  |
| 44.68 | -75.47 | NWS | OGDENSBURG INTL, NY | 14 |  |
| 43.23 | -75.40 | NWS | GRIFFISS AFB/ROME, NY | 15 |  |
| 43.14 | -75.38 | FAA | UTICA ONEIDA CO ARPT, UTICA, NY | 16 |  |
| 44.93 | -74.85 | FAA | MASSENA/RICHARDS, NY | 17 |  |
| 41.70 | -74.80 | NWS | MONTICELLO (AWOS), NY | 18 |  |
| 41.52 | -74.27 | FAA | MONTGOMERY/ORANGE CTY ARPT, NY | 19 |  |
| 44.40 | -74.20 | FAA | SARANAC LAKE/ADIRONDACK, NY | 20 |  |
| 41.50 | -74.10 | NWS | NEWBURGH/STEWART, NY | 21 |  |
| 42.85 | -73.93 | NWS | SCHENECTADY ARPT, NY | 22 |  |
| 41.63 | -73.88 | FAA | POUGHKEEPSIE DUTCHESS CO ARPT, NY | 23 |  |
| 42.69 | -73.83 | NWS | ALBANY WFO & SCD, NY | 24 |  |
| 42.75 | -73.80 | NWS | ALBANY, NY | 25 |  |
| 42.75 | -73.80 | NWS | ALBANY CO ARPT, ALBANY, NY | 26 |  |
| 41.70 | -73.73 | NWS | MILLBROOK/SKY ACRES, NY | 27 |  |
| 43.34 | -73.61 | FAA | GLENS FALLS ARPT,GLENS FALLS, NY | 28 |  |
| 44.69 | -73.52 | FAA | PLATTSBURGH/CLINTON CNTY ARPT, NY | 29 |  |
| 41.37 | -73.48 | FAA | DANBURY MUNI, CT | 30 |  |
| 44.65 | -73.47 | NWS | PLATTSBURGH AFB, NY | 31 |  |
| 42.42 | -73.29 | FAA | PITTSFIELD MUNI ARPT, PITTSFIELD, NY | 32 |  |
| 42.89 | -73.25 | NWS | BENNINGTON, VT | 33 |  |
| 42.89 | -73.25 | FAA | BENNINGTON MORSE STATE ARPT, VT | 34 |  |
| 42.70 | -73.17 | FAA | NORTH ADAMS HARRIMAN, MA | 35 |  |
| 42.70 | -73.17 | FAA | NORTH ADAMS MA | 36 |  |
| 44.47 | -73.15 | NWS | BURLINGTON INTL, VT | 37 |  |
| 41.48 | -73.13 | NWS | OXFORD (AWOS) CT | 38 |  |
| 43.53 | -72.95 | NWS | RUTLAND STATE (AWOS) VT | 39 |  |
| 41.51 | -72.83 | FAA | MERIDEN MARKHAM MUNI APT, CT | 40 |  |
| 42.15 | -72.72 | FAA | WESTFIELD/BARNES MA | 41 |  |
| 41.93 | -72.68 | NWS | HARTFORD/BRADLEY CT | 42 |  |
| 41.73 | -72.65 | FAA | HARTFORD/BRAINARD CT | 43 |  |
| 44.53 | -72.62 | FAA | MORRISVILLE (ASOS) VT | 44 |  |
| 44.20 | -72.57 | FAA | BARRE-MONTPELIER VT | 45 |  |
| 42.20 | -72.53 | NWS | CHICOPEE/WESTOVER MA | 46 |  |
| 43.35 | -72.52 | FAA | SPRINGFIELD/HARTNES VT | 47 |  |
| 43.63 | -72.30 | FAA | LEBANON MUNI NH | 48 |  |
| 42.57 | -72.28 | FAA | ORANGE MUNI ARPT, ORANGE, MA | 49 |  |
| 42.90 | -72.27 | NWS | KEENE/DILLANT (AWOS) NH | 50 |  |
| 44.93 | -72.20 | NWS | NEWPORT VT | 51 |  |
| 41.74 | -72.18 | FAA | WILLIMANTIC WINDHAM ARPT, CT | 52 |  |
| 44.42 | -72.02 | NWS | ST. JOHNSBURY (AMOS) VT | 53 |  |
| 42.80 | -72.00 | FAA | JAFFREY NH | 54 |  |
| 42.27 | -71.88 | NWS | WORCESTER (AMOS) MA | 55 |  |
| 41.35 | -71.80 | FAA | WESTERLY STATE ARPT, WESTERLY, RI | 56 |  |
| 42.55 | -71.76 | FAA | FITCHBURG MUNI ARPT, FITCHBERG, MA |  |  |
|  |  |  |  | # |  |
| Lat | Lon | Provider | Station Location | 57 |  |
| 43.78 | -71.75 | NWS | PLYMOUTH MUNI ARPT, NH | 58 |  |
| 42.57 | -71.60 | NWS | FORT DEVENS/AYER MA | 59 |  |
| 44.35 | -71.55 | FAA | WHITEFIELD (ASOS) NH | 60 |  |
| 42.78 | -71.52 | NWS | NASHUA/BOIRE FIELD NH | 61 |  |
| 41.92 | -71.50 | NWS | PAWTUCKET (AWOS) RI | 62 |  |
| 43.20 | -71.50 | NWS | CONCORD MUNI NH | 63 |  |
| 41.723 | -71.43 | NWS | PROVIDENCE GREEN STATE ARPT, RI | 64 |  |
| 42.93 | -71.43 | FAA | MANCHESTER AIRPARK NH | 65 |  |
| 41.60 | -71.42 | NWS | N. KINGSTON/QUONSET RI | 66 |  |
| 43.57 | -71.42 | NWS | LACONIA MUNI (AWOS) NH | 67 |  |
| 44.27 | -71.30 | NWS | MOUNT WASHINGTON NH | 68 |  |
| 42.47 | -71.29 | FAA | BEDFORD HANSCOM FIELD,BEDFORD,MA | 69 |  |
| 41.53 | -71.28 | FAA | NEWPORT STATE ARPT, NEWPORT, RI | 70 |  |
| 44.58 | -71.18 | FAA | BERLIN MUNI NH | 71 |  |
| 42.19 | -71.17 | FAA | NORWOOD MEMORIAL ARPT, NORWOOD, MA | 72 |  |
| 42.71 | -71.13 | FAA | LAWRENCE MUNI ARPT, LAWRENCE, MA | 73 |  |
| 42.22 | -71.12 | NWS | EAST MILTON (ASOS), MA | 74 |  |
| 42.37 | -71.03 | NWS | BOSTON MA | 75 |  |
| 41.88 | -71.02 | FAA | TAUNTON MUNI ARPT,TAUNTON,MA | 76 |  |
| 42.36 | -71.02 | NWS | BOSTON LOGAN INTL ARPT, BOSTON, MA | 77 |  |
| 41.68 | -70.96 | FAA | NEW BEDFORD MUNI ARPT, MA | 78 |  |
| 43.98 | -70.95 | FAA | FRYEBURG (ASOS) ME | 79 |  |
| 42.15 | -70.93 | NWS | SOUTH WEYMOUTH NAS MA | 80 |  |
| 43.28 | -70.93 | FAA | ROCHESTER/SKYHAVEN ARPT, NH | 81 |  |
| 43.28 | -70.92 | NWS | ROCHESTER, NH | 82 |  |
| 42.58 | -70.92 | FAA | BEVERLY MUNI ARPT, BEVERLY, MA | 83 |  |
| 43.08 | -70.82 | NWS | PEASE AFB/PORTSMOUT NH | 84 |  |
| 41.91 | -70.73 | FAA | PLYMOUTH MUNI ARPT, PLYMOUTH, MA | 85 |  |
| 43.40 | -70.72 | NWS | SANFORD MUNI (AWOS) ME | 86 |  |
| 42.10 | -70.68 | NWS | MARSHFIELD ARPT MA | 87 |  |
| 41.39 | -70.62 | FAA | MARTHA'S VINEYARD ARPT, MA | 88 |  |
| 44.53 | -70.53 | NWS | RUMFORD ME | 89 |  |
| 41.65 | -70.52 | NWS | OTIS ANGB MA | 90 |  |
| 43.65 | -70.32 | NWS | PORTLAND INTL JET ME | 91 |  |
| 44.05 | -70.28 | NWS | AUBURN-LEWISTON ME | 92 |  |
| 41.67 | -70.27 | FAA | HYANNIS BARNSTABLE MUNI ARPT, MA | 93 |  |
| 43.89 | -70.26 | NWS | GRAY ME | 94 |  |
| 42.07 | -70.22 | NWS | PROVINCETOWN (AWOS) MA | 95 |  |
| 41.69 | -69.99 | FAA | CHATHAM MUNI ARPT, CHATHAM, MA | 96 |  |
| 41.67 | -69.97 | NWS | CHATHAM MA | 97 |  |
| 43.88 | -69.93 | NWS | BRUNSWICK NAS ME | 98 |  |
| 44.32 | -69.80 | FAA | AUGUSTA STATE ARPT ME | 99 |  |
| 43.97 | -69.72 | FAA | WISCASSET ME | 100 |  |
| 44.53 | -69.68 | NWS | WATERVILLE (AWOS) ME | 101 |  |
| 45.47 | -69.58 | NWS | GREENVILLE (AMOS) ME | 102 |  |
| 45.47 | -69.58 | NWS | GREENVILLE (AMOS) ME | 103 |  |
| 46.62 | -69.53 | NWS | CLAYTON LAKE, ME | 104 |  |
| 44.07 | -69.10 | NWS | ROCKLAND/KNOX (AWOS) ME | 105 |  |
| 44.80 | -68.83 | FAA | BANGOR INTL ARPT ME | 106 |  |
| 45.65 | -68.68 | FAA | MILLINOCKET MUNI ME | 107 |  |
| 44.45 | -68.37 | NWS | BAR HARBOR (AWOS) ME | 108 |  |
| 47.28 | -68.32 | FAA | FRENCHVILLE, ME | 109 |  |
| 46.68 | -68.05 | NWS | PRESQUE ISLE (AWOS), ME | 110 |  |
| 46.87 | -68.02 | NWS | CARIBOU MUNI, ME | 111 |  |
| 46.95 | -67.88 | NWS | LORING AFB/LIMESTON, ME | 112 |  |
| 46.12 | -67.80 | FAA | HOULTON INTL ARPT, ME | 113 |  |

B-3 ASOS/METAR Platform Locations (cont’d)

Station

KELZ

KDSV

KROC

KPEO

KELM

KITH

KFZY

KSYR

KART

KBGM

KGTB

KAVP

KOGS

KRME

KUCA

KMSS

KMSV

KMGJ

KSLK

KSWF

KSCH

KPOU

KALY

KALB

KALB

K44N

KGFL

KPLB

KDXR

KPBG

KPSF

K5B5

KDDH

KAQW

KAQW

KBTV

KOXC

KRUT

KMMK

KBAF

KBDL

KHFD

KMVL

KMPV

KCEF

KVSF

KLEB

KORE

KEEN

K9B2

KIJD

K1V4

KAFN

KORH

KWST

KFIT

Station

K1P1

KAYE

KHIE

KASH

KSFZ

KCON

KPVD

KMHT

KOQU

KLCI

KMWN

KBED

KUUU

KBML

KOWD

KLWM

KMQE

KBOX

KTAN

KBOS

KEWB

KIZG

KNZW

KDAW

K6B1

KBVY

KPSM

KPYM

KSFM

K3B2

KMVY

KRUM

KFMH

KPWM

KLEW

KHYA

KGYX

KPVC

KCQX

KCHH

KNHZ

KAUG

KIWI

KWVL

KGNR

KGNR

K40B

KRKD

KBGR

KMLT

KBHB

KFVE

KPQI

KCAR

KLIZ

KHUL

KEPO

44.92

-67.00

NWS

EASTPORT ME

| # | Station | Lat | Lon | Station Location |
| --- | --- | --- | --- | --- |
| 1 | CWHY | 45.32 | -72.25 | MONT-ORFORD QB |
| 2 | CWQH | 45.36 | -71.81 | LENNOXVILLE, QB |
| 3 | CYSC | 45.43 | -71.68 | SHERBROOKE ARPT QB |
| 4 | CWPQ | 45.63 | -70.55 | MONTREAL-EST QB |
| 5 | CERM | 47.42 | -68.32 | EDMUNSTON NB |
| 6 | CYSL | 47.16 | -67.83 | ST LEONARD ARPT NB |
| 7 | CWIY | 47.15 | -67.82 | ST LEONARD AUTO8 NB |
| 8 | CWSS | 45.20 | -67.25 | ST STEPHEN (MARS) NB |
| 9 | CXGM | 44.71 | -66.80 | GRAND MANAN SAR CS NB |
| 10 | CWSG | 49.08 | -66.73 | CAP CHAT (MAPS) QB |
| 11 | CWZF | 45.97 | -66.65 | FREDERICTON AQUATIC NB |
| 12 | CAFC | 45.92 | -66.60 | FREDERICKTON CDA CS NB |
| 13 | CYFC | 45.87 | -66.53 | FREDERICTON ARPT NB |
| 14 | CWPE | 45.07 | -66.47 | POINT LEPREAU NB |
| 15 | CYCX | 45.83 | -66.43 | GAGETOWN (CAN-MIL) NB |
| 16 | CZCR | 47.98 | -66.33 | CHARLO AUTO NB |
| 17 | CWVU | 44.28 | -66.33 | BRIER ISLAND (AUT) NS |
| 18 | CYCL | 47.98 | -66.33 | CHARLO ARPT NB |
| 19 | CYQI | 43.83 | -66.08 | YARMOUTH ARPT NS |
| 20 | CYSJ | 45.32 | -65.88 | SAINT JOHN ARPT NB |
| 21 | CYID | 44.55 | -65.78 | DIGBY ARPT (AWRS) NS |
| 22 | CZBF | 47.63 | -65.75 | BATHURST ARPT (AWRS) NB |
| 23 | CWCP | 43.45 | -65.47 | BACCARO POINT NS |
| 24 | CWCQ | 47.00 | -65.47 | CHATHAM (AUTO8) NB |
| 25 | CACQ | 47.00 | -65.45 | MIRAMICHI RCS NB |
| 26 | CYCH | 47.00 | -65.45 | CHATHAM (CAN-MIL) NB |
| 27 | CWOC | 48.02 | -65.33 | NEW CARLISLE1 QB |
| 28 | CWSF | 49.25 | -65.33 | CAP MADELEINE (MAPS) QB |
| 29 | CWKG | 44.43 | -65.20 | KEJIMKUJIK 1 NS |
| 30 | CAKC | 46.77 | -65.00 | KOUCHIBOUGUAC CS NB |
| 31 | CAFY | 45.60 | -64.95 | FUNDY PARK CS NB |
| 32 | CYZX | 44.98 | -64.92 | GREENWOOD (CAN-MIL) NS |
| 33 | CWXS | 47.80 | -64.83 | BAS CARAQUET NB |
| 34 | CWPJ | 47.07 | -64.80 | PT. ESCUMINAC(MAPS) NB |
| 35 | CABT | 46.42 | -64.77 | BUCTOUCHE CDA CS NB |
| 36 | CYQM | 46.12 | -64.68 | MONCTON ARPT NB |
| 37 | CWWE | 43.98 | -64.67 | WESTERN HEAD (MARS) NS |
| 38 | CWMI | 48.02 | -64.50 | MISCOU ISL (MARS) NB |
| 39 | CYGP | 48.77 | -64.48 | GASPE ARPT QB |
| 40 | CXKT | 45.07 | -64.48 | KENTVILLE NS |
| 41 | CAPR | 45.42 | -64.35 | PARRSBORO NS |
| 42 | CWRZ | 48.42 | -64.32 | CAP D'ESPOIR (MAPS) QB |
| 43 | CWBY | 49.83 | -64.30 | PORT MENIER (MARS) QB |
| 44 | CXLB | 44.35 | -64.30 | LUNENBERG NS |
| 45 | CYPN | 49.83 | -64.28 | PORT MENIER (MAN) QB |
| 46 | CWAH | 45.84 | -64.26 | AMHERST NS |
| 47 | CXNP | 45.75 | -64.23 | NAPPAN AUTO NS |
| 48 | CWNE | 47.08 | -64.00 | NORTH POINT (AUTO8) PE |
| 49 | CWSD | 46.43 | -63.85 | SUMMERSIDE (AUTO8) PE |
| 50 | CYSU | 46.43 | -63.83 | SUMMERSIDE (CAN-MIL) PE |
| 51 | CXBP | 46.23 | -63.73 | CONFEDERATION BRIDGE PE |
| 52 | CWAZ | 44.72 | -63.68 | BEDFORD NS |
| 53 | CXMP | 46.30 | -63.58 | MAPLE PLAINS PE |
| 54 | CXMI | 44.60 | -63.53 | MCNABS ISLAND NS |
| 55 | CWAW | 44.63 | -63.52 | SHEARWATER JETTY NS |
| 56 | CYAW | 44.63 | -63.50 | SHEARWATER (CAN-MIL) NS |
| 57 | CYHZ | 44.88 | -63.50 | HALIFAX INTL ARPT NS |
| 58 | CZDB | 45.42 | -63.47 | DEBERT NS |
| 59 | CWUR | 45.37 | -63.27 | TRURO (MARS) NS |
| 60 | CAHR | 46.35 | -63.17 | HARRINGTON CDA CS PE |
| 61 | CYYG | 46.28 | -63.13 | CHARLOTTETOWN ARPT PE |
| 62 | CAOH | 45.23 | -63.05 | UPPER STEWIACKE RCS NS |
| # | Station | Lat | Lon | Station Location |
| 63 | CWBK | 45.76 | -62.68 | CARIBOU NS |
| 64 | CYTN | 45.61 | -62.62 | TRENTON MUNI ARPT NS |
| 65 | CZSP | 46.45 | -62.57 | ST PETERS PE |
| 66 | CXMY | 44.98 | -62.48 | MALAY FALLS NS |
| 67 | CWBV | 44.81 | -62.33 | BEAVER ISLAND (MAPS) NS |
| 68 | CWEP | 46.45 | -61.97 | EAST POINT (MARS) PE |
| 69 | CWGU | 45.87 | -61.90 | CAPE GEORGE (AUTO8) NS |
| 70 | CWGR | 47.42 | -61.80 | ILES-DE-LA-MADELEIN QB |
| 71 | CYGR | 47.42 | -61.78 | ILES DE LA MADELEIN QB |
| 72 | CWHP | 49.08 | -61.70 | HEATH POINT (MAPS) QB |
| 73 | CXTD | 45.61 | -61.68 | TRACADIE NS |
| 74 | CYPD | 45.67 | -61.38 | PORT HAWKESBURY NS |
| 75 | CWFU | 52.23 | -61.32 | LITTLE MACATINA NF |
| 76 | CWZQ | 46.53 | -61.08 | GRAND ETANG (AUTO8) NS |
| 77 | CWRN | 45.35 | -60.98 | HART ISLAND (MAPS) NS |
| 78 | CXNM | 46.82 | -60.67 | NORTH MOUNTAIN NS |
| 79 | CWXO | 43.88 | -60.55 | ROWAN GORILLA III NS |
| 80 | CWRW | 45.72 | -60.23 | FOURCHU HEAD (MAPS) NS |
| 81 | CWEF | 47.23 | -60.13 | ST PAUL ISL (MAPS) NS |
| 82 | CYQY | 46.17 | -60.05 | SYDNEY ARPT NS |
| 83 | CAQY | 46.17 | -60.05 | SYDNEY RCS NS |
| 84 | CWSA | 43.93 | -60.02 | SABLE ISLAND NS |
| 85 | CYSA | 43.93 | -60.00 | SABLE ISLAND (APT) NS |
| 86 | CXWR | 47.71 | -59.31 | WRECKHOUSE NF |
| 87 | CWOF | 47.57 | -59.18 | PORT BASQUES (AUTO) NF |
| 88 | CWZB | 47.56 | -59.16 | PORT-AUX-BASQUES, NF |
| 89 | CYJT | 48.53 | -58.55 | STEPHENVILLE ARPT NF |
| 90 | CXRH | 49.57 | -57.88 | ROCKY HARBOUR CS NF |
| 91 | CWBF | 47.62 | -57.63 | BURGEO (AUTO) NF |
| 92 | CWBD | 47.61 | -57.61 | BURGEO NF |
| 93 | CWDX | 50.23 | -57.60 | DANIEL’S HARBOUR (AUTO) NF |
| 94 | CWDH | 50.24 | -57.58 | DANIEL'S HARBOUR NF |
| 95 | CYDF | 49.22 | -57.40 | DEER LAKE ARPT NF |
| 96 | CWHU | 48.58 | -57.23 | STAR BROOK NF |
| 97 | CWXI | 51.02 | -57.10 | FEROLLE PT. (MAPS) NF |
| 98 | CWDA | 50.72 | -56.12 | ENGLEE (MAPS) NF |
| 99 | CWDW | 51.38 | -56.10 | ST ANTHONY NF |
| 100 | CYAY | 51.38 | -56.09 | ST ANTHONY ARPT NF |
| 101 | CWDI | 48.97 | -56.07 | BADGER (MARS) NF |
| 102 | CWZN | 47.37 | -55.80 | SAGONA ISL (MAPS) NF |
| 103 | CWAG | 49.91 | -55.66 | LA SCIE NF |
| 104 | CWAX | 51.37 | -55.63 | ST ANTHONY NF |
| 105 | CWFY | 48.17 | -55.48 | CONNE RIVER NF |
| 106 | CWDS | 46.92 | -55.38 | ST LAWRENCE NF |
| 107 | CXWT | 47.14 | -55.33 | WINTERLAND NF |
| 108 | CWHW | 49.27 | -54.88 | COMFORT COVE NF |
| 109 | CWDO | 49.68 | -54.80 | TWILLINGATE (MAPS) NF |
| 110 | CXGD | 48.94 | -54.58 | GANDER ARPT CS NF |
| 111 | CYQX | 48.95 | -54.57 | GANDER INTL ARPT NF |
| 112 | CWAR | 47.29 | -54.00 | ARGENTIA NF |
| 113 | CXTP | 48.56 | -53.97 | TERRA NOVA NATL PARK CS NF |
| 114 | CWXJ | 47.82 | -53.90 | BULL ARM AUTO8 NF |
| 115 | CWZY | 47.82 | -53.87 | GBS PLATFORM NF |
| 116 | CWYI | 49.12 | -53.58 | POOLS ISLAND NF |
| 117 | CWFH | 46.72 | -53.48 | ST SHOTTS NF |
| 118 | CXSA | 47.26 | -53.29 | SALMONIER NATURE PARK NF |
| 119 | CWVA | 48.67 | -53.12 | BONAVISTA NF |
| 120 | CWRA | 46.65 | -53.07 | CAPE RACE (MARS) NF |
| 121 | CWWU | 47.52 | -52.98 | LONG POND NF |
| 122 | CWVW | 48.17 | -52.93 | GRATES COVE (AUTO) NF |
| 123 | CXSW | 47.52 | -52.78 | ST JOHN'S WEST CDA CS NF |

B-3 ASOS/METAR Platform Locations (cont’d)

124

CYYT

47.62

-52.73

ST JOHN'S ARPT NF

| # | Station | Lat | Lon |  |
| --- | --- | --- | --- | --- |
| 1 | CTPP | 18.57 | -72.32 |  |
| 2 | KQAR | 18.58 | -72.30 |  |
| 3 | MDBH | 18.22 | -71.10 |  |
| 4 | MDHE | 18.47 | -69.97 |  |
| 5 | MDLR | 18.42 | -68.95 |  |
| 6 | MDPC | 18.57 | -68.37 |  |
| 7 | MDPP | 19.75 | -70.55 |  |
| 8 | MDSD | 18.43 | -69.67 |  |
| 9 | MDST | 19.47 | -70.70 |  |
| 10 | MKJP | 17.93 | -76.78 |  |
| 11 | MKJS | 18.50 | -77.92 |  |
| 12 | MTCH | 19.75 | -72.18 |  |
| 13 | MTPP | 18.57 | -72.30 |  |
| 14 | MUBA | 20.35 | -74.50 |  |
| 15 | MUBY | 20.40 | -76.62 |  |
| 16 | MUCA | 21.78 | -78.78 |  |
| 17 | MUCC | 22.45 | -78.32 |  |
| 18 | MUCF | 22.15 | -80.40 |  |
| 19 | MUCL | 21.62 | -81.55 |  |
| 20 | MUCM | 21.42 | -77.85 |  |
| 21 | MUCU | 19.97 | -75.85 |  |
| 22 | MUGM | 19.90 | -75.13 |  |
| 23 | MUGT | 20.08 | -75.15 |  |
| 24 | MUHA | 22.98 | -82.40 |  |
| 25 | MUHG | 20.78 | -76.32 |  |
| 26 | MUMO | 20.65 | -74.92 |  |
| 27 | MUMZ | 20.33 | -77.12 |  |
| 28 | MUNG | 21.83 | -82.78 |  |
| 29 | MUPR | 22.42 | -83.68 |  |
| 30 | MUSC | 22.50 | -79.95 |  |
| 31 | MUVR | 23.13 | -81.28 |  |
| 32 | MUVT | 20.95 | -76.95 |  |
| 33 | MWCB | 19.68 | -79.89 |  |
| 34 | MWCR | 19.28 | -81.35 |  |
| 35 | MYEF | 23.57 | -75.89 |  |
| 36 | MYEG | 23.50 | -75.77 |  |
| 37 | MYIG | 20.95 | -73.68 |  |
| # | Station | Lat | Lon |  |
| 38 MYSM | 24.05 | -74.53 |  |  |
| 39 | TAPA | 17.12 | -61.78 |  |
| 40 | TBPB | 13.07 | -59.48 |  |
| 41 | TDCF | 15.53 | -61.40 |  |
| 42 | TDPD | 15.53 | -61.30 |  |
| 43 | TDPR | 15.30 | -61.40 |  |
| 44 | TFFF | 14.60 | -61.00 |  |
| 45 | TFFJ | 17.90 | -62.85 |  |
| 46 TFFR | 16.27 | -61.52 |  |  |
| 47 | TGPY | 12.00 | -61.78 |  |
| 48 | TIST | 18.33 | -64.97 |  |
| 49 | TISX | 17.70 | -64.80 |  |
| 50 | TJBQ | 18.50 | -67.13 |  |
| 51 | TJMZ | 18.27 | -67.15 |  |
| 52 | TJNR | 18.25 | -65.63 |  |
| 53 | TJPS | 18.02 | -66.57 |  |
| 54 | TJSJ | 18.43 | -66.00 |  |
| 55 | TKPK | 17.30 | -62.68 |  |
| 56 | TKPN | 17.20 | -62.58 |  |
| 57 | TLPC | 14.02 | -61.00 |  |
| 58 | TLPL | 13.75 | -60.95 |  |
| 59 | TNCA | 12.50 | -70.02 |  |
| 60 TNCB | 12.15 | -68.28 |  |  |
| 61 TNCC | 12.20 | -68.97 |  |  |
| 62 | TQPF | 18.20 | -63.05 |  |
| 63 TRPM | 16.75 | -62.17 BLACKBURNE/PLYMOUTH AT |  |  |
| 64 | TSTT | 18.33 | -64.98 | CYRIL E. KING INTL ARPT, ST. TOMAS, USVI |
| 65 | TSTX | 17.7 | -64.80 | CHRISTIANSTED ARPT, |
| 66 | TTCP | 11.15 | -60.85 |  |
| 67 | TTPP | 10.62 | -61.35 |  |
| 68 | TTPT | 11.15 | -60.83 |  |
| 69 | TUPJ | 18.45 | -64.53 |  |

B-3 ASOS/METAR Platform Locations (cont’d)

Station Location

CAMP_CANARGUS HA

PORT_AU_PRINCE  HA

BARAHONA DR

HERRERA  DR

LA_ROMANA_INTL_ARPT  DR

PUNTA_CANA DR

PUERTO_PLATA_INTL DR

CAUCEDO/DE_LAS_AMER DR

SANTIAGO_INTL_ARPT DR

NORMAN_MANLEY/KINGS JM

SANGSTER/MONTEGO JM

CAP-HAITIEN_INTL HA

PORT-AU-PRINCE_ARPT HA

BARACOA_(CIV/MIL) CU

BAYAMO CU

CIEGO_AVILA/VENEZUE CU

CAYO_COCO/JARDINES_DEL_RAY

CIENFUEGOS(CIV/MIL)  CU

CAYO_LARGO_DEL_SUR CU

CAMAGUEY/IGANCIO CU

SANTIAGO_DE_CUBA CU

GUANTANAMO_BAY_NAS CU

GUANTANAMO CU

HAVANA/JOSE_MARTI CU

HOLGUIN_(CIV/MIL) CU

MOA_(MIL) CU

MANZANILLO CU

NUEVA_GERONA_(MIL) CU

PINAR_DEL_RIO_NORTE CU

ABEL_SANTAMARIA  CU

VARADERO CU

LAS_TUNAS/VICTORIA CU

GERRARD_SMITH_INTL/CAYMAN_

OWEN_ROBERTS_INTL GC

EXUMA_INTL BA

GEORGE_TOWN/EXUMA BA

MATTHEW_TOWN/INAGUA BA

Station Location

COCKBURN/SAN_SALVAD BA

COOLIDGE_ARPT AT

GRANTLEY_ADAMS_INTL BR

CANEFIELD_AIRPORT DO

MELVILLE_HALL_ARPT DO

ROSEAU DO

LE_LAMENTIN MR

GUSTAVIA/ST BARTHEL MF

LE_RAIZET/GUADELOUP MF

POINT_SALINES_INTL GD

C._AMALIE/CYRIL_E. VI

CHRISTIANSTED/ALEXA VI

AQUADILLA/BORINQUEN PU

MAYAGUEZ/EUGENIO PU

ROOSEVELT_ROADS_NAS PU

PONCE/MERCEDITA PU

LUIS MUNOZ ARPT, SAN JUAN PR

GOLDEN_ROCK AT

CHARLESTOWN/NEWCAST AT

CASTRIES/VIGIE LC

HEWANORRA_INTL_ARPT LC

REINA_BEATRIX_INTL NU

FLAMINGO_AIRPORT NU

HATO_ARPT_(CIV/MIL) NU

WALL_BLAKE VI

ST. CRIOX, USVI

CROWN_PT./SCARBOROU TD

PIARCO_INTL_ARPT TD

CROWN_POINT_ARPT TD

BEEF_ISL/ROADTOWN VI

70

TVSV

13.13

| -61.20 | ARNOS_VALE/KINGSTON LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| # | Station | Lat | Lon | Height |  |
| 1 | MMCN | 27.32 | -112.30 | 10 (m) |  |
| 2 | MMLT | 26.02 | -111.35 | 10 |  |
| 3 | MMHO | 29.07 | -110.97 | 10 |  |
| 4 | MMGM | 27.97 | -110.93 | 10 |  |
| 5 | MMLP | 24.07 | -110.37 | 10 |  |
| 6 | MMSD | 23.15 | -109.70 | 10 |  |
| 7 | MMLM | 25.68 | -109.08 | 10 |  |
| 8 | MMCL | 24.82 | -107.40 | 10 |  |
| 9 | MMMZ | 23.17 | -106.27 | 10 |  |
| 10 | MMCU | 28.70 | -105.97 | 10 |  |
| 11 | MMPR | 20.68 | -105.25 | 10 |  |
| 12 | MMEP | 21.52 | -104.90 | 10 |  |
| 13 | MMZO | 19.15 | -104.57 | 10 |  |
| 14 | MMDO | 24.13 | -104.53 | 10 |  |
| 15 | MMIO | 25.45 | -103.98 | 10 |  |
| 16 | MMIA | 19.27 | -103.58 | 10 |  |
| 17 | MMZP | 20.75 | -103.47 | 10 |  |
| 18 | MMTC | 25.53 | -103.45 | 10 |  |
| 19 | MMGL | 20.52 | -103.32 | 10 |  |
| 20 | MMZC | 22.90 | -102.68 | 10 |  |
| 21 | MMAS | 21.88 | -102.30 | 10 |  |
| 22 | MMLC | 17.98 | -102.22 | 10 |  |
| 23 | MMPN | 19.40 | -102.03 | 10 |  |
| 24 | MMLO | 21.00 | -101.48 | 10 |  |
| 25 | MMZH | 17.60 | -101.47 | 10 |  |
| 26 | MMMV | 26.88 | -101.42 | 10 |  |
| 27 | MMMM | 19.85 | -101.03 | 10 |  |
| 28 | MMSP | 22.15 | -100.98 | 10 |  |
| 29 | MMPG | 28.70 | -100.52 | 10 |  |
| 30 | MMQT | 20.60 | -100.38 | 10 |  |
| 31 | MMAN | 25.87 | -100.23 | 10 |  |
| # | Station | Lat | Lon | Height |  |
| 32 | MMMY | 25.78 | -100.1 | 10 (m) |  |
| 33 | MMAA | 16.77 | -99.75 | 10 |  |
| 34 | MMNL | 27.43 | -99.57 | 10 |  |
| 35 | MMTO | 19.35 | -99.57 | 10 |  |
| 36 | MMCB | 18.92 | -99.25 | 10 |  |
| 37 | MMMX | 19.43 | -99.10 | 10 |  |
| 38 | MMCV | 23.72 | -98.97 | 10 |  |
| 39 | MMTL | 20.08 | -98.37 | 10 |  |
| 40 | MMRX | 26.02 | -98.23 | 10 |  |
| 41 | MMPB | 19.03 | -98.20 | 10 |  |
| 42 | MMTM | 22.28 | -97.87 | 10 |  |
| 43 | MMMA | 25.77 | -97.53 | 10 |  |
| 44 | MMPA | 20.50 | -97.47 | 10 |  |
| 45 | MMPS | 15.87 | -97.08 | 10 |  |
| 46 | MMOX | 16.97 | -96.73 | 10 |  |
| 47 | MMBT | 15.78 | -96.27 | 10 |  |
| 48 | MMVR | 19.15 | -96.18 | 10 |  |
| 49 | MMIT | 16.45 | -95.08 | 10 |  |
| 50 | MMMT | 18.10 | -94.58 | 10 |  |
| 51 | MMTG | 16.75 | -93.12 | 10 |  |
| 52 | MMTB | 16.45 | -93.07 | 10 |  |
| 53 | MMVA | 18.00 | -92.82 | 10 |  |
| 54 | MMTP | 14.78 | -92.38 | 10 |  |
| 55 | MMCE | 18.65 | -91.80 | 10 |  |
| 56 | MMCP | 19.85 | -90.55 | 10 |  |
| 57 | MMMD | 20.98 | -89.65 | 10 |  |
| 58 | MMCT | 20.64 | -88.45 | 10 |  |
| 59 | MMCM | 18.50 | -88.30 | 10 |  |
| 60 | MMCZ | 20.53 | -86.93 | 10 |  |

B-3 ASOS/METAR Platform Locations (cont’d)

Station Location

SANTA ROSALIA

LORETO

HERMOSILLO INTL

GUAYMAS INTL ARPT

LA PAZ INTL ARPT

SAN JOSE DEL CABO

LOS MOCHIS ARPT

CULIACAN (CITY)

MAZATLAN/G BUELNA

CHIHUAHUA INTL ARPT

PUERTO VALLARTA/LIC

TEPIC

MANZANILLO INTL

DURANGO ARPT

SALTILLO

COLIMA

ZAPOPAN (MIL)

TORREON ARPT

DON MIGUEL/GUADALAJ

ZACATECAS ARPT

AGUASCALIENTES

LAZARO CARDENAS

URUAPAN/GEN RAYON

DEL BAJIO/LEON

IXTAPA-ZIHUATANEJO

MONCLOVA

MORELIA NEW

SAN LUIS POTOSI

PIEDRAS NEGRAS

QUERETARO

MONTERREY INTL ARPT

Station Location

MONTERREY/GEN MARIA

ACAPULCO/G. ALVAREZ

NUEVO LAREDO INTL

TOLUCA/JOSE MARIA

CUERNAVACA

MEXICO CITY/LICENCI

CIUDAD VICTORIA APT

TULANCINGO

REYNOSA INTL ARPT

PUEBLA

TAMPICO/GEN FJ MINA

MATAMOROS INTL

POZA RICA/PALIZADA

PUERTO ESCONDIDO

OAXACA/XOXOCOTLAN

BAHIAS DE HUATULCO

GEN. HERIBERTO JARA

IXTEPEC OX

MINATITLAN

TUXTLA GUTIERREZ A

TUXTLA GUTIERREZ

VILLAHERMOSA

TAPACHULA

CIUDAD DEL CARMEN

CAMPECHE/IGNACIO

MERIDA INTL ARPT

CHICHEN-ITZA

CHETUMAL

COZUMEL (CIV/MIL)

61

MMUN

21.03

| -86.87 | 10 | CANCUN INTL ARPT |  |  |
| --- | --- | --- | --- | --- |
|  |  | # | Station |  |
| Lon | Station Location | 1 | MGRT |  |
| -91.67 | RETALHULEU (MIL) GU | 2 | MGQZ |  |
| -91.52 | QUEZALTENANGO GU | 3 | MGHT |  |
| -91.47 | HUEHUETENANGO GU | 4 | MGSJ |  |
| -90.82 | SAN JOSE (CIV/MIL) GU | 5 | MGGT |  |
| -90.52 | GUATEMALA/LA AURORA GU | 6 | MGCB |  |
| -90.32 | COBAN GU | 7 | MGFL |  |
| -89.88 | FLORES/SANTA ELENA GU | 8 | MGTK |  |
| -89.85 | TIKAL GU | 9 | MGZA |  |
| -89.53 | ZACAPA GU | 10 | MHSR |  |
| -88.78 | SANTA ROSA DE COPAN HO | 11 | MGPB |  |
| -88.60 | PUERTO BARRIOS(MIL) GU | 12 | MZBZ |  |
| -88.30 | BELIZE INTL ARPT BH | 13 | MHLE |  |
| -88.17 | LA ESPERANZA HO | 14 | MHLM |  |
| -87.93 | LA MESA/PEDRO SULA HO | 15 | MHAM |  |
| -87.67 | AMAPALA/LOS PELONAS HO | 16 | MHSC |  |
| -87.62 | SOTO CANO AB HO | 17 | MHTE |  |
| -87.48 | TELA HO | 18 | MHTG |  |
| -87.22 | TEGUCIGALPA/TONCONT HO | 19 | MHCH |  |
| -87.18 | CHOLUTECA HO | 20 | MNCH |  |
| -87.13 | CHINANDEGA NK | 21 | MHYR |  |
| -87.12 | YORO HO | 22 | MHLC |  |
| -86.87 | LA CEIBA/GOLOSON HO | 23 | MHRO |  |
| -86.53 | ROATAN HO | 24 | MNMG |  |
| -86.17 | MANAGUA/AUGUSTO CES NK | 25 | MHNO |  |
| -86.07 | GUANAJA HO | 26 | MNJG |  |
| -85.98 | JINOTEGA NK |  |  |  |
|  |  | # | Station |  |
| Lon | Station Location | 27 | MHTR |  |
| -85.93 | TRUJILLO HO | 28 | MHNJ |  |
| -85.92 | GUANAJA HO | 29 | MHCA |  |
| -85.91 | CATACAMAS HO | 30 | MNRS |  |
| -85.83 | RIVAS NK | 31 | MRLB |  |
| -85.55 | LIBERIA/TOMAS GUARD CS | 32 | MRNC |  |
| -85.45 | NICOYA CS | 33 | MNJU |  |
| -85.37 | JUIGALPA NK | 34 | MRCH |  |
| -84.78 | CHACARITA CS | 35 | MROC |  |
| -84.22 | SAN JOSE/SANTAMARIA CS | 36 | MRPV |  |
| -84.15 | TOBIAS BOLANOS INTL CS | 37 | MHIC |  |
| -83.93 | ISLAS DEL CISNE HO | 38 | MHPL |  |
| -83.80 | PUERTO LEMPIRA HO | 39 | MNBL |  |
| -83.77 | BLUEFIELDS NK | 40 | MRPM |  |
| -83.47 | PALMAR SUR CS | 41 | MNPC |  |
| -83.37 | PUERTO CABEZAS(MIL) NK | 42 | MRLM |  |
| -83.05 | LIMON INTL ARPT CS | 43 | MPCH |  |
| -82.52 | CAPTAIN MANUEL NINO PM | 44 | MPDA |  |
| -82.42 | DAVID/ENRIQUE MALEK PM | 45 | MPBO |  |
| -82.25 | BOCAS DEL TORO INTL PM | 46 | MPSA |  |
| -80.95 | SANTIAGO PM | 47 | MPNU |  |
| -80.28 | AUGUSTO VERGARA PM | 48 | MPCF |  |
| -79.87 | ENRIQUE ADOLFO JIME PM | 49 | MPHO |  |
| -79.60 | HOWARD AFB PM | 50 | MPMG |  |
| -79.52 | MARCOS A. GELABERT PM | 51 | MPTO |  |

B-3 ASOS/METAR Platform Locations (cont’d)

Lat

14.53

14.83

15.32

13.92

14.58

15.48

16.92

16.90

14.97

14.78

15.72

17.53

14.33

15.45

13.28

14.38

15.72

14.05

13.30

12.63

15.17

15.73

16.32

12.15

16.47

13.08

Lat

15.93

16.47

14.90

11.42

10.60

10.15

12.10

9.98

10.00

9.95

17.40

15.22

12.00

8.95

14.05

10.00

9.43

8.40

9.35

8.08

7.83

9.35

8.92

8.98

9.05

-79.37

TOCUMEN/GEN. OMAR PM

52

MPLP

8.40

-78.13

LA PALMA PM

Appendix C: DOD/NWS RAWIN/RAOB and NWS Coastal Land-based Radar Locations

APPENDIX D: PRINCIPAL DUTIES OF THE NOAA SCIENTIFIC PERSONNEL

CAUTION

Flight operations are routinely conducted in turbulent conditions. Shock-mounted electronic and experimental racks surround most seat positions. Therefore, for safety onboard the aircraft all personnel should wear a flight suit and closed toed shoes. For comfort, personnel should bring a jacket or sweater, as the cabin gets cold during flight.

Smoking is prohibited within 50 ft of the aircraft while they are on the ground. No smoking is permitted on the aircraft at any time.

Section 4-401, of the NOAA Safety Rules Manual state that:  “Don’t let your attention wander, either through constant conversation, use of cell phone or sightseeing while operating vehicles.  Drivers must use caution and common sense under all conditions. Operators and passengers are not permitted to smoke or eat in the government vehicles. Cell phone use is permitted while car is parked.”

GENERAL INFORMATION FOR ALL SCIENTIFIC MISSION PARTICIPANTS

Mission participants are advised to carry the proper personal identification [i.e., travel orders, "shot" records (when appropriate), and passports (when required)]. Passports will be checked by AOC personnel prior to deployment to countries requiring it. All participants must provide their own meals for in-flight consumption. AOC provides a refrigerator, microwave, coffee, utensils, condiments, ice, water, and soft drinks for a nominal fee per flight.

D.1 Field Program Director/ IFEX Chief Scientist;

(1) Responsible to the HRD director for the implementation of the Hurricane Field Program Plan.

(2) Only official communication link to AOC. Communicates flight requirements and changes in mission to AOC.

(3) Only formal communication link between AOML and CARCAH during operations. Coordinates scheduling of each day's operations with AOC only after all (POD) reconnaissance requirements are completed between CARCAH and AOC.

(4) Convenes the Hurricane Field Program Operations Advisory Panel. This panel selects missions to be flown.

(5) Provides for pre-mission briefing of flight crews, scientists, and others (as required).

(6) Assigns duties of field project scientific personnel. Ensures safety during the field program.

(7) Coordinates press statements with NOAA/Public Affairs.

D.2 Assistant Field Program Director

(1) Assumes the duties of the field program director in his absence.

D.3 Miami Ground Operations Center: Senior Team Leader

(1) During operations, the MGOC senior team leader is responsible for liaison between HRD base and field personnel and other organizations as requested by the field program director, the director of HRD, or their designated representatives.

D.4 Named Experiment Lead Project Scientist

(1) Has overall responsibility for the experiment.

(2) Coordinates the project and sub-project requirements.

(3) Determines the primary modes of operation for appropriate instrumentation.

(4) Assists in the selection of the mission.

(5) Provides a written summary of the mission to the field program director (or his designee) at the experiment's debriefing.

D.5 Lead Project Scientist

(1) Has overall scientific responsibility for his/her aircraft.

(2) Makes in-flight decisions concerning alterations of: (a) specified flight patterns; (b) instrumentation operation; and (c) assignment of duties to on-board scientific project personnel.

(3) Acts as project supervisor on the aircraft and is the focal point for all interactions of project personnel with operational or visiting personnel.

(4) Conducts preflight and post flight briefings of the entire crew. Completes formal checklists of safety, instrument operations - noting malfunctions, problems, etc.

(5) Provides a written report of each mission day's operations to the field program director at the mission debriefing.

D.6 Cloud Physics Scientist

(1) Has overall responsibility for the cloud physics project on the aircraft.

(2) Briefs the on-board lead project scientist on equipment status before takeoff.

(3) Determines the operational mode of the cloud physics sensors (i.e., where, when, and at what rate to sample).

(4) Operates and monitors the cloud physics sensors and data systems.

(5) Provides a written preflight and post flight status report and flight summary of each mission day's operations to the on-board lead project scientist at the post flight debriefing.

D.7 Boundary-Layer Scientist

(1) Insures that the required number of AXCPs, AXBTs, and AXCTDs are on the aircraft for each mission.

(2) Operates the AXCP, AXBT, and AXCTD equipment (as required) on the aircraft.

(3) Briefs the on-board lead project scientist on equipment status before takeoff.

(4) Determines where and when to release the AXCPs, AXBTs, and AXCTDs (as appropriate) subject to clearance by flight crew.

(5) Performs preflight, inflight, and post flight checks and calibrations.

(6) Provides a written preflight and post flight status report and a flight summary of each mission day's operations to the on-board lead project scientist at the post flight debriefing.

D.8 Radar Scientist

(1) Determines optimum meteorological target displays. Continuously monitors displays for performance and optimum mode of operations. Thoroughly documents modes and characteristics of the operations.

(2) Provides a summary of the radar display characteristics to the on-board lead project scientist at the post flight debriefing.

(3) Maintains tape logs.

(4) During the ferry to the storm, the radar scientist should record a tape of the sea return on either side of the aircraft at elevation angles varying from -20� through +20�. This tape will allow correction of any antenna mounting biases or elevation angle corrections.

D.9 Dropsonde Scientist

(1) Processes dropsondes observations on HRD workstation for accuracy.

(2) Provides TEMP drop message for ASDL, transmission or insures correct code in case of automatic data transmission.

D.10 Workstation Scientist

(1) Operates HRD's workstation.

(2) Runs programs that determine wind center and radar center as a function of time, composite flight-level and radar reflectivity relative to storm center and then process and code dropwindsonde observations.

(3) Checks data for accuracy and sends appropriate data to ASDL computer.

(4) Maintains records of the performance of the workstation and possible software improvements.

APPENDIX E: NOAA RESEARCH OPERATIONAL PROCEDURES AND CHECK LISTS

- rricane Field Program Deployment Safety Checklist

- e Field Program Director is responsible for making sure safety is enforced and ensuring necessary materials are in place and/or any actions have been completed before the start of the HFP.  Field program participants are responsible for reviewing this checklist.

- ientist_______________________________       Date ______________

Before leaving AOML

_____ 1. Contact MGOC personnel to notify departure time.

_____ 2. Things to take

- Flight bag (s)

- Cell phone

- List of HFP important numbers

d. HRD Field program plan

Flight suit

Ground transportation

_____ 1. Arrange for ground transportation

- ___ 2. Visual inspection of government vehicle

- Make sure tires do not appear to be flat

- Check for any cracked/broken lights, windshield and mirrors

- Check for any major dents around the vehicle

- ___ 3. Inspection inside the government vehicle

a. Check all lights work properly (head and tail lights, dome lights, dashboard and turn signal lights)

b. Make sure the engine, oil, or temperature indicator lights does not flash.  If so, contact facilities management.

c. Note the gas and mileage

_____ 4. Contents inside the government vehicle

a. Make sure there is first aid kit and fire extinguisher

b. Proper jack and lug wrench

c. Spare tire

d. Basic auto repair kit (i.e. road hazard reflector or flares)

e. Consider carrying a flashlight

_____ 5. If possible, return vehicle with full tank (regular unleaded gasoline)

_____ 6. Contact MGOC personnel upon returning

E.1  "Conditions-of-Flight" Commands

Mission participants should be aware of the designated "conditions-of-flight." There are five designated basic conditions of readiness encountered during flight. The pilot will set a specific condition and announce it to all personnel over the aircraft's PA (public address) and ICS (interphone communications systems). All personnel are expected to act in accordance with the instructions for the specific condition announced by the pilot. These conditions and appropriate actions are shown below.

CONDITION 1:  TURBULENCE/PENETRATION. All personnel will stow loose equipment and fasten safety belts.

CONDITION 2: HIGH ALTITUDE TRANSIT/FERRY. There are no cabin stations manning requirements.

CONDITION 3: NORMAL MISSION OPERATIONS. All scientific and flight crew stations are to be manned with equipment checked and operating as dictated by mission requirements. Personnel are free to leave their ditching stations.

CONDITION 4: AIRCRAFT INSPECTION. After take-off, crew members will perform wings, engines, electronic bays, lower compartments, and aircraft systems check. All other personnel will remain seated with safety belts fastened and headsets on.

- NDITION 5: TAKE-OFF/LANDING. All personnel will stow or secure loose equipment, don headsets, and fasten safety belts/shoulder harnesses.

E.2  Lead Project Scientist

E.2.1 Preflight

_____ 1. Participate in general mission briefing.

_____ 2. Determine specific mission and flight requirements for assigned aircraft.

_____ 3. Determine from field program director whether aircraft has operational fix responsibility and discuss with AOC flight director/meteorologist unless briefed otherwise by field program director.

_____ 4. Contact HRD members of crew to:

a. Assure availability for mission.

b. Review field program safety checklist

c. Arrange ground transportation schedule when deployed.

d. Determine equipment status.

_____ 5. Meet with AOC flight director and navigator at least 3 hours before take-off for initial briefing.

_____ 5. Meet with AOC flight crew at least 2 hours before take-off for crew briefing. Provide copies of flight requirements and provide a formal briefing for the flight director, navigator, and pilots.

_____ 6. Report status of aircraft, systems, necessary on-board supplies and crews to appropriate HRD operations center (MGOC in Miami).

_____ 7. Before take-off, brief the on-board GPS dropsonde operator on times and positions of drop times.

_____ 7. Make sure each HRD flight crew members have life vests

_____ 7. Perform a headset operation check with all HRD flight crew members. Make sure everyone can hear and speak using the headset.

_____ 8. Collect “mess” fee ($2.00) from all on-board HRD flight crew members.

E.2.2 In-Flight

_____ 1. Confirm from AOC flight director that satellite data link is operative (information).

_____ 2. Confirm camera mode of operation.

_____ 3. Confirm data recording rate.

_____ 4. Complete Lead Project Scientist Form.

_____ 5. Check in with the flight director to make sure the mission is going as planned (i.e. turns are made when they are supposed to be made).

E.2.3 Post flight

_____ 1. Debrief scientific crew.

_____ 2. Report landing time, aircraft, crew, and mission status along with supplies (tapes, etc.) remaining aboard the aircraft to MGOC.

_____ 3. Gather completed forms for mission and turn in at the appropriate operations center. [

Note

: all data removed from the aircraft by HRD personnel should be cleared with the AOC flight director.]

_____ 4. Obtain a copy of the 10-s flight listing from the AOC flight director. Turn in with completed forms.

_____ 5. Obtain a copy of the radar DAT tapes and if possible a copy of the radar data-packet files should be copied onto a flash drive.  Turn in with completed forms.

_____ 6. Obtain a copy of the all VHS videos form aircraft cameras (3-4 approx.). Turn in with completed forms.

_____ 7. Obtain a copy of CD with all flight data. Turn in with completed forms.

_____ 8. Determine next mission status, if any, and brief crews as necessary.

_____ 9. Notify MGOC as to where you can be contacted and arrange for any further coordination required.

_____ 10. Prepare written mission summary using Mission Summary form (due to Field Program Director 1 week after the flight).

|  |  | Form E-2 |  |  | Page 1 of 5 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Date _______________ Aircraft ______________ Flight ID ____________________ |  |  | A. —Participants: |  |  |
|  | AOC |  | Function | Participant |  |  |
| Function | Participant |  | Lead Project Scientist |  |  |  |
|  | Radar |  |  |  |  |  |
|  |  |  | Workstation |  |  |  |
| Navigator |  |  | Cloud Physics |  |  |  |
| Systems Engineer |  |  | Photographer/Observer/Guests |  |  |  |
| Data Technician |  |  | Dropwindsonde |  |  |  |

Lead Project Scientist Check List

HRD

Flight Director

Pilots

Electronics Technician

AXBT/AXCP

Other

|  | B. Take-off and Landing Locations: | Take-Off: ________ Location: _______________ | Landing: ________ Location: _________________ |  |
| --- | --- | --- | --- | --- |
|  | C. Past and Forecast Storm Locations: |  | Date/Time | Latitude |
| MSLP | Maximum Wind |  |  |  |

Number of Eye Penetrations: ______

Longitude

|  |  | D. Mission Briefing: | Form E-2 Page 2 of 5 |  |
| --- | --- | --- | --- | --- |
| E. Equipment Status (Up ↑, Down ↓, Not Available —, Not Used O) |  | Equipment | Pre-Flight | In-Flight |
| # DATs / Cds | /Expendables/Printouts | Radar/LF |  |  |
|  |  | Doppler Radar/TA |  |  |
|  |  | Cloud Physics |  |  |
|  |  | Data System |  |  |
|  |  | GPS sondes |  |  |
|  |  | AXBT/AXCP |  |  |
|  |  | Ozone instrument |  |  |
|  |  | Workstation |  |  |

Post-Flight

Videography

REMARKS:

Form E-2

Page 3 of 5

Mission Summary

Storm name

YYMMDDA# Aircraft 4 _RF

Scientific Crew (4 RF)

Lead Project Scientist

Radar Scientist

Cloud Physics Scientist

Dropwindsonde Scientist

Boundary-Layer Scientist

Workstation Scientist

Observers

Mission Briefing: (include sketch of proposed flight track or page #)

Mission Synopsis: (include plot of actual flight track)

Evaluation: (did the experiment meet the proposed objectives?)

Problems:(list all problems)

Expendables used in mission:

GPS sondes : ________________

AXBTs :__________________

| Sonobuoys: _________________ | Form E-2 | Page 5 of 5 |  |
| --- | --- | --- | --- |
|  | Date ________________ Flight ________________ LPS _____________________ |  |  |
| Event | Position | Comments |  |

Lead Project Scientist Event Log

Time

E.3 Cloud Physics Scientist

The on-board cloud physics scientist (CPS) is responsible for cloud physics data collection on his/her assigned aircraft. Detailed operational procedures are contained in the cloud physics kit supplied for each aircraft. General procedures follow. (Check off and initial.)

E.3.1 Preflight

_____ 1. Determine status of cloud physics instrumentation systems and report to the on-board lead project scientist (LPS).

_____ 2. Confirm mission and pattern selection from the on-board LPS.

_____ 3. Select mode of instrument operation.

_____ 4. Complete appropriate instrumentation preflight check lists as supplied in the cloud physics operator's manual.

E.3.2 In-Flight

_____ 1. Operate instruments as specified in the cloud physics operator's manual and as directed by the on-board LPS.

E.3.3 Post flight

_____ 1. Complete summary checklist forms and all other appropriate forms.

_____ 2. Brief the LPS on equipment status and turn in completed check sheets to the LPS.

_____ 3. Take cloud physics data tapes and other data forms and turn these data sets in as follows:

a. Outside of Miami-to the LPS.

b. In Miami-to AOML/HRD. [Note: all data removed from the aircraft by HRD personnel should be cleared with the AOC flight director.]

_____ 4. Debrief as necessary at MGOC or the hotel during a deployment.

_____ 5. Determine the status of future missions and notify MGOC as to where you can be contacted.

Form E-3

Page 1 of 2

|  | Cloud Physics Scientist Check List |  |  |
| --- | --- | --- | --- |
|  |  | A. —Instrument Status and Performance: |  |
| Pre-Flight | In-Flight | Downtime |  |

Date ___________________ Aircraft ____________ Flight ID __________________

System

PMS Probes 2D-P

PMS Probes 2D-C

PMS Probes FSSP

Data System

DRI Field Mills

King Probe

NCAR/NOAA CIP

NCAR PIP

NCAR FSSP

B. —Remarks:

E.4 Boundary-Layer Scientist

The on-board boundary-layer scientist (BLS) is responsible for data collection from AXBTs, AXCPs, AXCTDs, BUOYs, and SST radiometers (if these systems are used on the mission). Detailed calibration and instrument operation procedures are contained in the air-sea interaction (ASI) manual supplied to each operator. General supplementary procedures follow. (Check off and initial.)

E.4.1 Preflight

_____ 1. Determine the status of equipment and report results to the on-board lead project scientist (LPS).

_____ 2. Confirm mission and pattern selection from the LPS.

_____ 3. Select the mode of operation for instruments after consultation with the HRD/BLS and the LPS.

_____ 4. Complete appropriate preflight check lists as specified in the ASI manual and as directed from the LPS.

E.4.2 In-Flight

_____ 1. Operate the instruments as specified in the ASI manual and as directed by the on-board LPS.

E.4.3 Post flight

_____ 1. Complete summary checklist forms and all other appropriate forms.

_____ 2. Brief the on-board LPS on equipment status and turn in completed checklists to the LPS.

_____ 3. Debrief as necessary at MGOC or the hotel during a deployment.

_____ 4. Determine the status of future missions and notify MGOC as to where you can be contacted.

Form E-4

Page 1 of 2

AXBT and Sonobuoy Check Sheet Summary

Flight _________________ Aircraft _____________ Operator _________________

Number

(1) Probes dropped _______

(2) Failures _______

(3) Failures with no signal _______

(4) Failures with sea surface temperature, but terminated above thermocline _______

(5) Probes that terminated above 250 m, but below thermocline  _______

(6) Probes used by channel number CH12   _______

CH14   _______

CH16   _______

CH__   _______

NOTES:

E.5 Radar Scientist

The on-board radar scientist is responsible for data collection from all radar systems on his/her assigned aircraft. Detailed operational procedures and checklists are contained in the operator's manual supplied to each operator. General supplementary procedures follow. (Check off and initial.)

E.5.1 Preflight

_____ 1. Determine the status of equipment and report results to the lead project scientist (LPS).

_____ 2. Confirm mission and pattern selection from the LPS.

_____ 3. Select the operational mode for radar system(s) after consultation with the LPS.

_____ 4. Complete the appropriate preflight calibrations and check lists as specified in the radar operator's manual.

E.5.2 In-Flight

_____ 1. Operate the system(s) as specified in the operator's manual and as directed by the LPS or as required for aircraft safety as determined by the AOC flight director or aircraft commander.

_____ 2. Maintain a written commentary in the radar logbook of tape and event times, such as the start and end times of F/AST legs. Also document any equipment problems or changes in R/T, INE, or signal status.

E.5.3 Post flight

_____ 1. Complete the summary checklists and all other appropriate check lists and forms.

_____ 2. Brief the LPS on equipment status and turn in completed forms to the LPS.

_____ 3. Hand-carry all radar tapes and arrange delivery as follows:

a. Outside of Miami-to the LPS.

b. In Miami-to MGOC or to AOML/HRD. [Note: all data removed from the aircraft by HRD personnel should be cleared with the AOC flight director.]

_____ 4. Debrief at MGOC or the hotel during a deployment.

_____ 5. Determine the status of future missions and notify MGOC as to where you can be contacted.

Form E-5

Page 1 of 2

HRD Radar Scientist Check List

Flight ID:

Aircraft Number:

Radar Operators:

Radar Technician:

Number of digital magnetic tapes on board:

Component Systems Status:

MARS __________________________ Computer ____________________________

DAT1___________________________ DAT2 _______________________________

LF _____________________________ R/T Serial # ___________________________

TA _____________________________ R/T Serial # ___________________________

Time correction between radar time and digital time:

Radar Post flight Summary

Number of digital tapes used: DAT1 _______________________

DAT2 _______________________

Significant down time:

DAT1 ______________________ Radar LF ____________________

DAT2 ______________________ Radar TA ____________________

Other Problems:

Form E-5

Page 2 of 2

|  | HRD Radar Tape Log | Flight ______________ Aircraft _______ Operator _______________ Sheet ____ of ____ | LF RPM _______________ TA RPM ______________ |
| --- | --- | --- | --- |
| (Include start and end times of DATs, as well as times of F/AST legs and any changes of radar equipment status) |  | Tape # | F/AST |
| Event Time | (HHMMSS) | Event |  |

On?

Item List: DAT1, DAT2, COMP, MARS, LF, and TA.

Include serial numbers of any new R/Ts.

E.6  Dropsonde Scientist

The lead project scientist (LPS) on each aircraft is responsible for determining the distribution patterns for dropwindsonde releases. Predetermined desired data collection patterns are illustrated on the flight patterns. However, these patterns often are required to be altered because of clearance problems, etc. Operational procedures are contained in the operator's manual. The following list contains more general supplementary procedures to be followed. (Check off and initial.)

E.6.1 Preflight

_____ 1. Determine the status of the AVAPS and HAPS.  Report results to the LPS.

_____ 2. Confirm the mission and pattern selection from the LPS and assure that enough dropsondes are on board the aircraft.

_____ 3. Modify the flight pattern or drop locations if requested by AOC to accommodate changes in storm location or closeness to land.

_____ 4. Complete the appropriate preflight set-up and checklists.

E.6.2 In-Flight

_____ 1. Operate the system as specified in the operator's manual.

_____ 2. Ensure the AOC flight director is aware of upcoming drops.

_____ 3. Ensure the AVAPS operator has determined that the dropsonde is (or is not) transmitting a good signal.  Recommend if a backup dropsonde should be launched in case of failure.

_____ 4. Report the transmission of each drop and fill in the Dropwindsonde Scientist Log.

E.6.3 Post flight

_____ 1. Complete Dropwindsonde Scientist Log.

_____ 2. Brief the LPS on equipment status and turn in reports and completed forms.

_____ 3. Hand-carry all dropwindsonde data tapes or CDs as follows:

a. Outside of Miami-to the LPS or PI.

b. In Miami-to AOML/HRD. [Note: all data removed from the aircraft by HRD personnel should be cleared with the AOC flight director.]

_____ 4. Debrief at the MGOC or the hotel during a deployment.

| _____ 5. Determine the status of future missions and notify MGOC as to where you can be contacted. |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Table F-1 Systems of measure: Units, symbols, and definitions |  | Quantity | SI Unit |
| Maritime | English | length meter (m) | centimeter (cm) |  |
| foot (ft) | distance | meter (m) | kilometer (km) |  |
| mile (nm) | mile (mi) | depth | meter (m) |  |
| fathom (fa) | foot (ft) | mass | kilogram (kg) |  |
|  |  | time | second (s) |  |
| second (s) | second (s) | speed | meter per second (mps) |  |
| knot (kt) (nm h-1) miles per hour (mph) |  |  | kilometers per hour (km h-1) |  |
|  | temperature | -sensible | degree Celsius (�C) |  |
| --- | degree Fahrenheit (�F) | -potential Kelvin (K) | Kelvin (K) | --- |
| force Newton (N) | (kg m s-2) dyne (dy) | (g cm s-2) poundal (pl) | poundal (pl) |  |

APPENDIX F: SYSTEMS OF MEASURE AND UNIT CONVERSION FACTORS

Early Metric

foot (ft)

nautical

meter (m)

gram (g)

second (s)

centimeter per second (cm s-1)

degree Celsius (�C)

Kelvin (K)

pressure

Pascal (Pa)

(N m-2)

millibar (mb)

(103 dy cm-2)

| inches (in) | mercury (Hg) | inches (in) |
| --- | --- | --- |
| Table F-2. Unit conversion factors | Parameter Unit |  |
| length | 1 in 1 ft 1 m |  |
| 30.480 cm | 3.281 ft | distance |
| 1.151 mi | 1.852 km | 6080 ft |
| 1.609 km | 5280 ft |  |
| 59.996 nm | 69.055 mi | 111.136 km |
| depth | 1 fa | 6 ft |
| mass | 1 kg 2.2 lb |  |
| force | 1 N | 105 dy |
| pressure | 1 mb | 102 Pa 0.0295 in Hg |
|  | 1 lb ft-2 | 4.88 kg m-2 |

mercury (Hg)

Conversions

2.540 cm

1 nm (nautical mile)

1 mi (statute mile)

1� latitude

1.829 m

speed

1 m s-1

1.9

| at. 6 h-1 | 10 kt |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| APPENDIX G: AIRCRAFT SCIENTIFIC INSTRUMENTATION | Instrument | Parameter | PI | Group | Electronics Location |
| Navigational |  |  |  |  |  |
| INE1/2 | lat, lon |  | AOC |  |  |
| GPS1/2 | lat, lon |  | AOC |  |  |
| Honeywell HG9550 altimeter | Radar altitude |  | AOC |  |  |
| Standard Met. |  |  |  |  |  |
| Buck1101c, Edgetech Vigilant, Maycom TDL | Td |  | AOC |  |  |
| Rosemount temp | T, T' |  | AOC |  |  |
| Static pressure | p |  | AOC |  |  |
| Dynamic pressure | p' |  | AOC |  |  |
| Horizontal wind | Vh |  | AOC |  |  |
| Vertical wind | w |  | AOC |  |  |
| Infrared Radiation |  |  |  |  |  |
| Side CO2 radiometer | T |  | AOC |  |  |
| AOC down radiometer | SST |  | AOC | Under floor |  |
| Weather Radar |  |  |  |  |  |
| LF radar | R | Gamache | AOC | Station 3 |  |
| TA Doppler radar, French antenna | V, R | Gamache | AOC | Station 3 |  |
| Passive Microwave |  |  |  |  |  |
| AOC SFMR/pod | V10, Z | Goldstein | AOC | pod |  |
| Active Microwave |  |  |  |  |  |
| ProSensing WSRA | HS, WPS, WDS | Popstefanija | HRD, NHC | Fore Press Dome |  |
| Passive GPS |  |  |  |  |  |
| GPS bistatic altimeter | ocean height | Walsh | GSFC, ESRL | Station 5 |  |
| Airborne Ocean Profiler |  |  |  |  |  |
| HRD/UM AXBT receivers (2), DAT recorders (4) | TS vs z | Shay | UM | Station 2 |  |
| AOC AXBT receivers | TS vs z | Smith | AOC | Station 5 |  |
| Dropsonde System |  |  |  |  |  |
| GPS AVAPS Dropsonde-8CH | V, T, RH, p vs z | Smith | AOC | Station 5 |  |
| Video Systems |  |  |  |  |  |
| Down video | F(%), WD |  | AOC |  |  |
| Side, nose video | LCL |  | AOC |  |  |
| Turbulence System |  |  |  |  |  |
| Friehe radome gust probe system | U',V',W',T' | Zhang, Drennan | HRD, UM | Nose radome bulkhead |  |
| On board processing |  |  |  |  |  |
| Mac Notebook | LPS/X-chat | Griffin | HRD |  |  |
| Mac Notebook (if RVP-8 non-operational) | Radar superobs | Griffin | HRD | Station 3 |  |
| HRD workstation (if RVP-8 non-operational) | Radar processing/Editsonde | Griffin | HRD | Station 3 |  |
| Real-time data communications systems | FL, radar data | Chang, Carswell | NESDIS | Station 3 |  |
| ASDL (100 baud) | V, T, Td, p, PA, D, V10, Z | Goldstein | AOC |  |  |

Instrument

Location

Down radiometer port

Lower fuselage

Fuselage tail

Inner left pylon

Fore Press Dome

up/down field mill ports

Free-fall chute

Aft station 5

Vert. Camera port

Side, nose camera port

Nose radome

Wind Lidar

Atlas

|  |  | Table G.1: NOAA/AOC WP-3D (N42RF) instrumentation |  |  |  |
| --- | --- | --- | --- | --- | --- |
| AIRCRAFT SCIENTIFIC INSTRUMENTATION (CONT’D) | Instrument | Parameter | PI | Group | Electronics Location |
| Navigational |  |  |  |  |  |
| INE1/2 | lat, lon |  | AOC |  |  |
| GPS1/2 | lat, lon |  | AOC |  |  |
| Honeywell HG9550 altimeter | Radar altitude |  | AOC |  |  |
| Standard Met. |  |  |  |  |  |
| Buck1101c, Edgetech Vigilant, Maycom TDL | Td |  | AOC |  |  |
| Rosemount temp | T, T' |  | AOC |  |  |
| Static pressure | p |  | AOC |  |  |
| Dynamic pressure | p' |  | AOC |  |  |
| Horizontal wind | Vh |  | AOC |  |  |
| Vertical wind | w |  | AOC |  |  |
| Infrared Radiation |  |  |  |  |  |
| Side CO2 radiometer | T |  | AOC |  |  |
| AOC down radiometer | SST |  | AOC | Under floor |  |
| Weather Radar |  |  |  |  |  |
| LF radar | R | Gamache | AOC | Station 3 |  |
| TA Doppler radar, NOAA/AOC antenna | V, R | Gamache | AOC | Station 3 |  |
| Passive Microwave |  |  |  |  |  |
| AOC SFMR/pod | V10, Z | Goldstein | AOC | pod |  |
| USFMR (UMASS) | V10, Z | Esteban, Carswell, Chang | UMass, NESDIS | Station 7 |  |
| Active Microwave |  |  |  |  |  |
| AWRAP (CSCAT, KSCAT) | V10, Z, V vs z | Zhang, Chang | Umass, NESDIS | Station 7 |  |
| Airborne Ocean Profiler |  |  |  |  |  |
| AOC AXBT receivers | TS vs z | Smith | AOC | Station 5 |  |
| Dropsonde System |  |  |  |  |  |
| GPS AVAPS Dropsonde-4CH | V, T, RH, p vs z | Smith | AOC | Station 5 |  |
| Video System |  |  |  |  |  |
| Down video | F(%), WD |  | AOC |  |  |
| Side, nose video | LCL |  | AOC |  |  |
| Cloud Microphysics/Sea Spray |  |  |  |  |  |
| DMT CCP probe | Cloud particle spectra | Black | AOC |  |  |
| DMT PIP probe | Precipitation particle spectra | Black | AOC |  |  |
| DMT CAS probe | Aerosol/cloud droplet spectra | Black | AOC |  |  |
| DMT DAS | processor | Black | AOC | Station 4 |  |
| TECO Ozone sampler | 03 | Carsey | AOML |  |  |
| CCN Counter (DMT or other) | Aerosol/cloud droplet spectra | Black | AOML |  |  |
| SEA probe | Total water | R. Black | AOC, HRD |  |  |
| Turbulence Systems |  |  |  |  |  |
| Friehe radome gust probe system | U',V',W',T' | Drennan | RSMAS | Nose radome bulkhead |  |
| LICOR-750 water vapor analyzer | q’ | Drennan | RSMAS, AOC | Nose radome bulkhead |  |
| On board processing |  |  |  |  |  |
| Mac Notebook | Radar superobs | Griffin | HRD | Station 3 |  |
| HRD workstation | Radar processing/Editsonde | Griffin | HRD | Station 3 |  |
| Mac Notebook | LPS/x-chat | Griffin | HRD |  |  |
| Real-time data communications systems | FL, radar data | Chang, Carswell | NESDIS |  |  |

Instrument Location

Down radiometer port

Lower fuselage

Fuselage tail

Inner left pylon

Laser hole

Fore/aft pressure domes

Aft station 5

Vert. Camera port

Side, nose camera port

Outer left pylon

Outer left pylon

Outer left pylon

Nose radome

Nose Radome bulkhead

ASDL (100 baud)

V, T, Td, p, PA, D, V10,  Z

Goldstein

AOC

Table G.2: NOAA/AOC WP-3D (N43RF) instrumentation

|  |  | APPENDIX G: AIRCRAFT SCIENTIFIC INSTRUMENTATION (CONT’D) | Instrument |
| --- | --- | --- | --- |
| PI | Group | Navigational |  |
|  |  | INE1/2 |  |
|  | AOC | GPS1/2 |  |
|  | AOC | Honeywell HG9550 altimeter |  |
|  | AOC | Standard Met. |  |
|  |  | Buck1101c, Edgetech Vigilant, Maycom TDL |  |
|  | AOC | Rosemount temp |  |
|  | AOC | Static pressure |  |
|  | AOC | Dynamic pressure |  |
|  | AOC | Horizontal wind |  |
|  | AOC | Vertical wind |  |
|  | AOC | Weather Radar |  |
|  |  | TA Doppler radar |  |
| Gamache | AOC | Passive Microwave |  |
|  |  | SFMR |  |
| Goldstein | AOC | Dropsonde Systems |  |
|  |  | GPS AVAPS Dropsonde-8CH |  |
| Smith | AOC | On board processing |  |
|  |  | Real-time data communications systems |  |
| Chang, Carswell | AOC | HP-UX Workstations |  |

Parameter

lat, lon

lat, lon

Radar altitude

Td

T, T’

p

p'

Vh

w

V, R

V10, Z

V, T, RH, p vs z

FL, radar data

radar data, sondes

Gamache

HRD

MacBook Laptops

radar data, x-chat

Gamache

HRD

Table G.3 (Cont’d): NOAA/AOC G-IV (N49RF) instrumentation

|  | APPENDIX H: NOAA EXPENDABLE AND RECORDING MEDIA |  |  |  | Experiment |  | GPS Dropwindsondes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CADs |  |  | G-IV |  | 42/43RF |  |  |
| 42/43RF | Three-Dim Doppler Winds |  | - |  | 20 |  |  |
| 9 | TC Synoptic Flow |  | 30 |  | 30 |  |  |
| - | Coyote UAS |  | - |  | 14 |  |  |
| 9 | Doppler Wind Lidar |  | - |  | 10 |  |  |
| - | Tropical Cyclogenesis |  | 25 |  | 25 |  |  |
| 9 | Rapid Intensification |  | 25 |  | 20 |  |  |
| 9 | TC-Ocean Interaction |  | - |  | 20 |  |  |
| 15 | Saharan Air Layer |  | 15 |  | 20 |  |  |
| - | TC Landfall and Decay |  | - |  | 15 |  |  |
| - | Arc Cloud |  | - |  | 10 |  |  |
| - | Boundary Layer entrainment |  | - |  | 12 |  |  |

AXBTs

42/43RF

9

-

9

-

9

9

15

-

-

-

6

6

Aerosol/Cloud droplet

-

-

| - | - |  | Table H-1.1: Required expendables for 2010 experiments per flight day for 42/43RF and the G-IV. |  |
| --- | --- | --- | --- | --- |
|  |  |  | DATs1 | CDs2 |
| DVD +R DL | Experiment |  |  |  |
| Nose/Side/Down |  |  |  |  |
|  | Three-Dim Doppler Winds 42/3: 2 / 2 / 1 = 5 | 3 / 2 / 1 = 6 | 6 |  |
| TC Synoptic Flow | 2 / 2 / 1 = 5 | 42/3: 3 / 2 / 1 = 6 | 49: 0 / 0 / 1 = 1 |  |
| 1 / 2 / 1 = 4 | Coyote UAS | - | - |  |
| - | Doppler Wind Lidar - | - | - |  |
| Tropical Cyclogenesis | 2 / 2 / 1 = 5 42/3: 3 / 2 / 1 = 6 | 49: 0 / 0 / 1 = 1 | 6 |  |
| Rapid Intensification | 2 / 2 / 1 = 5 | 42/3: 3 / 2 / 1 = 6 | 49: 0 / 0 / 1 = 1 |  |
| 1 / 2 / 1 = 4 | TC-Ocean Interaction 2 / 2 / 1 = 5 | 3 / 2 / 1 = 6 | 6 |  |
| Saharan Air Layer | 2 / 2 / 1 = 5 | 42/3: 3 / 2 / 1 = 6 | 49: 0 / 0 / 1 = 1 |  |
| 1 / 2 / 1 = 4 | TC Landfall and Decay | 2 / 2 / 1 = 5 | 3 / 2 / 1 = 6 |  |
| 1 / 2 / 1 = 4 | Arc Cloud | - | - |  |
| - | Boundary Layer entrainment | - | - |  |

D-Audio

AXBTs

1 / 2 / 1 = 4

-

6

-

1 / 2 / 1 = 4

6

1 / 2 / 1 = 4

-

-

-

6

-

Aerosol/Cloud droplet

-

-

-

-

1 DATs required for Slow and Fast flight-level / Radar data / HRD Workstation Data

2 CDs required for Slow and Fast flight-level / Cloud Physics / AVAPS

NOTE:  1 DAT and 1 CD are required for G-IV missions

Table H-1.2. Required recording media for 2006 experiments per flight day for 42RF and the G-IV

ACRONYMS AND ABBREVIATIONS

θe equivalent potential temperature

ABL atmospheric boundary-layer

A/C aircraft

ACLAIM Airborne Coherent Lidar for Advanced In-flight Measurements

AES Atmospheric Environment Service (Canada)

AFRES U. S. Air Force Reserve

AOC Aircraft Operations Center

AOML Atlantic Oceanographic and Meteorological Laboratory

ASDL aircraft-satellite data link

AXBT airborne expendable bathythermograph

AXCP airborne expendable current probe

AXCTD airborne expendable conductivity, temperature, and depth probe

CARCAH Chief, Aerial Reconnaissance Coordinator, All Hurricanes

CDO central dense overcast

CIRA Cooperative Institute for Research in the Atmosphere

C-MAN Coastal-Marine Automated Network

CP coordination point

CW cross wind

DLM deep-layer mean

DOD Department of Defense

DOW Doppler on Wheels

DRI Desert Research Institute (at Reno)

E vector electric field

EPAC Eastern Pacific

ETL Environmental Technology Laboratory

EVTD extended velocity track display

FAA Federal Aviation Administration

F/AST fore and aft scanning technique

FEMA Federal Emergency Management Agency

FL flight level

FP final point

FSSP forward scattering spectrometer probe

GFDL Geophysical Fluid Dynamics Laboratory

G-IV Gulfstream IV-SP aircraft

GOMWE Gulf of Mexico Warm Eddy

GPS global positioning system

HL Hurricanes at Landfall

HRD Hurricane Research Division

INE inertial navigation equipment

IP initial point (or initial positionIWRS Improved Weather Reconnaissance System

JW Johnson-Williams

Ku-SCAT Ku-band scatterometer

LF lower fuselage (radar)

LIP Lightning Instrument Package

LPS Lead Project Scientist

MCS mesoscale convective systems

MGOC Miami Ground Operations Center

MLD Mixed Layer Depth

MPO Meteorology and Physical Oceanography

NASA National Aeronautics and Space Administration

NCAR National Center for Atmospheric Research

NCEP National Centers for Environmental Prediction

NDBC NOAA Data Buoy Center

NESDIS National Environmental Satellite, Data and Information Service

NHC National Hurricane Center

NOAA National Oceanic and Atmospheric Administration

NWS National Weather Service

OML oceanic mixed-layer

PDD pseudo-dual Doppler

PMS Particle Measuring Systems

POD Plan of the Day

PPI plan position indicator

PV potential vorticity

RA radar altitude

RAOB radiosonde (upper-air observation)

RAWIN rawinsonde (upper-air observation)

RECCO reconnaissance observation

RHI range height indicator

RSMAS Rosenstiel School of Marine and Atmospheric Science

SFMR Stepped-Frequency Microwave Radiometer

SLOSH sea, lake, and overland surge from hurricanes (operational storm surge model)

SRA Scanning Radar Altimeter

SST sea-surface temperature

TA tail (radar)

TAS true airspeed

TC tropical cyclone

TOPEX The Ocean Topography Experiment

TPC Tropical Prediction Center (at NHC)

UMASS University of Massachusetts (at Amherst)

USACE United States Army Corps of Engineers

USAF United States Air Force

USWRP U. S. Weather Research Program

UTC universal coordinated time (U.S. usage; same as “GMT” and "Zulu" time)

VTD velocity-track display

Acknowledgments

The preparation of HRD's 2010 Hurricane Field Program Plan was a team effort. The authors would like to express their appreciation to: the HRD scientists that contributed information on specific experiments; Shirley Murillo for her efforts on updating the information in the appendices of the HFP Plan and Neal Dorst for expertly drafting several of the figures.