Explorer of the Seas METADATA - 2008

Class of Data:  Surface ocean carbon dioxide concentrations

Dataset Identifier:  AOML_Explorer

One File:  Explorer_2008

Statement of how to cite dataset:

Explorer website: http://www.aoml.noaa.gov/ocd/gcc/explorer_introduction.php

These data are made freely available to the public and the scientific community

in the belief that their wide dissemination will lead to greater understanding

and new scientific insights.  The availability of these data does not

constitute publication of the data.  We rely on the ethics and integrity of

the user to assure that AOML receives fair credit for our work.  Please send

manuscripts using this data to AOML for review before they are submitted

for publication so we can insure that the quality and limitations of the data

are accurately represented.

Measurement platform identifier:  Explorer of the Seas

Cruise Information:

Weekly cruises to Bermuda, Canada or through the Caribbean departing

Bayonne, New Jersey.

Project Information:

This project was sponsored by the underway pCO2 on ships project of the

NOAA Climate Program and represents a collaboration between Royal Caribbean

International, the University of Miami's Rosenstiel School of Marine

and Atmospheric Science, the National Oceanic and Atmospheric

Administration, the National Science Foundation, the Office of

Naval Research, and the National Aeronautics and Space Administration.

Scientists responsible for technical quality of dataset:

Rik Wanninkhof

NOAA/AOML/Ocean Chemistry Division

4301 Rickenbacker Causeway

Miami, Florida 33149

Rik.Wanninkhof@noaa.gov

Dr. Denis Pierrot

NOAA/AOML/Ocean Chemistry Division

4301 Rickenbacker Causeway

Miami, Florida 33149

Denis.Pierrot@noaa.gov

Contact person for this dataset:

Betty Huss

NOAA/AOML/Ocean Chemistry Division

4301 Rickenbacker Causeway

Miami, Florida 33149

Betty.Huss@noaa.gov

Timestamp for initial submission of dataset: 9/09/10

Timestamp for the most recent update of dataset:  9/09/10

Timestamp period the dataset refers to: 2/13/2008 – 11/22/2008

Geographic area the dataset refers to:

13 N to 46 N

88 W to 60 W

List of variables included in this dataset:

COLUMN   HEADER                        EXPLANATION

1.	   Group_Ship:	               AOML_Explorer for all Explorer of the Seas data.

2.       Cruise_ID:		         Cruise Name (In 2008, operations transitioned

to a fully automated system.  There are only three

cruises for this year – EX0801E, EX0824, and

EX0839.)

3.	   JD_GMT:	                     Decimal year day.

4.	   DATE_UTC_ddmmyyyy:	         UTC Date.

5.	   TIME_UTC_hh:mm:ss:	         UTC time.

6.	   LAT_dec_degree:	         Latitude in decimal degrees (negative values

are in the southern hemisphere).

7.	   LONG_dec_degree:	         Longitude in decimal degrees (negative values

are in the western latitudes).

8.	   xCO2_EQU_ppm:                 Mole fraction of CO2 in the equilibrator

at equilibrator temperature (Teq) in parts

per million.

9.	   xCO2_ATM_ppm:                 Mole fraction of CO2 in air in parts per million.

10       xCO2_ATM_interpolated_ppm:    xCO2 interpolated in parts per million.

11.	   Pres_EQU_hPa:                 Barometric pressure in the lab in hectopascals

(1 hectopascal = 1 millibar).

12.	   Pres_ATM@SSP_hPa:	         Barometric pressure from ship's barometer,

corrected to sea level in hectopascals

(1 hectopascal = 1 millibar).

13.	   TEMP_EQU_C:                   Temperature in equilibrator water in degrees

centigade.

14.	   SST_C:                        Temperature from the ship's thermosalinograph in

degrees centigrade.

15.	   SAL_permil:                   Salinity from the ship's thermosalinograph on the

Practical Salinity Scale.

16.	   fCO2_SW@SST_uATM:            Fugacity of CO2 in seawater in microatmospheres.

17.      fCO2_ATM_interpolated_uatm:  Fugacity of CO2 in air in microatmospheres.

18.	   dfCO2_uatm:                  Sea water fCO2 - air fCO2 in microatmospheres.

This uses the interpolated air measurements.

19.	   WOCE_QC_FLAG:                Quality control flag for seawater fCO2 values

with 2 = good value and 3 = questionable.

20.      QC_SUBFLAG:                  Description of the problem with the questionable

data.

Narrative description of system design:

CO2 ANALYTICAL SYSTEM:

During 2008, the analytical system and location of the system was changed.

For the first cruise Ex0801E, the system described in the previous annual

Metadata files (up to 2007) was used.  The following is a description of the

setup used for cruises Ex0824 and beyond.  This system was placed in the bow chamber

and air CO2 values were measured.

The concentration of carbon dioxide (CO2) in surface ocean water is determined

by measuring the concentration of CO2 in gas that is in contact with the water.

Surface water is pumped from an inlet in the ship's bow to the equilibration

chamber.  The equilibration chamber has an enclosed volume of gas, or headspace,

and a pool of seawater that continuously overflows to a drain.  As the water

flows through the chamber, the dissolved gases (like CO2) partition between the

water and the headspace.  At equilibrium, the ratio of CO2 in the water and in

the headspace is influenced most by temperature, and that relationship is known.

By measuring the concentration of CO2 in the headspace and the temperature in

the chamber, the partial pressure (or fugacity) of CO2 in the surface water

can be calculated.

INSTRUMENT DESCRIPTION:

The general principle of instrumental design can be found in Wanninkhof and

Thoning (1993), Ho et al. (1995), Feely et al. (1998) and Pierrot et al. (2009).

The concentration of CO2 in the headspace gas is measured using the adsorption of

infrared (IR) radiation, which results from changes in the rotational and

vibrational energy state of the CO2 molecule.  The LI-COR detector passes IR

radiation through two 6" cells.  The reference cell is flushed with a gas of zero

concentration.  The sample cell is flushed with the headspace gas.  A vacuum-sealed,

heated filament is the broadband IR source.  The IR radiation alternates between the

two cells via a chopping shutter disc spinning at 500 Hertz.  An optical filter

selects an adsorption band specific for CO2 (4.26 micron) to reach the detector.

The solid state (lead selenide) detector is kept at -12 degrees C for excellent

stability and low signal noise (less than 0.2 ppm).

Several steps are taken to reduce interferences and to increase the accuracy

of the measurements.  The IR adsorption band for water vapor is close to the

band for CO2.  After the equilibration chamber, the headspace travels through

a drying trap to remove water vapor.  During each analysis, the headspace gas

is compared to a reference gas of zero CO2 concentration.  To improve the accuracy

of the measurements, four different gaseous standards for CO2 are analyzed

every three hours instead of the headspace gas.

Analyzer:  LI-COR 7000 infrared (IR) analyzer.

Method of Analysis:  Differential analyses relative to a reference gas which

is close to the CO2 concentration of the middle standard.  Measures dried

equilibrator headspace gas.  Gas flow is stopped prior to IR readings.

Drying Method:  The equilibrator headspace sample gas first goes through an

air filter and a thermoelectric refrigerator (~6 C).  The sample and standard

gases pass through a Perma Pure (Nafion) dryer and a short column of magnesium

perchlorate before reaching the analyzer.  The counter flow in the Perma Pure

tube is the reference gas.

Equilibrator (setup, size, flows):  The equilibrator was fabricated using a

filter housing (ColeParmer, U-010509-00) with ~0.5 L water reservoir and

~0.8 L gaseous headspace.  Water flow rate is ~1.5 L/min.  Headspace

recirculation rate is ~80 ml/min.

Narrative statement identifying measurement method for each required parameter:

CALCULATIONS:

The mixing ratios of ambient air and equilibrated headspace air are calculated

by fitting a second-order polynomial through the hourly averaged response of

the detector versus mixing ratios of the standards.  Mixing ratios of dried

equilibrated headspace and air are converted to fugacity of CO2 in surface

seawater and water saturated air in order to determine the fCO2.  For ambient

air and equilibrator headspace the fCO2a, or fCO2eq is calculated assuming

100% water vapor content:

fCO2eq = xCO2eq(P-pH2O)exp(B11+2d12)P/RT

where fCO2eq is the fugacity in the equilibrator, pH2O is the water vapor

pressure at the sea surface temperature, P is the atmospheric pressure

(in atm), T is the SST or equilibrator temperature (in K) and R is the

ideal gas constant (82.057 cm^3·atm·deg^-1·mol^-1).  The exponential term

is the fugacity correction where B11 is the second virial coefficient of

pure CO2

B11 = -1636.75 + 12.0408T - 0.032795T^2 + 3.16528E-5 T^3

and

d12 = 57.7 - 0.118 T

is the correction for an air-CO2 mixture in units of cm^3·mol^-1 (Weiss, 1974).

The calculation for the fugacity at SST involves a temperature correction term

for the change of fCO2 due to changes in water temperature from passing through the

pump and through 5 cm ID PVC tubing within the ship.  The water in the equilibrator

is typically 0.1 °C cooler than sea surface temperature. The empirical temperature

from equilibrator temperature to SST correction is:

fCO2(SST) = fCO2(eq) Exp (-0.0423 (Teq-SST))

where SST is sea surface temperature and Teq is the equilibrator temperature in

degrees °C.

Sampling Cycle:

The system runs on a cycle during which 4 standard gases,  5 air samples from

The bow of the ship and 50 surface water samples from the equilibrator head

Space are analyzed on the following schedule:

- Zero and span of Licor

- Four standard gases

- Five air samples

- Fifty equilibrator headspace gas samples

- Repeat 2 – 4 nine more times

- Restart from step 1

NOTES ON DATA:

Columns have a default value of -9 in case of instrument malfunction,

erroneous readings or missing data.  Furthermore, if a suspicious xCO2 value,

pressure or temperature value is encountered, the fCO2 is not calculated.

Analytical Instrument Manufacturer/Model:

The Explorer of the Seas system is built by General Oceanics.  The

analyzer is a LI-COR 7000 infrared analyzer.

Standard Gases and Reference Gas:  The three standard gases come from CMDL

in Boulder and are directly traceable to the WMO scale.  The reference gas

is a zero CO2 gas from a commercial company.  While individual data points

above 460 or below 300 may not be accurate, the general trends should be

indicative of the seawater chemistry.

Description of any additional environmental control:

The system is located in the bow thruster space of the Explorer of the

Seas.

Resolution of measurement:

The resolution of the instrument is better than 0.1 ppm.

Estimated overall uncertainty of measurement:

The xCO2eq measurements are believed accurate to 0.1 ppm.  The

fCO2@SST measurements are believed to be precise to 0.2 ppm.

List of calibration gases used:

The standards used on Ex0801E cruise are:

STANDARD      TANK #       CONCENTRATION        VENDOR

STD1          CA02040         319.82             CMDL

STD2          CA02030         377.14             CMDL

STD3          CA02020         458.04			CMDL

The standard used on Ex0824 and Ex0839 are:

STANDARD      TANK #       CONCENTRATION        VENDOR

STD1                            0.0             Commercial

STD2          CA02020         458.04			CMDL

STD3          CA02040         319.82             CMDL

STD4          CA02030         377.14             CMDL

Traceability to an internationally recognized scale (including date/place

of last calibration made):

All standards are obtained from NOAA/CMDL, now called the Global

Monitoring Division of the Earth Research Laboratory and are directly

traceable to WHO scale.

Uncertainty of assigned value of each calibration gas:

The uncertainty based on pre and post cruise calibrations is less

than 0.05 ppm.

Units:

All xCO2 values are reported in parts per million (ppm) and fCO2 values

are reported in microatmospheres (uatm) assuming 100% humidity at the

equilibrator temperature.

Bibliography:

DOE (1994). Handbook of methods for the analysis of the various parameters of the

carbon dioxide system in sea water; version 2. DOE.

Feely, R. A., R. Wanninkhof, H. B. Milburn, C. E. Cosca, M. Stapp and P. P.

Murphy (1998).  A new automated underway system for making high precision

pCO2 measurements onboard research ships. Analytica Chim. Acta 377: 185-191.

Ho, D. T., R. Wanninkhof, J. Masters, R. A. Feely and C. E. Cosca (1997). Measurement

of underway fCO2 in the Eastern Equatorial Pacific on NOAA ships BALDRIGE and

DISCOVERER, NOAA data report ERL AOML-30, 52 pp., NTIS Springfield.

Pierrot, D., C. Neil, K. Sullivan, R. Castle, R. Wanninkhof, H. Lueger, T. Johannson,

- Olsen, R. A. Feely, and C. E. Cosca (2009).  Recommendations for

autonomous underway pCO2 measuring systems and data reduction routines,

Deep-Sea Res II, 56, 512-522.

Wanninkhof, R. and K. Thoning (1993) Measurement of fugacity of CO2 in surface water

using continuous and discrete sampling methods. Mar. Chem. 44(2-4): 189-205.

Weiss, R. F. (1974). Carbon dioxide in water and seawater: the solubility of a

non-ideal gas.  Mar. Chem. 2: 203-215.

Takahashi, T., J. Olafsson, J. G. Goddard, D. W. Chipman, and S. C. Sutherland

(1993).  Seasonal variation of CO2 and nutrients in the high-latitude surface

oceans: a comparative study, Global Biogeochem. Cycles, 7, 843-878.

Comments related to the individual legs:

EX0801E:  The system was down from 2/15/08 (10:47) to 2/16/08 (15:47).

The following questionable data was removed - JD 50.057 to JD 50.292 and

JD 52.655 to JD 52.696.  Due to low gas flow the following questionable

data was removed - JD 61.142 to JD 61.147, JD 61.154 to JD 61.156,

JD 61.191 to JD 61.192 and JD 61.196 to JD 61.198.  The equil pressure

was bad from JD 44.793 to JD 51.959.  From JD 44.793 to JD 46.417,

interpolated equil pressure using sealevel pressure.  From JD 47.668

to JD 51.959 there was no sealevel pressure to interpolate the

equil pressure so this data was not included in the final file.

Due to the slow response time of the system, the first 10 minutes of

Data for each hour was removed.  For all legs, the first hour of data

after leaving port was removed.

EX0824:  At the start of this cruise, the major reconfiguration of the

Oceanographic instruments on the flowing seawater system was incomplete.

Several days into this cruise the reconfiguration was completed, and data

acquisition was initiated around latitude 24.5 deg North.  The operation

of the pCO2 instrumentation was optimized over the first days of use.

An unexpected decrease in the atmospheric CO2 occurred around JD 235.3

and 32 degress North.  There was a coincident drop in sea surface temperature

and seawater CO2, though no analytical causes for these changes were found.

EX0839:  During a 22 hour time period (17:30 on December 9 to 15:45 on

December 10, 2008) the flow of seawater was highly variable.  This symptom is

indicative of an "air bound" state: air is regularly pulled into the inlet

to the seawater pump, which loses its prime.  The discontinuous flow resulted

in widely variable CO2 concentrations in the equilibrator headspace.  Many of

the data were flagged as bad and deleted.  Some data were included in the

reported data set, but were flagged as questionable.  Several times during

the transits between islands, the air analyses were slightly higher than

expected.  Terrestrial influence of the atmospheric CO2 concentrations was

suspected, and the results are reported.