Cruise Report

High-density XBT cruise - AX080610

Ship Rider: Andrew Birkett

a)	XBTs deployed: 271

b)	Drifters deployed:  Eight surface drifters were deployed as follows:

| Drifter ID | Date | Time | Lat | Lon |
| --- | --- | --- | --- | --- |
| 41769 | 06/25/10 | 19:14 | 05˚ 06.00’S | 016˚ 05.52’W |
| 90415 | 06/26/10 | 04:44 | 02˚ 59.74’S | 017˚ 56.49’W |
| 90418 | 06/26/10 | 14:12 | 00˚ 58.27’S | 019˚ 44.98’W |
| 90417 | 06/27/10 | 04:58 | 02˚ 04.52’N | 022˚ 40.89’W |
| 90419 | 06/27/10 | 13:11 | 04˚ 01.35’N | 024˚ 24.24’W |
| 90416 | 06/27/10 | 23:19 | 06˚ 02.42’N | 026˚ 14.31’W |
| 41708 | 06/28/10 | 09:16 | 07˚ 58.55’N | 028˚ 08.05’W |
| 41736 | 06/28/10 | 19:41 | 10˚ 00.72’N | 030˚ 12.63’W |

c)	Profiling floats deployed: Nine APEX ARGO floats (6 ARGOS and 3 IRIDIUM) were deployed as follows:

| Float ID | Date | Time | Lat | Lon | Type |
| --- | --- | --- | --- | --- | --- |
| 6634 | 06/21/10 | 09:24 | 24˚ 58.16’S | 005˚ 40.18’E | ARGOS |
| 6638 | 06/21/10 | 23:14 | 22˚ 20.10’S | 002˚ 28.14’E | ARGOS |
| 6628 | 06/22/10 | 16:56 | 19˚ 00.00’S | 001˚ 29.40’W | ARGOS |
| 6888 | 06/29/10 | 22:28 | 15˚ 04.27’N | 035˚ 21.20’W | IRIDIUM |
| 6882 | 06/30/10 | 09:02 | 17˚ 07.74’N | 037˚ 33.00’W | IRIDIUM |
| 6889 | 06/30/10 | 18:18 | 18˚ 59.76’N | 039˚ 59.46’W | IRIDIUM |
| 6631 | 07/01/10 | 04:52 | 20˚ 57.36’N | 041˚ 44.34’W | ARGOS |
| 6633 | 07/01/10 | 15:44 | 23˚ 03.78’N | 044˚ 08.40’W | ARGOS |
| 6632 | 07/02/10 | 01:59 | 25˚ 00.12’N | 046˚ 28.20’W | ARGOS |

d)	Equipment tools and supplies needed: None

e)	Problems: Primary auto-launcher tube #7 faulty. Everything working fine but often produces profiles ~1˚C higher throughout than adjacent profiles. Problem began occurring towards end of high density sampling so it was decided to exclude tube #7 from cycle and work with 7 tubes rather than change to secondary auto-launcher. Tube #8 pin jammed against probe twice as it retracted. Problem occurred during the middle of the cruise, thereafter was fine. Stopped receiving email towards end of cruise, data transmission and sending of mail was still possible.

f)	Recommendations: None

g)	Other narrative: The ship had to stop for 3 hours on the 25th for engine repairs during which time the ship’s power supply was shut down. Other than that, smooth sailing all the way across.

h)	Ship contact: Captain Craig Winterton

email: 2pdoranje@maersk.com