Summary of cruise AX080213 between Durban and Newark

AX080213 was conducted aboard the Maersk Vilnius. I boarded the ship at Durban on the morning of the 14th of February 2013 and did the installation and testing of the equipment. A test drop was done when leaving Durban on the 16th February. The spare backup computer had a problem so I re-installed SEAS on it and reset the port settings .I then checked and tested it. One of the cat5 cables also had a problem, which I found to be with the plug at the A/L side. I replaced the plug, but we will need a tester in our toolkit so that we can test these cables before installation. After leaving Cape Town the drops for the AX080813 commenced on the 25th February until the ship birthed at Newark during the morning of the 14th March 2013.

In total 511 XBT’s were deployed. High density deployments commenced from a depth of 200m once leaving Cape Town on the 25th February and continued until a depth of 200m was reached before Newark on the 14th March. There weren't any major problems this voyage with the only problems being the occasional wire snag/stretch and a couple of faulty XBT’s. One whole box which looked damaged from the outside actually contained mostly faulty XBT's probably damaged when the box was dropped. Two drops were also a bit spikey due to welding during the drops.

Eight SVP drifters were deployed as follows:

ID          DATE           TIME(GMT)    LATITUDE    LONGITUDE

118522   03/01/2013       01H31               16.00.00 S       005.00.50 W

118524   03/01/2013       11H57                13.59.11 S    007.15.87 W

118526   03/01/2013       22H25                12.00.15 S    009.31.50 W

118519   03/02/2013       08H42                09.59.58 S    011.36.00 W

118521   03/02/2013       18H45                07.59.47 S    013.34.57 W

118520   03/03/2013       04H42                05.58.00 S    015.32.98 W

118525   03/03/2013       14H25                03.59.39 S    017.27.20 W

118523   03/04/2013       00H10                 02.00.50 S    019.21.60 W

Overall the voyage went well. We had fine weather for most of the trip. However we experienced very rough sea conditions from the 10th of March till reaching Newark on the 14th .The Maersk Vilnius is an ideal ship for XBT deployments since it maintains a very steady speed and the point of release is close to the water resulting in very few faulty drops due to launching position. The distance from the cabin to the launcher and antenna isn’t far making installation a lot simpler.

The Captain and crew were extremely hospitable during the crew and eager to assist during the voyage.

F.Mckay

Port Meteorological Officer

Durban