High-density XBT cruise AX080214 report

Ship          :  Maersk Vilnius

Call sign    : 9V8503

Ship Rider :  Fergus Mckay

Dates  XBT’s deployed : February 19th to March 19th 2014

a)	XBTs deployed : 482

b)	Drifters deployed: 04 SVP Drifters were deployed.

ID       DATE         TIME        LATITUDE          LONGITUDE
129312   03/02/14     03H43Z      19 00.73 S        000 41.48 W
129311   03/02/14     15H43Z      16 58.20 S        002 45.66 W  
129313   03/03/14     03H19Z      15.00.47 S        004 44.22 W
129314   03/03/14     15H03Z      13 00.28 S        006 42.87 W

c) Profiling floats deployed: 08 floats were deployed .

FLOAT S/N        START UP DATE     DEPLOYMENT DATE    LATITUDE      LONGITUDE 
 (SOLO 1)                                     TIME          
1151/121380       2014/02/17       2014/03/01 03H41Z   22 50.53 S    003 39.50 E
1152/121980       2014/02/17       2014/03/02 03H43Z   19 00.73 S    000 41.48 W
1153/121781       2014/02/17       2014/03/03 20H59Z   11 59.35 S    007 42.24 W

FLOAT S/N          START UP DATE     DEPLOYMENT DATE   LATITUDE     LONGITUDE        (SOLO 2)                                       TIME
7136/159864       2014/02/28        2014/03/04 08H41Z   10 02.59 S  009 40.98 W
7157/092098       2014/02/28        2014/03/05 13H57Z   05 00.36 S  014 38.61 W
7184/024298       2014/02/28        2014/03/06 01H33Z   02 58.44 S  016 38.74 W
7152/146881       2014/02/28        2014/03/06 13H00Z   01 00.28 S  018 36.01 W
7186/034736       2014/02/28        2014/03/09 20H15Z   10 57.19 N  030 41.07 W

- Equipment tools and supplies needed: Primary  launcher .

e)	Problems: There were a few problems this trip. I had two bad drops due to the crew  welding, which caused a bit of a disturbance to the data and four bad drops due to faulty XBT’s .Tubes 4 and 7’s pins didn’t respond to commands so were taken out of the sequence. Tube 8 worked with a bit of assistance(AX18 primary launcher).The AX08 primary launcher only gave a ‘HELLO’ with the diagnostics check and didn’t respond correctly to the status report. I think the connector box needs replacing. The backup launchers holes for the base are slightly off so the base doesn’t fit securely. A bolt snapped off in the AX18’s launcher base so I used the base from AX08.With the assistance of the 2nd engineer we managed to remove the bolt during the voyage. Other than that no other problems were experienced.

f)	Recommendations: The Maersk Vilnius is a perfect ship for deploying XBT’s so I recommend we use her as much as Maersk allows us to.

g)	Other narrative: The Maersk Vilnius is an ideal ship for dropping XBT's, drifters and Argo floats, with it maintaining a very steady speed, and the point of release close to the water, hence very few faulty drops occurred during my trip. The distance from the cabin to the launcher and antenna isn’t far, making installation a lot simpler. The Captain, officers and crew were also extremely hospitable making for a pleasant voyage.

h)	Ship contact: Captain Rannier P Santos

email: 908VILNIUS@maersk.com